# 3GPP TSG-RAN WG4 Meeting # 112	R4-2412964

# Maastricht, Netherland, August 19 – 23, 2024

Source: 	Huawei, HiSilicon

Title: 	General discussion on Tx requirements for NR NTN HPUE

Agenda Item:	8.8.2.2

Document for:	Approval

Introduction

In the RAN#103 meeting, the WI [1] for Enhanced requirements and test methodology for NR and IoT NTN was established to introduce for HPUE feature. The objectives for HPUE Tx requirements are listed below.

- Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study. - Support other power classes based on the co-existence and feasibility study - Non-hand held UE is prioritized - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands - Necessary Tx requirements, at least including - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary - Necessary Rx requirements, at least including - Specify the RSD requirements on the example bands, if necessary - Specify the necessary signaling to support the above objectives

In this paper, we’d like to share our views which requirements should be investigated and discussed in general.

Operating bands / Power Class VS workloads

In this WI, a lot of objectives and requirements need the simulation works. We summarized them below.

table 1 Operating bands / Power Class VS workloads

| Power Class | Operating bands | Coexistence Study | General MPR requirements | AMPR requirements |
| --- | --- | --- | --- | --- |
| PC2 | n256 | Needed | Needed | Needed |
|  | n255 |  |  | Needed |
| PC1.5 | n256 | Needed | Needed | Needed |
|  | n255 |  |  | Needed |
| PC1 | n256 | Needed | Needed | Needed |
|  | n255 |  |  | Needed |

Observation 1: If more power classes and operating bands are included in this WI, the workloads will be increased dramatically, especially for simulation works.

In current WID [1], the exemplary bands n256 and n255 are listed for NR NTN HPUE. As the workloads for this WI are dramatically huge, any HPUE demands for other bands can be considered later once the general HPUE framework is completed in this WI.

Proposal 1: based on the WID, exemplary bands n256 and n255 can be considered in this WI for NR NTN HPUE feature.

Referring to the summary [2], the following observations can be found.

Moderator’s observations: - Companies have different views on which power classes as well as the co-existence study and what UE form factors for each power should be considered in this WI. Based on the moderators’ observations, most companies can support PC2 for both handheld UE and non-handheld UE and PC1 for non-handheld UE. - To keep the reasonable workload, the frequency ranges need be down-selected and the example bands may also need be decided. Based on the moderators’ observations, quite a number of companies explicitly excluded FR2 and other companies did not clearly mention whether FR2 needs be considered. So the moderator proposed to focus on FR1, and maybe focus on the example bands with more adjacent bands or overlapping bands - Besides, the Rx requirements including MSD should also be specified in this WI.

Currently, for FDD bands, only PC2 and PC3 were specified in TS 38.101-1 considering handheld device. The RF component chains are premature to support the other power class higher than 26dBm in FDD bands. Thus, it’s better to focus on the PC2 for handheld HPUE in Rel-18. For non-handheld UE, if PC1 is agreeable to be considered, there is no need to specify lower power class for the non-handheld UE.

Proposal 2: considering the workloads, UE implementation and demands, we can specify PC2 for handheld/non-handheld UE and PC1 for non-handheld UE in Rel-18.

Tx requirements

1) PC2 UE architecture

In the TS 38.101-1, there are two UE RF architectures, i.e. one Tx RF architecture and Dual Tx RF architecture. For NR NTN PC2 UE, the similar RF architectures can be reused.

Proposal 3: one Tx RF architecture and Dual Tx RF architecture can be reused for NR NTN PC2 UE.

2) ACLR requirements

This requirement depends on the outcome of coexistence study. But from UE perspective, the higher ACLR requirement is, more power will be consumed. As NTN UE is always transmitting higher power due to the limited link budget, it is better to balance the UE power consumption and ACLR requirements.

3) SAR

Currently, there are two methods to solve SAR issue in the TS 38.101-1. One is the P-MPR solution, and the other is the duty cycle solution. However, for NR NTN UE, different characteristics can be observed. It’s well known that the UL coverage is the bottleneck of satellite communication scenario and the system is very sensitive to the UE transmitter power reduction. Thus, the P-MPR solution is not very friendly to NTN UE. For FDD bands, there is no fixed UL/DL pattern, so there is no accurate duty cycle concept in FDD deployment. It’s necessary to study the additional method to guarantee the SAR compliance without additional power reduction.

Unlike TDD frame, FDD frame is not restricted by the uplink-downlink configuration. Specifically, as long as the UL transmission percentage does not exceed a threshold within several minutes based on SAR restriction, the requirement can be satisfied. Therefore, to optimize the UL capacity while satisfying the SAR restriction, it is possible to allow the UE to transmit continuously with high power for longer time than that in TDD.

The mechanism to support flexible scheduling in a longer duration in FDD should be further discussed in working groups. Introducing too much complexity at the gNB side, e. g. counting each UE’s UL percentage within a long time span, is not preferred. As a simple candidate solution for PC1 UE with 31dBm Tx power, which could only have 1/8 UL transmission ratio within the statistics period of SAR requirement, UL transmission pattern can be considered to be configured for UEs. Other enhancements for HPUE, including PC 2 UEs, should be further studied and specified to support flexible scheduling in a longer duration while avoid introducing too much complexity at the gNB side.

Proposal 4: The mechanism to support flexible scheduling in a longer duration in FDD can be considered as a SAR solution.

Summary

Observation 1: If more power classes and operating bands are included in this WI, the workloads will be increased dramatically, especially for simulation works.

Proposal 1: based on the WID, exemplary bands n256 and n255 can be considered in this WI for NR NTN HPUE feature.

Proposal 2: considering the workloads, UE implementation and demands, we can specify PC2 for handheld/non-handheld UE and PC1 for non-handheld UE in Rel-18.

Proposal 3: one Tx RF architecture and Dual Tx RF architecture can be reused for NR NTN PC2 UE.

Proposal 4: The mechanism to support flexible scheduling in a longer duration in FDD can be considered as a SAR solution.

- References

[1] RP-240857, New WID: Enhanced requirements and test methodology for NR and IoT NTN, RAN4 chair (Huawei)

[2] RP-240711, Moderator's summary for discussion on NTN evolution in RAN4, RAN4 chair (Huawei)