# 3GPP TSG-RAN WG4 Meeting # 112	R4-2412965

# Maastricht, Netherland, August 19 – 23, 2024

Source: 	Huawei, HiSilicon

Title: 	General discussion on Rx requirements for NR NTN HPUE

Agenda Item:	8.8.2.3

Document for:	Approval

Introduction

In the RAN#103 meeting, the WI [1] for Enhanced requirements and test methodology for NR and IoT NTN was established to introduce for HPUE feature. The objectives for HPUE Rx requirements are listed below.

- Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study. - Support other power classes based on the co-existence and feasibility study - Non-hand held UE is prioritized - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands - Necessary Tx requirements, at least including - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary - Necessary Rx requirements, at least including - Specify the RSD requirements on the example bands, if necessary - Specify the necessary signaling to support the above objectives

In this paper, we’d like to share our views which requirements should be investigated and discussed in general.

Rx requirements

1) REFSENS

For band n256, the Tx-Rx frequency separation is about 190MHz, which is same as the band n1. Thus, Single Tx and dual Tx Reference Sensitivity Degradation from PC3 to PC2 for band n1 can be reused for band n256.

For band n255, the Tx-Rx frequency separation is about 101.5MHz, which is similar to the band n3. If flexible Tx-Rx frequency separation is allowed, the Tx-Rx frequency separation can be lower than 70MHz.

Proposal 1: When RAN4 discussed RSD requirements, it’s better to confirm whether the flexible Tx-Rx frequency separation is allowed or not.

2) Maximum input level

Currently, -40dBm maximum input level is specified for NR NTN UE, which is much less than the normal UE. However, based on the DL link budget, it’s very hard to reach such DL input level. RAN4 can discuss whether it can be updated for PC2 NR NTN UE.

3) ACS requirements.

In Rel-18, 33dB ACS requirement is specified for NR NTN UE. For NR NTN HPUE, it seems that the DL coexistence scenario is unchanged. RAN4 can further discuss and investigate whether to evaluate ACS requirement again for NR NTN UE.

Summary

Proposal 1: When RAN4 discussed RSD requirements, it’s better to confirm whether the flexible Tx-Rx frequency separation is allowed or not.

- References

[1] RP-240857, New WID: Enhanced requirements and test methodology for NR and IoT NTN, RAN4 chair (Huawei)