3GPP TSG-RAN WG4 Meeting #112	R4-2412966

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	8.1.1.3.4

Source:            	Google

Title:                	Discussion on SRS IL imbalance for 6Rx UE

Document for:	Approval

# Introduction

In RAN#103, a new WID on RF enhancements for NR FR1/FR2 and EN-DC was approved as one of Release 19 work items [1]. One of the objectives in the WID is to discuss 6Rx requirements for handheld and FWA UEs, which is shown as follows. In this contribution, we will focus on the SRS IL (insertion loss) imbalance for 6Rx handheld and FWA UE.

6Rx for handheld and FWA UE - Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario - Example bands: n41, n77/n78, n79, n104 - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers - Specify the Rx requirements including reference sensitivity requirements for support 6Rx - Note: the specified requirements can be applicable to both handheld UE and FWA devices - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, t4r6 depending on UE capability - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

In RAN4#111, the SRS IL imbalance for 6Rx handheld and FWA UEs has been discussed and one open issue is captured in the approved WF [2]. In WF, the open issue is to first discuss the initial considerations for SRS IL imbalance issue and encourage companies to bring analysis on the existing UE behavior and NW performance degradation. In this paper, we would like to share our view in the following.

# Topic 4: SRS IL imbalance issue ## Sub-topic 4-1: General considerations for SRS IL imbalance issue Issue 4-1-2: Initial Considerations for SRS IL imbalance issue Way forward: Given the different views amongst companies, RAN4 to further discuss the set of initial considerations which will allow companies to have a common understanding for the study including existing UE behavior for SRS transmissions in case of SRS IL. - Companies are encouraged to bring analysis on the existing UE behavior and achievable power imbalance for SRS transmissions based on current specification and UE implementations in case of SRS IL. - Companies are encouraged to analyse the impact of SRS IL imbalance on NW performance degradation.

# Discussion

In Rel-15, SRS Tx switching feature is introduced to obtain more accurate channel state information (CSI) by utilizing channel reciprocity for TDD bands. The motivation to introduce SRS Tx switching feature is that Tx antenna port number is usually less than Rx antenna port number from UE implementation perspective. In other words, in order to obtain more accurate CSI for DL-MIMO transmission at the base station side, the UE can be scheduled to perform antenna switching for SRS transmission in the corresponding Rx antenna port sequentially instead of simultaneous SRS transmission on the corresponding Rx ports.

From UE capability signalling perspective, SRS Tx switching feature can be indicated by the corresponding IE srs-TxSwitch/srs-TxSwitch-v1610 with “xTyR”, e.g., 1T2R/1T4R/ 2T4R/2T2R/4T4R/1T4R-2T4R, where “x” is the capable antenna port for SRS transmission and “y” means the total number of receive antenna where SRS transmission can be performed for the target band or the target band in the band combination. In Rel-17, the xTyR has been extended to support at most “1T8R/2T8R/4T8R” in the corresponding IE srs-AntennaSwitchingBeyond4Rx-r17.

From implementation perspective, if the UE is configured to perform SRS antenna switching, SRS Tx transmission may go through different number of RFFE antenna switch components, e.g., SPXT, DPXT where X=2,3,4,…8, at each associated Rx path on PCB board, which may cause different insertion loss for each SRS Tx path. In addition, the PCB tracing loss may also be different for each SRS Tx associated Rx path. Hence, SRS Tx power among each associated Rx paths would be imbalanced due to RFFE insertion loss and PCB tracing loss, which may cause SRS IL imbalance when the UE performs SRS antenna switching.

In RAN4#111, the SRS IL imbalance issue for 6Rx handheld and FWA UEs has been discussed and one open issue is captured in the WF. In WF, the open issue is to first discuss the initial consideration for SRS IL imbalance issue. In our view, SRS-IL imbalance issue can be analyzed from two perspective, where the first is from conducted RF antenna connector perspective and the second is from OTA perspective.

From conducted RF antenna connector perspective, although RFFE insertion loss and PCB tracing loss may cause SRS Tx power imbalance issue when performing SRS antenna switching, the SRS-IL imbalance can be reduced a lot by factory RF calibration process for every commercial UE before shipment, which is also the common process in the industry. In addition, the cost for factory RF calibration would be minor that the UE may need to save all calibration data in the non-volatile (NV) memory. In our understanding, for all SRS Tx power level after well RF calibration, SRS-IL imbalance among each SRS Tx associated Rx paths would be only around 1~2 dB. Hence, in our view, the SRS-IL imbalance would not be a critical issue from conducted RF antenna connector perspective.

Observation 1: From conducted RF antenna connector perspective, although RFFE insertion loss and PCB tracing loss may cause SRS Tx power imbalance issue when performing SRS antenna switching, the SRS-IL imbalance can be reduced a lot by factory RF calibration process for every commercial UE before shipment, which is also the common process in the industry.

From OTA perspective, SRS Tx power imbalance at the network side needs to consider not only SRS-IL imbalance but also different antenna implementation. In our understanding, the antenna performance for the MIMO Rx diversity paths would be usually sacrificed because of more protection on main Tx/PRx. So SRS Tx power imbalance may often be occurred in the real live network even if the UE is well calibrated. On the other hand, the received SRS Tx power at the network side may be varying very fast due to the different channel condition. In addition, SRS Tx power imbalance at the network side may not be always caused by SRS-IL imbalance but sometimes caused by different antenna implementation and/or fading channel. Hence, in our view, the SRS-IL imbalance would not be a critical factor to impact SRS Tx power imbalance from OTA perspective, and it would be better to handle it from demodulation perspective.

Observation 2: From OTA perspective, SRS Tx power imbalance at the network side needs to consider not only SRS-IL imbalance but also different antenna implementation. In our understanding, the antenna performance for the MIMO Rx diversity paths would be usually sacrificed because of more protection on main Tx/PRx. So SRS Tx power imbalance may often be occurred in the real live network even if the UE is well calibrated.

In the previous meeting discussion, some SRS-IL imbalance reporting solutions are proposed to overcome the SRS-IL imbalance issue for 6Rx handheld and FWA UE. In our view, we think we should first analyze whether SRS-IL imbalance could be a critical issue or not. As above mentioned, from conducted perspective, it may be a minor issue after well factory RF calibration at each SRS Tx on the associated Rx paths. From OTA perspective, it may not be the critical factor to impact SRS Tx power imbalance at the network side because different antenna radiation pattern and channel condition may also impact. Therefore, considering that SRS-IL imbalance can be reduced a lot by factor RF calibration process from conductive perspective and that SRS-IL imbalance may not be the critical factor to impact SRS Tx power imbalance from OTA perspective, it is proposed not to introduce any SRS-IL imbalance reporting mechanism for 6Rx UE in Rel-19.

Proposal 1: Considering that SRS-IL imbalance can be reduced a lot by factor RF calibration process from conductive perspective and that SRS-IL imbalance may not be the critical factor to impact SRS Tx power imbalance from OTA perspective, it is proposed not to introduce any SRS-IL imbalance reporting mechanism for 6Rx UE in Rel-19.

# Conclusion

The proposals in this contribution are summarized in the following.

Observation 1: From conducted RF antenna connector perspective, although RFFE insertion loss and PCB tracing loss may cause SRS Tx power imbalance issue when performing SRS antenna switching, the SRS-IL imbalance can be reduced a lot by factory RF calibration process for every commercial UE before shipment, which is also the common process in the industry.

Observation 2: From OTA perspective, SRS Tx power imbalance at the network side needs to consider not only SRS-IL imbalance but also different antenna implementation. In our understanding, the antenna performance for the MIMO Rx diversity paths would be usually sacrificed because of more protection on main Tx/PRx. So SRS Tx power imbalance may often be occurred in the real live network even if the UE is well calibrated.

Proposal 1: Considering that SRS-IL imbalance can be reduced a lot by factor RF calibration process from conductive perspective and that SRS-IL imbalance may not be the critical factor to impact SRS Tx power imbalance from OTA perspective, it is proposed not to introduce any SRS-IL imbalance reporting mechanism for 6Rx UE in Rel-19.

# Reference

- RP-240828, “New WID: UE RF enhancements for FR1/FR2 and EN-DC , Phase 4”, RAN4 chair (Huawei), RAN#103

- R4-2410751, “WF on requirements for 6Rx”, AT&T, RAN4#111

- R4-2409053, “Discussion on SRS IL imbalance for 6Rx UE”, Google, RAN4#111