3GPP TSG-RAN WG4 Meeting #112	R4-2412967

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	8.2.3

Source:            	Google

Title:                	Discussion on IMT parameters for 7125 to 8400MHz

Document for:	Approval

# Introduction

In the latest World Radiocommunication Conference 2023 (WRC-23) meeting, three frequency ranges (4400-4800 MHz, 7125-8400 MHz, and 14800-15350 MHz) are identified as potential candidates for IMT-2030. ITU-R Working Party 5D (WP5D) was tasked with conducting sharing and compatibility studies, and a LS [1] was sent to request RAN4 to provide technical parameters for these candidate frequency range. In response, a study item [2] for these IMT technical parameters has been approved in RAN#103. The objective of the WID is shown as follows.

The following objectives are defined for the frequency ranges in question: - Study the IMT parameters relevant for sharing and compatibility for 7125 to 8400 MHz frequency range. - Study the IMT parameters relevant for sharing and compatibility for 14800 to 15350 MHz frequency range. NOTE: IMT parameters relevant for sharing and compatibility for 4400 to 4800 MHz are reused from the already available RAN4 evaluations.

In RAN4#111, the IMT parameters for the three frequency ranges 7125 to 8400 MHz has been discussed and some open issues are captured in the approved WF [3]. In WF, some open issues for RF Tx and Rx parameters for the frequency range 7125 to 8400 MHz are listed as follows and further discussion may be needed. In this paper, we would like to share our view in the following.

# 3 IMT parameters for 7125 to 8400 MHz frequency range Sub-topic 2-1 General / system issues Issue 2-4: Typical signal bandwidth Outcome of discussion (Not agreed): Option 1: 273 RB, 30k SCS as 38.104 Option 2: Quote formula of RBs * SCS without number of RBs Option 3: Directly quote the signal bandwidth in MHz Sub-topic 2-3 UE parameters Issue 2-15: Maximum output power Tentative agreement: - Capture 23dBm as typical maximum output power in the LS to ITU. - Other output powers are not precluded and refer to TR - Capture 20dBm, 23dBm, 26dBm, and higher power classes in TR

# Discussion

Signal bandwidth

In WF, there are three options for the signal bandwidth on the frequency range 7125 to 8400 MHz, where the Option 1 is to use 273 RB and 30KHz as 38.104, the Option 2 is to quote formula of RBs*SCS without number of RBs, and Option 3 is directly to quote the signal bandwidth in MHz. In our understanding, the signal transmission bandwidth for 5G NR is larger than 4G LTE under the same system channel bandwidth, e.g., 50 RBs with 15KHz SCS for 4G LTE 10 MHZ but 52 RBs with 15KHz SCS for 5G NR 10MHz, because the RF guard band for 5G NR is reduced due to the advancements in RF hardware technology. Considering the RF hardware technology for 6G may be more advanced than 5G NR, it would be suitable not to specify specific RB number with SCS in the reply LS without more detailed discussion. Hence, in our view, for signal bandwidth on the frequency range 7125 to 8400 MHz, it is proposed to quote formula of RBs * SCS without specifying the number of RBs.

Proposal 1: Considering the RF hardware technology for 6G may be more advanced than 5G NR, it would be suitable not to specify specific RB number with SCS in the reply LS without more detailed discussion. Hence, for signal bandwidth on the frequency range 7125 to 8400 MHz, it is proposed to quote formula of RBs * SCS without specifying the number of RBs.

Maximum output power (UE)

In WF, the UE maximum output power for the frequency range 7125 to 8400 MHz has no consensus but only tentative agreement is reached. In the tentative agreement, the typical maximum output power is captured as 23 dBm (PC3) and other power 20dBm (PC5), 23dBm (PC3), 26dBm (PC2), and higher power classes are captured in TR.

In our understanding, from incumber user perspective, it is observed in [4] that this frequency range in the US is primarily utilized by military radar systems, various satellite networks (both GSO and NGSO), fixed services (FS), and earth exploration services (EES). Consequently, releasing this band for public mobile networks (e.g., IMT-2030 or 6G) may be challenging, and the shared usage of the 7125-8400 MHz band might be a possible solution instead.

Figure 1. Spectrum allocation in the 7125 – 8400 MHz band in the USA. [4]

Regarding the new band spectrum sharing with the military spectrum, the CBRS band n48 can be a precedent reference example and the n48 spectrum sharing with the military radar system is defined in FCC part 96. The spectrum access for n48 is divided into three tiers and the spectrum access system (SAS) is designed to protect the Tier-1 incumbent users. For Tier-2 and Tier-3 users, the spectrum access for n48 is needed to be coordinated by SAS which can mitigate the interference among users. In order to minimize the interference, the power class of n48 is only agreed as 23 dBm (PC3) in TS 38.101-1 and no HPUE is introduced for this band.

Considering that the spectrum sharing and interference mitigation might be the main issue for this frequency range, the similar frequency range for NR-U band n96 (5925MHz – 7125MHz) can also be the reference for the technical parameters. In n96, the power class has been only defined as 23 dBm (PC3) and 20 dBm (PC5) without introducing HPUE. In our view, the rule not to introduce HPUE for n48 and n96 can be also applied to this frequency range for 7125 to 8400 MHz. Therefore, it is preferred not to introduce HPUE but only to apply 23 dBm (PC3) and 20 dBm (PC5) to the frequency range for 7125 to 8400 MHz. Considering the last meeting tentative agreement has already captured 20 dBm (PC5) in the TR, it can be compromised that 20 dBm (PC5) is not included as the UE typical maximum output power in the reply LS. Hence, it is proposed to confirm the last meeting tentative agreement on the UE maximum out power for the frequency range 7125 to 8400 MHz.

Proposal 2: It is preferred not to introduce HPUE but only to apply 23 dBm (PC3) and 20 dBm (PC5) to the frequency range for 7125 to 8400 MHz. Considering the last meeting tentative agreement has already captured 20 dBm (PC5) in the TR, it can be compromised that 20 dBm (PC5) is not included as the UE typical maximum output power in the reply LS. Hence, it is proposed to confirm the last meeting tentative agreement on the UE maximum out power for the frequency range 7125 to 8400 MHz.

# Conclusion

The proposals in this contribution are summarized in the following.

Proposal 1: Considering the RF hardware technology for 6G may be more advanced than 5G NR, it would be suitable not to specify specific RB number with SCS in the reply LS without more detailed discussion. Hence, for signal bandwidth on the frequency range 7125 to 8400 MHz, it is proposed to quote formula of RBs * SCS without specifying the number of RBs.

Proposal 2: It is preferred not to introduce HPUE but only to apply 23 dBm (PC3) and 20 dBm (PC5) to the frequency range for 7125 to 8400 MHz. Considering the last meeting tentative agreement has already captured 20 dBm (PC5) in the TR, it can be compromised that 20 dBm (PC5) is not included as the UE typical maximum output power in the reply LS. Hence, it is proposed to confirm the last meeting tentative agreement on the UE maximum out power for the frequency range 7125 to 8400 MHz.

# Reference

- R4-2400333, “Parameters of terrestrial component of IMT for sharing and compatibility studies in the frequency bands 4 400-4 800 MHz, 7 125-8 400 MHz and 14.8-15.35 GHz. (R23-WP5D-240131-TD-0028)”, ITU-R Working Party 5D, RAN4#110

- RP-240787, “New SI proposal: Study on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz”, Ericsson, RAN#103

- R4-2410741, “WF on IMT parameters study”, RAN4#111, Ericsson

- R4-2409740, “Technical parameters for the 7125 to 8400 MHz and 14800 to 15350 MHz IMT bands for sharing studies”, RAN4#111, CableLabs, Charter Communications, Cox Communications

- R4-2409063, “Discussion on IMT parameters for 7125 to 8400MHz”, Google, RAN4#111