# 3GPP TSG-RAN WG4 Meeting # 112						R4-2412969

# Maastricht, Netherlands, 19th - 23rd  August, 2024

Source:		Ericsson

Title:			A-IoT deployment scenario and spectrum usage

Agenda item:		8.20.2.1

Document for:		Approval

# Introduction

In this paper, we present our overview for A-IoT deployment scenario and spectrum usage.

# Discussion

In WF [1], the deployment scenario is stated as below:

Issue 2-1-1: deployment scenarios for D1T1

Option 1-1: Legacy NR gNB are outdoor macro gNB while AIoT reader/CW/devices are all indoors. Legacy NR UE is only allowed outdoors.

Option 1-2: Legacy NR gNB are outdoor macro gNB while AIoT reader/CW/devices are all indoors. Legacy NR UE is indoor accessing to outdoor NR marco gNB

Option 2-1: Legacy NR gNB are co-located with AIoT reader and CW. All of NR and AIoT BS/UE/Reader/Device/CW are indoors. AIoT reader /CW and Legacy gNB share same hardware

Option 2-2: Legacy NR gNB are co-located with AIoT reader and CW. All of NR and AIoT BS/UE/Reader/Device/CW are indoors. AIoT reader /CW and Legacy NR gNB do not share same hardware. (less limitation on the power boosting)

Option 1-1 and 1-2 is currently in coexisting simulation but option 2-1 is 2-2 is deprioritized.

Agreement in RAN4#111:

- Consider only adjacent RB/channel co-existence evaluation for in-band deployment scenario for NR and AIOT

- Encourage companies to provide the simulation results for option 1-1 and 1-2

Our understanding on option 2-1/2-2 is that this is the in-band/guard band deployment that is similar to the in-band NB-IoT deployment and it would be meaningful in the study phase to draw conclusion on this, i.e whether such deployment would be possible or not from coexisting perspective. As we point it out in our companion paper on general aspect, in-band deployment is within the scope of the SID. As device type 1 and 2a has no in-band selectivity, it will be difficult to deploy option 2-1/2-2 for these device types and therefore, RAN4 could make a conclusion on these with LLS simulation.

RAN4 continue to discuss the option 2-1/2-2.

For device 1/2a, the option 2-1/2-2 may not be possible and this could be confirmed with LLS simulation.

For  CW signal spectrum usage, RAN4 continue to investigate inside topology for both UL and DL spectrum.  The implication on the hardware may need to understand better.

For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, and for topology 1, the following cases for CW transmission are studied.

        - Case 1-1: CW is transmitted from inside the topology, transmitted in DL spectrum

        - Case 1-2: CW is transmitted from inside the topology, transmitted in UL spectrum

        - Case 1-4: CW is transmitted from outside the topology, transmitted in UL spectrum

Agreement:

- For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, consider the following for co-existence evaluation

    - CW transmits in either UL or DL spectrum

    - FFS on inside topology and outside topology.

For FDD band, when the CW signal transmits in UL spectrum for inside topology with the option of the shared hardware, clearly the co-location requirement will not be met. Therefore, it cannot be deployed like this. Defining a TDD operation or HD-FDD operation may get around the protection of the own receiver or co-location requirement but receiving a low back-scatter signal (e.g -70 dBm) in presence of co-located high power CW signal (e.g 23 dBm) may pose implementation challenge on the radio hardware. When CW transmit in DL spectrum, as RAN4 assume no frequency shift for UL transmission for device 1/2a, the TDD operation may be needed so reader can receive the backscattered device in UL/DL spectrum.

TDD/HD-FDD operation may be needed for inside topology when CW transmit in UL/DL spectrum.

# Conclusions

In this contribution, we present our overview on the A-IoT deployment scenario and spectrum usage with below proposal:

Proposal-1: RAN4 continue to discuss the option 2-1/2-2.

Proposal-2: For device 1/2a, the option 2-1/2-2 may not be possible and this could be confirmed with LLS simulation.

Observation 1 TDD/HD-FDD operation may be needed for inside topology when CW transmit in UL/DL spectrum.

# References

- R4-2410567, WF on co-existence evaluation for ambient IoT, CMCC