# 3GPP TSG-RAN WG4 Meeting # 112						R4-2412971

# Maastricht, Netherlands, 19th - 23rd  August, 2024

Source:		Ericsson

Title:			A-IoT UE as intermediate node RF overview

Agenda item:		8.20.3.3

Document for:		Approval

# Introduction

In this paper, we present our overview for A-IoT UE as intermediate node RF.

# Discussion

In SID [2] objective:

- Deployment Scenarios with the following characteristics, referenced to the tables in Clause 4.2.2 of TR 38.848:

- Deployment scenario 1 with Topology 1

  - Basestation and coexistence characteristics: Micro-cell, co-site

- Deployment scenario 2 with Topology 2 and UE as intermediate node, under network control

  - Basestation and coexistence characteristics: Macro-cell, co-site

  - The location of intermediate node is indoor

- FR1 licensed spectrum in FDD.

- Spectrum deployment in-band to NR, in guard-band to LTE/NR, in standalone band(s).

As the in-band, guard-band and standalone mode are all within the SID scope, we think for A-IoT UE as immediate node RF requirement discussion should start with the legacy NR UE as baseline. This is similar with the NB-IoT RF requirement discussion. As the operator can deploy the NB-IoT with existing network, it should be the same ambition of A-IoT deployment, which reusing the current infrastructure should be prioritized.

The existing NR UE specification should be starting point.

Then for DL transmission of A-IoT, currently the OFDM-based waveform is considered. So OFDM based transmitter should be considered as baseline for DL R2D.

Agreement

A-IoT DL study includes an OFDM-based waveform from A-IoT R2D (reader-to-device) perspective.

- Depending on what modulation(s) are decided to be studied:

  - Study whether/how to handle CP at transmitter/device/design

- Study other characteristics of the OFDM waveform, e.g.:

  - CP-OFDM

  - DFT-s-OFDM

  - Etc.

  - The type of OFDM waveform is transparent to A-IoT device.

Other waveforms from DL transmitter’s perspective can be proposed, and further discussion will consider whether or not they are included in the study.

OFDM based transmitter should be baseline for R2D.

For D2R waveform there is no RAN1 agreement yet, so RAN4 should wait RAN1 further progress on it.

RAN4 wait RAN1 further progress on the D2R waveform.

Though there is no D2R waveform agreed in RAN1, from RF perspective, the RF impact of CW signal transmission from inside topology or outside topology could be discussed.

Starting from the outside topology, where CW transmission is from different hardware than UE as intermediate node,  if CW is assumed to be transmitted in DL spectrum, the narrow band blocking could be referenced if the legacy UE RF specification could be impacted as minimum. The level of CW block is -55 dBm, which can be tolerated as adjacent channel interferer. When CW is received within the wanted channel, there is no in-channel selectivity in UE specification, so the tolerance of CW signal within the same channel of the backscattered signal needs to be further studied.

The tolerance of CW signal within the same channel of the backscattered signal as interferer needs to be further studied.

For the CW inside the topology, all the legacy receiver RF requirements will be impacted. This may start with a new UE RF architecture discussion as legacy RF hardware cannot transmit DL spectrum. RAN1 is currently investigating the CW signal suppression capability from the reader and RAN4 can wait RAN1 further progress on this.

Further study needed for the CW inside topology impact on the UE RF when UE is intermediate node.

RAN4 wait RAN1 further progressing on the CW signal suppression capability.

# Conclusions

In this contribution, we present our overview on the A-IoT BS RF with below proposal:

Proposal-1:The existing NR UE specification should be starting point.

Proposal-2: OFDM based transmitter should be baseline for R2D.

Proposal-3: RAN4 wait RAN1 further progress on the D2R waveform.

Proposal-4:The tolerance of CW signal within the same channel  of the backscattered signal as interferer needs to be further studied.

Proposal-5: Further study needed for the CW inside topology impact on the UE RF when UE is intermediate node.

Proposal-6: RAN4 wait RAN1 further progressing on the CW signal suppression capability.

# References

- R4-2410597, WF on impacts of A-IoT on RF requirements, Huawei

- RP-234058, New SID: Study on solutions for Ambient IoT (Internet of Things) in NR