# 3GPP TSG-RAN WG4 Meeting # 112						R4-2412973

Maastricht, Netherlands, 19th - 23rd  August, 2024

Source:		Ericsson

Title:			Coexisting study simulation assumptions for A-IoT

Agenda item:		8.20.2.2

Document for:		Approval

# Introduction

In this paper, we discuss additional simulation assumptions and also some initial simulation results for some of cases of coexisting for A-IoT.

# Discussion

## Some simulation parameters

Emission mask of CW transmitter:

In WF [1], the SINR for performance calibration purpose is agreed.  Though the CW interference for the co-location reader receiver is ignored for calibration purpose, the CW interference to the NR BS or NR UE on the adjacent RB should be considered. The CW transmits one tone and the emission level from the edge of the A-IoT channel BW will be different and therefore the interference is different for the RBs adjacent to the A-IoT RB in NR macro BS receiver. As the CW can be transmitted by the same hardware as A-IoT BS or different hardware, the emission mask of CW transmitter could be discussed separately. The A-IoT device emission mask is agreed last meeting for calibration purpose and we think such emission mask could be extended for the CW transmitter also as the device 1 and device 2a backscatter the CW signal. However,  the emission mask requirement in RF requirements starts from the edge of the channel, while current assumption of the emission mask is from second PRB which is the edge of the transmitted PRB. To facilitate the RF requirement discussion, it may be good to align the RF requirement with the coexisting study. Therefore, ACLR is proposed to be used instead of emission mask for the coexisting study. This is because ACLR is defined as the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency and is more relevant on calculating the interference level for adjacent channel.

Use the ACLR for interference modelling for coexisting study.

Use the same ACLR characteristic of the device 1/2a for the CW transmission.

Minimum NR BS – NR UE distance (2D)

In WF [1], the distance of min NR BS and NR UE in 2D is 35 m. Perhaps it is quoted from the distance from wide area BS type 1-O. However, there is no AAS BS for 900 MHz and therefore, we suggest to use the MCL of 70 dB for a wide area BS type 1-C.

Use MCL of 70 dB for Minimum NR BS – NR UE distance setting in coexisting simulation.

## Simulation methodology

### D1T1 and interference modelling

Figure 1: Interference source for coexisting case 1 to 10 except 5 and 6

Figure 2: Interference source for coexisting case 5, 6, 11 and 12

In WF[1], the Monte-Carlo method is agreed to be used for simulation. To further align the simulation results, the interference modelling should also be discussed. Figure 1 and Figure 2 shows the interference source for different cases for D1T listed in Table 1 and these cases are listed in excel sheet for calibration purpose.

Table 1: simulation cases for D1T1

| deployment scenario and topology​ |  | Evaluation case No.​ | (Aggressor -> Victim)​ | spectrum​ |
| --- | --- | --- | --- | --- |
| 1​ | D1T1-A2-legacy UE only outdoor​ | 1​ | device -> NR UL​ | R2D: DL​ CW2D and D2R: UL​ |
|  |  | 2​ | NR UL -> reader​ |  |
|  |  | 3​ | reader -> NR DL​ |  |
|  |  | 4​ | NR DL -> device​ |  |
|  |  | 5​ | device -> NR DL​ | R2D: DL​ CW2D and D2R: DL​ |
|  |  | 6​ | NR DL -> reader​ |  |
| 2​ | D1T1-A2-legacy UE indoor​ | 7​ | device -> NR UL​ | R2D: DL​ CW2D and D2R: UL​ |
|  |  | 8​ | NR UL -> reader​ |  |
|  |  | 9​ | reader -> NR DL​ |  |
|  |  | 10​ | NR DL -> device​ |  |
|  |  | 11​ | device -> NR DL​ | R2D: DL​ CW2D and D2R: DL​ |
|  |  | 12​ | NR DL -> reader​ |  |

Interference for case 1 and 7:

The interference source: CW transmission, device backscatter signal.

Related RF parameter: CW node emission mask or ACLR, device emission mask or ACLR

For case 1 and 7, as the number of received RBs from 3 UE at NR BS is more than 4 so the emission mask agreed for device 1/2a should be extended to more PRB outside the transmitted PRB. We suggest the ACLR is to be used instead of the emission mask. However, it is uncertain that ACLR specification will be specified with the alternative channel and we think it may be good enough to specify the simple ACLR model for now and also used for later RF requirement discussion.

In NB-IoT, the interference from ACLR is scaled with adjacent system BW and number of UE and described as effective ACLR as below in 36.802:

ACLR_e = ACLR – 10*log10((9MHz/(# of LTE users))/180kHz) [pessimistic assumption]

As the A-IoT system also transmit one PRB and therefore similar approach could be taken:

ACLR_e_AIOT_xx = ACLR – 10*log10((9MHz/(# of  NR users))/180kHz) [pessimistic assumption]

At NR BS , it could discuss whether additional channel filter will be implemented to attenuate the A-IoT signal, as this is legacy NR BS, so no new RF requirement should be added. However, it could be discussed whether an additional baseband filter could be added as implementation specific choice.

Interference calculation:

I  = PCW* ACLR_e_AIOT_CW +  PD *ACLR_e_AIOT_UE

Figure 3: Interference modelling to NR BS receiver

Interference for case 2 and 8:

The interference source: NR UE transmission

Related RF parameter: NR UE IBE mask, ACS of the A-IoT BS

For case 2 and 8, the NR UE IBE mask specification can be used.

Interference calculation:

I  = (PUE2 + PUE3)*ACSR-BS  + (PUE2 + PUE3)*IBE_first adjacent RB outside of the allocated bandwidth

Figure 4: interference from NR UE to A-IoT BS receiver

Interference for case 3 and 9:

The interference source: A-IoT BS transmission.

Related RF parameter: ACLR of A-IoT BS.

In this case, the reader will transmit one RB within A-IoT BS channel. Therefore, the interference modelling from A-IoT BS relates to the ACLR of A-IoT BS. The transmission bandwidth from RAN1 Ls[3] is 180kHz (M) and RAN4 should further discuss the guard band of the channel so the BW of channel could be decided.

As the A-IoT system also transmit one PRB and therefore similar approach could be taken:

ACLR_e_AIOT_BS = ACLR – 10*log10((9MHz/(# of  NR users))/180kHz) [pessimistic assumption]

Interference calculation:

I  = PAIoT_BS*ACLR_e_AIoT_BS

Figure 5: interference to NR UE receiver

Interference for case 4 and 10:

The interference source: A-IoT BS transmission.

Related RF parameter: N/A

In this case, though there is agreement in WF below, we think the interference calculation should be relating to the power difference between NR and wanted A-IoT signal and the noise bandwidth of receiver.  Therefore, it will be difficult to calculate the interference with simple assumption as the power difference is different from different NR BS. In this sense, we suggest RAN4 make a LLS for this case to calculate the interference.

Issue 2-4-2: SINR definition for R2D

Agreement in RAN4#111:

SINR for R2D for calibration purposes

- signal power of device to the noise and interference within 10MHz

  - Assume interference NR BW is 10MHz

RAN4 conduct the LLS for the R2D SINR calculation.

Interference for case 6 and 12:

The interference source: NR BS transmission.

Related RF parameter: A-IoT BS ACS

Figure 6: interference from NR BS to A-IoT BS receiver

For the case 6 and 12, the interference from NR BS on the A-IoT BS mainly decided by the iFFT.  The interference level can be illustrated by the simulation of OFDM modulation where one RB is blanked. Figure 7 shows that the interference to the blanked RB can be around 17 dB relative to the average NR transmission power.

Interference calculation:

I  = PNR_BS*IBE_model specific + PNR_BS *ACSR-BS

Figure 7: IBE for NR DL with one RB blanking

Interference for case 5 and 11:

The interference source: CW transmission, A-IoT device backscatter transmission

Related RF parameter: CW emission mask, A-IoT device emission mask

The interference is the same with case 1 and 7 , the victim is now NR UE instead of NR BS.

Interference calculation:

I  = PCW*ACLRCW + 2* PD *ACLR1D+4*PD*ACLR2D

Figure 8:Interference from A-IoT to NR UE receiver

### D2T2 and interference modeling

Figure 9:Interference source for coexisting case 13 to 20

Figure 7 shows the interference source for different cases for D2T2 in Table 2 and these cases are listed in excel sheet for calibration purpose.

Table 2: D2T2 coexisting cases

| deployment scenario and topology​ |  | Evaluation case No.​ | (Aggressor -> Victim)​ | spectrum​ |
| --- | --- | --- | --- | --- |
| 3​ | D2T2-A2-legacy UE only outdoor​ | 13​ | device -> NR UL​ | R2D: UL​ CW2D and D2R: UL​ |
|  |  | 14​ | NR UL -> reader​ |  |
|  |  | 15​ | reader -> NR UL​ |  |
|  |  | 16​ | NR UL -> device​ |  |
| 4​ | D2T2-A2-legacy UE indoor​ | 17​ | device -> NR UL​ | R2D: UL​ CW2D and D2R: UL​ |
|  |  | 18​ | NR UL -> reader​ |  |
|  |  | 19​ | reader -> NR UL​ |  |
|  |  | 20​ | NR UL -> device​ |  |

Interference for case 13 and 17:

The interference source: CW transmission, A-IoT device backscatter transmission

Related RF parameter: CW emission mask, A-IoT device emission mask

The interference is the same with case 1 and 7

Interference calculation:

I  = PCW*ACLRCW + 2* PD *ACLR1D+4*PD*ACLR2D

Interference for case 15 and 19:

The interference source: A-IoT reader (UE as intermediate node) transmission.

Related RF parameter: ACLR of Reader.

In this case, the reader (UE as reader) will transmit one RB within A-IoT BS channel. Therefore, the interference modelling from reader (UE) relates to the ACLR of A-IoT UE. The transmission bandwidth from RAN1 Ls[3] is 180kHz (M) and RAN4 should further discuss the guard band of the channel so the BW of channel could be decided. Assuming there will be some guard band and the emission mask should be starting from the edge of the channel. At NR BS , it could discuss whether additional channel filter will be implemented to attenuate the A-IoT signal, as this is legacy NR BS, so no additional filter could be assumed as starting points.

As the UE also transmit to the outdoor BS, from the RF perspective, RAN4 should discuss whether the reader (UE) in this case has different channel configured when it transmit to the A-IoT device. In such case, when UE transmits to the macro BS, the channel bandwidth will be switched to wider macro BS channel so the ACLR of the UE will be different.

UE as reader may switch different channel when communicating with macro BS.

In NB-IoT, the interference from ACLR is scaled with adjacent system BW and number of UE as below in 36.802:

ACLR_e = ACLR – 10*log10((9MHz/(# of LTE users))/180kHz) [pessimistic assumption]

As the A-IoT system also transmit one PRB and therefore similar approach could be taken:

ACLR_e_AIOT_reader = ACLR – 10*log10((9MHz/(# of  NR users))/180kHz) [pessimistic assumption]

Interference calculation:

I  = PAIoT_reader*ACLR_e_AIoT_reader

Interference for case 14 and 18:

The interference source: NR UE transmission

Related RF parameter: NR UE IBE mask, ACS of the A-IoT reader (UE as intermediate node)

Interference calculation:

I  = (PUE2 + PUE3)*ACSR-reader  + (PUE2 + PUE3)*IBE_first adjacent RB outside of the allocated bandwidth

Figure 10: interference from NR UE to A-IoT reader receiver

Interference for case 16 and 20:

The interference source: NR UE transmission.

Related RF parameter: N/A

In this case, it is similar with case 4 and 10 in D1T1. Though there is agreement in WF below, we think it interference calculation should be relating to the power difference between NR and wanted A-IoT signal and the noise bandwidth of receiver.  Therefore, it will be difficult to calculate the interference with simple assumption as the power difference is different from different NR BS. In this sense, we suggest RAN4 make a LLS for this case to calculate the interference.

Issue 2-4-2: SINR definition for R2D

Agreement in RAN4#111:

SINR for R2D for calibration purposes

- signal power of device to the noise and interference within 10MHz

  - Assume interference NR BW is 10MHz

RAN4 discuss the modelling of the interference source.

### Further consideration on the performance measurement

The SINR degradation can be derived from the SINR CDF curve for the victim system. However, from the calibration results in Annex, it is observed that there is little impact on the SINR degradation. Partly reason of this is that the SINR degradation may be averaged out when SINR statistic is derived from large number of the victim nodes. To show the degradation, we suggest to take the approach of the NB-IoT in WF[2] where the SINR cdf curve of individual adjacent RB of the NR system is calculated. Another alternative is to show the SINR only for the nearer NR BS, which may equally reduce the macro BS layer nodes numbers to a few.

Collect the SINR CDF curve for individual adjacent RB to the A-IoT RB

Alternatively collect the SINR CDF for nearer NR BSs around A-IoT BS deployments

## outside topology simulation

In WF [1], the evaluation cases are stated as below.

Issue 2-5-2: Evaluation cases for D1T1 for device 1 and 2a between NR and AIOT

Agreement:

- FFS on evaluation cases for D1T1 for device 1 and 2a between NR and AIOT. Corresponding evaluation cases are listed for information. Note that some duplicated cases are omitted in the table.

| Deployment scenario and topology | spectrum | aggressor | victim |
| --- | --- | --- | --- |
| · Case 1-1: CW is transmitted from inside the topology, transmitted in DL spectrum · Case 1-2: CW is transmitted from inside the topology, transmitted in UL spectrum | R2D: DL CW2D and D2R: UL | CW and/or device | NR UL |
|  |  | NR UL | device and/or reader |
|  |  | reader | NR DL |
|  |  | NR DL | device |
|  | R2D: DL CW2D and D2R: DL | CW and/or device | NR DL |
|  |  | NR DL | device and/or reader |
|  | R2D: UL CW2D and D2R: UL | Reader | NR UL |
|  |  | NR UL | reader |
| Self interference cancelation is needed for reader · Case 1-1: CW is transmitted from inside the topology, transmitted in DL spectrum · Case 1-2: CW is transmitted from inside the topology, transmitted in UL spectrum | R2D: DL CW2D and D2R: UL | CW and/or device | NR UL |
|  |  | NR UL | device and/or reader |
|  |  | reader | NR DL |
|  |  | NR DL | device |
|  | R2D: DL CW2D and D2R: DL | CW and/or device | NR DL |
|  |  | NR DL | device and/or reader |
|  | R2D: UL CW2D and D2R: UL | reader | NR UL |
|  |  | NR UL | reader |
| Self interference cancellation is needed for reader · Case 1-4: CW is transmitted from outside the topology, transmitted in UL spectrum | R2D: DL CW2D and D2R: UL | CW and/or device | NR UL |
|  |  | NR UL | device and/or reader |
|  |  | reader | NR DL |
|  |  | NR DL | device |
|  | R2D: UL CW2D and D2R: UL | reader | NR UL |
|  |  | NR UL | reader |

Issue 2-5-3: Evaluation cases for D2T2 for device 1 and 2a between NR and AIOT

Agreement:

- FFS on evaluation cases for D2T2 for device 1 and 2a between NR and AIOT. Corresponding evaluation cases are listed for information. Note that some duplicated cases are omitted in the table.

| Deployment scenario and topology | spectrum | aggressor | victim |
| --- | --- | --- | --- |
| Case 2-2: CW is transmitted from inside the topology (i.e., intermediate UE), transmitted in UL spectrum | R2D: UL CW2D and D2R: UL | CW and/or device | NR UL |
|  |  | NR UL | Device and/or reader |
|  |  | reader | NR UL |
|  |  | NR UL | device |
| \ Case 2-2: CW is transmitted from inside the topology (i.e., intermediate UE), transmitted in UL spectrum Self interference cancelation is needed for reader | R2D: UL CW2D and D2R: UL | CW and/or device | NR UL |
|  |  | NR UL | Device and/or reader |
|  |  | reader | NR UL |
|  |  | NR UL | device |
| Case 2-3: CW is transmitted from outside the topology, transmitted in DL spectrum Case 2-4: CW is transmitted from outside the topology, transmitted in UL spectrum Self interference cancelation is needed for reader | R2D: UL CW2D and D2R: UL | CW and/or device | NR UL |
|  |  | NR UL | Device and/or reader |
|  |  | reader | NR UL |
|  |  | NR UL | device |
|  | R2D: UL CW2D and D2R: DL | CW and/or device | NR DL |
|  |  | NR DL | device and/or reader |

Issue 2-5-4: Evaluation cases for device 2b between AIOT and NR

Agreement:

- FFS for evaluation cases for device 2b

Issue 2-5-5: Interference between AIOT systems

Agreement:

- RAN4 to first evaluate interference between AIOT and NR.

- FFS on interference between AIOT and AIOT

The main difference between a legacy 3GPP coexisting and the A-IoT coexisting is that a new node of carrier-wave transmitter (CWT) is introduced on top of the BS /UE coexisting. From this perspective, the CWT characteristic, the CWT deployment in relation to the BS deployment, the antenna characteristic, power, SNR etc. should be discussed first.

RAN1 discuss the CWT inside topology and CWT outside the topology, when CWT is inside topology, the CW is transmitted by either BS and UE and as such the maximum output power can be the same with BS or UE; when CWT is out of topology, the CWT node has its own transmitter and therefore the separate RF parameter needs to be defined. In Figure 1 below, the interference source is visualized when BS is victim for UL receiving and listed in Table 1. Similarly, for DL coexisting, the aggressor sources are listed in Table 2. Clearly there are more parameters to be modeled comparing to the legacy coexisting simulation.

Figure 11: UL coexisting overview for CWT outside the topology

Table 3: The UL interference from aggressors to BS receiving @f1

| - Aggressor | - Parameter | - Note |
| --- | --- | --- |
| - CW node transmitting at f1 | - In-band emission level relative to the transmitted power at certain frequency offset to the CW transmitted frequency | - The phase noise or PA nonlinear noise both impact this level. |
| - CW node transmitting at f2 | - ACLR of CWT | - The adjacent channel leakage power to the wanted signal channel |
| - Backscatter transmission at f2 | - ACLR of CWT attenuated by a backscattering loss at A-IoT | - The adjacent channel leakage power to the wanted signal channel |
| - Backscatter transmission at f1 by other A-IoT device | - CW signal will be modulated and backscattered not only by active UE within the cell, but also modulated and backscattered by other active UE in a cell operating by another operator | - Active A-IoT UE operating at f2 in another cell can backscatter the signal at f1 when both CW at f1 and f2 present at the same time. |

Figure 12:DL coexisting overview for CWT outside the topology

Table 4: The DL interference from aggressors to A-IoT receiving @f1

| - Aggressor | - Parameter | - Note |
| --- | --- | --- |
| - CW node transmitting at f1 | - In-band emission level relative to the transmitted power at certain frequency offset to the CW transmitted frequency | - The phase noise or PA noise both impact this level. |
| - CW node transmitting at f2 | - ACLR of CWT - Potential IMD degradation | - The adjacent channel leakage power to the wanted signal channel |
| - BS transmission at f2 | - ACLR | - The adjacent channel leakage power to the wanted signal channel |

For the CWT topology, when CWT node is transmitting the CW signal, the CW signal can be used for the A-IoT device type 1 to harvest energy. The maximum distance to the A-IoT should be corresponding to minimum power to power on A-IoT device. Such distance could be increased with a higher CWT power, but as CWT is also one interference source, the CWT power should be controlled in a reasonable level, with referencing to the coexisting results. As a starting point, the CWT node topology could be similar with BS topology but with different network grid shift. This could be a similar case with the different network layer with different grid shift. When CW is in topology, it can be regarded as a special case where CWT node is co-located with BS.

Model a dedicated CWT node layer with a grid shift to the network layer.

When CW is inside topology, the CW signal will be transmitting from BS or UE, in such a case, there is no dedicated CW node, and one network layer can be used.

The coexisting simulation scenarios are listed in Table 5 below, the cases number can be increased when RAN1 agrees also how the CW is transmitted (DL or UL) band.

Table 5: The coexisting simulation cases for D1T1 of outside topology

| Simulation cases/ Scenarios | Topology 1: indoor BS + indoor AIoT device |  |  |
| --- | --- | --- | --- |
|  | Victim | Aggressor | CW in/outside topology |
| 1 | BS UL receiving | Other BSs transmitting CWT in adjacent channel CWT node transmitting in cochannel CW node transmitting in adjacent channel | CW outside topology |
| 2 | A-IoT DL receiving | CW node transmitting at cochannel CW node transmitting at adjacent channel BS transmission at adjacent channel | CW outside topology |

Consider the simulation cases in above table in this meeting and update it when RAN1 agree on CW transmission spectrum (UL or DL spectrum) between A-IoT systems.

- In last meeting, the A-IoT and NR cases are prioritized, However, we think coexisting case of A-IoT and A-IoT may be more critical than coexisting case of NR and A-IoT. Because there is no in-band selectivity of the RF ED architecture in DL for device 1 and device 2a, this means the adjacent A-IoT carrier will be act as “co-channel” interferer as the RF ED will convert any channel to the baseband so the adjacent channel signal will overlap with the wanted signal fully. This creates a situation of the wanted channel and adjacent channel competing to each other when want channel and adjacent channel deployment share the same coverage. To be able to be received by A-IoT, the wanted channel power of A-IoT device in DL should be high enough and therefore, the adjacent cell should be grid shifted, or ideally, they should not even share the same coverage. This means the grid shift may not be the same as the legacy in percentage wise, but maybe the distance in meters.

- For UL, A-IoT from different operator could share the same coverage as the backscattered signal can be differentiated at A-IoT BS, this can be further studied in the coexisting case.

RAN4 continue to discuss the A-IoT coexisting with A-IoT scenario.

# Conclusions

In this contribution, we present our view on the coexisting simulation with below proposal and observation:

Observation 1 UE as reader may switch different channel when communicating with macro BS.

Proposal-1: Use the ACLR for interference modelling for coexisting study.

Proposal-2: Use the same ACLR characteristic of the device 1/2a for the CW transmission.

Proposal-3: Use MCL of 70 dB for Minimum NR BS – NR UE distance setting in coexisting simulation.

Proposal-4: RAN4 conduct the LLS for the R2D SINR calculation.

Proposal-5:RAN4 discuss the modelling of the interference source.

Proposal-6: Collect the SINR CDF curve for individual adjacent RB to the A-IoT RB

Proposal-7:Alternatively collect the SINR CDF for nearer NR BSs around A-IoT BS deployments

Proposal-8: Model a dedicated CWT node layer with a grid shift to the network layer.

Proposal-9: Consider the simulation cases in above table in this meeting and update it when RAN1 agree on CW transmission spectrum (UL or DL spectrum) between A-IoT systems.

Proposal-10: RAN4 continue to discuss the A-IoT coexisting with A-IoT scenario.

# References

- R4-2410567, WF on co-existence evaluation for ambient IoT, CMCC

- R4-158206， Way forward on coexistence evaluation methodology for NB-IOT in-band and guard band operation，Qualcomm, etc

- R1-2403782, LS on Ambient-IoT evaluation scenarios and assumptions

# Annex

## Case 1 - Victim: nrul

Figure A- 1: coupling loss distribution for victim system

Figure A- 2: SINR CDF wo aggressor system (baseline)

## Case 2 - Victim: d2r

Figure A- 3: coupling loss distribution for victim system

Figure A- 4: SINR CDF wo aggressor system (baseline)

## Case 3 - Victim: nrdl

Figure A- 5: coupling loss distribution for victim system

Figure A- 6: SINR CDF wo aggressor system (baseline)

## Case 4 - Victim: r2d

Figure A- 7: coupling loss distribution for victim system

Figure A- 8: SINR CDF wo aggressor system (baseline)

## Case 5 - Victim: nrdl

Figure A- 9: coupling loss distribution for victim system

Figure A- 10: SINR CDF wo aggressor system (baseline)

## Case 6 - Victim: d2r

Figure A- 11: coupling loss distribution for victim system

Figure A- 12: SINR CDF wo aggressor system (baseline)

## Case 7 - Victim: nrul

Figure A- 13: coupling loss distribution for victim system

Figure A- 14: SINR CDF wo aggressor system (baseline)

## Case 8 - Victim: nrdl

Figure A- 15: coupling loss distribution for victim system

Figure A- 16: SINR CDF wo aggressor system (baseline)

## Case 9 - Victim: nrdl

Figure A- 17: coupling loss distribution for victim system

Figure A- 18: SINR CDF wo aggressor system (baseline)

## Case 10 - Victim: r2d

Figure A- 19: coupling loss distribution for victim system

Figure A- 20: SINR CDF wo aggressor system (baseline)

## Case 11 - Victim: nrdl

Figure A- 21: coupling loss distribution for victim system

Figure A- 22: SINR CDF wo aggressor system (baseline)

## Case 12 - Victim: d2r

Figure A- 23: coupling loss distribution for victim system

Figure A- 24: SINR CDF wo aggressor system (baseline)

## Case 13 - Victim: nrul

Figure A- 25: coupling loss distribution for victim system

Figure A- 26: SINR CDF wo aggressor system (baseline)

## Case 14 - Victim: d2r

Figure A- 27: coupling loss distribution for victim system

Figure A- 28: SINR CDF wo aggressor system (baseline)

## Case 15 - Victim: nrul

Figure A- 29: coupling loss distribution for victim system

Figure A- 30: SINR CDF wo aggressor system (baseline)

## Case 16 - Victim: r2d

Figure A- 31: coupling loss distribution for victim system

Figure A- 32: SINR CDF wo aggressor system (baseline)

## Case 17 - Victim: nrul

Figure A- 33: coupling loss distribution for victim system

Figure A- 34: SINR CDF wo aggressor system (baseline)

## Case 18 - Victim: d2r

Figure A- 35: coupling loss distribution for victim system

Figure A- 36: SINR CDF wo aggressor system (baseline)

## Case 19 - Victim: nrul

Figure A- 37: coupling loss distribution for victim system

Figure A- 38: SINR CDF wo aggressor system (baseline)

## Case 20 - Victim: d2r

Figure A- 39: coupling loss distribution for victim system

Figure A- 40: SINR CDF wo aggressor system (baseline)