# 3GPP TSG-RAN WG4 Meeting # 112						R4-2412974

# Maastricht, Netherland, 19th - 23rd  August, 2024

Source:		Ericsson

Title:			BS RF requirement overview for LP-WUS

Agenda item:		8.22.3

Document for:		Approval

# Introduction

In this paper, we provide our view on BS RF requirement impact.

# Discussion

In WF[2], the different aspects of the RF impact of the LP-WUS signal are listed and we share our view on these issues in below discussion.

As in study phase, the focus of the evaluation of the RF impact of LP-WUS is on the power boosting capability. The boosting of signal may impact the emission performance especially when the WUS signal is located at the channel edge. In 38.104, the booting is specified in RE power control dynamic range as illustrated below.

Table 6.3.2.2-1: RE power control dynamic range

| Modulation scheme used | RE power control dynamic range (dB) |  |
| --- | --- | --- |
| on the RE | (down) | (up) |
| QPSK (PDCCH) | -6 | +4 |
| QPSK (PDSCH) | -6 | +3 |
| 16QAM (PDSCH) | -3 | +3 |
| 64QAM (PDSCH) | 0 | 0 |
| 256QAM (PDSCH) | 0 | 0 |
| 1024QAM (PDSCH) | 0 | 0 |
| NOTE: The output power per carrier shall always be less or equal to the maximum output power of the base station. |  |  |

In 38.141-1 and 38.141-2, for FR1 test model TM1.2 including a 3dB boosting at the channel edge with up to 40% of the total PDSCH RB allocated in the channel. So if the CP-OFDM symbol used for LP-WUS generation is QPSK modulation, the 3dB power boosting could be possible already for legacy NR Base station. For the applicable BS type, as NB-IoT power boosting applies to type 1-C, the LP-WUS power boosting can follow the same way as NB-IoT. In 38.141-1, the declaration for NB-IoT power boosting only for BS type 1-C.

Table 4.6-1 [2] Manufacturer declarations for BS type 1-C and BS type 1-H conducted test requirements

| Declaration identifier | Declaration | Description | Applicability |  |
| --- | --- | --- | --- | --- |
|  |  |  | BS type 1-C | BS type 1-H |
| D.41 | NB-IoT operation | Manufacturer shall declare the support of NB-IoT operation in NR in-band and the number of supported NB-IoT carriers in total and for each supported band, frequency range and channel bandwidth. | x |  |
| D.42 | NB-IoT sub-carrier spacing | If the BS supports NB-IoT operation in NR in-band, manufacturer shall declare if it supports 15 kHz sub-carrier spacing, 3.75 kHz sub-carrier spacing, or both for NPUSCH. | x |  |
| D.43 | NB-IoT power dynamic range | If the BS supports NB-IoT operation in NR in-band, manufacturer shall declare the maximum power dynamic range it could support with a minimum of +6dB or +3dB as specified in clause 6.3.4 of TS 38.104 [2] (Note 5). | x |  |

3 dB power boosting is possible if CP-OFDM symbol used for LP-WUS generation is QPSK modulation.

3 dB power boosting is applied to BS type 1-C, 1-H and 1-O in test model TM1.2.

Power boosting declaration only applies for BS type 1-C.

Limit the power boosting for LP-WUS to 3 dB for BS type 1-C.

However, there is no such power boosting test model defined in FR2, therefore, the power boosting can be assumed as the power boosting is not tested. As FR2 scope in Rel-19 is not clear, RAN4 could wait further RAN1 progress on this.

No test model for power boosting is defined for FR2 and power boosting capability cannot be assumed for FR2 BS.

- To have OOK1 waveform, the QPSK modulation can be mapped to CP-OFDM symbol which is used to generate LP-WUS signal. With OOK-4 (DFT-s or waveform fitting approaches) the gNB may need to support additional DFT (or another transform for LS approach) or pre-generate and store frequency domain symbols. Additionally, the frequency domain symbols used as inputs of IFFT may not be similar to those used for generating existing NR signals (e.g., M-QAM). Mapping the generated frequency domain values (before IFFT) to existing sequences/QAM modulations could be considered for this approach to reduce the impact on gNB. Therefore, for OOK4 waveform, the higher modulation than QPSK may be mapped to CP-OFDM symbols as illustrated in Figure 1. As Such, gNB can not be boosted according to the current base station RE dynamic range requirement.

Figure 1:Constellation mapping for multi-bit OOK WUS (4-bit OOK case mapped to 64QAM).

RAN4 should allow dynamic range requirements based on existing specification for OOK-1 waveform.

When the overlaid sequence is added on top of the LP-WUS, whether power boosting can be enabled needs to be discussed. RAN1 currently discuss either time domain or frequency domain generation of the overlaid sequence and for time domain sequency generation, the constellation mapping may not be lower modulation order and this should be further discussed after RAN1 progress further.

For overlaid LP-WUS signal, RAN4 investigate further after RAN1 make further progress.

As the power boosting is applied to LP-WUS feature and as such it would be appropriate that manufacture declare it. Another reason is that the LP-WUS signal bandwidth is not agreed yet.  If the system channel bandwidth would be the same with LP-WUS signal bandwidth, it is not possible to boost as the working assumption is below.

  - The rated carrier output power and rated total output power are not changed with LP-WUS power boosting.

The power boosting of the LP-WUS is up to manufacture declaration.

For the other requirement than the dynamic range (e.g EVM for OOK), we donot think it is necessary as the new waveform is synthesized with the legacy OFDM transmitter. The OOK waveform is only observable at the receiver side wide with WUR bandwidth filter. From the base station side, it is OFDM waveform it transmitting and transmitting LP-WUS within 5MHz bandwidth does not change this.

No other RF requirement than the dynamic range should be specified for LP-WUS.

# Conclusions

In this contribution, we present our view on the BS RF requirement with below observations and proposals:

Observation 1 3 dB power boosting is possible if CP-OFDM symbol used for LP-WUS generation is QPSK modulation.

Observation 2 3 dB power boosting is applied to BS type 1-C, 1-H and 1-O in test model TM1.2.

Observation 3 Power boosting declaration only applies for BS type 1-C.

Proposal-1: Limit the power boosting for LP-WUS to 3 dB for BS type 1-C.

Observation 4 No test model for power boosting is defined for FR2 and power boosting capability cannot be assumed for FR2 BS.

Proposal-2: RAN4 should allow dynamic range requirements based on existing specification for OOK-1 waveform.

Observation 5 For overlaid LP-WUS signal, RAN4 investigate further after RAN1 make further progress.

Proposal-3: The power boosting of the LP-WUS is up to manufacture declaration.

Proposal-4: No other RF requirement than the dynamic range should be specified for LP-WUS.

# References

- R4-2409956, Way Forward for [110bis][314] NR_LPWUS, Huawei

- 38.141-1, Base Station (BS) conformance testing Part 1: Conducted conformance testing