3GPP TSG-RAN WG4 Meeting # 112						R4-2412975

Maastricht, Netherland, 19th - 23rd  August, 2024

Source:		Ericsson

Title:			On general issues for WUR

Agenda item:		8.22.1

Document for:		Approval

# Introduction

In this paper, we present our overview on the general issues for WUR discussed lasted meeting.

# Discussion

One question remain to be clarified is that how to test OFDM WUR. In TR 38.869, the OFDM WUR related architecture is quoted as below:

- E)	OFDMA-based signals/channels detection

For OFDMA-based signals/channels, study the receiver architectures based on the following diagrams:

-	I/Q branches are required for digital BB processing.

-	Digital BB processing may or may not include FFT (companies to provide details on how).

-	For sequence-based OFDM signals/channels, digital BB processing includes sequence correlation in either time domain (without FFT) or frequency domain (after FFT).

So basically the OFDM WUR RF front-end should be similar with OFDM receiver. However, the NF may be higher for time domain correlation and documented between 7 to 25 dB in Table 7.1.1a-8 in TR 38.869.

According to the least RAN1 agreement on the SNR point to match the MSG3 PUSCH coverage, both OOK and OFDM WUR should be targeted to the same SNR and this leads to the same test requirement to cover both OFDM WUR and OOK WUR.

Agreement

For evaluation of LP-WUS and LP-SS design to achieve coverage of PUSCH for message3 from RAN1 perspective, at least the following SNR values should be considered:

- SNR=-3dB for NF of LR = NF of MR+ 8dB

- SNR= -0.5dB for NF of LR = NF of MR+ 5dB

- SNR=2dB for NF of LR = NF of MR+ 2dB

- Note 1: The NF of MR is assumed as 7dB

We believe both OFDM WUR and OOK WUR should be discussed together as the SNR target from RAN1 is set the same.

OFDM WUR and OOK WUR could be tested under the same RF requirements.

# Conclusions

In this contribution, we present our view on some of general aspects for WUR with below proposal:

Proposal-1: OFDM WUR and OOK WUR could be tested under the same RF requirements.

# References

- R4-2410569, WF for [111][136] NR_LPWUS_UERF, vivo

- TR 38.869.