# 3GPP TSG-RAN WG4 Meeting # 112						R4-2412976

# Maastricht, Netherland, 19th - 23rd  August, 2024

Source:		Ericsson

Title:			On system parameter for WUR

Agenda item:		8.22.2.1

Document for:		Approval

# Introduction

In this paper, we present our overview on the system parameter for WUR RF requirement perspective.

# Discussion

In WF[1], the channel raster for WUR is not decided.

Issue 1-1-7: Channel raster for LP-WUR

Agreements:

RAN4 further check whether no channel raster is needed for LP-WUR.

In TR 38.869, the WUS carrier/BW is defined in Figure 7.1.2.1-1 (quoted below). So it is the part of RB region within the MR transmission bandwidth. It can be observed that the WUS BW will occupy a range of the RB within the MR RB grid.

In latest RAN2 agreement, the LP-WUS will be configured in system information for RRC idle and inactive and this means UE can locate the LP-WUS when the SIB is obtained.

### Procedure and configuration of LP-WUS in RRC_IDLE INACTIVE

The LP-WUS related configuration for IDLE/INACTIVE state is provided via system information. FFS if dedicated configuration is needed. (e.g. dedicated signalling to enable/disable LP-WUS)

Working assumption: the LP-WUS configuration in SIB at least includes the following information:

- -	LP-SS configuration

- -	LP-WUS configuration

- -	FFS on Entry/exit condition for LP-WUS monitoring (chair: RRC config is not a possible entry/exit condition, i.e. full coverage/OFDM unfriendly)

From the above observation, the WUR will not be configured with a channel and therefore channel raster concept will no apply to WUR. Network needs to assure the LP-WUS signal should be transmitted together with other NR signals using the same iFFT modulator, this also means the LP-WUS signal position should be referenced with the serving cell RB grid.

RAN2 agrees to configure LP-WUS in SIB.

Channel raster does not apply to WUR.

(case 1: LP-WUS within large channel)

(case 2: WUS within narrow channel, WUS bandwidth is comparable with channel bandwidth)

Figure 7.1.2.1-1: Definition of the guardband of NR channel and guard RBs for both LP-WUS ACS and ASCS

In RAN1 agreement, it is agreed that X PRB for LP-WUS and LP-SS with SCS 30kHz is supported by all device.

Agreement

Update agreement in last meeting as below:

From RAN1 perspective, support X PRBs for LP-WUS and LP-SS with SCS 30kHz (blanked guard RBs are not included) for a channel bandwidth equal or larger than 5MHz

- X  = 11PRBs

- FFS if other number of PRBs needed, for LP-SS and LP-WUS with a channel bandwidth equal or less than 5MHz

FFS: Whether the above is applicable to FR2

From the above agreement, it can be observed that WUR receiver should be tested with the X PRB capability for SCS 30kHz, it may be different for SCS 15kHz.

RAN1 agree for the 11 PRBs for LP-WUS for SCS 30kHz.

This means that in RF requirement, the WUR BW could be specified with X PRB and this also means there is no need to add a clause in system parameter for WUR.

No specification impact on WUR in terms of system parameter.

The BW of WUR should be specified in X PRB of LP-WUS (X=11 for SCS =30kHz and BW > 5MHz) referencing to the RB grid of MR.

# Conclusions

In this contribution, we present our view on system parameter for WUR with below proposal:

Observation 1 RAN2 agrees to configure LP-WUS in SIB.

Proposal-1: Channel raster does not apply to WUR.

Observation 2 RAN1 agree for the 11 PRBs for LP-WUS for SCS 30kHz.

Proposal-2: No specification impact on WUR in terms of system parameter.

Proposal-3: The BW of WUR should be specified in X PRB of LP-WUS (X=11 for SCS =30kHz and BW > 5MHz) referencing to the RB grid of MR.

# References

- R4-2410569, WF for [111][136] NR_LPWUS_UERF, vivo