# 3GPP TSG-RAN WG4 Meeting # 112						R4-2412977

# Maastricht, Netherland, 19th - 23rd  August, 2024

Source:		Ericsson

Title:			On WUR RF requirement testability

Agenda item:		8.22.2.4

Document for:		Approval

# Introduction

In this paper, we present our overview on the WUR RF requirement testability issue.

# Discussion

In WF[1], there are couple of issues relating to the testing aspects.

Issue 1-1-9: Side condition for LP-WUR requirements

Agreements:

  - FFS RAN4 should consider both idle and connected mode conditions in the side conditions for the LPWUR requirements.

Issue 1-1-11: Metric for LP_WUR requirements

Agreements:

  - MDR is used as the metric for LP_WUR Rx RF requirements

    - FFS MDR values.

  - FAR will be defined as demodulation requirements

FFS FAR values

For test condition, we believe either idle or connected mode would be fine. This is because to achieve the certain MDR, the tests needs to be repeated with certain times and this may increase the test time. If testing time would be concern, there is no need to test both idle and connect mode for the same RF requirement.

One test condition is enough for same RF requirements, no need to repeat the same RF requirement with different test conditions.

For the test metric, the MDR of 1% could be used with quoted RAN1 agreement as below. For FAR, as it is demodulation discussion, RAN4 can postpone this till performance phase.

Agreement

For RRC idle/inactive state, down-select among the following options for at least indicating subgroup information using LP-WUS:

- The followings are considered when down-select among options:

  - The number of supported UE subgroups per PO: M

  - Average network overhead to indicate the number of UE subgroups M per PO

  - False wake up rate due to subgroup-based indication, which will impact the power saving gain

  - Paging latency

  - Note: Coverage target shall be met under 1%BLER, 1% FAR (for false alarm from noise)

1% MDR (BLER) should be used and postpone FAR discussion to performance phase.

RAN5 could also introduce new test mode if they deem the test time would be long and further optimization for it. This should be decided by RAN5 and RAN4 can send LS to RAN5 to trigger the discussion.

Send a LS to RAN5 to see if test time could be further optimized from testing perspective.

LS to RAN5 to confirm this with text below:

In case a LS to RAN5, below LS text is proposed:

RAN4 are discussing the test metric for wake up receiver RF performance test. As the WUR only detect LP-WUS/LP-SS and no other signals, legacy throughout monitoring for PDSCH is not possible anymore. Therefore, RAN4 propose the test metric of Miss Detection Rate of LP-WUS (target 1%). To test MDR of LP-WUS, there are two options to do it and RAN4 agree that it will be up to RAN5 to decide which options is suitable for WUR receiver test.

Two options below to test

- Using the legacy paging procedure to detect successfully LP-WUS reception by WUR

- New test mode for testing the successfully LP-WUS reception by WUR

# Conclusions

In this contribution, we present our view on WUR RF testability with below proposal:

Proposal-1: One test condition is enough for same RF requirements, no need to repeat the same RF requirement with different test conditions.

Proposal-2: 1% MDR (BLER) should be used and postpone FAR discussion to performance phase.

Proposal-3: Send a LS to RAN5 to see if test time could be further optimized from testing perspective.

Proposal-4: LS to RAN5 to confirm this with text below:

RAN4 are discussing the test metric for wake up receiver RF performance test. As the WUR only detect LP-WUS/LP-SS and no other signals, legacy throughout monitoring for PDSCH is not possible anymore. Therefore, RAN4 propose the test metric of Miss Detection Rate of LP-WUS (target 1%). To test MDR of LP-WUS, there are two options to do it and RAN4 agree that it will be up to RAN5 to decide which options is suitable for WUR receiver test.

Two options below to test

- Using the legacy paging procedure to detect successfully LP-WUS reception by WUR

- New test mode for testing the successfully LP-WUS reception by WUR

# References

- RP-240801, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), vivo etc

- TR 38.869,  Study on low-power wake-up signal and receiver for NR

- R4-2406619, WF on NR LP-WUS UE requirements, vivo

# Annex 1

3GPP TSG-RAN WG4 Meeting # 112						R4-240xyz

Maastricht, Netherland, 19th- 23nd August, 2024

Title:	LS on WUR testibility

Response to:

Release:	Rel-19

Work Item:	NR_LPWUS-Core

Source:	RAN WG4

To:	RAN WG5

Cc:

Contact Person:

Name:	Chunhui Zhang

E-mail Address:	Chunhui (dot) Zhang (at) ericsson (dot) com

Attachments:

1. Overall Description:

RAN4 are discussing the test metric for wake up receiver RF performance test. As the WUR only detect LP-WUS/LP-SS and no other signals, legacy throughout monitoring for PDSCH is not possible anymore. Therefore, RAN4 propose the test metric of Miss Detection Rate of LP-WUS (target 1%) as one option. To test MDR of LP-WUS, there are two options to do it and RAN4 agree that it will be up to RAN5 to decide which options is suitable for WUR receiver test.

Two options below to test

- Using the legacy paging procedure to detect successfully LP-WUS reception by WUR

- New test mode for testing the successfully LP-WUS reception by WUR

RAN4 would like to know which option would be appropriate to design the test case for WUR.

2. Actions:

To RAN WG1/WG2 group.

ACTION: 	RAN4 would like RAN5 consider the information above and provide answer for the RAN4 questions above.

3. References:

4. Date of Next TSG-RAN WG4 Meetings:

TSG RAN WG4 Meeting #112bis		Oct 14-18 , 2024						China

TSG RAN WG4 Meeting #113		Nov 18-22 , 2024						Orlando