# 3GPP TSG-RAN WG4 Meeting # 112						R4-2412979

# Maastricht, Netherland, 19th - 23rd  August, 2024

Source:		Ericsson

Title:			WUR RF requirement of REFSESN, ASC, ASCS

Agenda item:		8.22.2.2

Document for:		Approval

# Introduction

In this paper, we present our view on the WUR RF requirement of REFSENS, ASC, ASCS.

# Discussion

### REFSENS

The UE REFSENS is defined by the following equation:

REFSENS=kTB + SNR +10log10(RB number*SCS*12) +( NF+ IM) – Diversity gain

Where

-	kTB: Thermal noise level is [-174dBm(kT) + 10*log10(RX BW)] dBm.

-	NF: Noise figure.

-	IM: 2.5 dB is assumed.

-	Target SNR:

-	Diversity gain: 3dB

REFSENS is directly related to the coverage so in the end it is related to WUR coverage question. In WID[1], the coverage target is stated below:

- Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3.

RAN1 has below agreement from last meeting.

Agreement

The following SNR values for LP-WUR are reported by companies to achieve coverage of PUSCH for message3 for difference noise figures for calibration purposes:

- NF of LR = NF of MR+ 8dB: the reported SNR value range is [-9.05,2.94] dB

- NF of LR = NF of MR+ 5dB:  the reported SNR value range is [-6.5, 5.58] dB

- NF of LR = NF of MR+ 2dB: the reported SNR value range is [-4.04,7.95] dB

- Note 1: The above is observed based on the following assumptions used for calibration:

-	Carrier frequency: 2.6 GHz

-	The number of Tx chains: 1

-	MIL of MSG 3: use the average one in R17 coverage, i.e.,153.51 dB for non-redcap UE

-	Transmit antenna gain correction factors for WUS: up to company report

-	Noise Figure: All three values +2dB, +5dB, +8dB on top of NF of MR (7dB) are to be reported, SNR for different assumptions on NF are determined separately

Agreement

For evaluation of LP-WUS and LP-SS design to achieve coverage of PUSCH for message3 from RAN1 perspective, at least the following SNR values should be considered:

- SNR=-3dB for NF of LR = NF of MR+ 8dB

- SNR= -0.5dB for NF of LR = NF of MR+ 5dB

- SNR=2dB for NF of LR = NF of MR+ 2dB

- Note 1: The NF of MR is assumed as 7dB

During the SNR evaluation, RAN1 has assumed different WUS signal structure and in our RAN1 contribution, we evaluated below.

Table 1[5]: WUS resources required to match msg3-PUSCH coverage (4.32 MHz WUS bandwidth).

| WUS payload (modulation) | Msg3, no rtx | Msg3, two rtx |
| --- | --- | --- |
| 1bit (OOK, sequence-based) | 18 symbols | 5 slots |
| 8bit (OOK1, payload-based, +10 bits CRC) | 6 slots | 25 slots |
| 16bit (OOK1, payload-based, +10 bits CRC) | 9 slots | 36 slots |
| 48bit (OOK, payload-based) | 40 slots | 180 slots |
| 1bit (OFDM, SSS-based time-correlation) | 2 symbols | 5 symbols |
| 8bit (OFDM, SSS-based time-correlation) | 3 symbols | 7 symbols |
| Note1: MDR = 1% and FAR = 0.1% assumed for above comparison. Note 2: When duration is given in slots, all the symbols in a slot are assumed to be available for WUS. Note 3: For Msg-3 PUSCH, 2 PRBs, 56 bits TBS, 14 OFDM symbols are assumed per slot. |  |  |

The reported SNR value is from the SNR value for all the different WUS structure in Table 1.

Table 2 [5]: SNR table for coverage evaluations.

|  | Bandwidth for LP-WUS signal (MHz) | NF for LP-WUR (dB) | Gain of antenna element (dBi) assumed for LP-WUR: | Antenna correction factor for WUS | # of Tx chains for LP-WUS/LP-SS transmission, e.g., 2 The number of Tx chains for LP-WUS/LP-SS transmission is assumed the same as the number of RX chains for MSG3 reception | MIL value of MSG3: taking redcap UE /non-redcap UE @dense urban 2.6GHz (per RAN1 116#bis agreement 153.5 dB shown from R17 coverage engh TR) | The SNR (dB) to achieve the coverage of PUSCH for message3 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Ericsson (7dB+8dB NF) | 4.32 MHz | 15 | 0 | 12 dB | 1 | 153.5 | -9 |
| Ericsson (7dB+5dB NF) | 4.32 MHz | 12 | 0 | 12 dB | 1 | 153.5 | -6.5 |
| Ericsson (7dB+2dB NF) | 4.32 MHz | 9 | 0 | 12 dB | 1 | 153.5 | -4 |
| Ericsson (7dB NF) | 4.32 MHz | 7 | 0 | 12 dB | 1 | 153.5 | -2.7 |

RAN1 collected the reported value for different NF group (NF of MR + 2, +5 and +8) and further agreed with one SNR threshold each for the different NF of WUR. As the WUS structure may be different configuration, the same SNR means that WUS configuration should be configured in a way that the target BLER is achievable with the agreed SNR when the NF of WUR would be the same with NF listed in RAN1 agreements. Apparently, such grouping of the NF and agreed SNR make room for RAN4 discussion with below observations:

Agreed SNR target from RAN1 corresponding to the different NF

Network configure the WUS structure to make sure the certain BLER should be achievable with agreed SNR when NF of WUR is in certain range.

NF is band specific number and is also RF architecture dependent. RAN4 has not decided on the NF of the WUR yet [ 2]. However, with the consideration of the matching coverage requirement and RAN1 agreement, one option could be that RAN4 agree with the maximum NF of WUR considering the different bands and RF architectures. As the RAN1 SNR agreement based on both 700MHz for rural and 2.6 GHz, so such option could be possible to cover the low and mid-band.

Limit the NF of the WUR to 15 dB for low and mid-bands.

RAN4 could discuss whether the different maximum NF of WUR could be assumed for OFDM WUR and OOK WUR. In study phase, the NF of OFDM WUR is expected to be better than OOK WUR if OFDM WUR uses frequency domain correlation. The noise figure of OFDM WUR for frequency domain correlation is 7 dB to 12 dB in Table 7.1.1a-9 in TR 38.869 while NF can be more than 15 dB for time domain correlation. There is also benefit at network side if the NF of OOK WUR and OFDM WUR could be confirmed differently or the same. In idle mode, the predefined NF of OFDM WUR and NF of the OOK WUR could help the network to define the WUS signal structure so that the overlaid WUS signal can be configured accordingly. At connected mode, the WUS signal can be further adapted if network would know if the UE were equipped with an OOK WUR or OFDM WUR.

RAN4 confirm if the maximum NF of the OFDM WUR can be assumed differently with OOK WUR.

For the SNR threshold used in REFSENS, according to RAN1 agreement above, the SNR of -3 dB can be assumed when the maximum NF of WUR is 15 dB. For other NF less than 15 dB, the SNR is increased to offset the NF decrease, this means the same REFSENS may be specified as long as the NF of WUR is less than the maximum allowed NF. Therefore, it could be possible to specify one unified REFSENS for both OFDM WUR and OOK WUR.

It is possible to specify unified RF requirement for both OOK WUR and OFDM WUR.

### ASCS

From the LLS simulation, when filter order is 3th or 5th. The BLER performance is illustrated in Figure 1 to Figure 4. It shows that larger guard RB size has penalty on the SNR threshold due to less received energy. Such observation holds for both AWGN and TDL-C channel and for OOK1 and OOK4. Because of this, the guard RB of less or equal than 1 RB at each side of WUS signal is good enough. The guard RB of 1 corresponds to the case where 11 RB WUS is configured and LP-WUS signal is confined within a 5MHz frequency block (13 RB in total within 5MHz block).

Larger guard RB size has penalty on the SNR threshold due to less received energy

Maximum of 1 guard RB at each side of WUS signal should be used in ASCS case.

The simulation case for the parameter setting is below.

| Attributes | Assumptions |  |  |
| --- | --- | --- | --- |
| Case name (waveform) | OOK-1 waveform |  | OOK-1 waveform |
| Center frequency | 900MHz, 2.6GHz and 3.5GHz |  |  |
| Channel structure | 8/16 bits payload |  |  |
| Chip rate | M=1 |  |  |
| WUS duration | 16 OFDM symbols |  |  |
| Coding | 1/2 rate Manchester coding |  |  |
| Time error | 0 |  |  |
| Residual Frequency error | 0 ppm |  |  |
| SCS | 30kHz |  |  |
| UE Channel BW | 20MHz (51 RB)-case 1 |  |  |
| WUS RB | - Fixed 11RB ~ 3.96MHz for 10MHz and 20MHz cases |  |  |
| Position within channel | - For 10/20MHz CBW, Center for ASCS, edge for ACS [assume no ASCS impact] - For 5MHz CBW, fixed center of channel |  |  |
| Guardband of NR channel, both wanted cell and interfer cell (ACS) | - For wanted signal: 505kHz for 5MHz, 665kHz for 10MH, 805kHz for 20Mhz - For interference cell2 5MHz: fixed 505kHz |  |  |
| Guard RB | - For ASCS: 1/1.5/2 RB on each side of LP-WUS bandwidth |  |  |
| Filter | - 3th/5th Order lowpass Butterworth matching fixed 3.96MHz RF bandwidth for 10MHz/20MHz case |  |  |
| ASCS | PDSCH mapped on RBs not used for LP-WUS and Guard RB; EPRE of PDSCH /EPRE of LP-WUS = 0 dB Same PSD with WUS signal |  |  |
| Wanted signal level | Same PSD with NR signal |  |  |
| Sampling rate | 7.68MHz |  |  |
| Phase noise | Not modelled |  |  |
| Channel Model | Option 1: TDL-C 300 Option 2: AWGN Note: encourage companies to provide simulation results with both options |  |  |
| Performance metric | - 1% MDR/BLER - The following false alarm rate is used - 0.1% |  |  |

Figure 1: BLER for ASCS with different LPF order (3 and 5) and guardRB size (1 and 2) with AWGN channel and OOK1

Figure 2: BLER for ASCS with different LPF order (3 and 5) and guardRB size (1 and 2) with AWGN channel and OOK4

Figure 3: BLER for ASCS with different LPF order (3 and 5) and guardRB size (1 and 2) with TDL-c channel and OOK1

Figure 4:BLER for ASCS with different LPF order (3 and 5) and guardRB size (1 and 2) with TDL-c channel and OOK4

### ACS

The simulation assumption from below table is used.

| Attributes | Assumptions |  |  |
| --- | --- | --- | --- |
| Case name (waveform) | OOK-1 waveform |  | OOK-4 waveform |
| Center frequency | 900MHz, 2.6GHz and 3.5GHz |  |  |
| Channel structure | Total 8 bits |  |  |
| Chip rate | M=1 | M=2 |  |
| WUS duration | FFS |  |  |
| Coding | 1/2 rate Manchester coding |  |  |
| Time error | 0 |  |  |
| Residual Frequency error | 0/10/20 ppm |  |  |
| SCS | 30kHz |  |  |
| UE Channel BW | 20MHz (51 RB)-case 1 |  |  |
| WUS RB | - Fixed 11RB ~ 3.96MHz for 10MHz and 20MHz cases |  |  |
| Position within channel | - For 10/20MHz CBW, Center for ASCS, edge for ACS [assume no ASCS impact] - For 5MHz CBW, fixed center of channel |  |  |
| Guardband of NR channel, both wanted cell and interfer cell (ACS) | - For wanted signal: 505kHz for 5MHz, 665kHz for 10MH, 805kHz for 20Mhz - For interference cell2 5MHz: fixed 505kHz |  |  |
| Guard RB | - For ACS: 1/2/3/4 RB |  |  |
| Filter | - 3th/5th Order lowpass Butterworth matching fixed 3.96MHz RF bandwidth for 10MHz/20MHz case - Other order lowpass filter is not precluded - The filter bandwidth is adapted with actual WUS RBs, for 5MHz case |  |  |
| ACS | PDSCH mapped on interference RBs (11RB for 5MHz CBW), one side; EPRE of PDSCH /EPRE of in-band LP-WUS = 33dB NOTE: decide the interference level depending on SNR |  |  |
| Sampling rate | 7.68MHz |  |  |
| ADC bit width | 4/8 bits ADC for ASCS/ACS |  |  |
| Phase noise | Not modelled |  |  |
| Non-linearities | Not modelled |  |  |
| Power boosting | EPRE ratio: 0dB/3dB for OOK-1/OOK-4 NOTE: 3dB is optional for simulation |  |  |
| Channel Model | Option 1: TDL-C 300 Option 2: AWGN Note: encourage companies to provide simulation results with both options |  |  |
| Performance metric | - 1% MDR/BLER - The following false alarm rate can be considered - 0.1% |  |  |

In LLS simulation, the ACS cases are simulated with ACI =0 and ACI =33 with different shifted RB and filter order. The purpose of such simulation is to identify the needed wanted signal increase to compensate the noise increase caused by adjacent carrier interferer. For OOK1 waveform, it is illustrated in Figure 3 that the potential SNR increase is around 6 dB with low order of filter and 0 shift RB. If the shift RB size is increased to 2, the SNR increase will be reduced less than 1 dB and this is illustrated in Figure 4. For OOK4 waveform, there is a need to add at least 2 shift RB for the case of filter order of 3 and ACI=33. The SNR increase is around 5 dB in this case. When shift RB size is increased to 4, the SNR increase is less than 1 dB.  For OOK4 and TDL-C, it can be observed that filter order of 5 is needed in Figure 10.

Increasing the ACI from 0 to 33 for OOK-1 scenario with filter order 3 and no shift RB will deteriorate the BLER performance maximally by approximately 6 dB.

The SNR increase can be reduced when increasing either filter order or shifted RB size.

The SNR increase is around 5 dB for OOK4 with filter order 3 and shift RB size of 2 comparing between the ACI =0 and ACI=33 dB case.

Filter order of 5 is needed for OOK4 for TDL-C channel and ACI =33 dB case.

Figure 5: The BLER for ACI=0 and ACI=33 with LPF order of 3 and shift RB number of 0 and OOK1

Figure 6:The BLER for ACI=0 and ACI=33 with LPF order of 3 and shift RB number of 2 and OOK1

Figure 7:The BLER for ACI=0 and ACI=33 with LPF order of 3 &5 and shift RB number of 0&2 and OOK4

Figure 8: The BLER for ACI=0 and ACI=33 with LPF order of 3 and shift RB number of 4 and OOK4

Figure 9:The BLER for ACI=0 and ACI=33 with LPF order of 3 and shift RB number of 0/2 and OOK1 for TDL-C

Figure 10: The BLER for ACI=0 and ACI=33 with LPF order of 3/5 and shift RB number of 0/4 and OOK4 for TDL-C

Figure 11: BLER performance for OOK1 and ACI=33 with ADC bit 8

Figure 12: BLER performance for OOK1 and ACI=33 with ADC bit 4

Figure 13: BLER performance for OOK4 and ACI=33 with ADC bit 4

Figure 14: BLER performance for OOK4 and ACI=33 with ADC bit 8

It is illustrated in Figure 11 to Figure 14 for comparison of 4 bits ADC and 8 bits ADC, it is observed that additional shifted RB will be needed for ACI=33 dB cases for LPF order of 3 for OOK1 and OOK4.

Additional shifted RB may be needed for OOK1 and OOK4 with a 4 bits ADC for LPF order of 3.

There are two test cases for ACS, case #1 and case #2.  For case #1, the ACS interferer is set relative to the wanted signal with the power difference of the selectivity value. From the LLS simulation, WUR can achieve the same selectivity with the MR. As such, the WUR ACS can be tested with the same selectivity with MR.  As the ACS test focus testing the filter attenuation and therefore, the WUR ACS interference should be set the same way as the MR ACS, meaning the ACS interferer level is set relative to the WUR REFSENS in addition to the relaxation to compensate the noise raise and the ACS.

WUR can achieve the same ACS as the MR.

WUR ACS of case #1 can be tested as the similar with MR ACS case #1.

For ACS test case #2, it can be the same with MR channel bandwidth of 5MHz. The difference is that there is no need to transmit the UL for FDD bands.  As the WUS signal can be placed at the near edge of the MR channel, therefore, it is possible to test it only for the minimum MR channel bandwidth where has the minimum guard band among all channel bandwidth supported by MR.

WUR ACS of case #2 can be tested with the same wanted and interference level

WUR ACS of case #2 can be test with only the minimum MR channel bandwidth.

Specify the same ACS selectivity for WUR with MR.

# Conclusions

In this contribution, we present our view on WUR RF requirement for REFSENS, ASCS and ACS with below proposal:

Observation 1 Agreed SNR target from RAN1 corresponding to the different NF

Observation 2 Network configure the WUS structure to make sure the certain BLER should be achievable with agreed SNR when NF of WUR is in certain range.

Proposal-1: Limit the NF of the WUR to 15 dB for low and mid-bands.

Proposal-2: RAN4 confirm if the maximum NF of the OFDM WUR can be assumed differently with OOK WUR.

Proposal-3: It is possible to specify unified RF requirement for both OOK WUR and OFDM WUR.

Observation 3 Larger guard RB size has penalty on the SNR threshold due to less received energy

Proposal-4: Maximum of 1 guard RB at each side of WUS signal should be used in ASCS case.

Observation 4 Increasing the ACI from 0 to 33 for OOK-1 scenario with filter order 3 and no shift RB will deteriorate the BLER performance maximally by approximately 6 dB.

Observation 5 The SNR increase can be reduced when increasing either filter order or shifted RB size.

Observation 6 The SNR increase is around 5 dB for OOK4 with filter order 3 and shift RB size of 2 comparing between the ACI =0 and ACI=33 dB case.

Observation 7 Filter order of 5 is needed for OOK4 for TDL-C channel and ACI =33 dB case.

Observation 8 Additional shifted RB may be needed for OOK1 and OOK4 with a 4 bits ADC for LPF order of 3.

Observation 9 WUR can achieve the same ACS as the MR.

Observation 10 WUR ACS of case #1 can be tested as the similar with MR ACS case #1.

Observation 11 WUR ACS of case #2 can be tested with the same wanted and interference level

Observation 12 WUR ACS of case #2 can be test with only the minimum MR channel bandwidth.

Proposal-5:Specify the same ACS selectivity for WUR with MR.

# References

- RP-240801, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), vivo etc

- R4-2410569, WF for [111][136] NR_LPWUS_UERF, vivo

- R1-2405374, LP-WUS and LP-SS design, Ericsson