# 3GPP TSG-RAN WG4 Meeting # 112						R4-2412980

# Maastricht, Netherland, 19th- 23rd August, 2024

Source:		Ericsson

Title:			General issue for NTN RedCap

Agenda item:		8.25.1

Document for:		Approval

# Introduction

In this paper, we discuss the release independent for RedCap feature to support NTN.

# Discussion

In WID[1], there is no 38.307 as one of the impacted specification so the release independent aspect for RedCap needs to be discussed.

In 38.307, the additional operating band for RedCap is added as release independent feature for TN. As the NTN feature is separate clause in 38.307, the additional operating band for RedCap for NTN should be added under the NTN feature. For example:

# Release independent features for NR UE supporting satellite access operation

## 9.1	Additional NR operating bands for NR NTN in frequency range 1

Requirements for NR UE supporting satellite access operation, for additional NTN operating bands of TS 38.101-5 in Rel-P [8] are introduced via this clause.

Table 9.1-1: Additional NR operating bands for NR NTN in FR1-NTN

| Feature | Duplex-mode | Release independent from | Requirements to be fulfilled (see TS 38.307 of the release in which the band was introduced) |
| --- | --- | --- | --- |
| NTN operating bands | FDD | Rel-17 | Table F.1.1-1, Table F.1.2-1, Table F.1.3-1 |

Table 9.1-2: NR NTN UE power class in FR1-NTN

| Feature | Duplex-mode | Release independent from | Requirements to be fulfilled (see TS 38.307 of the release in which the power class was introduced) |
| --- | --- | --- | --- |
| Power Class 3 | FDD | Rel-17 | Table F.1.1-1 |

Table 9.1-3: NR NTN UE channel bandwidth in FR1-NTN

| Feature | Duplex-mode | Release independent from | Requirements to be fulfilled (see TS 38.307 of the release in which the UE channel bandwidth was introduced) |
| --- | --- | --- | --- |
| UE channel bandwidth | FDD | Rel-17 | Table F.1.1-1 |

## 9.2 Additional operating bands for Redcap for NR frequency range 1

Requirements for a Redcap UE for additional operating bands within FR1 of TS 38.101-5 in Rel-P [2] are introduced via this clause.

Table 9.2 -1: NR NTN RedCap operating band

| Feature | Duplex-mode | Release independent from |  | Requirements to be fulfilled (see TS 38.307 of the release in which the band was introduced) |
| --- | --- | --- | --- | --- |
| Redcap operating band | FDD | Rel-17 |  |  |

Though it could be argued if the 38.307 should be updated when additional new band is added, this indeed cause confusion for RedCap release independent feature update in 38.307 in later release. We suggest RAN4 discuss this and see if the 38.307 should be added in a updated WID.

RAN4 discuss if the 38.307 should be updated for RedCap NTN as release independent feature (operating band).

# Conclusions

In this contribution, we present our view on release independent RedCap feature with below proposal:

Proposal-1: RAN4 discuss if the 38.307 should be updated for RedCap NTN as release independent feature (operating band).

# References

- RP-234078, New WID: Non-Terrestrial Networks (NTN) for NR Phase 3 , Huawei