# 3GPP TSG-RAN WG4 Meeting # 112						R4-2412981

# Maastricht, Netherland, 19th - 23rd August, 2024

Source:		Ericsson

Title:			Other NTN UE RF impact

Agenda item:		8.25.2.2

Document for:		Approval

# Introduction

In this paper, we present our general overview on NTN UE RF impact on uplink capability objective.

# Discussion

In revised WID [1],  the objective for the uplink capacity is updated as below:

- Uplink Capacity/Cell Throughput Enhancement for FR1-NTN [RAN1, RAN2, RAN4]

- Specify Orthogonal Cover Codes (OCC) for DFT-s-OFDM PUSCH at least for multiplexing 2 or 4 UEs when PUSCH repetitions are used

  - Specify necessary signalling

  - Update RF requirements accordingly, if needed

- Notes for this objective:

  - The enhancement is not targeting improvements/impacts of MU-MIMO capability

  - The enhancement is not targeted to PUSCH DMRS

  - No enhancement for initial access

  - Enhancements to PRACH are not in scope.

  - This feature may be applicable for UEs operating in terrestrial networks based on a common design

For UL coverage enhancement objective, the RF requirement is relating to the OCC, which is mainly on inter-lost time domain OCC with PUSCH repetition type A.  RAN1 did not reach consensus on the down-selection of OCC scheme and RAN1 will continue to specify at least one of the OCC scheme in agreement below.

Agreement​

- For the normative phase, at least one of the OCC techniques will be specified:​

- Inter-slot time-domain OCC with PUSCH repetition Type A with OCC length 2 or 4​

- Inter-symbol(s) time domain OCC with OCC length 2 or 4​

- Intra-symbol pre-DFT-s OCC (comb-like structure as in PUCCH format 4) with OCC length 2 or 4​

- FFS Combination of OCC techniques including multiplexing of 8 UEs​

- FFS Use of OCC techniques with TBoMS​

- FFS Backward compatibility with non-Rel-19 UEs​

For inter-slot time domain OCC, as illustrated in Figure 1, the different OCC code will be used for different UE for the same frequency and time resources. At BS receiver, to decode one UE data, the combined signal with other UEs will multiply this UE’s own OCC code, at the same time, the other UE data with such OCC code would be cancelled out as the OCC codes between UEs are orthogonal. When decoding the intended UE data, the CFO of this UE can be compensated but the CFO of other UE cannot be compensated due to the different frequency offset from UEs, which shows up as interference and cause the impairment on the OCC orthogonality and therefore SNR degradation could occur.

Figure 1:Schematic view of OCC across slots

RAN1 has agreed the below impairment model:

Agreement

Adopt the table below for assumptions for modelling impairments for link level evaluation in NR NTN UL capacity and throughput enhancements

| Parameter | Value |
| --- | --- |
| TO | Reported by companies - With TO: Uniform selection from [-0.94us, 0.94us], where 0.94us=29Ts - Optional without TO |
| FO | Reported by companies - Uniform selection from [-0.1 ppm, +0.1 ppm], Variation of frequency error is negligible. - Optional: with lower maximum residual FO, to be reported by companies |
| Timing drift | Optional |
| Receiver algorithm | To be reported by companies, e.g. - MMSE |
| Channel estimation | - Real channel estimation |

From the above impairment model, it can be observed that the frequency offset is only relevant to the UE RF discussion. In [2], the FO impact on the LLS performance is observed and it is illustrated in Figure 2. Big CFO has negative impact on OCC performance and one of the mitigations could be the pairing the UE with selective FO. This also implies that UE should maintain a fixed CFO during the OCC transmission otherwise there is no guarantee on the OCC performance.

The CFO impairment has impact on the OCC performance in inter-slot OCC scheme.

Inter-slot OCC scheme is relevant to RAN4 RF discussion.

UE should maintain the same CFO during the OCC transmission so the OCC performance gain could be achieved.

(a) OCC across slots without impairments.

(b) OCC across slots with TO and FO impairments.

(c) FO spread impact on 4 UE OCC across slots

Figure 2: Link performance of VoIP on PUSCH with OCC across slots.

As it is not clear for now whether the inter-slot OCC scheme will be specified in RAN1 and also there is no common understanding in RAN1 on CFO impact. We think RAN4 could wait further progress from RAN1. RAN4 could also discuss whether the additional simulation campaign should be initiated to evaluate the CFO impact on the inter-slot OCC scheme when RAN1 decides to specify it.

Wait RAN1 reach conclusions on OCC feature before RAN4 start to evaluate the RF impact.

# Conclusions

In this contribution, we present our general overview for NTN UE RF impact for UL capacity objective with below proposal:

Observation 1 The CFO impairment has impact on the OCC performance in inter-slot OCC scheme.

Observation 2 Inter-slot OCC scheme is relevant to RAN4 RF discussion.

Observation 3 UE should maintain the same CFO during the OCC transmission so the OCC performance gain could be achieved.

Proposal-1:Wait RAN1 reach conclusions on OCC feature before RAN4 start to evaluate the RF impact.

# References

- RP-241667,	Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, THALES

- R1-2404418, Ericsson, On uplink capacity/throughput enhancement for NR NTN, RAN1#117, Fukuoka City, Fukuoka, Japan, May 20th – 24th, 2024