# 3GPP TSG-RAN WG4 Meeting # 112						R4-2412983

# Maastricht, Netherland, 19th - 23rd  August, 2024

Source:		Ericsson

Title:			RedCap UE RF impact on HD-FDD

Agenda item:		8.25.2.1

Document for:		Approval

# Introduction

In this paper, we present our general view on the RedCap UE RF remaining issue.

# Discussion

In WF[3], the remaining issue is below:

## Issue 1-2: HD-FDD refsens for 2 Rx

Agreement: [0.5] dB tightening for n254 and n256, FFS for n255. n256 to be confirmed in RAN4#112.

Note: RAN4#111 maintenance agreement on n256 refsens in R4-2408787 was not considered for this agreement.

For 0.5 dB tightening factor for n256, this is inherited from the RedCap in TN for HD-FDD operation and it is based on the NTN band generic requirement. R4-24008787 is agreed to change the general requirements of REFSENS in FD-FDD mode. Therefore, there is no impact on the 0.5 dB tightening for n256 and n254 for NTN Redcap UE. The tightening factor is the same as Redcap UE in TN.

0.5 dB tightening for n256 for NTN RedCap operating at HD-FDD mode based on the generic requirement of the REFSENS of the n256 operating at FD-FDD mode.

For n256, it can be discussed whether the same factor should be reused from TN. As Rel-17 RedCap UE operating at HD-FDD mode , a general scaling factor of 0.8 dB tightening with exception for some bands[2]. The exception is mainly because of comparison with the tightest TDD band REFSENS and set no tightening if REFSENS lower than -100 dBm. The agreement is quoted below[2].

[Agreement:]

Agree on the framework of Option 1a

-	Option 1a: per band exception and selected band for different scaling factor as below: [Apple]

The HD-FDD 5MHz REFSENS tightening from FD-FDD is proposed as in the table below.

| FD-FDD 5MHz REFSENS | HD-FDD REFSENS Tightening | Bands |
| --- | --- | --- |
| ≤ -100 dBm | 0 dB | n1, n18, n24, n70 |
| > -100 dBm and ≤ -99 dBm | 0.5 dB | n30, n65, n66, n74 |
| > -99 dBm | 0.8 dB | n2, n3, n5, n7, n8, n12, n13, n14, n20, n25, n26, n28, n71, n85 |

-	HD-FDD REFSENS for channel BW wider than 5 MHz can be calculated by REFSENS(5MHz) + 10log10(n x NRB/25), where NRB is the maximum transmission bandwidth configuration with n=1 for 15kHz SCS and n=2 for 30kHz SCS.

Such agreement could apply for the NTN band also, with only concern that the NTN RedCap UE operating at low level due to the large distance to the satellite. Therefore, it may be good to at least be tightened with 0.5 dB. The REFSENS of NTN band of 2 RX is quoted below.

Table 7.3.2-1: Two antenna port reference sensitivity QPSK PREFSENS for FDD bands

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
|  | 15 | -99.5 | -96.3 | -94.5 | -93.8 |  |  |  |  |  |  |
| n256 | 30 |  | -96.6 | -94.6 | -94.0 |  |  |  |  |  |  |
|  | 60 |  | -97.0 | -94.9 | -94.2 |  |  |  |  |  |  |
|  | 15 | -100.0 | -96.8 | -95.0 | -93.8 |  |  |  |  |  |  |
| n255 | 30 |  | -97.1 | -95.1 | -94.0 |  |  |  |  |  |  |
|  | 60 |  | -97.5 | -95.4 | -94.2 |  |  |  |  |  |  |
|  | 15 | -99.5 | -96.3 | -94.5 |  |  |  |  |  |  |  |
| n254 | 30 |  | -96.6 | -94.6 |  |  |  |  |  |  |  |
|  | 60 |  | -97.0 | -94.9 |  |  |  |  |  |  |  |
| NOTE：The transmitter shall be set to PUMAX as defined in clause 6.2.4 of 3GPP TS 38.101-1 [5]. |  |  |  |  |  |  |  |  |  |  |  |

Consider the 0.5 dB tightening for the n255.

For the suffix of the new NTN Redcap UE, we think using the same suffix with suffix in TS 38.101-1 is appropriate. As many NTN UE requirement refer to the same clause of the TN specification, the NTN UE specification structure arranged in this way has better readability and traceability.

Use the same suffix with suffix for RedCap in TS 38.101-1.

# Conclusions

In this contribution, we present our view on the remaining issue for  Redcap UE RF:

Proposal-1: 0.5 dB tightening for n256 for NTN RedCap operating at HD-FDD mode based on the generic requirement of the REFSENS of the n256 operating at FD-FDD mode.

Proposal-2:Consider the 0.5 dB tightening for the n255.

Proposal-3:Use the same suffix with suffix for RedCap in TS 38.101-1.

# References

- RP-234078, New WID: Non-Terrestrial Networks (NTN) for NR Phase 3 , Huawei

- MeetingReport__2022-01-29_06h03_UTC_R2.zip

- R4-2410574, WF on Rel-19 phase 3 NR-NTN UE RF requirements, Qualcomm Inc