# 3GPP TSG-RAN WG4 Meeting # 112						R4-2412984

# Maastricht, Netherlands, 19th  – 23rd August, 2024

Source:		Ericsson

Title:			PC2 RedCap RF overview

Agenda item:		8.7.2

Document for:		Approval

# Introduction

In this paper, we present our view on PC2 RedCap UE RF.

# Discussion

.In [1], the scope of the RedCap PC2 is quoted as below:

The objectives of the core part are as follows:

- Apply the PC2 RF requirements for non-RedCap UE to RedCap UE, based on 1Tx architecture with 26 dBm PA.

  - The PC2 RF requirements include: UE maximum output power, Tx power tolerance, MPR, A-MPR and RSD requirements if needed.

  - For TDD bands, PC2 RF requirements for non-RedCap UE can be re-used for PC2 RedCap UE in a band agnostic way.

    - Note: Not differentiate the applicable form factors for TDD bands in the specification.

  - Check at RAN #106 on whether to define PC2 RF requirements for FDD bands and the applicable form factors for FDD bands (if defined).

Note 1: Only the operating bands for which PC2 on non-RedCap UE has been introduced can be considered for RedCap UE.

Note 2: PC2 for Rel-17 Redcap and Rel-18 eRedcap are considered for this WI.

The main RF impact when introducing the ReCap is the REFSENS clause, where 1 Rx and HD-FDD is introduced in RedCap compared to NR UE. For TDD band of PC2 NR UE, there is no additional RSD so the REFSENS of the PC2 RedCap UE could be kept the same as legacy.

No impact on Redcap REFSENS in Rel-17 and Rel-18 for PC2 RedCap for TDD band.

For Tx requirement, as objective indicates that UE maxim output power, Tx power tolerance. MPR and A-MPR needs to discuss. As 1 Tx architecture with 26 dBm PA is assumed, the NR non-RedCap UE MPR and A-MPR can be reused for the PC2 RedCap UE. This is also agreed in WF [2].

NR PC2 UE MPR and A-MPR can be reused for PC2 RedCap.

For FDD bands, there is additional relaxation factor for REFSENS in NR UE. The RSD for 1 Tx PC2 could be considered as a starting point and further checking if the relaxation factor is enough for small factor RedCap UE.

For FDD band, RSD for 1Tx PC2 could be considered as starting point.

According to WID objective, the FDD band will be checked at RAN#106 and therefore, RAN4 can focus on TDD band first.

RF impact for FDD band can be discussed before RAN#106.

# Conclusions

In this contribution, we present our overview on the PC2 RedCap RF overview with below proposal:

Observation 1 No impact on Redcap REFSENS in Rel-17 and Rel-18 for PC2 RedCap for TDD band.

Observation 2 NR PC2 UE MPR and A-MPR can be reused for PC2 RedCap.

Observation 3 For FDD band, RSD for 1Tx PC2 could be considered as starting point.

Observation 4 RF impact for FDD band can be discussed before RAN#106.

# References

- RP-241665, New WID on NR power class 2 RedCap (Reduced Capability) UE in FR1, China Telecom, MediaTek

- R4-2314748, WF on NR power class 2 RedCap UE in FR1