# 3GPP TSG-RAN WG4 Meeting # 112						R4-2412992

# Maastricht, Netherland, 19th- 23rd August, 2024

Source:		Ericsson

Title:			IoT NTN UE RF impact

Agenda item:		8.26.2

Document for:		Approval

# Introduction

In this paper, we present our IoT NTN UE RF impact on uplink capability objective.

# Discussion

In WID [1],  the objective for the uplink capacity is as below:

- Support of Capacity enhancements for uplink

  - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2, RAN4]

    - Multi-tone support for 15 kHz SCS should also be considered

    - Specify necessary signalling, if needed

    - Update RF requirements accordingly, if needed

Note: Impact of impairment shall be taken into account

For UL coverage enhancement objective, the RF requirement (if needed) is related to the OCC, which for NPUSCH format 1 has under discussion whether to support either a symbol-level OCC or a slot-level OCC.

During RAN1# 117 RAN1 reached the following agreement:

Agreement

- For 3.75kHz single-tone OCC for NPUSCH format 1, RAN1 supports either symbol-level OCC or slot-level OCC. Other OCC schemes are not pursued.

- For 15kHz single-tone OCC for NPUSCH format 1, RAN1 supports either symbol-level OCC or slot-level OCC. Other OCC schemes are not pursued.

RAN1 did not reach consensus on the down-selection of OCC scheme and RAN4 can wait the progress of RAN1 on this.

RAN1 will down select symbol-level OCC or slot-level OCC

For inter-slot time domain OCC the different OCC code will be used for different UE for the same frequency and time resources. At BS receiver, to decode one UE data, the combined signal with other UEs will multiply this UE’s own OCC code, at the same time, the other UE data with such OCC code would be cancelled out as the OCC codes between UEs are orthogonal. When decoding the intended UE data, the CFO of this UE can be compensated but the CFO of other UE cannot be compensated due to the different frequency offset from UEs, which shows up as interference and cause the impairment on the OCC orthogonality and therefore SNR degradation could occur.

RAN1 has agreed the below impairment model:

Agreement

The following evaluation assumptions are used for the study of OCC for NPUSCH format 1:

|  | Parameter | value |  |
| --- | --- | --- | --- |
| scenario | orbit | GEO | LEO600 |
|  | Elevation angle | 12.5 degree | 30degree |
| Channel and impairments | carrier frequency | 2GHz |  |
|  | Channel model | NTN-TDL-C The channels from different UE are independent. |  |
|  | Frequency error | Uniform random selection from [-0.1 ppm, +0.1 ppm] for all UEs Variation of frequency error is negligible. |  |
|  | Timing error | Uniform random selection from [-97Ts, +97Ts] for all UEs Timing drift 80us/s for LEO600 and 0 for GEO. |  |
|  | Power imbalance | Uniformly distributed between +Pimb and -Pimb for all UEs Proponent to report the value of Pimb (can be zero) and justification for the chosen value |  |
| transmitter | SCS | 3.75KHz and 15KHz | 15kHz |
|  | Number of tones | Single tone | Single tone and multi tone up to 12 tones |
|  | Waveform | DFT-s-OFDM |  |
|  | Frequency hopping | w/o frequency hopping |  |
|  | MIMO scheme | SISO |  |
|  | DMRS configuration | For baseline evaluations: OS#3 per slot for 3.75kHz OS#4 per slot for 15kHz For OCC evaluations: Up to proponent | For baseline evaluations: OS#4 per slot for 15kHz For OCC evaluations: Up to proponent |
|  | Number of resource unit () | Up to proponent | Up to proponent |
|  | Modulation order | Up to proponent | Up to proponent |
|  | TBS () | Up to proponent | Up to proponent |
|  | Number of repetitions () | Up to proponent |  |
|  | OCC length | Up to 4 |  |
|  | OCC sequence | Up to proponent |  |
|  | Number of UE | Up to 4 |  |
|  | Velocity of UE | 3km/h |  |
| receiver | Receiver algorithm | MMSE |  |
|  | Channel estimation | Real channel estimation |  |
| KPI | SNR at 10% BLER | Report for baseline and OCC schemes |  |
|  | Aggregated throughput | Total throughput of up to 4 UEs multiplexed |  |

Above all, RAN1 also agrees the additional frequency error assumption:

Agreement

For the NPUSCH evaluation assumptions, update the frequency error assumption, as follows.

| Frequency error | Uniform random selection from [-0.1 ppm, +0.1 ppm] for all UEs Variation of frequency error is negligible. For GEO, the same frequency error is applied to each subframe of a transport block. For LEO, the same frequency error is applied to each subframe of a segment (if applied in the evaluation). Companies to report their assumption on frequency error across segments. |
| --- | --- |

From the above impairment model, it can be observed that the frequency offset is only relevant to the UE RF discussion. In [2], the LLS performance is discussed but no agreed observations is reached.

RAN1 still discuss on the OCC performance.

As it is not clear for now whether a symbol-level OCC or a slot-level OCC will be specified in RAN1 and also there is no common understanding in RAN1 on CFO impact,  we think RAN4 could wait further progress from RAN1.

Wait RAN1 reach conclusions on OCC feature before RAN4 start to evaluate the RF impact.

# Conclusions

In this contribution, we present our general overview for IoT NTN UE RF impact for UL capacity objective with below proposal:

Observation 1 RAN1 will down select symbol-level OCC or slot-level OCC

Observation 2 RAN1 still discuss on the OCC performance.

Proposal-1: Wait RAN1 reach conclusions on OCC feature before RAN4 start to evaluate the RF impact.

# References

- RP-240776,	Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, MediaTek Inc

- R1-2405494, FL Summary #1 for IoT-NTN, Sony