3GPP TSG-RAN WG4 Meeting #112	R4-2412993

Maastricht, Netherlands, 19th – 23rd August 2024

Agenda Item:	8.9.2 - Coexistence study based on ITU regulations

Source:	Eutelsat Group

Title:	Regulatory status of NTN in bands above 10 GHz post WRC-23

Document for:	Information

# Introduction

This is an update of [1] submitted to RAN4 #110 but not treated.

The present document provides the regulatory status of bands above 10 GHz applicable to NTN, incorporating outcomes of the WRC-23 ITU conference in December 2023. WRC-23 added mobile VSAT to the Ka band (17.7 - 30.0 GHz), and re-enforced the global consensus for continued priority for satellite operation in the Ku band (10.7 -14.5 GHz).

# Ka band 17.7 – 30.0 GHz

The now completed Rel-18 specifications for NTN in bands above 10 GHz cover deployment requirements and use cases for the Ka band within the scope of the ITU-R following WRC-19. The NTN WI did not include connectivity to what the ITU-R terms as non-GSO ESIM (Earth Station In Motion) – which 3GPP refer to as connectivity between a mobile VSAT and NGSO satellite (e.g. LEO, MEO).

Non-GSO ESIM in Ka was reviewed at WRC-23 under agenda item 1.16 and resolution COM5/3 (WRC-23) updates use of the frequency bands 17.7-18.6 GHz, 18.8-19.3 GHz, 19.7-20.2 GHz (space-to-Earth) and 27.5-29.1 GHz and 29.5-30 GHz (Earth-to-space). The resolution adds aeronautical and maritime ESIMs communicating with non-geostationary space stations in the fixed-satellite service [3].

In [4] we proposed that the Rel-18 standards gap of Mobile VSAT to NGSO should be addressed in the Rel-19 RAN4 non spectrum package now that the regulatory blocker has been removed. This proposal will be re-introduced in Dec at RAN #106.

# Ku Band 10.7 – 14.5 GHz

Eutelsat Group operates fixed and mobile satellite services in Ku band and is planning to launch a new constellation to provide 5G NR NTN services in 2027 to operate commercial services in 2028. Following the release 18 work on NTN in bands above 10 GHz which used Ka as the example band, Eutelsat Group will be proposing to add support for the Ku band within release 19. This activity was successfully started at RAN #104 in [5]. As with Ka, consensus from WRC-23 removes any remaining regulatory uncertainties about the use of Ku.

Figure 1 shows the frequency ranges of the Ku band downlink and uplinks, as well as the adjacent space science service below 10.7 GHz and the proposed future IMT band above 14.8 GHz.

Figure 1 Frequency Range of Ku band and adjacent services

## Note 1: Radio Astronomy, Earth Exploration Satellite (passive) Services and passive band below 10.7 GHz. The 10.6-10.7 GHz band involves two allocations. The band contains a primary allocation to RAS (Radio Astronomy Service) and (passive) EESS (Earth Exploration Space Service) and the 10.68-10.7 GHz band is also a passive band (No. 5.340).

## Note 2: Ku downlink extends to 12.75 GHz in Regions 1 & 3. The upper end of the Ku downlink range is 12.70 GHz in Region 2 and 12.75 GHz in regions 1 & 3.

## Note 3: Proposals to consider IMT operation in the range 12.75-13.25 GHz were rejected at WRC-23 confirming the primary allocation for satellite services. In region 2, earth-space is permitted between 12.70-12.75 GHz, so could extend uplink1 from 12.70-13.25 GHz.

## Note 4: Small antennas for Ku uplink 13.75-14.0 GHz. For 13.75-14.0 GHz, ITU has minimum terminal antenna size requirements of 1.2m for GSO and 4.5m for NGSO. 14.0-14.5 GHz only has restrictions for earth stations on vessels of 1.2m with flexibility for smaller antennas where 1.2m requirements can be met. The 4.5m requirement for 13.75 - 14.0 GHz is on the WRC-27 agenda and may consider a smaller terminal size.

## Note 5: Future IMT study band 14.8-15.35 GHz. The set of future IMT study bands on the agenda for WRC-27 are as follows. All proposals to consider IMT within the Ku band frequency range were rejected at WRC-23.

## GEO Arc protection applies across all FR2-NTN bands with directive VSAT UEs

Where RR Article 22.2 applies, GSO can claim protection from NGSO. In practice this means neither NGSO SAN nor UE can transmit to each other when aligned with a GSO arc.

# Ku band definition

Due to the split uplink, antenna size restrictions and variations between regions on the 12.7 GHz or 12.75 GHz boundary the mapping of 3GPP bands onto the Ku frequency range is not straightforward so will be confirmed as part of the new WI in [5].

# References

[1] R4-240516 Regulatory status of NTN in bands above 10 GHz post WRC-23 (Eutelsat Group, RAN4#110, Athens)

[2] RP-232732 3GPP NTN Based Satellite Networks - Deployment Considerations (GSOA, RAN#102, Edinburgh)

[3] World Radiocommunication Conference 2023 (WRC-23) Provisional Final Acts (ITU WRC, Dubai 2023)

[4] R4-2400511 Motivation to support mobile VSAT in NGSO deployment scenarios (Eutelsat Group, RAN4#110, Athens)

[5] RP-241690 New WID on Introduction of Ku Band for NR NTN (Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat RAN #104 Shanghai)