3GPP TSG-RAN WG4 Meeting #112	R4-2412994

Maastricht, Netherlands, Aug 19 – Aug 23, 2024

Source:	Nokia

Title:	Testability and interoperability issues for beam management

Agenda item:	8.17.2

Document for:	Discussion

Introduction

The discussions on AI/ML enabled BM use case were continued in RAN4#111 in the scope of Rel-19 WI based on the outcomes of the Rel-18 SI. Agreements achieved during this meeting are summarized in the WF [1] and the ad hoc minutes [2].

Some of the issues related to AI/ML enabled BM-Case1 and BM-Case2 require further discussion, as follows:

- Ground truth reporting and measurement accuracy

- Indicated/activated TCI states requirements

- LCM related requirements

- Generalization and test coverage related aspects

- Testing setup

In this paper, we provide some additional views on the topics listed above.

Discussion

Ground truth reporting and measurement

One of the challenges in defining new performance requirements on beam prediction is availability of the ground truth, i.e., a reference, to compare the predicted value and/or the best beam(s) based on these values with.

At RAN4#111, the following agreement was achieved for the ground truth extraction for testing of AI/ML BM use case:

Agreement: - The ground truth for the predicted RSRP is the ideal measurement of RSRP on the predicted Tx beam - In RAN4, the ground truth is the approximate as the reported RSRP measurement under the certain SNR on the predicted Tx beam - FFS on SNR level to ensure that SNR is high enough for sufficient accuracy of reported RSRP - FFS on impact of multiple-AoA test setup - FFS on the channel condition - FFS on whether the ground truth will be changing or not - Other solutions are not precluded

In legacy, the requirements on measurement accuracy are defined in TS 38.133, Clause 10:

- 10.1.20 L1-RSRP accuracy requirements for FR2

  - 10.1.20.1 SSB based L1-RSRP accuracy requirements

  - 10.1.20.2 CSI-RS based L1-RSRP accuracy requirements

Usually, measurement accuracy requirements should be followed during the measurement tests, also defined in RAN4, e.g.,

- A.17.6.3.1 SSB based L1-RSRP measurement when DRX is not used

- A.17.6.3.3 CSI-RS based L1-RSRP measurement when DRX is not used

The current approach is based on comparison of the measurement value reported by the UE with the known/reference/ground truth value that is defined by the testing setup parameters.

More insightful details on test tolerance topics for radiated test cases can be found in the TR 38.903 [3], Clause A.2.6 UE Reported RSRP and UE gain:

An example is provided here for a scenario where the UE reports SS-RSRP for a signal arriving from a direction within the UE spherical coverage, to illustrate variation from both UE spherical coverage and variation from UE gain “G”. UE-measured SS-RSRPnom = Applied SSB_RP + UE Spherical coverage gain midpoint + UE gain G midpoint where: Applied SSB_RP is specified in the test case, either directly as Es or derived from Noc and Es/Noc, and is in dBm per subcarrier UE Spherical coverage gain midpoint in dB is derived as (UE Refsens - UE Spherical coverage)/2 UE gain G midpoint in dB is derived as (Min value of G + Max value of G)/2 As an example for a UE power class 3 in band n257, measuring SS_RSRP from a spherical coverage direction, UE-measured SS-RSRPnom = Applied SSB_RP -5.45dB +5.0dB. Figure A.2.1.2-1 shows the derivation of UE Spherical coverage gain midpoint. Variation about the midpoint is handled as a UE uncertainty. For signals arriving from Rx Beam Peak direction, this gain is 0dB and does not vary. Figure A.2.6.2-1 shows the derivation of UE gain G midpoint. Variation about the midpoint is handled as a UE uncertainty. Figure A.2.6.2-1: Example modelling of UE Gain “G” variation To calculate the range of valid SS-RSRP values that can be reported by the UE, contributions from Spherical coverage gain variation, UE gain variation and UE reporting accuracy are considered: Figure A.2.6.2-2: modelling of contributions affecting SS-RSRP reported values Reported SS-RSRP = UE measured SS-RSRPnom ±Spherical coverage gain variation ±UE gain variation ±UE accuracy where: UE measured SS-RSRPnom is the nominal value derived from Applied SSB_RP, UE Spherical coverage gain midpoint and UE gain G midpoint Spherical coverage gain variation is derived from Refsens and Spherical coverage, as shown in Figure A.2.1.2-1 UE gain variation is derived from Minimum and maximum values of G, as shown in Figure A.2.6.2-1 UE accuracy is the absolute accuracy from the core requirement referred to in A.2.6.1 As an example for a UE power class 3 in band n257, measuring SS_RSRP from a spherical coverage direction with applied Io > -70dBm, the variation would be (±5.45dB ±15dB ±8dB) = ±28.45dB These variations are added arithmetically in the test case analysis, as each could be systematic and not random. For signals arriving from Rx Beam Peak direction, spherical coverage gain variation is 0dB.

As it can be seen from the example above for a UE power class 3 in band n257, the range of valid RSRP values is huge (on the level of several 10s of dB) due to test tolerances and uncertainties. Hence, the requirements that can be defined based on legacy principles, i.e., based on the comparison to the specified value in the test setup, are not very meaningful for testing the performance/accuracy of the predictions.

Existing principles of testing UE-reported RSRP accuracy based on setup parameters and measurement uncertainties allow for very wide range of valid RSRP values. This makes testing for verification of AI/ML-enabled RSRP/best beam prediction a very futile exercise.

The main sources of uncertainty are the unknown device implementation in respect of beam patterns (beam gains and spherical coverage), orientation of the device the testing setup, elements of the testing setup, etc. Hence, we need to find a way to minimize or exclude completely the impact of such uncertainties on the requirement/test:

- One of the directions is to consider UE-related uncertainties as a part of the ground truth. A corresponding option was discussed at the previous meeting and is based on using UE-reported RSRP measurement values.

- Another possible direction that we also discuss below can be based on excluding measurement uncertainties completely, i.e., using the relative difference in prediction to verify the AI/ML-enabled functionality.

In the last meeting, there was a proposal to use reported measurements as ground truth under certain SINR on the predicted Tx beam. The idea was to use higher SINR in order to achieve better L1-RSRP measurement accuracy. However, as shown in below simulation results, higher SINR (such as changing side conditions from -3 to 0 dB), doesn’t provide any significant improvements in L1-RSRP measurement accuracy.

Figure 1: L1-RSRP measurements error at 95th percentile for different SINR side conditions

No significant improvement in L1-RSRP measurement accuracy is observed while changing SINR side condition from -3 to 0 dB.

Due to above observation, one needs to be careful in directly using error-prone L1-RSRP measurements reports as ground truth for AI/ML BM requirements and test cases.

Furthermore, as shown in Figure. 2, if legacy tolerance margin is used for L1-RSRP measurement accuracy while using measured RSRP as ground truth, the performance of AI/ML BM may be worse than legacy for L1-RSRP measurement accuracy metric. Therefore, it is important to be very cautious in using error-prone L1-RSRP measurement reports as ground truth. If no other solution, than using measurement reports as ground truth, is available then RAN4 should consider tightening tolerance margin for L1-RSRP measurement accuracy in case of AI/ML BM use case.

Figure 2: L1-RSRP measurement accuracy – legacy vs AI/ML BM

If the requirement on predicted RSRP accuracy is based on the error-prone measured RSRP, the resulting absolute error of the prediction can be higher than in legacy requirements.

If reported RSRP measurements are used as an approximate of the ground truth, a tighter than legacy measurement accuracy should be considered for the definition of requirements for AI/ML BM.

There are also other concerns related to the approach when the prediction accuracy is defined relative to the measurements performed by the UE:

- What is the mechanism to report the measurements and prediction?

  - Will UE be able to make measurement together with the prediction? Otherwise, it will be a time gap in between the measurement and prediction causing additional uncertainty among measured and predicted value.

- How to ensure that measurements that are used as an approximation of the ground truth and are known only to the UE are not used to enhance the predictions?

If both measurements and predictions are reported by the UE, there is no objective/external criteria to verify the independence of these reports.

The requirement based on RSRP measurements used as an approximate of the ground truth should not be the only ones specified for BM use cases.

Another alternative to define the requirements is not to use UE-based measurement as a reference, but to verify the expected behaviour of AI/ML-enabled functionality based on the change in the configurations/inputs of the functionality. For example, the best predicted beam should not change, if the power of all measured beams is scaled in the same way or when one of the measured beams cannot be received anymore, etc. In other words, the AI/ML functionality can be tested without explicit definition of the ground truth.

The behaviour of beam prediction functionalities can be tested without explicit definition of the ground truth by comparing the predictions to each other.

RAN4 to introduce additional requirements based on relative difference in between the predictions due to change in the conditions/inputs, i.e., without dependency on the measurement accuracy errors and uncertainties.

Indicated/activated TCI states requirements

The core requirements related to beam indication (TCI indication/activation) is discussed in this Section. In Rel-19 AI/ML BM, the measurements of the Set B of beams are used as input to the ML model, which outputs the best K beams (beam indices and/or L1-RSRP values) from Set A beams. For a UE-sided model, Set A and B can be provided with the CSI reporting configurations. The NW can determine whether any changes are required in the set of active TCI states, and if yes, the NW can adapt the set of active TCI states based on the prediction report received from the UE. Here, the NW needs to ensure that the RS resource corresponding to the activated/indicated beams can be measured by the UE prior any beam indication takes place corresponding to the predicted RS resource.

Accordingly, the UE may not have measured the RS resource included in the indicated/activated TCI state since the beam prediction may be based on a subset of beam measurements (Set B) to predict beam(s) from Set A. In this case, if the predicted beam(s) is not measured by the UE for longer period, the predicted beam(s) will be corresponding to unknown TCI state. The delay for switching the beam from known TCI states to unknown TCI states and vice versa is higher. Therefore, in our view, for a UE that supports beam prediction, there is a need to relax the time period defined in Clause 8.10.2 of TS 38.133 for the following part:

- “TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement”.

If this condition is relaxed, then the predicated beam (which may not be part of Set B beams) corresponding to a Set A/subset of set A, may be measured with a longer delay than 1280 ms and still fulfil TCI known state condition. In other words, a higher value of the delay requirements should be used for known TCI state conditions in Clause 8.10.2 of TS 38.133 for predicted beams.

When the TCI state is known, it is assumed that UE is aware of corresponding beam/RS, it has measured it recently, UE can switch to known TCI state faster than to an unknown TCI state.

RAN4 should consider relaxing known conditions for TCI states (Clause 8.10.2 of TS 38.133) for BM-Case1 and BM-Case2

LCM related requirements

Following the agreement from RAN4#111, general aspects agenda, the agreement related LCM procedures are as follow:

Agreement:

RAN 4 to consider defining RRM requirements for LCM procedures based on the specific use cases (beam management and positioning) and candidate are:

- Identification

- Selection

- Activation

- Deactivation

- Switching

- Fallback to non-AI operation

- Performance monitoring

- Others are not precluded

The core requirements related LCM procedure is further discussed in this section.

RAN2 is currently discussing LCM for the UE-side model for beam management.  One aspect is related to the activation/deactivation of the functionality after inference configuration. We recall that an active functionality can refer to functionalities that the UE starts to predict beam results via model inference.

For the beam management use case, the activation is when the UE receives the CSI report configuration that enables AI/ML model usage and the corresponding UE-sided operations such as measurements of the RS, use of the AI/ML model to obtain the predictions and report of the prediction are performed.

The delay considered for these operations necessary to activate the functionality depends on the use case (spatial beam prediction or temporal beam prediction) and should be carefully analysed.

Once a functionality is activated, it is reasonable to assume that performance monitoring will be in place. For NW-sided performance monitoring for beam prediction at UE, the NW will need to configure an RS resource set to consider as monitoring RS resources, where the UE can measure these monitoring RS resources and report back to NW. Based on the reported measurements regarding the monitoring RS resources and inference-related reports, the NW will calculate performance metrics or KPIs, e.g., beam prediction accuracy, L1-RSRP difference, etc. Then, the calculated performance metric(s) or KPI(s) will be used to evaluate the operability of the CSI report associated with beam prediction.

If performance monitoring detects a performance degradation to a point where a decision to either switch this functionality with another functionality is taken or a fallback to a legacy/default algorithm, it means that the AI/ML functionality is degrading the system performance and if this functionality, with detected performance degradation, keeps running then the impact on performance may be catastrophic.

Therefore, it is crucial to stop this functionality, either by falling back to legacy method or by switching to another functionality, within a specified time. The specified time allowed to switch/disable the functionality should guarantee that the system performance remains within the acceptable levels.

If an LCM action is required and it is not taken in a timely manner, the performance for AI/ML enabled BM use case may be degraded to undesirable level.

Core requirements should be considered to limit latency of LCM actions typical for AI/ML enabled BM-Case1 and BM-Case2.

In RAN4#111 meeting, various candidates were considered for defining RRM requirements for LCM procedures based on the specific use cases (beam management and positioning). Since RRM requirements may not be necessary for all listed candidates, we consider that following candidates should be prioritized, which in our view have potential impacts on RRM requirements:

- Activation

- Deactivation

- Switching

- Fallback to non-AI operation

- Performance monitoring

Activation, Deactivation, Switching, Fallback to non-AI operation and Performance monitoring should be prioritized for defining RRM requirements for LCM AI/ML BM use case.

Generalization and test coverage related aspects

Agreement:

- Vendors may take into account the test environment/conditions defined by RAN4 when training the UE

  - sufficient test environment/conditions should be defined to enable vendors to create the data needed for training.

- FFS on proposals to augment training data to avoid overfitting of UE models to the test environment

- RAN4 to strive to make the test conditions similar to field deployment conditions

Following agreement was achieved in RAN4#111 meeting:

In the previous meeting, the concern, regarding overfitting of UE models to the test environment, was raised by some companies and it was captured in the agreement and companies were encouraged to bring proposals to avoid overfitting issue.  In our view, testing generalization aspects of UE models/functionalities provide a rather realistic and feasible solution to the overfitting issue. Since, testing a UE model/functionality in different scenarios and configuration would ensure that the model/functionality is/are not overfitted to a certain test setup/environment. A UE model/functionality, if tested in different generalization scenarios, would not be overfitted to a certain test environment.

A UE model/functionality, if tested in different generalization scenarios/configuration, would be considered not overfitted to a certain test environment.

Testing of generalization aspects should also be considered to avoid overfitting of UE models to the test environment.

Furthermore, testing of generalization aspect is one of the main challenges for RAN4 testing. RAN4 may define multiple tests with different conditions. In each of the test, TE configures the same specified UE configuration for AI/ML BM, therefore the same UE configuration is tested under different scenarios to verify the UE’s model capability to generalize. (environment for AI/ML based BM-Case1 and BM-Case2 differs in each test but not changing dynamically during the test)

For identified scenarios in AI/ML enabled BM-Case1 and BM-Case2, the minimum level of performance may also need to be defined. Similarly acceptable tolerance margin for other scenarios may also need to be agreed, potentially based on simulation results.

In the tables below, we attempt to capture different parameters that may be needed to define different generalization and scalability scenarios for AI/ML enabled BM use case.

- Generalization Scenarios – Known set of parameters/configurations to both DUT / TE.

Parameters that should be considered for the definition of different generalization/scalability scenarios:

Table 3: Parameters for Generalization Scenarios

| Parameters | Description |
| --- | --- |
| Propagation Model | AWGN/CDL/Uma/UMi |
| SINR | Good / Bad Radio conditions |
| UE Speed | Slow / Medium / Fast |
| Channel propagation conditions | LOS/NLOS |

- Scalability Scenarios only – Network may or may not know the change of the parameters/configurations.

Table 4: Parameters for Scalability Scenarios

| Parameters | Description |
| --- | --- |
| gNB antenna configurations | gNB antenna array 2x4/4x8/8x16 |
| Variable number of Set B beams | Set B 16/32/64 beams |
| UE Rx beams | UE Rx beams 4/8 beams per panel |

For the verification/testing of generalization related aspects in RAN4 for AI/ML BM, RAN4 should define different scenarios based on parameters listed in Table 3.

For the verification/testing of scalability related aspects in RAN4 for AI/ML BM, RAN4 should define different scenarios based on parameters listed in Table 4.

In order to address the testability issue of generalization aspects of AI/ML BM use case, we propose a new metric to indicate the generalization capabilities of an AI/ML model/functionality, which may be referred as “generalization score”. In case of AI/ML BM use case, this may be used to verify/check whether the BM functionality/model (which may be developed for a specific scenario and/or configuration) generalize well to other scenario(s) and/or configuration(s).

As an example, if it may be identified that there are 3 scenarios and 3 configurations of interest, then there will be 9 different pairs (Scenario 1, Configuration 1), (Scenario 1, Configuration 2), (Scenario 1, Configuration 3), (Scenario 2, Configuration 1), …, (Scenario 3, Configuration 3). Any one pair out of these 9 pairs can be the baseline setup (depending on functionality/model implementation) and the remaining 8 pairs will then be used as other set ups to test the generalization aspects. An overall generalization score can be calculated as a function of individual pair verdicts, e.g., as simple average or weighted average of the individual generalization verdict of each Pair ‘j’ (i.e., Pair ‘1’ to Pair ‘N), where the verdict of Pair ‘j’ = 1 if the functionality/model generalize to Pair ‘j’ else it is equal to 0.

Overall Generalization Score= (VerdictPair1+…+VerdictPairN)N×100%

RAN4 needs to design a new metric, indicative of generalization capabilities of model/functionality for AI/ML BM, to verify the generalization performance of the model/functionality in different scenarios.

Testing setup

The feasibility of testability aspects of AI/ML enabled BM use case and the corresponding testing setup is one of the most important topics which have been discussed over the past few RAN4 meetings and the following agreements were made in the RAN4#111 meeting [1].

Agreement: maximum number of set B Tx beams that test system should be able to emulate: [8-16] maximum number of set A Tx beams that test system should be able to emulate: [64-128] FFS on AoAs UE rotation during the test: FFS UE rotation/repositioning between different tests: Yes

However, there are some issues which are still open: (1) details of the AoAs to be supported during the conformance test (i.e., the number of different AoAs and their values), (2) whether to use the UE rotation or not during the conformance test of the AI/ML enabled BM use-case. In this section, we provide our further views on these open issues.

Before RAN4 agrees on a particular channel model (e.g., CDL or AWGN) to be used for the conformance testing of AI/ML enabled BM use case, it is important to first discuss and understand the objective(s) of the conformance testing for this use case.

As per our understanding, the focus of conformance testing here should be to verify the correctness/accuracy of AI/ML based predictions for DL Tx beam(s) only and should not be on testing the multipath/MIMO functionalities as these are the legacy features and the corresponding test case(s) already exist. Furthermore, if the input to the AI/ML model/functionality for BM use case will be L1-RSRP (based on RAN1 agreements) to predict the best (set) of DL Tx beam(s) at the UE, RAN4 should focus on developing the requirements and test case for AI/ML based prediction of the best (set) of beam(s) for a given L1-RSRP value only. Hence, it is redundant to test how the DUT measures the L1-RSRP during the AI/ML enabled BM conformance test as it will not add any significant value to the conformance testing of the AI/ML enabled BM use case but will increase the test complexity for sure.  Therefore, as a starting point, RAN4 should not focus on the use of complex testing setup for the conformance testing of the AI/ML enabled BM use case as it will not bring substantial benefits over the simpler testing setups.

With L1-RSRP being input to the AI/ML model/functionality for BM use case, it is redundant to test how the DUT measures the L1-RSRP during the AI/ML enabled BM conformance test and hence AWGN channel model may be sufficient for testing setup of this use case.

Use of complex testing setups for the conformance testing of AI/ML enabled BM use case will not add any significant value over the simpler testing setups but will increase the test complexity for sure.

As a starting point, RAN4 should focus on simpler test setups (e.g., AWGN based) for the conformance testing of AI/ML enabled beam management use case and not on the complex ones.

In the RAN4 discussions related to the use of channel model during the conformance testing, the views of different companies were quite diverse where some companies were supporting the use of AWGN propagation condition based simpler testing solution while the others were in favor of CDL channel model-based conformance testing. The reasons for preferring the AWGN channel based testing are: (1) less complexity relative to CDL model based test, (2) questions on the feasibility of CDL based test due to the limitations of the test system to emulate the total number of taps in the CDL model, (3) longer test time with CDL model, (4) possibility of using AWGN model with UE rotation to emulate the different AoAs, (5) using the different test probes with different transmit power to emulate the different L1-RSRP values, (6) less attractiveness of scaling MIMO OTA test systems in terms of time, cost and very limited use, etc.

Therefore, we next propose a simple solution which combines the AWGN based test systems and UE orientation to perform the conformance testing of AI/ML based BM use case to enable the beam sweeping of Set A and/or Set B beams in an iterative manner, by using a combination of the available test probes and different UE orientations/rotations, during the RAN4 conformance testing using the OTA test chamber where single or multiple test probe(s) are available. The proposed solution assumes: (1) OTA test chamber is equipped with single or multiple test probe(s), (2) AWGN channel model to be used during the test, (3) One beam is generated by using only one of the available test probe(s) during the conformance test.

The solution proposes the DUT to perform beam sweeping in multiple iterations and to measure the RSRP of the corresponding beams in Set A and/or Set B during the conformance test for which a new test command is to be defined and to be sent to the DUT. After the DUT receives this test command from the TE and configures itself to perform beam sweeping in multiple iterations, one of the possible/allowed UE orientations is selected and TE selects the power level (typically different) for each of the available test probes. It is worth mentioning that in this solution, only one test probe is active at any given point in time and represents one Tx beam. This implies that with ‘P’ test probes in the OTA testing (anechoic) chamber, using one probe at a time will implicitly mean emulating ‘P’ different Tx beams (one at a time) for a given UE orientation. Thereafter, if more than ‘P’ Tx beams are required, TE can change the UE orientation but uses the same set of available test probes (with the Tx power of each test probe to be held constant as previously selected or to be set differently) to emulate another set of ‘P’ Tx beams. In this fashion, multiple UE orientations can be selected depending on the number of Tx beams required and number of UE orientations should be equal to the ceil value of the ratio of the total number of Tx beams to the number of available test probes. The above solution is valid for emulating all Set A and/or Set B beams in an iterative manner to perform beam sweeping during the conformance testing.

Figure 2: An example of the proposed AWGN based test system with UE rotation.

Given the Set A and/or Set B beams can be generated iteratively during the conformance test as proposed above, DUT can measure the L1-RSRP of each beam in Set B and provides it as input to its AI/ML model for Set A beam prediction.

A combination of AWGN based test system and UE rotation can be a potential candidate for the conformance testing of AI/ML based BM use case.

RAN4 to discuss and explore the feasibility of performing beam sweeping in multiple iterations during the conformance testing of AI/ML based BM use case.

Conclusion

In this paper we share our views on potential RAN4 impacts from issues related to AI/ML enabled BM-Case1 and BM-Case2. Specifically, we cover following aspects:

- Ground truth reporting and measurement accuracy

- Indicated/activated TCI states requirements

- LCM related requirements

- Generalization and test coverage related aspects

- Testing setup

In the paper, the following Observations and Proposals were made:

Observation 1: Existing principles of testing UE-reported RSRP accuracy based on setup parameters and measurement uncertainties allow for very wide range of valid RSRP values. This makes testing for verification of AI/ML-enabled RSRP/best beam prediction a very futile exercise.

Observation 2: No significant improvement in L1-RSRP measurement accuracy is observed while changing SINR side condition from -3 to 0 dB.

Observation 3: If the requirement on predicted RSRP accuracy is based on the error-prone measured RSRP, the resulting absolute error of the prediction can be higher than in legacy requirements.

Proposal 1: If reported RSRP measurements are used as an approximate of the ground truth, a tighter than legacy measurement accuracy should be considered for the definition of requirements for AI/ML BM.

Observation 4: If both measurements and predictions are reported by the UE, there is no objective/external criteria to verify the independence of these reports.

Proposal 2: The requirement based on RSRP measurements used as an approximate of the ground truth should not be the only ones specified for BM use cases.

Observation 5: The behaviour of beam prediction functionalities can be tested without explicit definition of the ground truth by comparing the predictions to each other.

Proposal 3: RAN4 to introduce additional requirements based on relative difference in between the predictions due to change in the conditions/inputs, i.e., without dependency on the measurement accuracy errors and uncertainties.

Observation 6: When the TCI state is known, it is assumed that UE is aware of corresponding beam/RS, it has measured it recently, UE can switch to known TCI state faster than to an unknown TCI state.

Proposal 4: RAN4 should consider relaxing known conditions for TCI states (Clause 8.10.2 of TS 38.133) for BM-Case1 and BM-Case2

Observation 7: The delay considered for these operations necessary to activate the functionality depends on the use case (spatial beam prediction or temporal beam prediction) and should be carefully analysed.

Observation 8: If an LCM action is required and it is not taken in a timely manner, the performance for AI/ML enabled BM use case may be degraded to undesirable level.

Proposal 5: Core requirements should be considered to limit latency of LCM actions typical for AI/ML enabled BM-Case1 and BM-Case2.

Proposal 6: Activation, Deactivation, Switching, Fallback to non-AI operation and Performance monitoring should be prioritized for defining RRM requirements for LCM AI/ML BM use case.

Observation 9: A UE model/functionality, if tested in different generalization scenarios/configuration, would be considered not overfitted to a certain test environment.

Proposal 7: Testing of generalization aspects should also be considered to avoid overfitting of UE models to the test environment.

Proposal 8: For the verification/testing of generalization related aspects in RAN4 for AI/ML BM, RAN4 should define different scenarios based on parameters listed in the Table below.

Parameters for Generalization Scenarios

| Parameters | Description |
| --- | --- |
| Propagation Model | AWGN/CDL/Uma/UMi |
| SINR | Good / Bad Radio conditions |
| UE Speed | Slow / Medium / Fast |
| Channel propagation conditions | LOS/NLOS |

Proposal 9: For the verification/testing of scalability related aspects in RAN4 for AI/ML BM, RAN4 should define different scenarios based on parameters listed in the Table below.

Parameters for Scalability Scenarios

| Parameters | Description |
| --- | --- |
| gNB antenna configurations | gNB antenna array 2x4/4x8/8x16 |
| Variable number of Set B beams | Set B 16/32/64 beams |
| UE Rx beams | UE Rx beams 4/8 beams per panel |

Proposal 10: RAN4 needs to design a new metric, indicative of generalization capabilities of model/functionality for AI/ML BM, to verify the generalization performance of the model/functionality in different scenarios.

Observation 10: With L1-RSRP being input to the AI/ML model/functionality for BM use case, it is redundant to test how the DUT measures the L1-RSRP during the AI/ML enabled BM conformance test and hence AWGN channel model may be sufficient for testing setup of this use case.

Observation 11: Use of complex testing setups for the conformance testing of AI/ML enabled BM use case will not add any significant value over the simpler testing setups but will increase the test complexity for sure.

Proposal 11: As a starting point, RAN4 should focus on simpler test setups (e.g., AWGN based) for the conformance testing of AI/ML enabled beam management use case and not on the complex ones.

Observation 12: A combination of AWGN based test system and UE rotation can be a potential candidate for the conformance testing of AI/ML based BM use case.

Proposal 12: RAN4 to discuss and explore the feasibility of performing beam sweeping in multiple iterations during the conformance testing of AI/ML based BM use case.

- References

- R4-2410570, WF on AI/ML, RAN4#111, Fukuoka city, Japan, May 20th – 24th, 2024.

- R4-2410571, AI/ML ad-hoc meeting minutes, Qualcomm, RAN4#111, Fukuoka city, Japan, May 20th – 24th, 2024.

- TR 38.903, Derivation of test tolerances and measurement uncertainty for User Equipment (UE) conformance test cases, Rel-17.