3GPP TSG-RAN WG4 Meeting #112	R4-2412995

Maastricht, Netherlands, Aug 19 – Aug 23, 2024

Source:	Nokia

Title:	DL interruption for Tx switching across 2 bands and 2 TAGs case

Agenda item:	5.20.2

Document for:	Discussion

Introduction

The Rel18 WI Multi-carrier enhancements for NR was discussed during RRM sessions on RAN4#110 and RAN4#111 meetings, where core requirements were finalized. Some companies brough the idea to update DL interruption requirements for UL Tx switching across 2 bands and 2 TAGs case. In this paper, we provide our view on this topic.

Discussion

During RAN4#110 meeting, some companies raised the issue to update DL interruption requirements for UL Tx switching in the case of 2 bands and 2 TAGs case. In RAN4#111, this discussion on DL interruption for Tx switching across 2 bands and 2 TAGs case was postponed to RAN4#112 meeting.

In our view, these already are agreed legacy requirements, where no distinction was made between single TAG and dual TAGs cases, as mentioned in Table 8.2.2.210-1 and Table 8.2.2.210A-1 of TS 38.133 and shown below as reference.

Table 8.2.2.2.10-1 of TS 38.133: DL interruption length on NR carrier(s) in the unit of OFDM symbols (X) for switching between two uplink carriers

|  | NR Slot length (ms) | Uplink Tx switching period Note1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | 35us | 140us | 210us |
| 0 | 1 | 2 | 3 | 4 |
| 1 | 0.5 | 3 | 6 | 7 |
| 2 | 0.25 | 4 | 10 | 14 |
| Note 1: Uplink Tx switching period depends on UE capability uplinkTxSwitchingPeriod |  |  |  |  |

Table 8.2.2.2.10A -1 of TS 38.133: DL interruption length on NR carrier(s) in the unit of OFDM symbols (X) for switching between two uplink carriers

|  | NR Slot length (ms) | Uplink Tx switching period Note1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | 35us | 140us | 210us |
| 0 | 1 | 2 | 3 | 4 |
| 1 | 0.5 | 3 | 6 | 7 |
| 2 | 0.25 | 4 | 10 | 14 |
| Note 1: Uplink Tx switching period depends on UE capability [uplinkTxSwitchingPeriod2T2T]. |  |  |  |  |

Therefore, in our view, these legacy requirements for 2 bands 2 TAGs case should not be added at this stage.

No distinction was made in 1-TAG and 2TAGs cases for 2 bands scenario in the legacy case.

Existing requirements for DL interruption for Tx switching across 2 bands can be applied to DL interruption for Tx switching across 2 bands and 2 TAGs case.

Conclusion

This contribution discusses the issue of additional requirements for DL interruption for Tx switching across 2 bands and 2 TAGs case.

The observations and proposals are summarized as below:

- No distinction was made in 1-TAG and 2TAGs cases for 2 bands scenario in the legacy case.

- Existing requirements for DL interruption for Tx switching across 2 bands can be applied to DL interruption for Tx switching across 2 bands and 2 TAGs case.

References

- R4-2401987 CR on Implementation of two-band Tx switching with dual TAG, ZTE Corporation, RAN4#110, Athens, Greece, February 19 – March 01, 2024.