3GPP TSG RAN WG4 Meeting #112	R4-2412996

Maastricht, NL, 19 Aug. 2024 – 23 Aug. 2024

Agenda Item:	5.11.3

Source: 	Ericsson

Title:	Discussion on performance part for NR in less than 5 MHz bandwidth

Document for:	Discussion

# Introduction

In this contribution, we provide values for the BFD and RLM test cases for NR in less than 5 MHz bandwidth.

# Discussion

## BFD and RLM out of sync and Insync thresholds

Based on the simulation results submitted in the annexes we summarize the threshold values for BFD and RLM test below.

BFD test

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| SNR_SSB of set q0 | Config 1 | dB | 5 | -3 | -12 | -12 | -12 |
| SNR_SSB of set q1 | Config 1 | dB | -10 | -10 | 10 | 10 | 10 |
| SSB_RP of set q1 | Config 1 | dBm/SCS kHz | -108 | -108 | -88 | -88 | -88 |

For RLM test,

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| SNR on RLM-RS | Config 1 | dB | 1 | -9 | -15 | -4.5 | -1 |
|  | Config 2 |  | 1 | -7 | -15 | -4.5 | 3 |

- RAN4 to agree following thresholds for the BFD test

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| SNR_SSB of set q0 | Config 1 | dB | 5 | -3 | -12 | -12 | -12 |
| SNR_SSB of set q1 | Config 1 | dB | -10 | -10 | 10 | 10 | 10 |
| SSB_RP of set q1 | Config 1 | dBm/SCS kHz | -108 | -108 | -88 | -88 | -88 |

- RAN4 to agree following thresholds for the RLM test

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| SNR on RLM-RS | Config 1 | dB | 1 | -9 | -15 | -4.5 | -1 |
|  | Config 2 |  | 1 | -7 | -15 | -4.5 | 3 |

# Summary and conclusions

- RAN4 to agree following thresholds for the BFD test

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| SNR_SSB of set q0 | Config 1 | dB | 5 | -3 | -12 | -12 | -12 |
| SNR_SSB of set q1 | Config 1 | dB | -10 | -10 | 10 | 10 | 10 |
| SSB_RP of set q1 | Config 1 | dBm/SCS kHz | -108 | -108 | -88 | -88 | -88 |

- RAN4 to agree following thresholds for the RLM test

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| SNR on RLM-RS | Config 1 | dB | 1 | -9 | -15 | -4.5 | -1 |
|  | Config 2 |  | 1 | -7 | -15 | -4.5 | 3 |

# References

- R4-2410431

# Annexe

In previous meeting, following PDCCH parameters are agreed for RLM or BFD OOS and IS.

Table 8.1.2.1-3: PDCCH transmission parameters for out-of-sync evaluation for [less than 5MHz UE]

| Attribute | Value for BLER Configuration #0 |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3MHz (12 PRBs) | 3MHz (15 PRBs) | 5MHz (20 PRBs) |  |
| DCI format | 1-0 |  |  |  |
| Number of control OFDM symbols | [2] | [3] | [3] |  |
| Aggregation level (CCE) | [4] | [8] | [8] |  |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 4dB |  |  |  |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 4dB |  |  |  |
| Bandwidth (PRBs) | 12 | 15 | 20 |  |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP |  |  |  |
| DMRS precoder granularity | REG bundle size |  |  |  |
| REG bundle size | 6 |  |  |  |
| CP length | Normal |  |  |  |
| Mapping from REG to CCE | Distributed | Non-Distributed and Distributed |  | Distributed |

Table 8.1.2.1-4: PDCCH transmission parameters for in-sync evaluation for [less than 5MHz UE]

| Attribute | Value for BLER Configuration #0 |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3MHz (12 PRBs) | 3MHz (15 PRBs) | 5MHz (20 PRBs) |  |
| DCI format | 1-0 |  |  |  |
| Number of control OFDM symbols | [2] | [3] | [3] |  |
| Aggregation level (CCE) | [2] | [4] | [4] |  |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 0dB |  |  |  |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 0dB |  |  |  |
| Bandwidth (PRBs) | 12 | 15 | 20 |  |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP |  |  |  |
| DMRS precoder granularity | REG bundle size |  |  |  |
| REG bundle size | 6 |  |  |  |
| CP length | Normal |  |  |  |
| Mapping from REG to CCE | Distributed | Non-Distributed and Distributed |  | Distributed |

We have performed following set of simulations based on the above assumptions and the simulation parameters agreed during the core part of the discussion.

Out of sync simulations:

- 12 PRB interleaved with TDL-C and above OOS parameters

- 15 PRB interleaved with TDL-C and above OOS parameters

- 15 PRB non-interleaved with TDL-C and above OOS parameters

- 20 PRB interleaved with TDL-C and above OOS parameters

- 12 PRB interleaved with HST and above OOS parameters

In sync simulations:

- 12 PRB interleaved with TDL-C and above IS parameters

- 15 PRB interleaved with TDL-C and above IS parameters

- 15 PRB non-interleaved with TDL-C and above IS parameters

- 20 PRB interleaved with TDL-C and above IS parameters

- 12 PRB interleaved with HST and above IS parameters

We present results in the following table for relevant simulation assumption.

Simulation Assumption:

PDCCH BW: 12 PRBs, Number of CORESET symbols: 2; Carrier freq.: 900 MHz; CORESET bandwidth 12 PRBs and non-punctured and inter-leaved. Antenna config: 2*2, low.

Simulation Result:

| Scenario | AL | Channel | OOS (with -4 dB adjustment to 10% BLER) | SNR dB for 2% BLER In-Sync | Difference between IS-OOS |
| --- | --- | --- | --- | --- | --- |
| OOS | 8 | TDL-C 300ns 100 Hz | -6.94 |  | 10.04 |
| IS | 4 | TDL-C 300ns 100 Hz |  | 3.1 |  |

Simulation Assumption:

PDCCH BW: 15 PRBs, Number of CORESET symbols: 3; Carrier freq.: 900 MHz; the CORESET is generated from 24-PRB CORESET 0 of which top 9 PRBs are punctured and inter-leaved, Antenna config: 2*2, low

Simulation Result:

| Scenario | AL | Channel | OOS (with -4 dB adjustment to 10% BLER) | SNR dB for 2% BLER In-Sync | Difference between IS-OOS |
| --- | --- | --- | --- | --- | --- |
| OOS | 8 | TDL-C 300ns 100 Hz | -8.66 |  | 7.8 |
| IS | 4 | TDL-C 300ns 100 Hz |  | -0.855 |  |

Simulation Assumption:

PDCCH BW: 15 PRBs, Number of CORESET symbols: 3; Carrier freq.: 900 MHz; the CORESET is generated from 24-PRB CORESET 0 of which top 9 PRBs are punctured and non-inter-leaved, Antenna config: 2*2, low

Simulation Result:

| Scenario | AL | Channel | OOS (with -4 dB adjustment to 10% BLER) | SNR dB for 2% BLER In-Sync | Difference between IS-OOS |
| --- | --- | --- | --- | --- | --- |
| OOS | 8 | TDL-C 300ns 100 Hz | -7,23 |  | 2,86715 |
| IS | 4 | TDL-C 300ns 100 Hz |  | -4,36285 |  |

Simulation Assumption:

PDCCH BW: 20 PRBs, Number of CORESET symbols: 3; Carrier freq.: 900 MHz; the CORESET is generated from 24-PRB CORESET 0 of which top 4 PRBs are punctured and inter-leaved, Antenna config: 2*2, low

Simulation Result:

| Scenario | AL | Channel | OOS (with -4 dB adjustment to 10% BLER) | SNR dB for 2% BLER In-Sync | Difference between IS-OOS |
| --- | --- | --- | --- | --- | --- |
| OOS | 8 | TDL-C 300ns 100 Hz | -8,04762 |  | 3,27125 |
| IS | 4 | TDL-C 300ns 100 Hz |  | -4,77637 |  |