3GPP TSG-RAN WG4 Meeting #112 	R4-2413000

Maastricht, NL, 19 Aug. 2024– 23 Aug. 2024

Agenda Item:	5.13.2

Source: 	Ericsson

Title:	maintenance of RRM performance for multi-rx

Document for:	Discussion

# Introduction

In this contribution we provide our views on the test cases for FR2 multi-RX WI.

# Discussion

In last meeting 2 new AoA set up introduced for test cases of the multi-rx WI. The introduced AoA setup are copied below for reference.

### Setup 5: 2 AoAs for simultaneous reception with QCL Type-D There are 2 active probes in the test. The DL signals, and noise if applicable, transmitted from the two active probes. The 2 AoAs [(AoA1 and AoA2)] for simultaneous reception with different QCL-typeD is from the set of [qualified AoA pairs] according to the spherical coverage requirement for simultaneous reception from multiple directions as defined in clause 7.3K.3 of TS 38.101-2 for power class 3 supporting simultaneous reception from multiple directions, the angular separation between the directions (AoAs) of the 2 active probes is declared from Table 7.3K.3-1 in clause 7.3K.3 of TS 38.101-2 [19] and [shall not be changed for each test iteration]. [The relative angular offset between the directions of the AoA pair is based on the UE’s declared [AoA separation and UE] orientation as defined in clause 7.3K.3 of TS 38.101-2.] [One of the AoAs (AoA1) needs to satisfy the spherical coverage requirement in Table 7.3.4.3-1 of TS 38.101-2]. Editor’s note: FFS how single AoA RF test and 2 AoA RF test could ensure this. ### Setup 6: 3 AoAs for simultaneous reception with different QCL Type-D There are 3 active probes in the test and the DL signals and noise are transmitted from the three active probes. Out of the three AoA, one AoA [(AoA1)] is aligned to a direction which is from the set of directions corresponding to the EIS spherical coverage percentile of the DUT as defined in clause 7.3.4 of TS 38.101-2 [19] for UE power class 3 and other 2 AoAs [(AoA2, AoA3)] are from the set of [qualified AoA pairs] according to the spherical coverage requirement for simultaneous reception from multiple directions as defined in clause 7.3K.3 of TS 38.101-2 for power class 3 supporting simultaneous reception from multiple directions. The relative angular offset between the directions of the AoA pair is based on the UE’s declared [AoA separation and UE] orientation as defined in clause 7.3K.3 of TS 38.101-2 and [shall not be changed for each test iteration].

In the newly introduced AoA setups there are few open issues with [] for further check. One such open issue is whether to use the AoA numbering in the test set up or not. We think specifying which AoA UE needs receive during which stage of the test is essential for the ease of the test.  We think specifying the AoA numbering in the test setup would simplify the test description across the multiple test cases.

For example, for the setup 3, AoA2 and AoA3 can be received using the beam pair and in this case AoA1 has to be received using a single beam. If we do not have a notation of which AoA can be received using a single beam and which AoA can be received using a beam pair in the test setup, in each and every test, we need to specify that particular AoA is assumed to be received using single beam and other AoA(s) can be received using the beam pair. To make the test description simplified and consistent across the tests, we prefer to specify the AoA number and whether which AoA can be pair or which AoA can be received using single beam.

- Specify the AoA numbering and their description in the test set up.

Scheduling restriction test cases:

In the agreed test cases the reporting period is considered as 320 ms and CSI-RS resource periodicity is 80ms. Since we are testing the scheduling restriction and whether UE can measure CSI-RS and receive data simultaneously, it is important to configure report periodicity equal to CSI-RS resource periodicity. If not then we cannot really test whether UE can measure simultaneously or not.

- In the scheduling and measurement restriction test cases set CSI-RS periodicity as 80ms and CSI report periodicity as 80ms.

# Summary

The following have been proposed in this contribution.

- Specify the AoA numbering and their description in the test set up.

- In the scheduling and measurement restriction test cases set CSI-RS periodicity as 80ms and CSI report periodicity as 80ms.

# References

[1] RP-221753, WI description, Requirement for NR frequency range 2 (FR2) multi-Rx chain DL reception, Qualcomm Inc., June 2022.

[2] R4-2410356