3GPP TSG RAN WG4 Meeting #112	R4-2413005

Maastricht, NL, 19 Aug. 2024 – 23 Aug. 2024

Agenda Item:	5.24.1

Source: 	Ericsson, Qualcomm Incorporated

Title:	RRM requirements for L1/L2 based inter-cell mobility

Document for:	Discussion

# Introduction

In this contribution, we provide our views on the some of the open issues on L1/L2 triggered inter-cell mobility (LTM).

# Discussion

In last meeting it was agreed to support LTM without L1-RSRP report. In this contribution we discuss requirements to support the LTM without L1-RSRP report in the subsequent sections.

## TCI state activation requirements for LTM candidate cell

As per RAN2 design, one MAC-CE activates TCI states of a single candidate cell. If UE supports multiple TCI states activation, NW may activate the TCI states of different cells at the same time. For multiple candidate cells TCI state activation, NW may send MAC CE activation command at the same time in the same MAC PDU or in a different MAC PDU at different time instances. In this scenario, TCI state activation of the UE depends on different cases. The TCI state activation requirements depend on different aspects such as whether SSB are overlapping among the candidate cells or whether the candidate cell is intra-frequency candidate cell or inter-frequency candidate cell or whether the cell operates on FR1 or FR2.

For intra-frequency FR1 cells: Irrespective of whether SSB of the candidate cells overlap or not, if the candidate cell is an intra-frequency cell, UE can perform the T/F tracking in parallel (i.e., at the same measurement occasion).

For inter-frequency FR1 or intra-frequency/inter-frequency FR2 cells: If the SSB are overlapping, UE need to perform T/F tracking in sequential order as UE measures one frequency layer at a time in FR1 and one cell at a time in FR2. If the SSB are not overlapping, UE can perform T/F tracking in parallel (that means on non-overlapping resources).

Further in previous meeting, RAN4 agreed to define the TCI state activation requirements for known and unknown TCI state activation in FR1.  In the rest of this section, we discuss the requirements for FR1 known and unknown TCI state activation and FR2 known TCI state activation requirements.

While defining the TCI state activation requirements we think we can borrow the principles used in legacy TCI state activation.

- Principle 1: If the TCI state is known, and if the TCI state is not in the active TCI state list, to activate the TCI state in FR1 or FR2, UE needs one SSB sample to obtain T/F tracking.

- Principle 2: If the TCI state is unknown, in FR1, UE need one SSB sample for T/F tracking.

- Principle 3: If the TCI state is unknown in a FR2 cell, UE needs to perform L1-RSRP measurement and additional T/F tracking to complete the TCI state activation.

In this section, we first look at the single cell TCI state activation requirements and then define the scaling rules when more than one cell is to be activated. If a TCI state activation command is received at slot n, UE shall be able to finish the TCI state activation within slot n+ THARQ +3Nslotsubframe,µ+TTF + TSSB-proc. Where TTF for single cell in different scenario is mentioned below.

|  | TTF | Comments |
| --- | --- | --- |
| FR1 intra-frequency cell, known and unknown TCI state | max (Tfirst-SSB_TCI1, Tfirst-SSB_TCI2 .., Tfirst-SSB_TCIn) | Tfirst-SSB_TCIn is the time for first SSB associated to TCI state n. |
| FR1 inter-frequency cell without MG, known and unknown TCI state | max (Tfirst-SSB_TCI1, Tfirst-SSB_TCI2 .., Tfirst-SSB_TCIn) | Tfirst-SSB_TCIn is the time for first SSB reception associated to TCI state n. |
| FR1 inter-frequency cell with MG, known and unknown TCI state | TFirstMG + MGL | TFirstMG is the time to start of first MG after slot n+THARQ+3ms, MGL is measurement gap length |
| FR2 intra-frequency cell, known TCI state | max (Tfirst-SSB_TCI1, Tfirst-SSB_TCI2 ..., Tfirst-SSB_TCIn) | Tfirst-SSB_TCIn is the time for first SSB reception associated to TCI state n. |
| FR2 inter-frequency cell without MG, known TCI state | max (Tfirst-SSB_TCI1, Tfirst-SSB_TCI2 ..., Tfirst-SSB_TCIn) | Tfirst-SSB_TCIn is the time for first SSB reception associated to TCI state n. |
| FR2 inter-frequency cell with MG, known TCI state | TFirstMG + MGL | TFirstMG is the time to start of first MG after slot n+THARQ+3ms, MGL is measurement gap length |

When a UE supports more than one candidate cell TCI states activation, NW may activate the TCI states from multiple cells. Since one cell TCI states are activated using one MAC CE, NW may send the different MAC CE to activate more than one cell in same slot or different slot. Following scenario may occur for activating the more than one cells TCI states.

- Scenario 1: MAC CE for TCI state activation for different candidate cells are received within same slot.

- Scenario 2: MAC CE for different candidate cells are not received within same slot but received before the TCI state activation completion of another cell.

Since supporting more than one candidate cells TCI state activation is up to UE capability, if UE supports and reports this capability, it is important for the NW to know the UE behaviour for the TCI state activation. We further discuss the expected UE behaviour for the scenario 1 and 2.

Scenario 1: UE activates both cells either in parallel or sequential depending on the intra or inter-frequency cases.

Scenario 2: this case may be bit complicated as we may need to define a window to define a UE measurement behaviour.

To simply the specification work, we can leave this scenario to UE implementation. That means, if NW would like to schedule multiple candidate cells TCI state activation, it can either do using scenario 1 or after the completion of the other ongoing TCI state activation.

In the next table we discuss the requirements for scenario 1.

|  | TTF | Comments |
| --- | --- | --- |
| FR1 intra-frequency cell, known and unknown TCI state | max (Tfirst-SSB_TCI1, Tfirst-SSB_TCI2 .., Tfirst-SSB_TCIn) | Tfirst-SSB_TCIn is the time for first SSB associated to TCI state n. |
| FR1 inter-frequency cell without MG, known and unknown TCI state | max (Tfirst-SSB_TCI1, Tfirst-SSB_TCI2 .., Tfirst-SSB_TCIn) + (M-1)*TSSB | Tfirst-SSB_TCIn is the time for first SSB reception associated to TCI state n. M is the number of cells to activate the TCI states TSSB is the SSB burst periodicity. |
| FR1 inter-frequency cell with MG, known and unknown TCI state | TFirstMG + (M-1) *MGRP+MGL | TFirstMG is the time to start of first MG after slot n+THARQ+3ms, MGL is measurement gap length M is the number of cells to activate the TCI states MGRP is the MG repetition periodicity |
| FR2 intra-frequency cell, known TCI state | max (Tfirst-SSB_TCI1, Tfirst-SSB_TCI2 .., Tfirst-SSB_TCIn) + (M-1)*TSSB | Tfirst-SSB_TCIn is the time for first SSB reception associated to TCI state n. M is the number of cells to activate the TCI states TSSB is the SSB burst periodicity |
| FR2 inter-frequency cell without MG, known TCI state | max (Tfirst-SSB_TCI1, Tfirst-SSB_TCI2 .., Tfirst-SSB_TCIn) + (M-1)*TSSB | Tfirst-SSB_TCIn is the time for first SSB reception associated to TCI state n. M is the number of cells to activate the TCI states TSSB is the SSB burst periodicity |
| FR2 inter-frequency cell with MG, known TCI state | TFirstMG + (M-1) *MGRP+MGL | TFirstMG is the time to start of first MG after slot n+THARQ+3ms, MGL is measurement gap length M is the number of cells to activate the TCI states MGRP is the MG repetition periodicity |

Based on the above analysis we make following proposal.

- When one or more of TCI state activation commands are received at slot n, UE shall be able to finish the TCI state activation within slot n+ THARQ +3Nslotsubframe,µ+TTF + TSSB-proc. where TTF  is mentioned in the below table.

|  | TTF | Comments |
| --- | --- | --- |
| FR1 intra-frequency cell, known and unknown TCI state | max (Tfirst-SSB_TCI1, Tfirst-SSB_TCI2 .., Tfirst-SSB_TCIn) | Tfirst-SSB_TCIn is the time for first SSB associated to TCI state n. |
| FR1 inter-frequency cell without MG, known and unknown TCI state | max (Tfirst-SSB_TCI1, Tfirst-SSB_TCI2 .., Tfirst-SSB_TCIn) + (M-1)*TSSB | Tfirst-SSB_TCIn is the time for first SSB reception associated to TCI state n. M is the number of cells to activate the TCI states TSSB is the SSB burst periodicity. |
| FR1 inter-frequency cell with MG, known and unknown TCI state | TFirstMG + (M-1) *MGRP+MGL | TFirstMG is the time to start of first MG after slot n+THARQ+3ms, MGL is measurement gap length M is the number of cells to activate the TCI states MGRP is the MG repetition periodicity |
| FR2 intra-frequency cell, known TCI state | max (Tfirst-SSB_TCI1, Tfirst-SSB_TCI2 .., Tfirst-SSB_TCIn) + (M-1)*TSSB | Tfirst-SSB_TCIn is the time for first SSB reception associated to TCI state n. M is the number of cells to activate the TCI states TSSB is the SSB burst periodicity |
| FR2 inter-frequency cell without MG, known TCI state | max (Tfirst-SSB_TCI1, Tfirst-SSB_TCI2 .., Tfirst-SSB_TCIn) + (M-1)*TSSB | Tfirst-SSB_TCIn is the time for first SSB reception associated to TCI state n. M is the number of cells to activate the TCI states TSSB is the SSB burst periodicity |
| FR2 inter-frequency cell with MG, known TCI state | TFirstMG + (M-1) *MGRP+MGL | TFirstMG is the time to start of first MG after slot n+THARQ+3ms, MGL is measurement gap length M is the number of cells to activate the TCI states MGRP is the MG repetition periodicity |

FR2 unknown TCI state activation:

In the FR2 unknown TCI state activation, UE need to make L1-RSRP measurement to determine the best TX and RX beam combination. Once the best TX and RX beam is determined UE need one more sample to get fine time tracking for the selected best TX and RX beam. This process can be quite long for even simple scenarios. When there is only one neighbour cell to measure, if the serving cell is in FR2, neighbour cell measurement and additional SSB for T/F tracking can take up to 980ms. That means, NW needs to send TCI state activation 980ms before and after the 980ms, the same neighbour cell may not be strong enough or best neighbour cell may change during this time. When a best neighbour cell, changes NW may need to perform another TCI state action. Due to the long delay involved, we do not see much value in this scenario. Hence, for the sake of time we prefer to deprioritize the defintion of FR2 unknown TCI state activation requirements in Rel-18.

- FR2 unknown TCI state activation requirements are not specified in Rel-18.

Other open issue in last meeting was the applicability of the unknown TCI state requirements for FR1 and FR2. One of the concerns raised in last meeting was about the TCI state being completely unknown (i.e., blind TCI state activation) without any measurement report. We think the TCI state can be unknown if the measurement report (e.g., L3-RSRP or L1-RSRP) was sent more than certain time ago. We can understand the concern from companies about the blind TCI state activation and we are fine to restrict the unknown TCI state activation only when there was a measurement report within certain time.

- Unknown TCI state activation requirements are applicable if the TCI state activated is based on the measurement report (e.g., L3-RSRP or L1-RSRP) within last 5 seconds and the SNR of the associated SSB is above -3dB.

Maximum time gap between TCI state activation and cell switch command:

Other open issue is what is the maximum time gap between completion of TCI activation and cell switch command to skip the T/F tracking. In last meeting following WF is agreed.

Issue 1-3-6: cell switch delay and PDCCH order RACH delay requirements without L1 measurement

Ad hoc agreement

< Agreement>:

- In cell switch delay requirement:

  - In FR1, for UE not supporting/configured with L1 measurement, one SSB occasion is needed from RAN4 requirement point of view for T/F fine tracking, if

    - the time gap between completion of TCI activation and cell switch command is larger than [160ms], or

    - the time gap between early RACH transmission and cell switch command is larger than [160ms].

- In PDCCH order RACH delay requirement:

  - In FR1, for UE not supporting/configured with L1 measurement, one SSB occasion is needed from RAN4 requirement point of view.

We think the tentative time gap of 160ms agreed in last meeting can put quite a bit of restriction on the NW implementation. For the case of inter-CU LTM, when UE sends PRACH to target cell, we expect the following sequence of operations before the cell switch command is received at the UE.

- Target DU estimate the TA

- Target DU sends TA command to target CU.

- Target CU sends TA command to source CU

- Source CU sends TA command to source DU

- Source DU prepares cell switch command

- Source DU send cell switch command to the UE

- Potential retransmissions of the cell switch command due to weak channel conditions in the HO region

Since different CU can be involved, and the CU can belong to different vendors, we may not guarantee to perform all these operations within 160ms. Having said that, we understand that Rel-18 consider intra-CU only in the scope of the WI. However, we think RAN4 may not be defining TCI activation requirements for inter-CU in Rel-19 as part of the inter-CU objective. Hence, we suggest revising the 160ms time gap to larger value such as 480ms to support realistic NW implementations.

Further, in FR1, there is no measurement occasion sharing between L1 and L3 measurement. That means UE use the same SSB occasion for L3 and L1 measurements as the raw SSB measurements are same for L1 and L3 measurement and L3 measurement involves additional filtering at the RRC layer. We think UE can obtain or maintain the obtained timing as long as UE has opportunity to measure L3 measurement at least once every 160 ms. We think it is reasonable assumption to not have additional SSB for T/F tracking to for the scenario of L3 measurement interval is shorter than or equal to 160ms.

- In FR1, for UE not supporting/configured with L1 measurement, one SSB occasion is needed from RAN4 requirement point of view for T/F fine tracking, if

    - the time gap between completion of TCI activation and cell switch command is larger than 480ms and the L3 measurement interval is larger than 160 ms, or

    - the time gap between early RACH transmission and cell switch command is larger than 480ms and the L3 measurement interval is larger than 160 ms.

## UL pre-synchronisation to LTM candidate cells

Issue 4-4-3: The value of additional time for RF/BB preparation and RF re-tuning: when PRACH bandwidth is not within any of the configured UL BWPs of any active serving cell

< Agreement>

  - For the case of PRACH bandwidth not within any of the configured UL BWPs of any active serving cell, if UE supports PRACH transmission outside any of the configured UL BWPs of any active serving cell, candidate values of the time needed for RF/BB preparation and RF retuning: 1ms, 3ms, 5ms, 10ms.

  - If identified, RAN4 to further discuss issues due to concurrencies, in terms of max number of carriers for SRS carrier switch and PRACH transmission, between ‘PRACH transmission outside any of the configured UL BWPs of any active serving cell’ and ‘SRS carrier switch.’

For the UE supporting ‘PDCCH-order PRACH transmission outside any of the configured UL BWPs of any active serving cell’ and ‘SRS carrier switch,’ the number of SRS carrier switch and/or SRS carrier switch delay can be potentially impacted due to RF script build, load, and program. Although there can be multiple ways of addressing such a concurrency issue depending on different cases, in order to avoid unnecessary prolonging, the interruption duration and defining complicated solutions, it was agreed to extend the interruption length to both DL and UL before/after the PDCCH-order PRACH by [1]ms in RAN4#110bis. We propose to remove [] in the agreement from RAN4#110bis.

Issue 4-4-2: Interruption when PRACH is not fully contained in any of UE’s configured UL BWP(s) of active serving cells

<Agreement at RAN4#110bis - Changsha>

  - Interruption to both DL and UL duration before/after PDCCH-order LTM PRACH is extended by [1ms] for the following case:

    - The PDCCH-order PRACH is not fully contained in any of UE’s configured UL BWP(s) of active serving cells, and

    - the number of RRC-configured LTM cells whose PRACH resources are not fully contained in any of UE’s configured UL BWP(s) of serving cell(s) is more than [2], and

    - UE is configured with SRS carrier switching.

- Interruption to both DL and UL duration before/after PDCCH-order LTM PRACH is extended by 1ms for the following case:

- The PDCCH-order PRACH is not fully contained in any of UE’s configured UL BWP(s) of active serving cells, and

- the number of RRC-configured LTM cells whose PRACH resources are not fully overlapping in the frequency domain is more than 2, and

- UE is configured with SRS carrier.

## LTM L1-RSRP measurement requirements

Measurement period of serving cell L1-RSRP measurement

<Way Forward >: Further discuss the following option:

- introduce NLayer in serving cell L1 RSRP measurement requirement and clarify that it is for UE capable of RTD>CP configured with L1 RSRP measurement on neighbour cell.

The above issue was discussed saying there is a misalignment between intra-frequency neighbour cell measurement and serving cell measurement in FR1. As per the proponents, current measurement period requirement for UE supporting RTD>CP, there is a scaling factor NLayer for neighbour cell, but in serving cell measurement there is no such scaling factor. However, we think NW may not need the L1 measurement from neighbour cell as it is needed from the serving cell. Hence, we think we do not need to scale the serving cell measurements with NLayer.

- RAN4 not to modify the serving cell measurement period by scaling it with NLayer for UE supporting RTD > CP.

Issue 2-4-1: L1 report for unmeasured candidate cells

<Way Forward> Further discuss the following options

  - Option 1 (MTK):

    - In L1-RSRP measurement report, for unmeasured candidate cells, UE reports measured quantity value corresponding to any of the invalid codepoints in Table 10.1.6.1-1.

  - Option 2 (vivo):

    - In L1-RSRP measurement report, for unmeasured candidate cells, UE reports measured quantity value corresponding to one of the invalid codepoints in Table 10.1.6.1-1, preferably RSRP_0.

  - Option 3 ([Apple], Nokia):

    - RAN4 needs to discuss more in which scenario this case is needed.

There can be a case where UE needs to report L1-RSRP measurement result even for a candidate cell left out from L1-RSRP measurements, which is allowed by RAN4 LTM L1-RSRP measurement definition, e.g. UE may not be able to perform measurements on some candidate cells particularly in FR2 because of UE beam sweeping. It’s left to UE implementation whether and for which specific cell the UE will perform LTM L1-RSRP measurements. In such a scenario, most likely RRC configuration will correspond to the yellow text. And the unmeasured candidate cell’s L1-RSRP can never be the largest one, meaning the report for such a cell’s RSRP will be reported as a relative value. In ‘Table 10.1.6.1-2: Differential SS-RSRP and CSI-RSRP measurement (for L1 reporting and L3 reporting) report mapping’ defined in RRM spec (38.133) does not have any codepoint reserved as an invalid value. With this observation, it makes most sense to report the value that can be interpreted as the lowest L1-RSRP value for the unmeasured candidate cell.

For L1-RSRP reporting, if the higher layer parameter nrofReportedRS in CSI-ReportConfig is configured to be one, or if the higher layer parameters [noOfReportedCells] and [noOfReportedRS-PerCell] are both configured to be one, the reported L1-RSRP value is defined by a 7-bit value in the range [-140, -44] dBm with 1dB step size, if the higher layer parameter nrofReportedRS is configured to be larger than one, or if the higher layer parameter groupBasedBeamReporting is configured as 'enabled', or if the higher layer parameter groupBasedBeamReporting-r17 is configured, or if any of the higher layer parameters [noOfReportedCells] and [noOfReportedRS-PerCell] is configured to be larger than one, the UE shall use differential L1-RSRP based reporting, where the largest measured value of L1-RSRP is quantized to a 7-bit value in the range [-140, -44] dBm with 1dB step size, and the differential L1-RSRP is quantized to a 4-bit value. The differential L1-RSRP value is computed with 2 dB step size with a reference to the largest measured L1-RSRP value which is part of the same L1-RSRP reporting instance. The mapping between the reported L1-RSRP value and the measured quantity is described in [11, TS 38.133].

- In L1-RSRP measurement report, for unmeasured candidate cells, UE reports DIFFRSRP_15 in Table 10.1.6.1-2.

## LTM Cell switch delay requirements

In this section we discuss the following open issues.

### Detail of cell switch delay requirements for PCell/PSCell

### PL-RS measurements

Since UE can be configured to measure and report the SSB based L1/L3-RSRP or can be configured with TCI state activation for the LTM candidate cell, we think UE can derive PL without any additional time required for PL computation. UE transmits PRACH as first transmission to candidate cell, hence UE would know the transmit power to use for the first UL transmission during the LTM Cell switch. We think no additional delay or additional conditions are required as the legacy HO do not have any conditions for PL-RS maintenance or how long before UE measured the PL-RS.

- For the cell switch delay, no additional delay or conditions are needed for PL-RS measurement provided that the following condition are fulfilled:

- UE has reported L3-RSRP on the SSB associated with PL-RS before reception of LTM configuration and UE is configured to perform L3 or L1 measurements after LTM configuration.

### Known conditions

In previous meetings, RAN4 agreed that the cell switch delay requirement is applicable for the case without L1 measurement and report. Following options are discussed in last meeting on how to support this in the spec.

Issue 3-3-1: known TCI state conditions Online Agreement < Agreement > - For FR1, the cell switch delay requirement is applicable for the case without L1 measurement and report, under certain conditions. - Further discuss the how the requirement applies together with the condition. < Agreement>: - In FR2: the RAN4 Rel-18 related LTM requirements are not applicable if L1 measurement of LTM candidate cell is not supported or configured. <Way Forward >: Further discuss the following options in FR1: - Option 1: change known TCI state conditions to make cell switch delay requirements applicable to the case without L1 measurement and report - Option 2: Allow cell switch delay requirements applicable to unknown TCI state case with conditions. - Regarding the conditions, further discuss the following options when SNR of the TCI state≥ -3dB - Condition 1: TCI state has been activated and - Condition 1A: the TCI state activation command was received not more than 1280 ms before the cell switch command - Condition 1B: Beam-level L3 measurement results have been reported in 1280 ms - Condition 1C: The RS associated to the target TCI state is available at least every 1280 ms after TCI state activation command - Condition 2: UE has reported L3-RSRP measurements for the SSB associated to the target TCI state in [1280]ms before the cell switch command <Way Forward >: Further discuss the following options in FR2: - Condition 3: TCI state has been activated and - Condition 3A: the TCI state activation command was received not more than 1280 ms before the cell switch command - Condition 3B: the TCI state activation command was received not more than 160 ms before the cell switch command - Condition 3C: The RS associated to the target TCI state is available at least every 1280 ms after TCI state activation command - Condition 3D: - Beam-level L3 measurement results have been reported in 1280ms - L1-RSRP measurement period is no larger than 1280ms in FR2 - Condition 4: UE has reported L3-RSRP measurements for the SSB associated to the target TCI state in [1280]ms before the cell switch command

Among the options discussed in last meeting, we think option 2 has least amount of workload. In other words, allowing cell switch delay requirements for unknown TCI state activation is the easiest option.

- RAN4 to allow the LTM cell switch requirements to be applicable for unknown TCI state.

### TCI state indication and activation in cell switch command

Current specification defines the cell switch delay requirement for “known” TCI state condition. The requirements also indicate that [for TCI state indicated in LTM cell switch command to be “known”, it has to be activated]. The requirement is shown below.

The target joint DL/UL TCI state or separate DL and UL TCI states in the LTM cell switch command are known if the following conditions are met: [- The target TCI state in the cell switch command is activated not more than TBD ms before the reception of the cell switch command and SNR of the SSB associated to TCI state ≥ -3dB; or] [- The target TCI state in cell switch command is activated before receiving the cell switch command and the SSB associated to target TCI state is available at least once every TBD ms after the TCI state activation command is received and SNR of the SSB associated to TCI state ≥ -3dB; or]

As per RAN1/2 design its allowed to either TRS or SSB based TCI states to be activated and indicated in the LTM cell switch command (based on 38.213, clause 21). Below table shows different scenarios, considering single beam for SSB/ TRS. Scenario 1-3 fulfil current cell switch delay requirements specified in RAN4 spec. However, Scenario 4 seems to not fulfil the current requirements (at least in the current wording).

If TCI state indicated in LTM cell switch command is associated with TRS, which in turn is associated (QCL relation) with SSB (which was activated), then it should be okay if either TCI state associated with SSB or TRS was activated in Candidate Cell TCI state activation command. However, it’s not clear in the current specification with the current wording whether NW can activated SSB based TCI state and indicate TS based TCI state in the cell switch command.

| TCI state activation/indication scenarios |  | Candidate Cell TCI State Activation Command | Cell Switch Command |
| --- | --- | --- | --- |
| 1 |  | SSB | SSB |
| 2 |  | SSB and TRS | TRS |
| 3 |  | TRS | TRS |
| 4 |  | SSB | TRS |
| 5 | DL sync failed or UE do not support DL sync | Either SSB or TRS |  |

- We suggest modifying the TCI known condition to following

The target joint DL/UL TCI state or separate DL and UL TCI states in the LTM cell switch command are known if the following conditions are met:

[-	The TCI state is activated not more than TBD ms before the reception of the cell switch command and SNR of the SSB associated to TCI state is ≥ -3dB; where the TCI state is considered activated if the activated TCI state and target TCI state in the cell switch command are same or the SSB associated to target TCI state in cell switch command and the SSB associated to activated TCI state are same; or]

[-	The TCI state is activated before the reception of the cell switch command (where the TCI state is considered activated if the activated TCI state and target TCI state in the cell switch command are same or the SSB associated to target TCI state in cell switch command and the SSB associated to activated TCI state are same) and the SSB associated to target TCI state is available at least once every TBD ms after the TCI state activation command is received and SNR of the SSB associated to TCI state ≥ -3dB; or]

UE behaviour for Scenario 5 is also not clarified, since current cell switch delay requirements is only defined for “known” TCI state condition, or if TCI state indicated in the LTM Cell switch command is activated. If the TCI state in cell switch command is not activated (with consideration of above exception), consequence is not clear at all. On the other hand, 38.213 Chapter 21 indicates LTM cell switch command can activate as well as indicate a TCI state, if it has not been activated by TCI state activation command.

- RAN4 to clarify, if TCI state in LTM Cell Switch command is “unknown”/not activated, UE should perform the cell switch with additional time for T/F tracking in the cell switch delay.

## LTM UE features

### Capability related to cell switch delay

Issue 4-5-1: Early ASN.1 decoding and validity/compliance check

< Agreement>

  - Update the feature group description of ‘early processing of an LTM candidate cell RRC configuration’ to ‘fast processing of an LTM candidate cell RRC configuration’

  - Add the following component(s):

    - maxServingAndCandidateCells: max number of serving and candidate cells

    - maxLTMCandidateConfig: the maximum number of LTMCandidateConfigs that UE can support fast processing

Due to the limited UE memory size, depending on the number of current serving cells and candidate cells across cell groups, the fast processing of LTM candidate cell RRC configuration (ltm-CandidateConfig IE) may or may not be applicable. In order to accommodate such a restriction, we can introduce two components, one for the number of candidate cell groups (the number of ltm-CandidateConfig IEs) and the other for the number of cells including both the current serving cells and candidate cells in ltm-CandidateConfig IE. Besides, the selection of candidate cell groups (ltm-CandidateConfig IEs) for the fast RRC processing can be based on so-called early DL synchronization (candidate TCI state activation) and UL synchronization (PDCCH-order PRACH).

- For ‘fast processing of an LTM candidate cell RRC configuration,’ RAN4 to adopt the following additional UE capabilities and applicable conditions:

- Add the following components:

  - maxLTMCandidateConfig (2nd component of 39-6)

  - maxServingAndCandidteCells (1st component of 39-6)

- Fast RRC processing is applicable to the following candidate cells (ltm-CandidateConfig):

  - The ltm-CandidateConfig IEs associated with at least one active TCI state

  - The ltm-CandidateConfig IEs associated with previously performed PDCCH-order PRACH.

  - If the number of the ltm-CandidateConfig IEs associated with active TCI state and PDCCH-order PRACH transmission is larger than maxLTMCandidateConfig, the ltm-CandidateConfig IEs for fast RRC processing are chosen in reverse chronological order of Candidate Cell TCI States Activation MAC CE and PDCCH-order PRACH, i.e. maxLTMCandidateConfig ltm-CandidateConfig IEs with the most recently activated TCI states and PDCCH-order PRACH transmission. And in case a tie-break rule is needed, the ltm-CandidateConfig associated with the most recent PDCCH-order PRACH transmission will be chosen.

  - The above applies only if each of the following conditions are fulfilled

    - The current serving cells and the cells inside the ltm-CandidateConfig, chosen by the above condition, across cell groups (i.e. MCG and SCG) is not larger than maxServingAndCandidteCells

    - The time gap from the slot where the UE received the candidate cell TCI state activation MAC CE to the slot where the UE received the LTM cell switch MAC CE is larger than THARQ+13ms, if the condition of ‘fast RRC processing’ is met by the candidate cell TCI state activation.

    - The time gap from the slot where the UE received the PDCCH triggering the PDCCH-order PRACH transmission to the slot where the UE received the LTM cell switch MAC CE is larger than NT,2+10ms, if the condition of ‘fast RRC processing’ is met by the PDCCH-order PRACH transmission.

According to the current UE capability structure of the feature, the UE can separately indicate its capabilities for the maximum number of candidate cells for early PRACH transmission and the maximum number of candidate cells for LTM RRM configuration. To maximize the benefit of LTM feature support, UE will need to report the same value for both capabilities. Otherwise, LTM candidate cells would need to be reconfigured via RRC to add or remove the early PRACH-related field from each LTM candidate cell based on the reported L1/L3 measurement results, which is undesirable due to significant RRC signalling overhead and latency.

Considering the limited UE capability in terms of memory and other resources to keep track of the time and frequency of all candidate cells to which early PRACH transmission was conducted, we propose a new UE capability for RACH-less LTM cell switch support. This capability would effectively limit the number of target cells for LTM cell switch based on the reverse chronological order of early PRACH transmission.

- RAN4 to define the following UE capability:

- RACH-less LTM cell switch can be conducted to one of ‘N’ cells to which the UE most recently transmitted the ‘PDCCH-order PRACH’ except for the cell configured as SCell.

  - N = {[1], 2, …, 7}, if not reported, N=8.

  - Granularity: Per UE

# Summary and Conclusion

In this contribution we have analysed RAN4 aspects for L1/L2 based inter-cell mobility and made following proposals.

- When one or more of TCI state activation commands are received at slot n, UE shall be able to finish the TCI state activation within slot n+ THARQ +3Nslotsubframe,µ+TTF + TSSB-proc. where TTF  is mentioned in the below table.

|  | TTF | Comments |
| --- | --- | --- |
| FR1 intra-frequency cell, known and unknown TCI state | max (Tfirst-SSB_TCI1, Tfirst-SSB_TCI2 .., Tfirst-SSB_TCIn) | Tfirst-SSB_TCIn is the time for first SSB associated to TCI state n. |
| FR1 inter-frequency cell without MG, known and unknown TCI state | max (Tfirst-SSB_TCI1, Tfirst-SSB_TCI2 .., Tfirst-SSB_TCIn) + (M-1)*TSSB | Tfirst-SSB_TCIn is the time for first SSB reception associated to TCI state n. M is the number of cells to activate the TCI states TSSB is the SSB burst periodicity. |
| FR1 inter-frequency cell with MG, known and unknown TCI state | TFirstMG + (M-1) *MGRP+MGL | TFirstMG is the time to start of first MG after slot n+THARQ+3ms, MGL is measurement gap length M is the number of cells to activate the TCI states MGRP is the MG repetition periodicity |
| FR2 intra-frequency cell, known TCI state | max (Tfirst-SSB_TCI1, Tfirst-SSB_TCI2 .., Tfirst-SSB_TCIn) + (M-1)*TSSB | Tfirst-SSB_TCIn is the time for first SSB reception associated to TCI state n. M is the number of cells to activate the TCI states TSSB is the SSB burst periodicity |
| FR2 inter-frequency cell without MG, known TCI state | max (Tfirst-SSB_TCI1, Tfirst-SSB_TCI2 .., Tfirst-SSB_TCIn) + (M-1)*TSSB | Tfirst-SSB_TCIn is the time for first SSB reception associated to TCI state n. M is the number of cells to activate the TCI states TSSB is the SSB burst periodicity |
| FR2 inter-frequency cell with MG, known TCI state | TFirstMG + (M-1) *MGRP+MGL | TFirstMG is the time to start of first MG after slot n+THARQ+3ms, MGL is measurement gap length M is the number of cells to activate the TCI states MGRP is the MG repetition periodicity |

- FR2 unknown TCI state activation requirements are not specified in Rel-18.

- Unknown TCI state activation requirements are applicable if the TCI state activated is based on the measurement report (e.g., L3-RSRP or L1-RSRP) within last 5 seconds and the SNR of the associated SSB is above -3dB.

- In FR1, for UE not supporting/configured with L1 measurement, one SSB occasion is needed from RAN4 requirement point of view for T/F fine tracking, if

    - the time gap between completion of TCI activation and cell switch command is larger than 480ms and the L3 measurement interval is larger than 160 ms, or

    - the time gap between early RACH transmission and cell switch command is larger than 480ms and the L3 measurement interval is larger than 160 ms.

- Interruption to both DL and UL duration before/after PDCCH-order LTM PRACH is extended by 1ms for the following case:

- The PDCCH-order PRACH is not fully contained in any of UE’s configured UL BWP(s) of active serving cells, and

- the number of RRC-configured LTM cells whose PRACH resources are not fully overlapping in the frequency domain is more than 2, and

- UE is configured with SRS carrier.

- RAN4 not to modify the serving cell measurement period by scaling it with NLayer for UE supporting RTD > CP.

- In L1-RSRP measurement report, for unmeasured candidate cells, UE reports DIFFRSRP_15 in Table 10.1.6.1-2.

- For the cell switch delay, no additional delay or conditions are needed for PL-RS measurement provided that the following condition are fulfilled:

- UE has reported L3-RSRP on the SSB associated with PL-RS before reception of LTM configuration and UE is configured to perform L3 or L1 measurements after LTM configuration.

- RAN4 to allow the LTM cell switch requirements to be applicable for unknown TCI state.

- We suggest modifying the TCI known condition to following

The target joint DL/UL TCI state or separate DL and UL TCI states in the LTM cell switch command are known if the following conditions are met:

[-	The TCI state is activated not more than TBD ms before the reception of the cell switch command and SNR of the SSB associated to TCI state is ≥ -3dB; where the TCI state is considered activated if the activated TCI state and target TCI state in the cell switch command are same or the SSB associated to target TCI state in cell switch command and the SSB associated to activated TCI state are same; or]

[-	The TCI state is activated before the reception of the cell switch command (where the TCI state is considered activated if the activated TCI state and target TCI state in the cell switch command are same or the SSB associated to target TCI state in cell switch command and the SSB associated to activated TCI state are same) and the SSB associated to target TCI state is available at least once every TBD ms after the TCI state activation command is received and SNR of the SSB associated to TCI state ≥ -3dB; or]

- RAN4 to clarify, if TCI state in LTM Cell Switch command is “unknown”/not activated, UE should perform the cell switch with additional time for T/F tracking in the cell switch delay.

- For ‘fast processing of an LTM candidate cell RRC configuration,’ RAN4 to adopt the following additional UE capabilities and applicable conditions:

- Add the following components:

  - maxLTMCandidateConfig (2nd component of 39-6)

  - maxServingAndCandidteCells (1st component of 39-6)

- Fast RRC processing is applicable to the following candidate cells (ltm-CandidateConfig):

  - The ltm-CandidateConfig IEs associated with at least one active TCI state

  - The ltm-CandidateConfig IEs associated with previously performed PDCCH-order PRACH.

  - If the number of the ltm-CandidateConfig IEs associated with active TCI state and PDCCH-order PRACH transmission is larger than maxLTMCandidateConfig, the ltm-CandidateConfig IEs for fast RRC processing are chosen in reverse chronological order of Candidate Cell TCI States Activation MAC CE and PDCCH-order PRACH, i.e. maxLTMCandidateConfig ltm-CandidateConfig IEs with the most recently activated TCI states and PDCCH-order PRACH transmission. And in case a tie-break rule is needed, the ltm-CandidateConfig associated with the most recent PDCCH-order PRACH transmission will be chosen.

  - The above applies only if each of the following conditions are fulfilled

    - The current serving cells and the cells inside the ltm-CandidateConfig, chosen by the above condition, across cell groups (i.e. MCG and SCG) is not larger than maxServingAndCandidteCells

    - The time gap from the slot where the UE received the candidate cell TCI state activation MAC CE to the slot where the UE received the LTM cell switch MAC CE is larger than THARQ+13ms, if the condition of ‘fast RRC processing’ is met by the candidate cell TCI state activation.

    - The time gap from the slot where the UE received the PDCCH triggering the PDCCH-order PRACH transmission to the slot where the UE received the LTM cell switch MAC CE is larger than NT,2+10ms, if the condition of ‘fast RRC processing’ is met by the PDCCH-order PRACH transmission.

- RAN4 to define the following UE capability:

- RACH-less LTM cell switch can be conducted to one of ‘N’ cells to which the UE most recently transmitted the ‘PDCCH-order PRACH’ except for the cell configured as SCell.

  - N = {[1], 2, …, 7}, if not reported, N=8.

  - Granularity: Per UE

# References

- R4-2410302 WF on mobility enhancement’s part 1, Mediatek

- RP-221799 “WID on Mobility enhancements”, MediaTek