3GPP TSG-RAN4 Meeting #112	R4-2413006

Maastricht, NL, Aug. 19  – Aug. 23, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 4881 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to 38.133 on LTM requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Maintenance on LTM requirements |  |  |  |  |  |  |  |  |
| Summary of change: |  | Maintenance of LTM requirements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Corresponding RRM requirement will be incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3, 8.25 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change 1

## 6.3	L1/L2-Triggered Mobility

### 6.3.1	LTM PCell Cell Switch

#### 6.3.1.1	Introduction

The purpose of LTM cell switch is to switch the PCell or PSCell to a target cell indicated in LTM cell switch command. The requirements in this section are applicable to LTM PCell switch.

The requirements in this clause are applicable to SA and NR-DC, and to both intra-frequency and inter-frequency LTM cell switch.

The requirements for inter-frequency cell switch to an FR2 target cell in this clause are only applicable, when

-	network has configured UE to perform L3 measurement with SSB index or L1 measurement for the target cell before the cell switch command, or

-	the SFN of the serving cell from which cell switch command is received and the SFN of the target cell are the same.

The requirements in this clause are applicable to SA for the following scenarios:

PCell switch to a neighboring LTM candidate cell

-	FR1 cell to FR1 cell

-	FR1 cell to FR2 cell

-	FR2 cell to FR2 cell

-	FR2 cell to FR1 cell

PCell switch to an LTM candidate cell that is a serving SCell in MCG

-	FR1 cell to FR1 cell

-	FR2 cell to FR2 cell

The requirements in this clause are applicable to NR-DC for the following scenarios:

PCell switch to a neighboring LTM candidate cell

-	FR1 cell to FR1 cell

PCell switch to an LTM candidate cell that is a serving SCell in MCG

-	FR1 cell to FR1 cell

#### 6.3.1.2	LTM Cell Switch delay

LTM cell switch delay DLTM is the delay from the end of the last TTI containing the MAC-CE command for cell switch until the time the UE transmits the first UL message on the target cell.

When the target cell and the target joint UL/DL TCI state or separate UL and DL TCI states in the MAC-CE LTM cell switch command are known, the LTM cell switch delay is defined as:

DLTM = Tcmd  + TLTM-interrupt

Where:

Tcmd equals to THARQ + 3ms, where THARQ is the timing between cell switch command and acknowledgement as specified in TS 38.213.

TLTM-interrupt is as stated in section 6.3.1.3.

The target cell in the LTM cell switch command is known if the following conditions are met:

-	During the last 5 seconds before the reception of the cell switch command:

-	The UE has sent a valid L1 or L3 measurement report for the target cell, and

-	One of the SSBs measured from the NR target cell configured for measurement remains detectable according to the cell identification conditions specified in clause 9.2 for intra-frequency cell and in clause 9.3 for inter-frequency cell,

-	One of the SSBs measured from the NR target cell configured for measurement remains detectable according to the cell identification conditions specified in clause 9.2 for intra-frequency cell and in clause 9.3 for inter-frequency cell,

Otherwise, the cell is unknown.

The target joint DL/UL TCI state or separate DL and UL TCI states in the LTM cell switch command are known if the following conditions are met:

[-	The TCI state in the cell switch command is activated not more than TBD ms before the reception of the cell switch command and SNR of the SSB associated to TCI state ≥ -3dB; where the TCI state is considered activated if the activated TCI state and target TCI state in the cell switch command are same or the SSB associated to target TCI state in cell switch command and the SSB associated to activated TCI state are same; or]

[-	The TCI state in cell switch command is activated before receiving the cell switch command (where the TCI state is considered activated if the activated TCI state and target TCI state in the cell switch command are same or the SSB associated to target TCI state in cell switch command and the SSB associated to activated TCI state are same) and the SSB associated to target TCI state is available at least once every TBD ms after the TCI state activation command is received and SNR of the SSB associated to TCI state ≥ -3dB; or]

-	During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target DL/UL TCI state to the completion of LTM cell switch, where the RS resource for L1-RSRP measurement is the RS in target DL/UL TCI state or QCLed to the target DL/UL TCI state

-	LTM cell switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement

-	The UE has sent at least 1 L1-RSRP report for the target DL/UL TCI state before the LTM cell switch command

-	The target DL/UL TCI state remains detectable during the LTM cell switching period

-	The SSB associated with the target DL/UL TCI state remain detectable during the cell switching period

-	SNR of the TCI state ≥ -3dB

Otherwise, the target joint DL/UL TCI state or separate DL and UL TCI state is unknown.

#### 6.3.1.3	Interruption time

The interruption time TLTM-interrupt is the time from the end of Tcmd stated in section 6.3.1.2  until the time the UE transmits the first UL message on the target cell.

TLTM-interrupt  = TLTM-RRC-processing + TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU ms,

Where:

TLTM-RRC-processing is the time for ASN.1 decoding and validity/compliance check for the RRC configuration of the LTM target cell indicated in the LTM cell switch command.

TLTM-RRC-processing = 0, if the UE supports [earlyDecodingAndValidityCheck] capability, and at least one of the following conditions is met:

-	The number of candidate cells in the LTM candidate cell configuration does not exceed [number of candidate cells for early ASN.1 decoding and validity check],

-	UE has received LTM candidate cell TCI state activation command for the target cell at least THARQ + 13 ms before the LTM cell switch command, and the number of candidate cells with TCI state(s) in LTM candidate cell active TCI state list does not exceed [number of candidate cells for early ASN.1 decoding and validity check],

-	UE has received PDCCH order for early RACH for the target cell at least NT,2+10 ms before the LTM cell switch command, where NT,2 is defined in section 8.1 of TS 38.213 [3]. [FFS further conditions]

-	[FFS: SCell is not part of the cell switch.]

Otherwise TLTM-RRC-processing = 10 ms.

TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change.

If the UE supports [faster LTM processing] capability, the value of TLTM-processing equals to

[faster intra-FR processing delay (10 ms or 15 ms)] for FR1 to FR1 and FR2 to FR2 LTM cell switch.

[faster inter-FR processing delay (20 ms or 30 ms)] for FR1 to FR2 and FR2 to FR1 LTM cell switch.

Otherwise, the value of TLTM-processing equals to

-	20 ms for FR1 to FR1 and FR2 to FR2 LTM cell switch.

-	40 ms for FR1 to FR2 and FR2 to FR1 LTM cell switch.

Editor’s note: FFS whether a smaller value can be considered in some scenarios or under certain conditions, when the UE does not support [faster LTM processing] capability.

Tfirst-RS is the time for fine time tracking and acquiring full timing information of the target cell.

TRS-proc is the time for SSB processing.

Tfirst-RS = 0 and TRS-proc= 0 under the following conditions:

-	The target TCI state indicated in the LTM cell switch command is in the serving cell active TCI state list, or

-	The UE is configured with LTM L1 intra- and/or inter-frequency measurements for the target cell, and

-	The target TCI state in the cell switch command is in the LTM candidate cell active TCI state list, and

-	the time gap between receiving the LTM candidate cell TCI state activation MAC-CE and the cell switch command is at least TCI state activation delay stated in section 8.x, and

- 	the time gap between receiving the LTM candidate cell TCI state activation MAC-CE and the cell switch command is not more than [160 ms], or

-	the measurement period of the SSB associated to target TCI state is not larger than 160 ms after the LTM candidate cell TCI state activation MAC-CE is received, or

-	The target cell is an FR1 cell, and the UE is not configured with LTM L1 intra- and/or inter-frequency measurements for the target cell, and

-	The target TCI state in the cell switch command is in the LTM candidate cell active TCI state list, and

-	the time gap between receiving the LTM candidate cell TCI state activation MAC-CE and the cell switch command is at least TCI state activation delay stated in section 8.x, and not more than TCI state activation delay stated in section 8.x + [480 ms], or the L3 measurement interval is smaller than 160 ms, or

-	The time gap between the latest PDCCH ordered RACH preamble transmission on the target cell and the cell switch command is not more than [480 ms] or the L3 measurement interval is shorter than 160 ms.

Otherwise,

Tfirst-RS is the time to the first SSB transmission on the target cell [after Tcmd].

Editor’s note: FFS whether TRS transmission is also considered.

TRS-proc = 2 ms.

TLTM-IU is the interruption uncertainty during LTM cell switch.

For RACH-based LTM cell switch, TLTM-IU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TLTM-IU can be up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in the table 8.1-1 of TS 38.213 [3].

For RACH-less LTM cell switch, TLTM-IU_is the uncertainty on transmitting the first uplink transmission on the target cell.

End of Change 1

Start of Change 2

## 8.25	TCI state activation for LTM candidate cell

### 8.25.1	Introduction

The requirements in this clause apply for a UE configured with one or more TCI state configurations on LTM candidate cell. UE shall complete the activation of TCI state within the delay defined in this clause after receiving a MAC CE indicating LTM candidate cell TCI state activation before cell switch command as specified in clause 6.1.3.75 of TS 38.321.

The requirements in this clause apply for

-	Known TCI state in FR1 and FR2, and

-	Unknown TCI state in FR1, provided the following conditions are met:

-	UE has reported L3 measurement result with the associated SSB index of the TCI state within 5 seconds before the LTM TCI state activation command.

-	SNR of the associated SSB is above -3dB.

### 8.25.2	Known TCI state conditions

The candidate TCI state in the TCI state activation list is known if the following conditions are met:

-	During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target downlink TCI state to the completion of active downlink TCI state activation, where the RS resource for L1-RSRP measurement is the RS in target downlink TCI state or QCLed to the target downlink TCI state

-	The MAC CE command indicating the activation of downlink TCI state is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement

-	The UE has sent at least 1 valid L1-RSRP report for the target downlink TCI state before the MAC CE command is received.

-	The target downlink TCI state remains detectable during the downlink TCI state activation period

-	The SSB associated with the downlink TCI state remain detectable during the downlink TCI activation period

-	SNR of the downlink TCI state ≥ -3dB

Otherwise, the candidate TCI state is unknown.

### 8.25.3	SSB based TCI state activation delay

In FR1, when the UE receives one or more of LTM candidate cell TCI state activation command at slot n, the UE shall have completed the LTM TCI state list update in slot n + THARQ +3Nslotsubframe,µ + (TTF + TSSB-proc) / NR slot length. Where TTF  is mentioned in the below table.

Table 8.25.3-1: Time frequency tracking delay for LTM TCI state activation

| Cell and TCI state condition | TTF |
| --- | --- |
| FR1 intra-frequency cell, known and unknown TCI state | max (Tfirst-SSB_TCI1, Tfirst-SSB_TCI2 .., Tfirst-SSB_TCIn) |
| FR1 inter-frequency cell without MG, known and unknown TCI state | max (Tfirst-SSB_TCI1, Tfirst-SSB_TCI2 .., Tfirst-SSB_TCIn) + (M-1)*TSSB |
| FR1 inter-frequency cell with MG, known and unknown TCI state | TFirstMG + (M-1) *MGRP+MGL |
| FR2 intra-frequency cell, known TCI state | max (Tfirst-SSB_TCI1, Tfirst-SSB_TCI2 .., Tfirst-SSB_TCIn) + (M-1)*TSSB |
| FR2 inter-frequency cell without MG, known TCI state | max (Tfirst-SSB_TCI1, Tfirst-SSB_TCI2 .., Tfirst-SSB_TCIn) + (M-1)*TSSB |
| FR2 inter-frequency cell with MG, known TCI state | TFirstMG + (M-1) *MGRP+MGL |
| Where, - Tfirst-SSB_TCIn is the time to first SSB occasion after slot n + THARQ + 3Nslotsubframe,µ. - M is the number of cells to activate the TCI states. - TSSB is the SSB burst periodicity - TFirstMG is the time to start of first MG after slot n + THARQ + 3Nslotsubframe,µ - MGL is measurement gap length - MGRP is the MG repetition periodicity |  |

3Nslotsubframe,µ

Where:

-	THARQ is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3].

-	TSSB-proc = 2 ms.

3Nslotsubframe,µ

3Nslotsubframe,µ

-	TSSB-proc = 2 ms.

End of Change 2