3GPP TSG-RAN WG4 Meeting # 112	R4-2413011

Maastricht, Netherlands, 19 Aug.  2024 – 23 Aug. 2024

Agenda Item:	5.27.2

Source: 	Ericsson

Title:	Performance requirements for TDCP

Document for:	Discussion

# Introduction

In this contribution we provide our views on the RRM performance requirements for Rel-18 TDCP measurements.

# Discussion

In the previous meeting following agreements were made for the TDCP test case (TC) definition:

Issue 2-1-1: Test metric of TDCP test cases:

< Agreement>	:

- Lower Doppler (10Hz, 20dB): reported TDCP index is no bigger than a certain value (e.g., 6), with at least 80% probability

  - TC1 for low doppler condition+15kHz SCS FDD: value 6

  - TC2 for low doppler condition+30kHz SCS TDD: value 5

- Higher Doppler

  - TC3 for high doppler condition+15kHz SCS FDD, 300 Hz, SNR=[10]dB

    - Reported TDCP index is larger than 8, with at least 80% probability

  - TC4 for high doppler condition+30kHz SCS TDD, 300Hz, SNR=[10]dB

    - Reported TDCP index is [equal to or] larger than 6, with at least 80% probability

One open issue after last meeting is the reported TDCP index for TC4. In last meeting the TDCP reported values are discussed and chooses as a package for TC1 to TC4, where the configuration (Doppler, SCS, and SNR) and reported values are chosen to be non-overlapping among the test cases. Hence, we suggest keeping the reported value to be greater than 6 (removing equal to 6) in the TC4 reported value.

- For TC4, reported TDCP index is larger than 6, with at least 80% probability

# Summary and Conclusion

In this contribution we analyzed the methods to define TDCP accuracy requirements and made following proposals.

- For TC4, reported TDCP index is larger than 6, with at least 80% probability.

# References

[1] 	R4-2410317

# Annex

| Doppler [Hz] | SNR [dB] | Companies results |  |  |  |  |  | Ideal value |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 5 | 0,7028 | 0,7243 | 0,902291 | 0,995415 | 0,983412 | 0,7069 | 0.99975 |
|  | 10 | 0,8715 | 0,8757 | 0,970982 | 0,996408 | 0,992663 | 0,8881 |  |
|  | 15 | 0,9516 | 0,9587 | 0,991936 | 0,994709 | 0,996049 | 0,9611 |  |
|  | 20 | 0,9809 | 0,9862 | 0,997 | 0,995189 | 0,998341 | 0,9873 |  |
| 30 | 5 |  | 0,7166 | 0,903111 | 0,987968 | 0,981535 | 0,7106 |  |
|  | 10 |  | 0,8811 | 0,968989 | 0,990522 | 0,991117 | 0,8862 |  |
|  | 15 |  | 0,9552 | 0,989825 | 0,984471 | 0,994646 | 0,9609 |  |
|  | 20 |  | 0,9842 | 0,995 | 0,983903 | 0,997191 | 0,987 |  |
| 200 | 5 | 0,6182 | 0,5908 |  | 0,853547 | 0,937058 | 0,6747 | 0.90371 |
|  | 10 | 0,8002 | 0,7646 |  | 0,85233 | 0,949576 | 0,8557 |  |
|  | 15 | 0,895 | 0,8532 |  | 0,856702 | 0,952684 | 0,9339 |  |
|  | 20 | 0,9322 | 0,8935 | 0,902 | 0,855079 | 0,954907 | 0,9592 |  |
| 300 | 5 | 0,5976 | 0,551 |  | 0,768931 | 0,893131 | 0,644 | 0.78996 |
|  | 10 | 0,7648 | 0,7138 |  | 0,772939 | 0,90208 | 0,8031 |  |
|  | 15 | 0,8555 | 0,7892 |  | 0,778584 | 0,905922 | 0,9019 |  |
|  | 20 | 0,8913 | 0,8216 | 0,807 | 0,782067 | 0,90851 | 0,9264 |  |