3GPP TSG RAN WG4 Meeting #112															R4-2413013

Maastricht, NL, 19 Aug. 2024 – 23 Aug. 2024

Agenda Item:	5.29.1

Source: 	Ericsson

Title:	Remaining issues on NES general

Document for:	Discussion

# Introduction

In last meeting, RAN4 achieved agreements on most open issues. In this contribution, we continue to discuss the remaining issue on NES.

- SSB less SCell activation for inter-band collocated CA

In last meeting following agreement is reached for the power difference side condition to support SSB less SCell on inter-band CA.

Clarify that EPRE difference is based on the EPRE normalized by SCSs of SSB of reference Cell and A-TRS/P-TRS of SSB-less Cell. Only change [9] dB to [12] dB in this meeting. Further discuss whether and how to support larger EPRE difference in next meeting: - Option 1: Clarify that EPRE difference is smaller or equal to [12] dB + ¦20*log (f1/f2)¦ - Margin, where f1 and f2 is the center frequency of reference Cell and SSB-less Cell and the value of Margin is FFS. - Option 2: Further increase [12] dB and the value is FFS - Option 3: Update the requirements with one more P-TRS sample for larger EPRE difference Other options are no precluded.

We provide our views on the different options which are on the table.

Option 1:

In previous meetings, when EPRE was considered for the metric for received power difference, it was agreed under the assumption that UE can perform the pre-compensation that was arising from carrier frequency and the BW of the SSB and TRS. In previous meetings there were some concerns raised regarding the PL precompensation calculation may be UE implementation dependent. Some UE may clacualte using free space progation model or some other UE may use some other sophistaicated method to estimate the PL comensation.  To allow different UE implementations for calculating the PL difference arising from carrier frequency and BW difference, option 1 allows UE implementation margin. If we go with option 1, we need to agree on the margin value.

Margin value: We think agreeing the margin value may not be very easy as some UE may be implementing one method of PL estimation, and some other UE may be estimating PL using some other method. Ideally, we should do simualtaions and see what the value for the margin is. However, since the WI is already completed, doing the simulations for determining margin may be not feasible.

Observation 1: We think option 1 is not feasible from the workload perspective.

Option 2:

We think the intention behind option 2 is to remove the wording of precompensation and use a larger value (e.g., such as 25 dB) with out UE doing precompensation based on the carrier frequency and BW. If we recall the discussion, initial RAN4 disucssion started with this and in this approach, we need to consider the band combo that are separated across the FR1 such as 700 MHz and 2.6 GHz or more. In this case also progation delay difference due to frequency difference can be quite large and agreeing on a value that covers all the band combo is not an easy task.

Observation 2: We think option 2 is not feasible from the workload perspective as we need to consider different band combination across the FR1 to arrive at a receive power difference without precompensation.

Option 3:

We think option 3 may not work well if one additional TRS sample is needed when the EPRE is more than certain threshold. This is mainly because UE experienced receive power difference (RPD) is not known at the NW and NW does not know whether UE complete the SCell activation with two TRS samples or needs one additional sample. This results in dynamic SCell activation requirement which is not known at the NW.

Observation 3: Option 3 can result in dynamic requirement which is not known at NW.

If we look at the workload and pros and cons for going to any of the above option, we think RAN4 should agree on the EPRE difference of 12 dB with precompensation in the core part and deifne a margin in the test case to allow different UE implementations to pass the test.

- RAN4 to agree that EPRE side condition for reference cell and SSB-less SCell as 12 dB while considering the precompensation.

Other open issue that was discussed but not reached consensus was relation of rel-18 SSB less SCell and Rel-15 intra-band SSB less SCell. Following WF is agreed.

Issue 1-1-5: Relation to R15 intra-band SSB-less Agreement: From RAN4 understanding, for UE supports both R18 inter-band SSB-less and R15 intra-band contiguous SSB-less, - It is not expected that the to-be-activated SCell is configured with QCL source to both intra-band contiguous and inter-band Cells [when the Rel-18 reference cell indication is not configured]. - When R18 reference cell indication is configured, network configure with QCL source to both intra-band contiguous and inter-band Cells, there is no RAN4 agreement on whether Rel-15 or Rel-18 requirement is applied. The above RAN4 understanding does not intend to trigger any further RAN2 discussion.

When UE is configured with both Rel-18 inter-band SSB less SCell and Rel-15 intra-band SSB less SCell, we think though there is explicit indication on the rel-18 inter-band SSB less SCell about the reference cell for QCL source, we think the rel-15 SSB less SCell activation takes 3ms to complete and rel-18 SSB less SCell activation takes additional time for obtaining AGC and fine timing. From this perspective finshing the SCell activation earlier is also useful from the UE to enjoy higher data rates.

- When R18 reference cell indication is configured, network configure with QCL source to both intra-band contiguous and inter-band Cells, SSB less activation should follow Rel-15 requirement.

- Summary

In this contribution, we have discussed the MUSIM gaps requirements. Based on the discussions, we have made following proposals and observations:

- RAN4 to agree that EPRE side condition for reference cell and SSB-less SCell as 12 dB while considering the precompensation.

- When R18 reference cell indication is configured, network configure with QCL source to both intra-band contiguous and inter-band Cells, SSB less activation should follow Rel-15 requirement.

- References

[1]. R4-2410269, “WF on RRM requirements for NR network energy saving”, Huawei, HiSilicon