3GPP TSG RAN WG4 Meeting #112							R4-2413015

Maastricht, NL, 19 Aug. 2024 – 23 Aug. 2024

Agenda Item:	5.29.2

Source:	Ericsson

Title:	Discussion on NES test cases

Document for:	Discussion

# Introduction

In previous meeting [1], it was discussed on how to configure the receive power difference side condition in the conducted test for FR1 as there is no propagation delay difference that can be tested in conducted test. There was proposal to configure the EPRE as [12db] + ΔPL, where ΔPL is the propagation delay difference UE supposed to pre-compensate which was arising from carrier frequency difference and BW difference.

How to calculate and reflect this ΔPL in the test:

The ΔPL can be calculated based on the exact band combinations used in the test. PL difference is based on the carrier frequency difference and BW difference. We suggest using the free space propagation delay difference to compute the ΔPL. In last meeting companies raised concern that, in the UE implementation, UE may calculate the pathloss based on different mechanism than the free space propagation model and apply different pathloss delay than the free space propagation. When UE can estimate the pathloss delay better than the free space propagation delay, by applying different value in the test may make the UE fail the test. However, we are not sure if this is such a big issue as if UE estimated PL by few dB than what is applied in test, we do not think it would alter the test result.

Having said that different UEs may implement PL pre compensation using different methods, we think it is reasonable to have a margin define the test case.

- RAN4 to set EPRE difference as 12 dB + ΔPL - margin in the test cases

- RAN4 to use free space propagation delay difference to compute the ΔPL based on the carrier frequency difference and BW difference.

# Conclusion

In the contribution, we discuss the test case design for NES. We have the following proposals:

- RAN4 to set EPRE difference as 12 dB + ΔPL - margin in the test cases

- RAN4 to use free space propagation delay difference to compute the ΔPL based on the carrier frequency difference and BW difference.

# Reference

[1] R4-2410269, “WF on RRM requirements for Netw_Energy_NR”, Huawei