3GPP TSG RAN WG4 Meeting #112	R4-2413018

Maastricht, NL, 19 Aug. 2024 – 23 Aug. 2024

Agenda Item:	8.23.2

Source: 	Ericsson

Title:	Discussion Rel-19 LTM RRM requirements

Document for:	Decision

# Introduction

In this contribution we analyze the potential RAN4 RRM impacts for NR mobility enhancements Phase 4.

# Discussion

WID is copied below for reference [1]:

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support - Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - RAN1 and RAN2 to progress independently on the event triggered measurements objectives of their respective MIMO and Mobility enhancement WIs. Review progress at RAN#105 to see if any modification of objectives is required to avoid/manage any overlap in the work - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management, and/or other necessary physical layer operations on candidate cells before LTM [RAN1] - Specify support of conditional LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Prioritize intra-CU LTM - Checkpoint to review objective at RAN#105. RAN WG work to not start before this checkpoint - Specify RRM requirements related to the above objectives as necessary [RAN4]

From the above WID, we can observe that RAN4 scope depends on the RAN2/1 specification introduced for each objective. In the subsequent sections we look at each objective and the corresponding RAN4 impact.

## Objective 1: Support for inter-CU LTM

Objective of the WID is copied here for further analysis.

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3]

  - Prioritize the case when CU is acting as MN when DC is not configured

  - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases:

    - As secondary priority, support the case where CU is acting as SN and MN is unchanged

    - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released

  - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM

    - Coordination with SA3 needed with respect to security key handling

  - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support

RAN4 introduced following requirements as part of the Rel-18 LTM requirements definition:

- Cell switch delay and interruption requirements

- L1-RSRP measurement requirements

- PDCCH order-based RACH delay and interruption requirements

- Early TCI state activation requirements

Since the above specification work is between CUs, the UE physical layer and baseband is agnostic to the inter-CU, we do not foresee RAN4 impact on the above mentioned RRM requirements due to addition of inter-CU scenario.

- Intra-CU LTM requirements to be reused for inter-CU LTM unless some issue is identified.

## Objective 2: Measurements related enhancements for purpose of supporting LTM

- Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1]

  - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM

  - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1]

    - RAN1 and RAN2 to progress independently on the event triggered measurements objectives of their respective MIMO and Mobility enhancement WIs. Review progress at RAN#105 to see if any modification of objectives is required to avoid/manage any overlap in the work

  - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management, and/or other necessary physical layer operations on candidate cells before LTM [RAN1]

In Rel-19, RAN2 introduced event triggered L1 measurement reporting to reduce the signalling overhead arising from the periodic LTM CSI reports in the UL. Though RAN2 is still discussing the event triggered measurement reporting, based on the RAN2 agreements, we summarize the framework for UE initiated beam report in figure 1.

Figure 1: Framework for event triggered LTM reporting

As shown in figure 1, event triggered measurement reporting framework can be divided into configuration aspects and UE actions aspects. From the RAN2 agreements, we can observe that RAN2 is still discussing the resource configuration, event configuration, events definition, and the report configuration. As the configuration aspects are specified in RAN2/1, RAN4 do not need to discuss the configuration aspects and RAN2 progress may not have impact on RAN4.

UE actions: From the figure 1, we think following UE actions may have impact on the RAN4 requirements and needs to be discussed in RAN4.

- L1 measurements for event triggered beam report

- Event evaluation

- Measurement Report transmission

  - Indicate gNB about the event using UL signal (details are FFS in RAN2 and RAN1)

  - Get the grant from the NW for LTM CSI report (details are FFS in RAN2 and RAN1)

  - Transmit L1 Measurement report (details are FFS in RAN2 and RAN1)

L1 measurements for event triggered beam reporting

RAN2 agreed to support at least L1-RSRP for the event triggered beam report. Further Rel-19 needs to specify the CSI-RS based measurement requirements. Since the event framework and event evaluation criteria should be same for CSI-RS and SSB, we think RAN4 requirements for event triggered LTM reporting should be applicable for SSB and CSI-RS.

- RAN4 to specify the requirements for SSB based event triggered L1-RSRP reporting, and CSI-RS based event triggered L1-RSRP reporting.

Event evaluation:

LTM L1-RSRP is used for mobility management and periodic L1-RSRP can be configured with periodicity of the SSB or CSI-RS. When the report is configured with same periodicity as SSB, NW generally gets one measurement report for each measurement occasion. Since the LTM L1 measurement report is used for mobility decisions, UE should be able to send the report as soon as event condition is met. Event triggering conditions can be changed as fast as the measurement occasions. UE is expected to evaluate the event after every measurement occasion to check if any of the configured event is met. If the time between measurement occasion and reporting occasion is short UE may not be ready to transmit on the first reporting occasion. In this case UE may need to wait for next transmission opportunity of the reporting occasion. To avoid the scheduling of the reporting occasion so close to the measurement occasions, RAN4 should define the event evaluation time.

- RAN4 to discuss the event evaluation time (i.e., the minimum time required to complete the event evaluation and be ready to send on the first reporting occasion from the measurement occasion)

Measurement Report transmission

Once the event is met, as shown in figure 1, UE needs to send the measurement report to the NW. The reporting procedure and the steps involved in the reporting procedure is FFS and being discussed in other WG. The impact of the reporting procedure on the RAN4 requirements can be revisited after further progress in RAN1/2.

CSI-RS based measurement requirements:

Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management, and/or other necessary physical layer operations on candidate cells before LTM

Main WG responsible for defining CSI-RS based measurements for LTM is RAN1 and RAN1 has not yet started discussing this objective as the RAN1 start for Rel-19 LTM is Aug 2024. Hence, RAN4 to wait for sufficient RAN1 progress to define CSI-RS based LTM L1-RSRP requirements.

- RAN4 to define CSI-RS based LTM L1-RSRP requirements after sufficient progress is achieved in RAN1.

## Objective 3: Specify support of conditional LTM

- Specify support of conditional LTM [RAN2, RAN3, RAN1]

  - Specify UE evaluated conditions for triggering LTM

  - Aim to support conditional LTM including subsequent LTM

  - Prioritize intra-CU LTM

  - Checkpoint to review objective at RAN#105. RAN WG work to not start before this checkpoint

Since the checkpoint to review objective is at RAN#105 and RAN WG work to not start before this checkpoint, we can revisit this after RAN#105.

- RAN WG work to not start before this checkpoint.

# Summary and Conclusion

In this contribution we analyzed potential RAN4 RRM requirements impact and made following observations and proposals.

- Intra-CU LTM requirements to be reused for inter-CU LTM unless some issue is identified.

- RAN4 to specify the requirements for SSB based event triggered L1-RSRP reporting, and CSI-RS based event triggered L1-RSRP reporting.

- RAN4 to discuss the event evaluation time (i.e., the minimum time required to complete the event evaluation and be ready to send on the first reporting occasion from the measurement occasion).

- RAN4 to define CSI-RS based LTM L1-RSRP requirements after sufficient progress is achieved in RAN1.

# References

[1] 	RP-241515, New WID: NR mobility enhancements Phase 4