# 3GPP TSG-RAN WG4 Meeting #112		    R4-2413027

# Maastricht, Netherlands, 19th – 23rd August, 2024

Source: 	Huawei, HiSilicon

Title:	On Rel-19 work for band n28

Agenda Item:	7.20.3

Document for:	Approval

# 1	Introduction

The objectives of this WI are to introduce requirements for UE Power Class 2 and 40MHz Channel Bandwidth for NR band n28 as optional features, including the following aspects:

- Introduce requirements for PC2 and those associated with following bullets

  - Both 1Tx and 2Tx PC2 are supported

  - UL-MIMO is supported.

  - Specify A-MPR requirements for PC2

- Introduce 40MHz Channel bandwidth located in UL: 703 - 743MHz and DL: 758 - 798MHz for band n28 supporting PC3 and PC2.

  - Allow flexible channel allocations within the frequency range (UL: 703 - 743MHz and DL: 758 - 798MHz) for BW<40MHz.

- Release independence of UE 40MHz Channel bandwidth to Rel-15 is anticipated.

- No changes to the existing requirements (38.101-1 v18.5.0) (including A-MPR values and RB allocation regions) for power class 3 with existing channel bandwidths up to and including 30MHz to avoid impacts on existing power class 3 networks. In the event of new regulatory requirements, this assumption can be revisited.

A Rel-19 WI is set up to introduce PC2 and 40MHz CBW for band n28 [1]. The key objectives are as follows:

# 2	Discussion

The following issues have been discussed for band n28 during the Rel-18 WI:

- PC2 RSD for 1Tx and 2Tx for BW=5~30MHz

- PC2 A-MPR requirements for NS_18

- PC2 A-MPR requirements for NS_17

- Requirements for spurious emissions for UE co-existence

The last three issues were initially discussed under dual-duplexer assumption, and later considered for full-duplexer implementations.

The PC2 RSD requirements for band n28 are completed for BW=5~30MHz [2].

For NS_18, the PC2 A-MPR for BW=5MHz to 30MHz have been agreed and captured in WF [3].

For NS_17, it has been agreed that no A-MPR is needed for dual-duplexer implementations [4].

For full-band duplexer implementations, however, there’re still open issues regarding NS_17 and UE co-existence requirements.

Proposal 1: The study and agreements completed in Rel-18 for band n28 can be reused as much as possible, including PC2 RSD, PC2 A-MPR for NS_18 (BW=5~30MHz).

Since the new CBW of 40MHz is added, the PC3 REFSENS should be determined first, and then the PC2 RSDs are decided. It’s unclear whether 40MHz should be added to NS_18.

To evaluate the REFSENS for 40MHz, the same 25 RB UL configuration can be reused.

Table 7.3.2-3: Uplink configuration for reference sensitivity

| Operating Band | SCS | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n28 | 15 | 15 | 25 | 251 | 251 | 251 | 251 | 251 |  |  | FDD |
|  | 30 |  |  | 101 | 101 | 101 | 101 | 101 |  |  |  |

In summary, the work to be completed in Rel-19 include:

- 40MHz CBW triggered

  - PC3 REFSENS for 40MHz

  - PC3 A-MPR for NS_18 if 40MHz is added to NS_18

- PC2 triggered

  - PC2 RSD for CBW=3~40MHz

  - PC2 A-MPR for NS_17 and NS_18

It has been discussed intensively in Rel-18 about the duplexer implementations [5-10]. The majority view is that both dual-duplexer and full-band duplexer should be allowed without compromising the existing network performance.

Proposal 2: Both dual-duplexer and full-band duplexer implementations should be supported without compromising the existing network performance.

It has been shown that UEs with dual-duplexers require zero A-MPR for NS_17 while UEs with full-band duplexer need more back-off. Furthermore, dual-duplexer implementations could provide extra attenuation to UL in adjacent bands, which is crucial for L-L CA/DC such as n18/18+n28/28, n5/5+n28/28.

Due to the considerable difference in Tx and Rx performance between the two types of duplexers, it’s important for the network to be aware of the distinction and optimise the scheduling accordingly. The signalling via modifiedMPR was proposed in [11], which indirectly indicates the underlying duplexer type. Other direct signalling approaches should not be excluded at this stage.

Proposal 3: Allow a UE to report to the network about its underlying duplexer implementation.

# 3	Conclusion

In this paper, we have performed initial analysis on the Rel-19 WI on band n28, and made the following proposals:

Proposal 1: The study and agreements completed in Rel-18 for band n28 can be reused as much as possible, including PC2 RSD, PC2 A-MPR for NS_18 (BW=5~30MHz).

Proposal 2: Both dual-duplexer and full-band duplexer implementations should be supported without compromising the existing network performance.

Proposal 3: Allow a UE to report to the network about its underlying duplexer implementation.

# References

- RP-241660 New WID: Introduction of Power Class 2 and UE 40MHz Channel Bandwidth in NR band n28, CMCC, CBN

- R4-2306544 WF on FDD HPUE Basket WIs, China Unicom, RAN4#106bis-e

- R4-2310245 WF on PC2 A-MPR for band n8 and n28, Huawei, HiSilicon, Apple, China Unicom, RAN4#107

- R4-2321715 WF on HPUE for FDD bands, China Unicom, RAN4#109

- R4-2400170 On n28 with full band duplexer, Apple, RAN4#110

- R4-2404878 Remaining issues for FDD HPUE A-MPR, Huawei, HiSilicon, RAN4#110bis

- R4-2405712 PC2 A-MPR for n7 and n28, Qualcomm, RAN4#110bis

- R4-2405962 n28 NS_17 PC2, Skyworks, RAN4#110bis

- R4-2403629 WF on HPUE for FDD bands, China Unicom, RAN4#110

- R4-2409638 PC2 HPUE for band n28, Huawei, HiSilicon, RAN4#111

- R4-2408804 CR to TS 38.101-1: Addition of PC2 for n28 and n83, Qualcomm Inc., Samsung, KDDI Corporation, Rakuten Mobile