# 3GPP TSG-RAN WG4 Meeting #112	R4-2413029

# Maastricht, Netherlands, 19th – 23rd August, 2024

Source: 	Huawei, HiSilicon

Title:	Discussion on increasing UE transmission power limit

Agenda Item:	8.1.1.1.3

Document for:	Approval

# 1	Introduction

A WF [1] was agreed in RAN4#111. In the following, we will share our views on the two open issues listed in the WF, i.e. R19 scenarios and the method for increasing transmission power limit.

# 2	Discussion

For convenience, the WID objective is duplicated below:

Investigate and if feasible, support increasing UE transmission power limit up to the sum of maximum output power per band for NR inter-band uplink CA and EN-DC HPUE with the different existing power classes which have already been specified

Observation 1: The WI scope is to study UL bands with different existing power classes. Except that, the WID imposes no restrictions on power class configurations, 2Tx/3Tx, FDD/TDD.

In Rel-18, the higherPowerLimit-r17 feature was excluded from the 3Tx inter-band CA/DC WI. It can be studied now in the Rel-19 WI.

Observation 2: Increasing UE transmission power limit for 3Tx band combinations can be studied in the Rel-19 WI.

Proposal 1: Consider the following list of scenarios for increasing UE transmission power limit as shown in Table 1 below.

Table 1: A list of applicable scenarios for increasing UE transmission power limit

| CA power class | Power class configuration Band A + Band B |  | Power limit increase (dB) | 2Tx | 3Tx |
| --- | --- | --- | --- | --- | --- |
| PC3 | PC3 | PC5 | 1.8 | Completed in R18 | N/A |
| PC2 | PC2 | PC3 | 1.8 | Completed in R17 | R19 |
|  |  | PC5 | 1.0 | R19 | R19 |
| PC1.5 | PC1.5 | PC2 | 1.8 | N/A | R19 |
|  |  | PC3 | 1.0 | N/A | R19 |
|  |  | PC5 | 0.5 | N/A | R19 |

As pointed out before, 26dBm+29dBm = 30.8dBm, 23dBm+29dBm = 30.0dBm, which are close to PC1 of 31dBm. Generally, PC1 is not targeted for handheld UEs.

Proposal 2: Increasing the total Tx power limit beyond PC1.5 is only for FWA UEs, not for handheld UEs.

For inter-band UL CA, there is no real need to define the CA power class, as long as the power class for each component band is indicated, e.g. via ue-PowerClassPerBandPerBC. This is because that each UL band has its own PA and can perform power control independently. On the contrary, if a CA power class is defined, the Scell may have to be dropped when the Tx power of the Pcell reaches the limit set by the CA power class.

Ideally, the CA power class could be removed from the PCMAX limit for inter-band UL CA. And the PCMAX can be simply bounded by the sum of power class per band (10 log10 ∑ pPowerClass,c). In this way, even the higherPowerLimit-r17 indication is redundant.

Observation 3: For inter-band UL CA, the CA power class is not must-have. The PCMAX for CA can simply be bounded by the sum of the power limit per band (10 log10 ∑ pPowerClass,c).

For backward compatibility, the limit (PPowerClass,CA) set by the CA power class may be kept in the PCMAX bounds when ΔPPowerClass, CA ≠ 0.

Proposal 3: To increase the transmission power limit, consider directly replacing the limit (PPowerClass,CA) set by the CA power class with the sum of the power limit per band (10 log10 ∑ pPowerClass,c) in PCMAX_H or both PCMAX_H and PCMAX_L.

# 3	Conclusion

In this paper, we have shared our views on the open issues listed in the WF for increasing UE transmission power limit. The following observations and proposals are made:

Observation 1: The WI scope is to study UL bands with different existing power classes. Except that, the WID imposes no restrictions on power class configurations, 2Tx/3Tx, FDD/TDD.

Observation 2: Increasing UE transmission power limit for 3Tx band combinations can be studied in the Rel-19 WI.

Proposal 1: Consider the following list of scenarios for increasing UE transmission power limit as shown in Table 1 below.

Table 1: A list of applicable scenarios for increasing UE transmission power limit

| CA power class | Power class configuration Band A + Band B |  | Power limit increase (dB) | 2Tx | 3Tx |
| --- | --- | --- | --- | --- | --- |
| PC3 | PC3 | PC5 | 1.8 | Completed in R18 | N/A |
| PC2 | PC2 | PC3 | 1.8 | Completed in R17 | R19 |
|  |  | PC5 | 1.0 | R19 | R19 |
| PC1.5 | PC1.5 | PC2 | 1.8 | N/A | R19 |
|  |  | PC3 | 1.0 | N/A | R19 |
|  |  | PC5 | 0.5 | N/A | R19 |

Proposal 2: Increasing the total Tx power limit beyond PC1.5 is only for FWA UEs, not for handheld UEs.

Observation 3: For inter-band UL CA, the CA power class is not must-have. The PCMAX for CA can simply be bounded by the sum of the power limit per band (10 log10 ∑ pPowerClass,c).

Proposal 3: To increase the transmission power limit, consider directly replacing the limit (PPowerClass,CA) set by the CA power class with the sum of the power limit per band (10 log10 ∑ pPowerClass,c) in PCMAX_H or both PCMAX_H and PCMAX_L.

# References

- R4-2406583 WF on HPUE UE for UL CA and EN-DC, Samsung, RAN4#110bis

- R4-2404885 Initial consideration on R19 UE RF enhancements for HPUE in TN, Huawei, HiSilicon, RAN4#110bis