3GPP TSG-RAN WG4 Meeting #112	R4- 2413030

Maastricht, NL, 19 - 23 August, 2024

Source: 	Huawei, HiSilicon

Title:	On the RF Requirements for Ambient IoT Devices

Agenda Item:	8.20.3.2

Document for:	Approval

# 1	Introduction

As per the Ambient IoT SID [1], RAN4 is tasked to study the following issues:

  - Coexistence study of Ambient IoT and NR/LTE.

  - RF requirements study for Ambient IoT:

    - Ambient IoT BS transmission and reception

    - Ambient IoT Device, as per the General Scope, transmission and reception

    - Intermediate node (UE), as per the General Scope, transmission and reception

Furthermore, based on the WF [2] on impacts of A-IoT on RF requirements agreed in RAN4#111, we share our views on the potential RF requirements for A-IoT system parameters as well as Tx and Rx from the perspective of A-IoT devices.

# 2	Discussion

### 2.1 System Parameters

A device that supports a given operating band should be able to receive in the DL frequency range of the band and backscatter (device 1/2a) or transmit (device 2b) in the UL frequency range of the band. For D2T2, the device might need to operate in the UL frequency range of the band for both reception and transmission.

For example, band 8/n8 spans from 880-915MHz (UL)/925-960MHz (DL). Hence, the RF front-end of a device needs to cover the frequency range of 880-960MHz.

UHF RFIDs typically support the frequency range of 860-930MHz. And for RFIDs, the Tx/Rx performance in the supported frequency range could vary. Figure 1 shows the performance of an example RFID tag duplicated from its datasheet.

Figure 1: An example RF Performance of a UHF RFID tag (Source: General specification for CTU07601-465)

Observation 1: It is challenging for devices to achieve consistent RF performances across the whole frequency range of an operating band due to the simplistic-transceiver design target. As a result, frequency-dependent relaxation in performance requirements is expected for a A-IoT device to support a given operating band.

From the perspective of devices, there’s no need for sync raster, since there will not be cell search/synchronisation procedure. On the other hand, it might be necessary to define channel bandwidth for devices. For the R2D channel, one or multiple PRBs are under consideration in RAN1. For D2R channel, the transmission bandwidth of 15kHz or more is being discussed. The specification of channel bandwidth might be needed to define the boundary for in-channel and adjacent channel emission requirements.

Observation 2: The definition of channel bandwidths based on multiple PRBs or sub-carriers should be considered for A-IoT devices.

### 2.2 Receiver Sensitivity

The receiver block diagram for device 1 [3] is shown in Figure 1 below.

Figure 2: Receiver Block Diagram for Device 1.

Due to the employment of RF envelope detector, the thermal noise within the whole Rx RF bandwidth will affect the SNR at the output of the detector. The baseband LPF cascaded with the envelope detector can improve the SNR by limiting the noise BW seen by the comparator.

A typical comparator has an inherent threshold voltage. When the input voltage is above the threshold voltage, a bit “1” is generated; otherwise, a bit “0” is output. Hence, it’s critical for the received signal modulated by ON/OFF keying to be large enough (compared with the threshold voltage) during the ON duration, so that the comparator can detect it correctly.

In the following, more details will be discussed on the sensitivity limited by SNR as well as sensitivity limited by the comparator threshold voltage.

### 2.2.1 Sensitivity limited by SNR

The noise characteristics of the RF envelope detector have been analysed in [4]. Given a target SNR at the output of the BB LPF, the minimum required Rx signal power (i.e. Rx sensitivity) is derived as [4]:

For example, assume the target output SNR for BB demodulation and decoding is 15dB, and the front-end noise figure is 24dB. The Rx sensitivity can be calculated for varying RF BW and BB BW using the above formula.

Figure 3: Rx sensitivity based on target SNR requirement for BB decoding

Knowing the required input signal power, the SNR at the input of the RF envelope detector can be readily derived by deducting the total noise power in the RF BW.

Figure 4: Minimum input SNR for Rx sensitivity.

Observation 3: The minimum input SNR for a RF envelope detector is required to be around 7.5dB, given that the RF BW=10MHz and target SNR=15dB.

For device 2a/2b, a BB amplifier may be utilised after the envelope detector. The effect of BB noise including the flicker noise is also analysed in [4], which shows the corner frequency of the 1/f noise is a key factor to determine the SNR degradation. Further study is needed to evaluate the Rx sensitivity for device 2a/2b.

Observation 4: For the Rx sensitivity for device 2a/2b, the effect of 1/f noise of the BB amplifier needs to be further considered.

### 2.2.2 Sensitivity limited by threshold voltage

After the envelope detector and the BB LPF, the Rx signal is fed to the comparator, which has an inherent threshold voltage. The output voltage of a typical envelope detector implemented by multi-stage CMOS rectifiers is shown to be [5]:

where VT is the thermal voltage≈ 26mV, VA is the input voltage amplitude and N is the number of rectifier stages.

Figure 5: The output voltage versus input power level for a RF envelope detector implemented by 10-stage CMOS rectifiers

Observation 5: To meet the 10mv threshold voltage of the comparator, the input power to the RF envelope detector (implemented by 10-stage rectifiers) is at least -36dBm.

Increasing the number of rectifier stages in the envelope detector or adding a BB amplifier could boost the output voltage. For the same threshold voltage, the required input signal power is reduced, i.e. the Rx sensitivity is improved. Both measures are at the expense of increased complexity and cost.

### 2.2.3 Summary

It can be seen that for devices using RF envelope detector and comparator-based receivers, the Rx sensitivity is limited by both target SNR and the threshold voltage. A-IoT devices are targeted for ultra-low power consumption and ultra-low cost. Hence, the Rx sensitivity limited by the threshold voltage is generally much higher than that limited by SNR.

In the presence of external interference, the interference power is added on top of the thermal noise power. The increase of the total noise power can be modelled as the rise of effective noise figure. To meet the target output SNR, the same input SINR needs to be maintained. As a result, the wanted signal power needs to be increased dB-by-dB.

Proposal 1: For device receivers based on RF envelop detector and comparator, the effects of both target SNR and the threshold voltage should be considered when determining the Rx sensitivity.

### 2.3 Other Receiver Characteristics

For UHF RFIDs, a number of RF requirements are defined in ISO/IEC 18046-3 [6], such as:

- Minimum power operation threshold for identification, read and write

- Sensitivity degradation

- Maximum operating power of tag

- Survival electromagnetic power of tag

- Interference rejection

- Maximum fade rate

For A-IoT devices, Maximum input level in addition to Rx sensitivity should be considered. Since A-IoT devices are likely deployed in-band of NR/LTE, the interference rejection to adjacent wideband interferer should also be considered.

Proposal 2: For A-IoT device Rx RF requirements, consider to define maximum input level, interference rejection (to wideband interferer) in addition to Rx sensitivity.

# 3	Conclusion

In this paper, we have shared further analysis on the potential RF requirements for A-IoT devices. The following observations and proposals are made:

Observation 1: It is challenging for devices to achieve consistent RF performances across the whole frequency range of an operating band due to the simplistic-transceiver design target. As a result, frequency-dependent relaxation in performance requirements is expected for a A-IoT device to support a given operating band.

Observation 2: The definition of channel bandwidths based on multiple PRBs or sub-carriers should be considered for A-IoT devices.

Observation 3: The minimum input SNR for a RF envelope detector is required to be around 7.5dB, given that the RF BW=10MHz and target SNR=15dB.

Observation 4: For the Rx sensitivity for device 2a/2b, the effect of 1/f noise of the BB amplifier needs to be further considered.

Observation 5: To meet the 10mv threshold voltage of the comparator, the input power to the RF envelope detector (implemented by 10-stage rectifiers) is at least -36dBm.

Proposal 1: For device receivers based on RF envelop detector and comparator, the effects of both target SNR and the threshold voltage should be considered when determining the Rx sensitivity.

Proposal 2: For A-IoT device Rx RF requirements, consider to define maximum input level, interference rejection (to wideband interferer) in addition to Rx sensitivity.

# References

- RP-240826, “Study on solutions for Ambient IoT (Internet of Things) in NR”, 3GPP TSG RAN Meeting #103, Maastricht, Netherlands, March 18-21, 2024

- R4-2410597 WF on impacts of A-IoT on RF requirements, Moderator (Huawei), RAN4#111

- R1-2401835 Final FL Summary#4 for 9.4.1.2 Ambient IoT Device Architecture, Qualcomm, RAN1#116

- Noise and Sensitivity in RF Envelope Detection Receivers, X. Huang et al, IEEE Trans. on Circuits and Systems, Oct 2013

- A -32dBm Sensitivity RF Power Harvester in 130nm CMOS, S. Oh et al, IEEE Radio Frequency Integrated Circuits Symposium, 2012

- ISO/IEC 18046-3 Information technology – Radio frequency identification device performance test methods – Part 3: Test methods for tag performance