3GPP TSG-RAN WG4 Meeting#112							         			R4-2413031

Maastricht, Netherland, 19 – 23 August 2024

Title: 	On architecture options for fragmented spectrum reception

Source: 	Skyworks Solutions, Inc.

Agenda Item:	8.6.2	Methods for reducing the number of UE Rx chains	[FS_NR_FR1_DL_Frag_Carrier]

Document for:	Approval

# 1	Background

In RAN#104, a new study [1] was agreed upon to enhance UE receiver capability to receive fragmented spectrum within one band. In this contribution, we explore possible architecture enhancements and their limitations.

# Discussion

## 2.1 Study item description inputs

Below in italics, we extracted the objectives from the study item description [1].

Identify methods for reducing the number of UE Rx chains (e.g. from separate RF chains per CC to shared RF chains ) needed for single DL band with frequency span ≤ 100 MHz, containing two non-contiguous CCs within a CA combination for the inter-operator co-located scenario, considering:

  - Which RF requirements could be adjusted for the inter-operator co-located BS scenario, e.g. existing UE RF requirements such as ΔRIBNC, ACS and in-band blocking [RAN4];

    - Other requirements are not precluded, if identified

  - The ability to semi-statically switch hardware resources (i.e. Rx chains) [RAN4, RAN2 – See note 2];

  - Up to 6 dB DL received power spectral density imbalance between the two non-contiguous CCs [RAN4];

  - Determine a reasonable level for the power spectral density difference between carriers of co-located adjacent channel operators for study [RAN4]

  - Impacts on DL performance [RAN4] ;

  - Means for a UE to inform the network of appropriate CA configuration it can support with adjusted RF requirements [RAN4, RAN2 – See note 2].

NOTE 1:	No RAN1 impact is foreseen

NOTE 2:	RAN2 work, if necessary, will be triggered by RAN4 LS

NOTE 3:	This study starts from single DL band. Sharing RF chain is not considered among inter-band DL carriers.

- Whether and how to apply the conclusions to intra-band component of inter-band CA will be decided after the study on 2CC in a single DL band is completed.

Observations:

- In the first statement, it is unclear what is meant by RF chains as it could be interpreted as antenna and LNA RF paths which can already share multiple DL CCs today.

- It seems that Rx chain sharing is the goal by receiving multiple CCs par analog BB and/or Digital BB paths.

- If the PSD imbalance of 6dB between the co-located CCs is captured, it is unclear if other channels in the gap or the edges of the CCs are collocated and thus which blocking levels may be suffered in-gap and on the side adjacent channels.

- It would be useful to have an example scenario/band.

Proposal:

- The SID should better clarify what is understood by RF chains and Rx chains, as it is our understanding that within the same band both the antenna and the LNA is already shared between non-contiguous CCs.

- The power of adjacent channels in-gap and on the sides should be clarified for this fragmented carrier scenario.

- An example scenario should be added in the SID to have a more concrete case to study.

In this contribution, we discuss which architecture options could be worth of studying and their potential issues.

## 2.2 Architectures to reduce the number of UE Rx chains

For DL contiguous CCs, it is possible to receive multiple CCs within one receive path and the separate CCs only being separated at BB and if the CCs are SCS aligned, it may be possible to use a single FFT.

For DL non-contiguous CCs (fragmented spectrum) however, each CCs have separate BB paths. In this case, the only shared hardware is the antenna and the LNA, while each CC has a separate down mixing.

In order to increase the support of more DL CCs and/or MIMO layer per CC, there is an incentive to receive non-contiguous DL spectrums within the same Rx path.

In this case, we may discuss non-contiguous DL clusters rather than non-contiguous DL CC as, for example, a band where an operator would operate two contiguous CCs and a separate non-contiguous CC would constitute only two non-contiguous DL clusters.

From a UE side, provided it has sufficient LO sources and mixers, the total number of receive paths must be distributed across the number of band and CCs and the number of Rx per band.

With the current architecture it may already be possible to trade two of the receivers dedicated to 4Rx into more 2Rx CCs by falling back to 2Rx in a similar way than what was specified for an overlapping band non-collocated CA scenario. The benefit of this is that there is no degradation in the reception of the additional CCs.

Proposal: In a similar way as specified for overlapping band non-collocated CA scenario, RAN4 studies the possibility of supporting only 2Rx in the 4Rx band in order to free up more receive paths to support more DL bands and/or CCs:

- This may already be supported by current architecture.

- The network may configure in a semi-static way the number of Rx per band to trade some of the 4Rx receive path to higher number of DL bands and/or CCs.

It is understood that the above proposal does not fully satisfy what the operator needs but there may already be solutions that support higher number of DLs provided the 4Rx bands could fallback to 2Rx when beneficial.

To achieve the SI objective there may be multiple approaches:

- The main goal is to share Rx chain from the mixer onwards, which means the non-contiguous DL CCs or clusters share the antenna, LNA, mixer/LO and analog BB path.

  - This means that the analog BB path has sufficient BW to encompass the two CCs/clusters and thus will have limited selectivity for the left and right adjacent channels, but none for adjacent channels in the gap which will result in reduced blocker immunity.

  - In-gap, there may be a strong desense if a blocker falls on top of the shared LO frequency.

- As a second step, there may be different digital BB chain approaches:

  - Separate digital BB chains per CC after digital down conversion which would enable a dedicated digital BB filter and thus good ACS.

  - Common digital BB chain assuming that CCs are SCS aligned such that a shared FFT/digital filter are used. In this case it may be difficult to maintain good ACS and blocking performance, especially in the gap.

- Note that if a single FFT is used to receive two non-contiguous CCs/cluster that are SCS aligned, if the CCs are 15kHz SCS the maximum BW is 50MHz and if 30kHz it is 100MHz.

Figure 1 illustrates some of the above aspects and terminology. The CCs in the illustration are the co-located operator non-contiguous CCs/clusters to be received.

Figure 1: illustration of shared Analog path and shared/separate digital BB paths

Proposal on architecture options: RAN4 studies different architectures where:

- A single analog BB chain is used to receive two non-contiguous CCs within a wide analog filter:

  - Blocker requirement may need to be revisited separately for in-gap blockers and adjacent side blockers.

  - Specific issues may need to be addressed when a blocker falls on the shared DL LO frequency.

- Additionally, study:

  - A separate digital BB path per CC/cluster is used allowing good blocker and ACS performance.

  - A shared digital BB path for both CCs/clusters with its potential ACS limitations

    - Single FFT may be used if the CCs are SCS aligned.

- Maximum received BW separation for two CCs/clusters in one RX path is 50MHz for 15kHz CCs and 100MHz for 30kHz CCs.

# Conclusion

In this contribution, we extended the inner, outer and edge allocation triangle concept used for the UE MPR by using the BS parameters instead and make the following proposal.

Proposal:

- The SID should better clarify what is understood by RF chains and Rx chains, as it is our understanding that within the same band both the antenna and the LNA is already shared between non-contiguous CCs.

- The power of adjacent channels in-gap and on the sides should be clarified for this fragmented carrier scenario.

- An example scenario should be added in the SID to have a more concrete case to study.

Proposal: In a similar way as specified for overlapping band non-collocated CA scenario, RAN4 studies the possibility of supporting only 2Rx in the 4Rx band in order to free up more receive paths to support more DL bands and/or CCs:

- This may already be supported by current architecture.

- The network may configure in a semi-static way the number of Rx per band to trade some of the 4Rx receive path to higher number of DL bands and/or CCs.

Proposal on architecture options: RAN4 studies different architectures where:

- A single analog BB chain is used to receive two non-contiguous CCs within a wide analog filter:

  - Blocker requirement may need to be revisited separately for in-gap blockers and adjacent side blockers.

  - Specific issues may need to be addressed when a blocker falls on the shared DL LO frequency.

- Additionally, study:

  - A separate digital BB path per CC/cluster is used allowing good blocker and ACS performance.

  - A shared digital BB path for both CCs/clusters with its potential ACS limitations

    - Single FFT may be used if the CCs are SCS aligned.

- Maximum received BW separation for two CCs/clusters in one RX path is 50MHz for 15kHz CCs and 100MHz for 30kHz CCs.

# References

[1] RP‑241669 New SID: Study on NR FR1 DL Fragmented Carriers, MediaTek Inc., RAN#104