3GPP TSG-RAN4 WG4 Meeting #112	R4-2413035

# Maastricht, Netherlands, 19th August 2024 – 23rd August 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 2471 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 38.101-1 Rel-18 Intra-band CA REFSENS corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Skyworks Solutions, Inc., T-Mobile USA, Murata Manufacturing Corp., Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | HPUE_NR_FR1_FDD_R18 NR_CA_R18_Intra-Core |  |  |  |  |  | Date: |  |  | 2024-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Changes to ΔRIBC and ΔRIBNC are discussed in R4-2400365/R4-2400366 and agreed in WF R4-2403628. PC2 FDD REFSENS requirements need to be introduced and the MSD table format was agreed in WF R4-2406683. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changes to Symbols: as agreed in WF R4-2403628. Changes to core requirements: - 7.3A.2.1: clarified PC3 applicability based on WF R4-2403628. - 7.3A.2.1: Added clarifications for PC2 applicability based on WF R4-2406683. - 7.3A.2.2: clarified PC3 applicability based on WF R4-2403628 and aligned text with clause 7.3A.2.1 for consistency. Changes to REFSENS Tables: - Changed the title of Table 7.3A.2.1-1 to clarify these are PC3 requirements and to bring consistency with other table titles - PC3 one uplink carrier requirements were meant to be introduced in Table 7.3A.2.1-2, but this table was voided in 18.6.0. These requirements are captured in new Table 7.3A.2.1-1a - Aligned the title of PC2 Table 7.3A.2.1-3 with other tables for consistency - Table 7.3A.2.2-1: - Changed title to clarify PC3 applicability and align text with table 7.3A.2.1-1 - Added the missing term “SCC” to the title of column ΔRIBNC, - Changed the title of Table 7.3A.2.2-2 to align with the title of Table 7.3A.2.1-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The PC3 REFSENS requirements are not clear, the PC2 requirements are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 7.3A.2.1, 7.3A.2.2. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.521 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

---Start of changes---

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

ΔFGlobal	Granularity of the global frequency raster

ΔFRaster	Band dependent channel raster granularity

ΔfOOB	Δ Frequency of Out Of Band emission

ΔFTX-RX	Maximum deviation to the Tx-Rx carrier center frequency separation for asymmetric uplink/downlink channel bandwidth operation

∆MPRc	Allowed Maximum Power Reduction relaxation for serving cell c

ΔPPowerClass	Adjustment to maximum output power for a given power class

RB	The starting frequency offset between the allocated RB and the measured non-allocated RB

ΔRIB,c	Allowed reference sensitivity relaxation due to support for inter-band CA operation, for serving cell c

ΔRIBC	Allowed relaxation to the power class 3 reference sensitivity level  due to support for intra-band contiguous CA operation

ΔRIBNC	Allowed relaxation to the power class 3 reference sensitivity level due to support for intra-band non-contiguous CA operation

ΔRIB,4R	Reference sensitivity adjustment due to support for 4 antenna ports

ΔRIB,8R	Reference sensitivity adjustment due to support for 8 antenna ports

ΔR1R	Reference sensitivity adjustment due to support for 1 antenna ports

ΔRXR,2R	Reference sensitivity adjustment for two antenna ports XR UEs on bands defined in Table 7.3.2-2b

ΔShift	Channel raster offset

TC	Allowed operating band edge transmission power relaxation

TC,c	Allowed operating band edge transmission power relaxation for serving cell c

ΔTIB,c	Allowed maximum configured output power relaxation due to support for inter-band CA operation, inter-band NR-DC operation and due to support for SUL operations, for serving cell c

BWChannel	Channel bandwidth

BWChannel,block	Sub-block bandwidth, expressed in MHz. BWChannel,block= Fedge,block,high- Fedge,block,low

BWChannel_CA	Aggregated channel bandwidth, expressed in MHz

BWChannel,max	Maximum channel bandwidth supported among all bands in a release

BWGB	max(GBChannel,low, GBChannel,high)

BWDL	Channel bandwidth for DL

BWUL	Channel bandwidth for UL

BWinterferer	Bandwidth of the interferer

Ceil(x)	Rounding upwards; ceil(x) is the smallest integer such that ceil(x) ≥ x

Floor(x)	Rounding downwards; floor(x) is the greatest integer such that floor(x) ≤ x

FC	Center frequency of a carrier for a numerology defined by the RF reference frequency on the channel raster mapped to the carrier according to sub-clause 5.4.2.2FC,block, high	Fc of the highest transmitted/received carrier in a sub-block

FC,block, low	Fc of the lowest transmitted/received carrier in a sub-block

FC,low	The Fc of the lowest carrier, expressed in MHz

FC,high	The Fc of the highest carrier, expressed in MHz

FDL_low	The lowest frequency of the downlink operating band

FDL_high	The highest frequency of the downlink operating band

FUL_low	The lowest frequency of the uplink operating band

FUL_high	The highest frequency of the uplink operating band

Fedge,block,low	The lower sub-block edge, where Fedge,block,low = FC,block,low - Foffset, low.

Fedge,block,high	The upper sub-block edge, where Fedge,block,high = FC,block,high + Foffset, high.

Fedge , low	The lower edge of aggregated channel bandwidth, expressed in MHz. Fedge,low = FC,low - Foffset,low.

Fedge, high	The higher edge of aggregated channel bandwidth, expressed in MHz. Fedge,high = FC,high + Foffset,high.

FInterferer (offset)	Frequency offset of the interferer (between the center frequency of the interferer and the carrier frequency of the carrier measured). For intra-band contiguous CA, the FInterferer (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the interferer

FInterferer	Frequency of the interferer

FIoffset	Frequency offset of the interferer (between the center frequency of the interferer and the closest edge of the carrier measured)

Foffset	Frequency offset from FC_high to the higher edge or FC_low to the lower edge.

Foffset,high	Frequency offset from FC,high to the upper UE RF Bandwidth edge, or from FC,block, high to the upper sub-block edge

Foffset,low	Frequency offset from FC,low to the lower UE RF Bandwidth edge, or from FC,block, low to the lower sub-block edge

FOOB	The boundary between the NR out of band emission and spurious emission domains

FREF	RF reference frequency

FREF-Offs	Offset used for calculating FREF

FREF, shift	RF reference frequency for Supplementary Uplink (SUL) bands, the uplink of all FDD bands, and TDD bands

Fuw (offset)	The frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the interferer

Gn100post connector	Declared value of the post chipset unit antenna connector gain for band n100, used for conversion of the radiated requirement into a conducted requirement (see principles described in annex M)

Gn101post connector 	Declared value of the post chipset unit antenna connector gain for band n101, used for conversion of the radiated requirement into a conducted requirement (see principles described in annex M)

GBChannel	Minimum guard band defined in clause 5.3.3, expressed in kHz

GBChannel(i)	Minimum guard band defined in clause 5.3.3 of carrier i

GBChannel,low	Minimum guard band defined in clause 5.3.3 for the lowest assigned component carrier in clause 5.3A.3

GBChannel,high	Minimum guard band defined in clause 5.3.3 for the highest assigned component carrier in clause 5.3A.3

LCRB	Transmission bandwidth which represents the length of a contiguous resource block allocation expressed in units of resources blocks

Max()	The largest of given numbers

Min()	The smallest of given numbers

Physical resource block number

NRACLR	NR ACLR

NRB	Transmission bandwidth configuration, expressed in units of resource blocks

NRB_agg	The number of the aggregated RBs within the fully allocated aggregated channel bandwidth

NRB_agg=1jNRBj*2μj for carrier 1 to j, where μ is defined in TS 38.211 [6]

NRB,c	The transmission bandwidth configuration of component carrier c, expressed in units of resource blocks

NRB,cj=NRBj*2μj for carrier j, where μ is defined in TS 38.211 [6]

NRB,largest BW	The largest transmission bandwidth configuration of the component carriers in the bandwidth combination, expressed in units of resource blocks

NRB,low	The transmission bandwidth configurations according to Table 5.3.2-1 for the lowest assigned component carrier in clause 5.3A.1

NRB,high	The transmission bandwidth configurations according to Table 5.3.2-1 for the highest assigned component carrier in clause 5.3A.1

NREF	NR Absolute Radio Frequency Channel Number (NR-ARFCN)

NREF-Offs	Offset used for calculating NREF

PCMAX	The configured maximum UE output power

PCMAX, c	The configured maximum UE output power for serving cell c

PCMAX, f, c	The configured maximum UE output power for carrier f of serving cell c in each slot

PEMAX	Maximum allowed UE output power signalled by higher layers

PEMAX, c	Maximum allowed UE output power signalled by higher layers for serving cell c

PInterferer	Modulated mean power of the interferer

Plargest BW	Power of the largest transmission bandwidth configuration of the component carriers in the bandwidth combination

PPowerClass	The nominal UE power (i.e., no tolerance)

Pmax,c,AC	Maximum output power defined as the sum of measurement of all antenna connectors

Pmax,c,TABC	Maximum carrier output power defined as the sum of measurement of all TAB connectors

Prated,c,AC	Rated maximum output power defined as the sum of power over all antenna connectors

Prated,c,TABC	Rated maximum output power defined as the sum of power over all TAB connectors

P-MPRc	Power Management Maximum Power Reduction for serving cell c

PRB	The transmitted power per allocated RB, measured in dBm

PREFSENS_SL 	The REFSENS power for Sidelink

PUMAX	The measured configured maximum UE output power

Puw	Power of an unwanted DL signal

Pw	Power of a wanted DL signal

RBstart	The lowest RB index of transmitted resource blocks

RBstart_CA	The lowest RB index of transmitted resource blocks for intra-band contiguous CA

SCSc	SCS for the component carrier c, expressed in kHz

SCSlargest BW	SCS for the largest transmission bandwidth configuration of the component carriers in the bandwidth combination, expressed in kHz

SCSlow	SCS for the lowest assigned component carrier in clause 5.3A.1, expressed in kHz

SCShigh	SCS for the highest assigned component carrier in clause 5.3A.1, expressed in kHz

tp	Transient Period value signalled by the UE

tpstart	Start position of transient period relative to the symbol boundary

T(PCMAX, f, c)	Tolerance for applicable values of PCMAX, f, c for configured maximum UE output power for carrier f of serving cell c

TL,c	Absolute value of the lower tolerance for the applicable operating band as specified in clause 6.2.1

SSREF	SS block reference frequency position

UTRAACLR	UTRA ACLR

---Unchanged sections are skipped ---

### 7.3A.2	Reference sensitivity power level for CA

#### 7.3A.2.1	Reference sensitivity power level for Intra-band contiguous CA

For intra-band contiguous carrier aggregation, the throughput of each component carrier shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3.2-1a, Table 7.3.2-1b, Table 7.3.2-2, Table 7.3.2-2a, and Table 7.3.2-3.

For UE(s) supporting one uplink carrier, the uplink configuration of the PCC shall be in accordance with Table 7.3.2-3 and the downlink PCC carrier center frequency shall be configured closer to uplink operating band than any of the downlink SCC center frequency. For power class 3, the reference sensitivity power level is increased by ΔRIBC for specific uplink and downlink test points which are specified in Table 7.3A.2.1-1a.

For aggregation of two or more downlink FDD carriers with two uplink carriers, the reference sensitivity is defined only for the specific uplink and downlink test points which are specified in Table 7.3A.2.1-1 and the power class 3 reference sensitivity power level increased by ΔRIBC. The requirements apply with all downlink carriers active. Unless given by Table 7.3.2-4, the reference sensitivity requirements shall be verified with the network signaling value NS_01 (Table 6.2.3.1-1) configured.

Table 7.3A.2.1-1: Power class 3 intra-band contiguous CA  reference sensitivity with two uplink carriers

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | UL PCC allocation (LCRB) | UL SCC allocation (LCRB) | PCC ΔRIBC (dB) | SCC ΔRIBC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n3B5 | 15/15 | 20MHz + 20MHz | 25 (RBSTART = 0) | 25 (RBSTART = 81) | 25.1 | 12.4 | FDD |
| CA_n5B | 15/15 | 10MHz + 10MHz | 10 (RBstart = 0) | 10 (RBstart = 42) | 30.8 | 26.1 | FDD |
| CA_n5B5 | 15/15 | 5MHz + 20MHz | 4 (RBSTART = 0) | 16 (RBSTART = 90) | 44.6 | 23.0 | FDD |
| CA_n7B | 15/15 | 10MHz + 40MHz | 9 (RBstart = 26) | 36 (RBstart = 180) | 34 | 25 | FDD |
| NOTE 1: All combinations of channel bandwidths defined in Table 5.5A.1-1. NOTE 2: The carrier centre frequency of SCC in the UL operating band is configured closer to the DL operating band. NOTE 3: The transmitted power over both PCC and SCC shall be set to PUMAX as defined in subclause 6.2A.4. NOTE 4: The PCC allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.3.2-1. NOTE 5: Applicable only to BCS 1. |  |  |  |  |  |  |  |

Table 7.3A.2.1-1a: Power class 3 intra-band contiguous CA reference sensitivity with one uplink carrier.

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | UL PCC allocation (LCRB) | SCC ΔRIBC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- |
| CA_n71B1 | 15/15 | 30MHz + 5MHz | 20 (RBSTART = 0) | 3.8 | FDD |
| NOTE 1: Applicable only to BCS 4 and 5 and for UEs supporting the optional symmetrical UL/DL channel bandwidths. |  |  |  |  |  |

Table 7.3A.2.1-2: Void

For power class 2, the reference sensitivity power level is increased by ΔRIBC for specific uplink and downlink test points which are specified in Table 7.3A.2.1-3. The requirements apply with all downlink carriers active. Unless given by Table 7.3.2-4, the reference sensitivity requirements shall be verified with the network signalling value NS_01 (Table 6.2.3.1-1) configured.

Table 7.3A.2.1-3: Power class 2 intra-band contiguous CA reference sensitivity with two uplink carriers.

| CA configuration | SCS (kHz) | Aggregated channel bandwidth (PCC+SCC) | UL PCC allocation | SCC ΔRIBNC1 (dB) | SCC ΔRIBNC2 (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n71B | 15/15 | 30 MHz + 5 MHz | 20 (RBstart = 0) | 5.6 3 | 7.73 | FDD |
| NOTE 1: Applicable to UE supporting PC2 with single Tx. NOTE 2: Applicable to UE supporting PC2 with dual Tx. NOTE 3: Applicable only to BCS 4 and 5 and for UEs supporting the optional symmetrical UL/DL channel bandwidths. |  |  |  |  |  |  |

#### 7.3A.2.2	Reference sensitivity power level for Intra-band non-contiguous CA

For intra-band non-contiguous carrier aggregation with one uplink carrier and two or more downlink sub-blocks, throughput of each downlink component carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) and parameters specified in Table 7.3.2-1a, Table 7.3.2-1b, Table 7.3.2-2, Table 7.3.2-2a and Table 7.3A.2.2-1 .

For aggregation of two or more downlink FDD carriers with one uplink carrier, the power class 3 reference sensitivity is increased by ΔRIBNC only for the specific uplink and downlink test points which are specified in Table 7.3A.2.2-1. For power class 2, the reference sensitivity power level is increased by ΔRIBNC for specific uplink and downlink test points which are specified in Table 7.3A.2.2-1a. The requirements apply with all downlink carriers active. Unless given by Table 7.3.2-4, the reference sensitivity requirements shall be verified with the network signaling value NS_01 (Table 6.2.3.1-1) configured.

Table 7.3A.2.2-1: Power class 3 intra-band non-contiguous CA  reference sensitivity with one uplink.

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation (LCRB) | SCC ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n1(2A) | 15/15 | 5MHz + 5MHz | 0.0 < Wgap ≤ 50.0 | 25 | 0.5 | FDD |
| CA_n2(2A) | 15/15 | 5MHz + 5MHz | Wgap = 50.0 | 105 | 5.0 | FDD |
|  |  |  | Wgap = 30.0 | 25 | 0.0 |  |
| CA_n3(2A) | 15/15 | 5MHz + 5MHz | Wgap = 65.0 | 125 | 4.7 | FDD |
|  |  |  | Wgap = 45.0 | 255 | 0.0 |  |
| CA_n5(2A) | 15/15 | 15MHz + 5MHz | Wgap = 5.0 | 55 | 6.3 | FDD |
| CA_n7(2A) | 15/15 | 10MHz + 5MHz | Wgap = 55 | 325 | 0.0 | FDD |
|  |  |  | Wgap = 30 | 505 | 0.0 |  |
| CA_n12(2A) | 15/15 | 5MHz + 5MHz | 0.0 < Wgap ≤ 7.0 | 5 (RBstart=12) | 3 | FDD |
| CA_n25(2A) 9 | 15/15 | 5MHz + 5MHz | Wgap = 55.0 | 105 | 5.0 | FDD |
|  |  |  | Wgap = 30.0 | 25 | 0.0 |  |
| CA_n25(2A) 10 CA_n25(3A) | 15/15 | 40MHz + 5MHz | Wgap = 20.0 | 40 (RBstart = 176) | [24.6] 8 | FDD |
| CA_n26(2A) | 15/15 | 15MHz + 10MHz | Wgap = 10.0 | 5 (RBstart = 74) | 25.2 | FDD |
| CA_n66(2A) CA_n66(3A) | N/A | NOTE 1 | NOTE 2 | NOTE 3, NOTE 4 | 0.0 | FDD |
| CA_n71(2A) | 15/15 |  |  |  |  |  |
|  |  | 15MHz + 10MHz | Wgap = 10.0 | 5 (RBstart = 2) | 22.2 |  |
|  |  | 25MHz + 5MHz11 | Wgap = 5.0 | 20 (RBstart = 19) | 25.1 |  |
| NOTE 1: All combinations of channel bandwidths defined in Table 5.5A.2-1. NOTE 2: All applicable sub-block gap sizes. NOTE 3: The PCC allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.3.2-1. NOTE 4: The carrier center frequency of PCC in the DL operating band is configured closer to the UL operating band. NOTE 5: Refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission. NOTE 6: Wgap is the sub-block gap between the two sub-blocks. NOTE 7: The carrier centre frequency of SCC in the DL operating band is configured closer to the UL operating band. NOTE 8: For operation with three or more non-contiguous component carriers, ΔRIBNC applies to all secondary component carriers. NOTE 9: Bandwidth Combination Set 0. NOTE 10: Bandwidth Combination Set 1 NOTE 11: Applicable only to Bandwidth Combination Set 4 and 5 and for UEs supporting the symmetrical UL/DL channel bandwidths. |  |  |  |  |  |  |

Table 7.3A.2.2-1a: Power class 2 intra-band non-contiguous CA reference sensitivity with one uplink carrier.

| CA configuration | SCS (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation | SCC ΔRIBNC1 (dB) | SCC ΔRIBNC2 (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n71(2A) | 15/15 | 15MHz + 10MHz | Wgap = 10.0 | 5 (RBstart = 2) | 24.8 | 29.3 | FDD |
|  |  | 25MHz + 5MHz1 | Wgap = 5.0 | 20 (RBstart = 8) | 27.23 | 31.83 |  |
| CA_n25(2A) 9 | 15/15 | 5MHz + 5MHz | Wgap = 55.0 | 105 | 7.3 | 10.0 | FDD |
|  |  |  | Wgap = 30.0 | 25 | 0.0 | 0.0 |  |
| CA_n25(2A) 10 CA_n25(3A) | 15/15 | 40MHz + 5MHz | Wgap = 20.0 | 40 (RBstart = 176) | [27.6] 8 | 32.68 | FDD |
| CA_n66(2A) CA_n66(3A) | N/A | NOTE 4 | NOTE 5 | NOTE 6, NOTE 7 | 0.0 | 0.0 | FDD |
| NOTE 1: Applicable to UE supporting PC2 with single Tx. NOTE 2: Applicable to UE supporting PC2 with dual Tx. NOTE 3: Applicable only to BCS 4 and 5 and for UEs supporting the optional symmetrical UL/DL channel bandwidths. NOTE 4: All combinations of channel bandwidths defined in Table 5.5A.2-1. NOTE 5: All applicable sub-block gap sizes. NOTE 6: The PCC allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.3.2-1. NOTE 7: The carrier center frequency of PCC in the DL operating band is configured closer to the UL operating band. NOTE 9: Bandwidth Combination Set 0. NOTE 10: Bandwidth Combination Set 1 |  |  |  |  |  |  |  |

For intra-band non-contiguous carrier aggregation with two uplink carriers and two or more downlink sub-blocks, throughput of each downlink component carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) and parameters specified in Table 7.3.2-1a, Table 7.3.2-1b, Table 7.3.2-2, Table 7.3.2-2a and Table 7.3A.2.2-2 with the power class 3 reference sensitivity power level increased by ΔRIBNC  only for the specific uplink and downlink test points which are specified in Table 7.3A.2.2-2.

The requirements apply with all downlink carriers and two uplink carriers active. The reference sensitivity requirements shall be verified with the network signaling value NS_01 (Table 6.2.3.1-1) configured.

Table 7.3A.2.2-2: Power class 3 intra-band non-contiguous CA reference sensitivity with two uplink carriers.

| CA configuration | PCC/SCC (SCS, BW) | PCC/SCC UL Fc (MHz) | UL PCC/SCC allocation (LCRB) | PCC/SCC DL Fc (MHz) | PCC ΔRIBNC (dB) | SCC ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n26(2A) | (15kHz, 5MHz)/ (15kHz, 5MHz) | 816.5 / 839 | 12 (RBSTART = 0) / 12 (RBSTART = 3) | 861.5 / 884 | 38.0 | 13.0 | FDD |

---End of changes---