3GPP TSG-RAN WG4 Meeting #112	R4-2413036

# Maastricht, Netherlands, August 19th – August 23rd, 2024

# Title: 	Consideration of Linear Polarization in Ku Band Co-existence Studies

# Source:	SKY Perfect JSAT

# Type:	Discussion

# Document for:	Discussion

# Agenda Item:	8.9.2 Coexistence study based on ITU regulations

Release	Rel-19

- Introduction

In RP#104, a new WI [1] for Ku band was approved and the phrase 'Reuse Ka-band coexistence study assumptions as much as possible, with modifications where necessary' is stated in the Work Item Description (WID) for the Ku-band standardization. The following is an excerpt from the WID related to the coexistence study.

- Use the regulation requirements in ITU Regions 1, 2 and 3 to identify relevant adjacent band co-existence scenarios for NTN Ku Band covering the following frequency ranges, considering targeted deployment scenarios: - For ITU regions 1 and 3: - Downlink 10.70 – 12.75 GHz - Uplink 13.75-14.5 GHz - For ITU regions 2 excluding US: - Downlink 10.70 – 12.70 GHz - Uplink 13.75-14.5 GHz - For ITU regions 1 and 3 - Downlink 10.70 – 12.75 GHz - Uplink 12.75 - 13.25 GHz - For ITU regions 2 excluding US - Downlink 10.70 – 12.70 GHz - Uplink 12.70 - 13.25 GHz - Note: Appropriate TX/Rx separation for uplink downlink pairing will be required. - Conduct coexistence study. Reuse Ka band coexistence study assumptions as much as possible, with modifications where necessary, e.g. for frequency range and subcarrier spacing. Conduct new co-existence studies based on adjacent services .

In this document, we will provide our views on whether it is necessary to consider polarization under the condition of determining whether additional practical co-existence scenarios need to be considered while re-using the Ka-band coexistence study assumptions as much as possible.

- Discussion

The results from the NTN-related co-existence study are documented in TR 38.863. Within this document, the antenna and beamforming pattern modelling for polarization shown in table 2-1 is used.

Table 2-1 Beam layout definition for single satellite simulation

|  | Option 1: FRF=1 | Option 2: FRF=3 | Option 3: FRF=2 |
| --- | --- | --- | --- |
| Polarization re-use | Option 1: Disable Option 2: Enable Note: Polarization re-use should apply only if circular polarization for terminal antenna is considered |  |  |

Since GEO satellites in Ku-band use linear polarization in most cases, it is necessary to discuss whether co-existence study for the linear polarization model is required.

Observation 1: GEO satellites in Ku-band use linear polarization in most cases and this does not appear to have been considered in the NTN-related coexistence study of TR 38.863.

Proposal 1: RAN4 to discuss whether it is also required to study shared use considerations for the linear polarization model.

- Conclusions

Observation 1: GEO satellites in Ku-band use linear polarization in most cases and this does not appear to have been considered in the NTN-related coexistence study of TR 38.863

Proposal 1: RAN4 to discuss whether it is also required to study shared use considerations for the linear polarization model.References.

- References

- RP-241690, New WID on Introduction of Ku Band for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat