# 3GPP TSG-RAN WG4 Meeting #112                                                       R4-2413043

# Maastricht, Netherlands, 19th – 23rd August, 2024

Source:	ZTE Corporation, Sanechips

Title:	Discussion on RRM requirements for NTN enhancement

Agenda item:	8.8.3.4

Document for:	Approval

Introduction

At RAN 104# meeting, the revised WID on NR NTN evolution [1] has been approved, and several objectives in the WID have RAN4 RRM involvement. As this is the first meeting for RAN4 RRM to work on this WI, we will provide our initial views on potential impacts on RRM requirements.

Discussion

The objectives of Rel-19 in NR NTN evolution are described in [1] including:

2.1 Less than 5MHz for NTN

- In this meeting, we will discuss the scenarios and express our views on potential impact to RRM requirements towards the above objective

  - 2.1 Less than 5MHz for NTN

Less than 5MHz for NTN - Specify the system parameters for channel bandwidth of less than 5 MHz for NR-NTN (NR based Non-Terrestrial Networks) in FR1-NTN bands - Consider the NR-NTN bands n255, n256, and n254 - Add less than 5MHz channel bandwidth - 3MHz with the RAN1 design in Rel-18 - Investigate and if necessary, specify the additional synchronization (sync) raster(s) for the additional channel bandwidth - Leverage the work on less than 5MHz for TN - Note: no RAN1 impact is expected - Specify the necessary SAN RF core requirements for NR-NTN in FR1-NTN bands - Specify the necessary NR-NTN UE RF core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Focus on power class 3 (PC3) for Tx RF requirements - Specify the necessary RRM core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Specify the necessary signaling to support the above objectives

- In R18, RAN4 just discussed less than 5MHz feature for TN, and the less than 5MHz feature will be applied to NTN scenario.

2.1.1 Measurement

- In legacy discussion of  TN less than 5MHz, the agreements for L1,L3 measurements can be seen as below:

- RAN4 will define both intra-frequency and inter-frequency measurement requirements - RAN4 will define SSB-based L3 measurement requirements. RAN4 will not define CSI-RS based L3 measurement requirements - RAN4 will define SSB-based L1 measurement requirements. - Do not define CSI-RS-based L1 measurements, RLM, BFD, CBD requirements in Rel-18 - RAN4 will not define L1-SINR measurement requirements - Apply the existing SSB based L1-RSRP measurement period TL1-RSRP_Measurement_Period_SSB for CBW less than 5MHz.

- Firstly, we can see that for both L1 and L3 measurements, no consideration is given to CSI-RS-based measurement requirements; only SSB-based requirements are defined. This is because, for CSI-RS, it occupies 48 PRBs, which corresponds to a bandwidth of 8.64 MHz when the subcarrier spacing is 15 kHz. This bandwidth is already greater than 5 MHz. For the SSB design, it occupies 20 PRBs. When the subcarrier spacing is 15 kHz, the SSB occupies a bandwidth of 3.6 MHz, which conforms to the scenario of being less than 5 MHz. Therefore, for measurements, the legacy system only defined SSB-based measurements. Consequently, when considering the definition of NTN requirements for less than 5 MHz, we also need to avoid defining CSI-RS-based requirements and should only consider SSB-based measurements.

- Observation 1: In legacy discussion of less than 5MHz in TN, RAN4 did not define the CSI-RS based requirements and only the SSB based requirements have been considered.

- Proposal 1: RAN4 shall follow the same principle as legacy and only define the SSB based requirements. Also, the legacy L1 and L3 measurement requirements can be the baseline.

2.1.2 Radio link monitoring

- In legacy, RAN4 just discussed the PDCCH transmission parameters as below:

The BW in Hypothetical PDCCH transmission parameters: Agreement: For 3MHz case, and band n100 the BW is 12PRBs for SSB based RLM. FFS for other bands with 3MHz CBW Agreement: For 5MHz case the BW is 24PRBs for SSB based RLM PDCCH transmission parameters for SSB-based RLM and BFD: Issue 1-5: The BW in hypothetical PDCCH transmission parameters for SSB-based RLM Define SSB-based RLM requirements for 12 PRBs [and 15 PRBs] hypothetical PDCCH transmission bandwidth. FFS on applicability rules of RLM requirements for different bands.

| Attribute | Value for BLER Configuration #0 |  |  |
| --- | --- | --- | --- |
| Channel BW | 3MHz |  | 5MHz |
| [DL Transmission BW] | 12 PRBs | 15 PRBs | 20 PRBs |

- We find that the BW is 12PRBs and 15PRBs for SSB based RLM in 3MHz case and also 20PRBs in 5MHz. 12PRBs and 15PRBs are enough to covering transmission of PSS/SSS which can effectively and continuously monitor the performance of radio link for RLM. For 5MHz, the final BW is 20PRBs which has already covered the whole SSB transmission. Thus, we deem that the same BW can be reused in NTN scenario.

- Observation 2: For 3MHz, dowlink transmission BW is 12PRBs and 15PRBs. For 5MHz, downlink transmission BW is 20PRBs.

- For RLM, RAN4 also defined the related evaluation in legacy, we list the legacy TN/ legacy less than 5MHz/ legacy NR NTN PDCCH transmission parameters for evaluation as below:

- Legacy ： Table 8.1.2.1-1: PDCCH transmission parameters for out-of-sync evaluation Attribute; Value for BLER Configuration #0. DCI format; 1-0. Number of control OFDM symbols; 2. Aggregation level (CCE); 8. Ratio of hypothetical PDCCH RE energy to average SSS RE energy; 4dB. Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy; 4dB. Bandwidth (PRBs); 24. Sub-carrier spacing (kHz); SCS of the active DL BWP. DMRS precoder granularity; REG bundle size. REG bundle size; 6. CP length; Normal. Mapping from REG to CCE; Distributed. Table 8.1.2.1-2: PDCCH transmission parameters for in-sync evaluation Attribute; Value for BLER Configuration #0. DCI payload size; 1-0. Number of control OFDM symbols; 2. Aggregation level (CCE); 4. Ratio of hypothetical PDCCH RE energy to average SSS RE energy; 0dB. Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy; 0dB. Bandwidth (PRBs); 24. Sub-carrier spacing (kHz); SCS of the active DL BWP. DMRS precoder granularity; REG bundle size. REG bundle size; 6. CP length; Normal. Mapping from REG to CCE; Distributed. Less than 5M: Table 8.1.2.1-3: PDCCH transmission parameters for out-of-sync evaluation for a UE operating on a cell with less than 5MHz BW Attribute; Value for BLER Configuration #0. Channel BW; 3MHz; 5MHz. [DL Transmission BW]; 12 PRBs; 15 PRBs; 20 PRBs. DCI format; 1-0. Number of control OFDM symbols; 2; 3; 3. Aggregation level (CCE); 4; 8; 8. Ratio of hypothetical PDCCH RE energy to average SSS RE energy; 4dB. Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy; 4dB. Bandwidth (PRBs); 12; 15; 20. Sub-carrier spacing (kHz); SCS of the active DL BWP. DMRS precoder granularity; REG bundle size. REG bundle size; 6. CP length; Normal. Mapping from REG to CCE; Distributed; Non-DistributedNote 1; Distributed. NOTE 1: For 15 PRB Transmission BW, distributed mapping may lead to unexpected out-of-sync indication.. Table 8.1.2.1-4: PDCCH transmission parameters for in-sync evaluation for a UE operating on a cell with less than 5MHz BW Attribute; Value for BLER Configuration #0. Channel BW; 3MHz; 5MHz. [DL Transmission BW]; 12 PRBs; 15 PRBs; 20 PRBs. DCI format; 1-0. Number of control OFDM symbols; 2; 3; 3. Aggregation level (CCE); 2; 4; 4. Ratio of hypothetical PDCCH RE energy to average SSS RE energy; 0dB. Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy; 0dB. Bandwidth (PRBs); 12; 15; 20. Sub-carrier spacing (kHz); SCS of the active DL BWP. DMRS precoder granularity; REG bundle size. REG bundle size; 6. CP length; Normal. Mapping from REG to CCE; Distributed; Non-Distributed; Distributed. Legacy NTN: Table 8.1C.2.1-1: PDCCH transmission parameters for out-of-sync evaluation Attribute; Value for BLER Configuration #0. DCI format; 1-0. Number of control OFDM symbols; 2. Aggregation level (CCE); 8. Ratio of hypothetical PDCCH RE energy to average SSS RE energy; 4dB. Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy; 4dB. Bandwidth (PRBs); 24. Sub-carrier spacing (kHz); SCS of the active DL BWP. DMRS precoder granularity; REG bundle size. REG bundle size; 6. CP length; Normal. Mapping from REG to CCE; Distributed. Table 8.1C.2.1-2: PDCCH transmission parameters for in-sync evaluation Attribute; Value for BLER Configuration #0. DCI payload size; 1-0. Number of control OFDM symbols; 2. Aggregation level (CCE); 4. Ratio of hypothetical PDCCH RE energy to average SSS RE energy; 0dB. Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy; 0dB. Bandwidth (PRBs); 24. Sub-carrier spacing (kHz); SCS of the active DL BWP. DMRS precoder granularity; REG bundle size. REG bundle size; 6. CP length; Normal. Mapping from REG to CCE; Distributed.

- As the table in above, the legacy NR NTN evaluation are the same as legacy TN and the downlink transmission bandwidth is 24PRBs. As for less than 5MHz, RAN4 defined different evaluation/requirements for different channel bandwidth. Thus, to my understanding, RAN4 shall consider reusing legacy less than 5MHz evaluation/requirements in NTN scenario.

- Observation 3: The legacy NR NTN evaluation is the same as legacy TN  and the downlink transmission bandwidth is only 24PRBs. As for less than 5MHz, RAN4 defined different requirements for different channel bandwidth.

- Proposal 2: The legacy  evaluation/requirements for less than 5MHz in TN shall be reused or be the baseline when RAN4 considers defining the evaluation/requirements for less than 5MHz in NTN scenario.

- The time period for out-/in-sync for RLM has also been discussed：

RLM OOS evaluation period: Agreement: For SSB-based OOS in 3MHz, RAN4 will keep the existing SSB based RLM evaluation periods TEvaluate_out_SSB for CBW less than 5MHz Agreement: For SSB-based OOS in 5MHz, RAN4 will keep the existing SSB based RLM evaluation periods TEvaluate_out_SSB for CBW less than 5MHz RLM IS evaluation period: Agreement: for SSB-based IS in 3MHz, RAN4 will keep the existing SSB based RLM evaluation periods TEvaluate_in_SSB for CBW less than 5MHz Agreement: For SSB-based IS in 5MHz, RAN4 will keep the existing SSB based RLM evaluation periods TEvaluate_in_SSB for CBW less than 5MHz

- In legacy, RAN4 kept all the existing SSB related RLM evaluation period for CBW less than 5MHz whatever for RLM OOS or RLM IS. Thus, we  consider that the legacy RLM OOS/IS evaluation period can be reused or be the baseline when defining the requirements for less than 5MHz in NTN scenario.

- Observation 4: In legacy, RAN4 kept all the existing SSB related RLM evaluation period for CBW less than 5MHz whatever for RLM OOS or RLM IS.

- Proposal 3: The legacy RLM OOS/IS evaluation period can be reused or be the baseline when defining the related requirements for less than 5MHz in NTN scenario.

2.1.3 CGI reading

- In R18, RAN4 has reached the agreement for CGI reading as below:

Agreement: - RAN4 will not define new CGI reading requirements - RAN4 agreed that the legacy CGI reading requirements do not apply

- There is no new CGI reading requirements were defined and also the legacy CGI reading requirements do not apply, that is, there is no CGI requirements for less than 5MHz. Thus, this requirement is not defined in legacy TN scenario and also RAN4 shall not define the related requirements in NTN scenario.

- Observation 5: There is no CGI requirements for less than 5MHz.

- Proposal 4: RAN4 shall not define the CGI requirements for less than 5MHz in NTN scenario.

2.1.4 Link recovery procedure

First of all, we can see the following agreements for less than 5MHz in TN scenario:

Beam Failure Detection evaluation period: The agreements made for RLM OOS can be applied also for BFD procedure. Agreement: for SSB-based BFD in 3MHz, RAN4 will keep the existing SSB based BFD evaluation periods TEvaluate_BFD_SSB for CBW less than 5MHz Agreement: For SSB-based BFD in 5MHz, RAN4 will keep the existing SSB based BFD evaluation periods TEvaluate_BFD_SSB for CBW less than 5MHz Candidate beam detection evaluation period: Agreement: for SSB-based CBD in 3MHz, RAN4 will apply the existing SSB based CBD evaluation period TEvaluate_CBD_SSB for CBW less than 5MHz. Agreement: For SSB-based CBD in 5MHz, RAN4 will apply the existing SSB based CBD evaluation period TEvaluate_CBD_SSB for CBW less than 5MHz.

- The same situation as RLM, the existing BFD and CBD requirements are reused for less than 5MHz in TN scenario. Thus, the same method as RLM can be considered when considering defining the requirements for less than 5MHz in NTN scenario.

- Observation 6:  In legacy, RAN4 kept all the existing SSB related BFD and CBD evaluation period for CBW less than 5MHz.

- Proposal 5: The legacy BFD and CBD evaluation period can be the baseline when defining the related requirements for less than 5MHz in NTN scenario.

- In summary, RAN4 shall define the requirements for less than 5MHz in NTN scenario based on the legacy requirements and the latest WID description.

2.1.5 PBCH detection

2.1.5.1 Time period for time index detection

- In this section, we mainly discuss one key point which is the PBCH detection, that is, detection with 12PRBs SSB. Compared to legacy SSB with 20PRBs, the 12PRBs SSB can guarantee the transmission of PSS/SSS but part of PRBs of PBCH are punched. Thus, the new measurement requirements for less than 5MHz have been defined such as handover, cell identification etc. We also can see the following agreements defined in less than 5MHz WI:

Agreement: - SSB index identification delay - Unknown intra-frequency cell (Es/Iot≥-6 dB): [3+4] samples - Unknown inter-frequency cell (Es/Iot≥-4 dB): [3+3] samples

- For unknown cell, we find that the measurement samples were increased based on 12PRBs SSB in order to  compensate the impact of punching for unknown cell searching, and the relevant requirements have been captured in the existing TS38.133 as below:

- Legacy time index detection requirements: Table 9.2.5.1-3: Time period for time index detection (FR1) DRX cycle; TSSB_time_index_intra. No DRX; max(120ms, ceil( 3 x Kp ) x SMTC period)Note 1 x CSSFintra. DRX cycle≤ 320ms; max(120ms, ceil (M2 Note 2 x 3 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320ms; Ceil(3 x Kp) x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1 NOTE 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. NOTE 4: When highSpeedMeasCA-Scell-r17 is configured and UE supports measurementEnhancementCA-r17, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1. - Less than 5MHz time index detection requirements: Table 9.2.5.1-23: Time period for time index detection for a UE operating on a target cell with 12 PRB SSB (Frequency range FR1) DRX cycle; TSSB_time_index_intra_less_than_5Mhz. No DRX; max(120ms, 7 x max(MGRP, SMTC period)) x CSSFintra_less_than_5Mhz. DRX cycle≤ 320ms; max(120ms, ceil(1.5 x 7) x max(MGRP, SMTC period,DRX cycle) x CSSFintra_less_than_5Mhz). DRX cycle>320ms; 7 x max(MGRP, DRX cycle) x CSSFintra_less_than_5Mhz. NOTE 1: FFS When highSpeedMeasInterFreq-r17. - Legacy NTN time index detection requirements: Table 9.2C.5.1-2: Time period for time index detection (FR1) DRX cycle; TSSB_time_index_intra. No DRX; max(120ms, ceil( 3 x Kp x Klayer1_measurement) x Kmulti_SMTC x SMTC period)Note 1 x CSSFintra. DRX cycle≤ 320ms; max(120ms, ceil (1.5 x 3 x Kp x Klayer1_measurement) x Kmulti_SMTC x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320ms; Ceil(3 x Kp x Klayer1_measurement) x Kmulti_SMTC x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified.

- Based on the table in above, we find that the time period for time index detection for NTN is different from legacy considering Klayer1_measurement (considering without measurement gap),Kgap (considering with measurement gap) and Kmulti_SMTC. However, the number of samples is same. Hence, from my perspective, when considering defining the requirements for less than 5M in NTN scenario, RAN4 shall refer to time period defined in less than 5M with Klayer1_measurement , Kmulti_SMTC and Kgap. An example can be seen as below:

Time period for time index detection for a UE operating on a target cell with 12 PRB SSB (Frequency range FR1) for intra-frequency without measurement gap

| DRX cycle | TSSB_time_index_intra_less_than_5Mhz |
| --- | --- |
| No DRX | max(120ms, 7 x max(MGRP, SMTC period))x Klayer1_measurement x Kmulti_SMTC x SMTC period)Note 1 x CSSFintra_less_than_5Mhz |
| DRX cycle≤ 320ms | max(120ms, ceil(1.5 x 7) x Klayer1_measurement x Kmulti_SMTC x max(MGRP, SMTC period,DRX cycle) x CSSFintra_less_than_5Mhz) |
| DRX cycle>320ms | 7 x max(MGRP, DRX cycle) x Kmulti_SMTC x CSSFintra_less_than_5Mhz |
| NOTE 1: FFS When highSpeedMeasInterFreq-r17 NOTE 2: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Time period for time index detection for a UE operating on a target cell with 12 PRB SSB (Frequency range FR1) (Frequency range FR1) for intra-frequency with measurement gap

| DRX cycle | TSSB_time_index_intra_less_than_5Mhz |
| --- | --- |
| No DRX | max(120ms, 7 x Kgap x Kmulti_SMTC x max(MGRP, SMTC period)) x CSSFintra_less_than_5Mhz |
| DRX cycle≤ 320ms | max(120ms, ceil(1.5 x 7) x Kgap x Kmulti_SMTC x max(MGRP, SMTC period,DRX cycle) x CSSFintra_less_than_5Mhz) |
| DRX cycle>320ms | 7 x Kgap x Kmulti_SMTC x max(MGRP, DRX cycle) x CSSFintra_less_than_5Mhz |
| Note 1: CSSFintra_less_than_5Mhz is 1 NOTE 2: FFS When highSpeedMeasInterFreq-r17 |  |

- We just provide the time period for time index detection for intra-frequency without/with measurement gap. The same situation can be studied for inter-frequency measurement. RAN4 shall refer to the legacy less than 5MHz requirements and legacy NTN requirements when considering defining the new requirements.

- Observation 7: Compared to legacy SSB with 20PRBs, the 12PRBs SSB can guarantee the transmission of PSS/SSS but part of PRBs of PBCH are punched.

- Observation 8: The measurement samples were increased based on 12PRBs SSB in order to  compensate the impact of punching for unknown cell searching, and the relevant requirements have been captured in the existing TS38.133.

- Proposal 6: RAN4 shall define the related requirements for less than 5MHz in NTN scenario with considering Klayer1_measurement , Kmulti_SMTC and Kgap (intra-frequency with/without measurement gap as an example):

Time period for time index detection for a UE operating on a target cell with 12 PRB SSB (Frequency range FR1) for intra-frequency without measurement gap

| DRX cycle | TSSB_time_index_intra_less_than_5Mhz |
| --- | --- |
| No DRX | max(120ms, 7 x max(MGRP, SMTC period))x Klayer1_measurement x Kmulti_SMTC x SMTC period)Note 1 x CSSFintra_less_than_5Mhz |
| DRX cycle≤ 320ms | max(120ms, ceil(1.5 x 7) x Klayer1_measurement x Kmulti_SMTC x max(MGRP, SMTC period,DRX cycle) x CSSFintra_less_than_5Mhz) |
| DRX cycle>320ms | 7 x max(MGRP, DRX cycle) x Kmulti_SMTC x CSSFintra_less_than_5Mhz |
| NOTE 1: FFS When highSpeedMeasInterFreq-r17 NOTE 2: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Time period for time index detection for a UE operating on a target cell with 12 PRB SSB (Frequency range FR1) (Frequency range FR1) for intra-frequency with measurement gap

| DRX cycle | TSSB_time_index_intra_less_than_5Mhz |
| --- | --- |
| No DRX | max(120ms, 7 x Kgap x Kmulti_SMTC x max(MGRP, SMTC period)) x CSSFintra_less_than_5Mhz |
| DRX cycle≤ 320ms | max(120ms, ceil(1.5 x 7) x Kgap x Kmulti_SMTC x max(MGRP, SMTC period,DRX cycle) x CSSFintra_less_than_5Mhz) |
| DRX cycle>320ms | 7 x Kgap x Kmulti_SMTC x max(MGRP, DRX cycle) x CSSFintra_less_than_5Mhz |
| Note 1: CSSFintra_less_than_5Mhz is 1 NOTE 2: FFS When highSpeedMeasInterFreq-r17 |  |

2.1.5.2 Side condition for handover and requirements

- In less than 5MHz WI, RAN4 discussed the side conditions for handover and updated the handover delay, the following agreements can be seen:

Agreement: - Unknown intra-frequency target cell: target cell Es/Iot≥-2 dB. - Unknown inter-frequency target cell: target cell Es/Iot≥-2 dB. - Unknown intra-frequency target cell: [3]*Trs ms (target cell Es/Iot≥-2 dB). - Unknown inter-frequency target cell: [5]*Trs ms (target cell Es/Iot≥-2 dB). - T∆ = Trs for both known and unknown target cells operating with 20 PRB SSB BW. - T∆ = 3*Trs for both known and unknown target cells operating with 12 PRB SSB BW.

- We find that under different side conditions the handover delay will be different especially TΔ, the following description in TS38.133 can be comparable:

- Legacy TΔ： - If the target cell is an unknown inter-frequency cell and Tsearch = 3* Trs ms if the target cell Es/Iot≥-2 dB. T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ = Trs for both known and unknown target cells operating with 20 PRB SSB BW. T∆ = 3*Trs for both known and unknown target cells operating with 12 PRB SSB BW. - Legacy NTN TΔ: - T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ = Trs.

- In legacy, the TΔin legacy NTN is the same as legacy known and unknown target cells operating with 20PRB or 24PRB SSB BW. However, compared with the legacy TΔwith 20 or 24 PRBs, the TΔ for both known and unknown target cells operating with 12 PRB SSB BW are three times as large under certain side conditions. To my understanding, when considering defining requirements for less than 5MHz in NTN, the legacy less than 5MHz side conditions for unknown inter-/intra-frequency target cells can be reused and the values of  TΔ can be reused.

- Observation 9: In legacy, the TΔin legacy NTN is the same as legacy known and unknown target cells operating with 20PRB or 24PRB SSB BW. However, compared with the legacy TΔwith 20 or 24 PRBs, the TΔ for both known and unknown target cells operating with 12 PRB SSB BW are three times as large under certain side conditions.

- Proposal 7: When considering defining requirements for less than 5MHz in NTN, the legacy less than 5MHz side conditions for unknown inter-/intra-frequency target cells can be reused and the values of  TΔ can be reused.

Conclusion

In this paper we provided our views on RRM requirements on R18 NR NTN enhancement.

- Observation 1: In legacy discussion of less than 5MHz in TN, RAN4 did not define the CSI-RS based requirements and only the SSB based requirements have been considered.

- Proposal 1: RAN4 shall follow the same principle as legacy and only define the SSB based requirements. Also, the legacy L1 and L3 measurement requirements can be the baseline.

- Observation 2: For 3MHz, dowlink transmission BW is 12PRBs and 15PRBs. For 5MHz, downlink transmission BW is 20PRBs.

- Observation 3: The legacy NR NTN evaluation is the same as legacy TN  and the downlink transmission bandwidth is only 24PRBs. As for less than 5MHz, RAN4 defined different requirements for different channel bandwidth.

- Proposal 2: The legacy  evaluation/requirements for less than 5MHz in TN shall be reused or be the baseline when RAN4 considers defining the evaluation/requirements for less than 5MHz in NTN scenario.

- Observation 4: In legacy, RAN4 kept all the existing SSB related RLM evaluation period for CBW less than 5MHz whatever for RLM OOS or RLM IS.

- Proposal 3: The legacy RLM OOS/IS evaluation period can be reused or be the baseline when defining the related requirements for less than 5MHz in NTN scenario.

- Observation 5: There is no CGI requirements for less than 5MHz.

- Proposal 4: RAN4 shall not define the CGI requirements for less than 5MHz in NTN scenario.

- Observation 6:  In legacy, RAN4 kept all the existing SSB related BFD and CBD evaluation period for CBW less than 5MHz.

- Proposal 5: The legacy BFD and CBD evaluation period can be the baseline when defining the related requirements for less than 5MHz in NTN scenario.

- Observation 7: Compared to legacy SSB with 20PRBs, the 12PRBs SSB can guarantee the transmission of PSS/SSS but part of PRBs of PBCH are punched.

- Observation 8: The measurement samples were increased based on 12PRBs SSB in order to  compensate the impact of punching for unknown cell searching, and the relevant requirements have been captured in the existing TS38.133.

- Proposal 6: RAN4 shall define the related requirements for less than 5MHz in NTN scenario with considering Klayer1_measurement , Kmulti_SMTC and Kgap (intra-frequency with/without measurement gap as an example):

Time period for time index detection for a UE operating on a target cell with 12 PRB SSB (Frequency range FR1) for intra-frequency without measurement gap

| DRX cycle | TSSB_time_index_intra_less_than_5Mhz |
| --- | --- |
| No DRX | max(120ms, 7 x max(MGRP, SMTC period))x Klayer1_measurement x Kmulti_SMTC x SMTC period)Note 1 x CSSFintra_less_than_5Mhz |
| DRX cycle≤ 320ms | max(120ms, ceil(1.5 x 7) x Klayer1_measurement x Kmulti_SMTC x max(MGRP, SMTC period,DRX cycle) x CSSFintra_less_than_5Mhz) |
| DRX cycle>320ms | 7 x max(MGRP, DRX cycle) x Kmulti_SMTC x CSSFintra_less_than_5Mhz |
| NOTE 1: FFS When highSpeedMeasInterFreq-r17 NOTE 2: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Time period for time index detection for a UE operating on a target cell with 12 PRB SSB (Frequency range FR1) (Frequency range FR1) for intra-frequency with measurement gap

| DRX cycle | TSSB_time_index_intra_less_than_5Mhz |
| --- | --- |
| No DRX | max(120ms, 7 x Kgap x Kmulti_SMTC x max(MGRP, SMTC period)) x CSSFintra_less_than_5Mhz |
| DRX cycle≤ 320ms | max(120ms, ceil(1.5 x 7) x Kgap x Kmulti_SMTC x max(MGRP, SMTC period,DRX cycle) x CSSFintra_less_than_5Mhz) |
| DRX cycle>320ms | 7 x Kgap x Kmulti_SMTC x max(MGRP, DRX cycle) x CSSFintra_less_than_5Mhz |
| Note 1: CSSFintra_less_than_5Mhz is 1 NOTE 2: FFS When highSpeedMeasInterFreq-r17 |  |

- Observation 9: In legacy, the TΔin legacy NTN is the same as legacy known and unknown target cells operating with 20PRB or 24PRB SSB BW. However, compared with the legacy TΔwith 20 or 24 PRBs, the TΔ for both known and unknown target cells operating with 12 PRB SSB BW are three times as large under certain side conditions.

- Proposal 7: When considering defining requirements for less than 5MHz in NTN, the legacy less than 5MHz side conditions for unknown inter-/intra-frequency target cells can be reused and the values of  TΔ can be reused.

- References

- RP-240857, New WID: Enhanced requirements and test methodology for NR and IoT NTN.

- TS38.133