3GPP TSG-RAN WG4 Meeting #112	R4-2413056

Maastricht, Netherlands, 19th – 23rd August, 2024

Source: 	Qualcomm Incorporated

Title: 	Initial Considerations on the study on SCM for demodulation performance requirements

Agenda item:	8.14.2

Document for:	Discussion

- Introduction

In this paper, we share our initial considerations on the Study Item on the use spatial channel model for demodulation performance requirements.

The paper will provide a list of proposals on the topics of assumptions and SCM implementation alignment for the evaluation of the performance through simulations.

Then, we will present some general observations pertaining to the usage of SCM in the context of RAN4 demodulation requirements.

# Objectives of the Study Item

For reference, here we are including an excerpt from the SID RP-241610 [1], in particular from the section referring to the objectives of the Study Item:

The study item [1] includes the following objectives:

- Study practical spatial channel modelling methodology for both SU- and MU-MIMO demodulation requirements and CSI reporting requirements:

  - Identify the limitations of the current (i.e. up to and including Release 18) channel models and corresponding scenarios and how they relate to UE MIMO performance

  - Consider both Clustered Delay Line (CDL)-based and TDL-based channel modelling approaches

    - For CDL-based channel modelling, use the tuned repeatable spatial channel model of TR38.827 as the starting point and identify any necessary changes.

  - Study and verify test methodology feasibility including test complexity and achievable results uncertainty. The test complexity shall not be significantly increased.

The methodology shall include both FR1 (conducted) and FR2 (wireless cable), with first priority for FR1.

# SI Simulation Alignment

## SCM Implementation for Simulations

According to the Study Item Objective, the SCM approach described in TR 38.827 [2] is to be used as reference for the implementation of the Spatial Channel Model.

TR 38.827 [2], is focused on the study of radiated tests methodology, in particular for the case of Multi-Probe Anechoic Chamber (MPAC) and Radiated Two Stage (RTS) setup for FR1, and 3D MPAC setup for FR2. However, again according to the scope of the SI, RAN4 should focus on FR1 conducted tests with priority, and then discuss FR2 radiated requirements.

According to the proposal below, RAN4 should make sure to separate in [3] the aspects related to the radiated tests setup.

Proposal 1: For implementation alignment RAN4 should focus on discussing SCM details assuming FR1 conducted test first, according to the SI description;

Focusing on FR1, we see that Section 7.2.1 in [2] includes a number of Channel model parameter tables for CDL-A, B, C, D, and E for UMa and UMi at 3.5 GHz. Again according to the scope of the SI document [1], we recommend RAN4 to consider the channel models parameters included in these tables.

Proposal 2: For FR1 Conducted Tests, RAN4 to consider CDL Channel Models parameters as defined in Section 7.2.1 of 38.827 as a starting point;

At the same time, there are still multiple details of the 38.827 implementation that need to be clarified, as it was also raised in the Appendix C in [3]. We believe that all these aspects need to be addressed before the simulation alignment can start.

Proposal 3: RAN4 to discuss any proposed modifications to the Channel Models in 38.827 before moving to the simulation alignment step;

Additionally, we believe that part of RAN4’s task should be, according to the SID, to identify the benefits of using the CDL models recommended in Proposal 2, compared to the existing TDL models.

Proposal 4: Once agreements is reached on the SCM modeling, RAN4 should identify the benefits of the chosen approach (e.g. CDL) compared to existing models (ie. TDL);

## Impact on performance of UE MIMO

The initial simulation results presented by other companies in R4-2402277 [3] show the impact of the SCM on the performances when transmitting multiple codewords compared to TDL models already used for RAN4 requirements. The performances were studied using SU-MIMO parameter configurations.

Observation 1: Simulation results shown in [2] for UE MIMO show a different behaviour across multiple CW scheduled when using SCM, compared to homogeneous behaviour in TDL;

We propose to down-select a number of existing MIMO requirements in 38.101-4, to be used as reference parameters for performance alignment in this work item. In particular, we think it is reasonable to limit the initial set for alignment to SU-MIMO test cases, and only after a reasonable level of alignment is reached with this setup, introduce MU-MIMO test cases.

Proposal 5: RAN4 to down-select a number of FR1 conducted UE-MIMO requirements in 38.101-4 as reference test cases for performance alignment in this SI.

Proposal 6: For the initial alignment RAN4 to focus on SU-MIMO requirements only, for both UE and BS Demodulation requirements. MU-MIMO test cases can be simulated after SU-MIMO results are reasonably aligned.

## Filtering using Base Station Antenna Beams

For the purposes of alignment, it is also worth highlighting that the channel models in Section 7.2.1 in [2] do not include the effect of base station antenna filtering using beams, which is separately described in Section 7.3 in [2].

Observation 2: Channel Models defined in Section 7.2.1 of TR 38.827 do not include base station antenna filtering;

Addressing that spatial filtering by the base station antenna, we have extrapolated the note below from [3] on how to address the beamforming capabilities:

Note: We understand the intent behind this AE weight definition is to create an abstraction between baseband (BB) visible RF ports, i.e., the Tx antennas in RAN4 requirements, and beamforming capabilities (due to many AEs) behind each BB visible RF port. Hence, one target beam direction is needed for 2 RF ports (2x2 MIMO), 2 directions for 4 ports (4x4), and 4 directions for 8 ports (8x8), with directions determined as in section 7.3.

However, from the above it is not clear what is the interplay between beamforming, intended in the following as application of a Precoding configuration (e.g. a typical randomized precoder selection for every PRB bundle, chosen from section 5.2.2.2.1 of TS 38.214), and antenna beamsteering, obtained through the use of phase shifters at the Antenna Elements (e.g. see example figure below).

Proposal 7: RAN4 should discuss how to distinguish in the model, the application of the beamforming (intended as precoder selection at the gNB/TE TX antennas) and the application of the beamsteering (spatial beam direction towards the intended receiver);

# General Observations on SCM Models usage in RAN4

## Randomness and Impact on Test Methodologies

It is our view that one of the main challenges of defining requirements based on stochastic channel models is the uncertainty of the results due to the randomness of the channel realisations. This randomness causes the results to vary significantly depending on the chosen seed and the number of iterations, and during the process of simulation alignment for the definition of the requirements, this variance compounds with the (unavoidable) variance in baseline performance of different UE implementations.

Observation 3: During the process of simulation alignment for the definition of the requirements, both the randomness due to the usage of stochastic channel models and the variance in UE implementations play a role;

This is a process by which the results may vary significantly, with varying amount of impact on the final requirements: if there is large variance introduced by the channel models, the problem of defining a “minimum” requirement after collecting a number of requirements can be considered naturally more subject to the potential risk of being too tight (unduly penalizing valid UE implementations) or too relaxed (not reaching the stated goal of feature testing).

It is our understanding, after reviewing the material in [2] and [3], the SCM derived in [2] have been designed starting from the original model described in TR 38.901 with the goal to reduce the amount of statistical randomness and facilitate the usage of these models for the purposes of performance alignment.

However, we believe it will be important to consider a careful analysis of:

- Input: The random component that play a role when using the SCM defined in [2] for the purposes of simulation alignment, and the way they are used;

- Output: the average and variance of the results over multiple trials, as a starting point toward an informed assessment of the confidence level and convergence criteria of the tests.

Proposal 8:RAN4 should discuss the random components that influence the stochastic Spatial Channel Models under study and how to configure them during the simulation;

Proposal 9:RAN4 should consider the expected average and variance of the simulation results over multiple trials and simulation durations, and how this impacts the definition of the requirements;

Also, RAN4 should consider that the current RAN5 methodology for testing is based on testing the device for a duration of time (typically up to 75-150 seconds) starting the channel from a single random seed, which means that only one channel realization is used for each test case. If the analysis of the results shows large variation across seeds, RAN4 should evaluate how to address this in testing.  The opinion of our colleagues from RAN5 might also be considered as an input.

Proposal 10: RAN4 to consider the impact on testing duration when using SCM within the framework of the current RAN5 procedure (using a single random seed), and evaluate whether to ask for RAN5’s input if necessary;

## Impact of Fixed Power Angular Distribution

According to [2], we would like to highlight the following section, related to the generation of the CDL channel model:

[…]. The distribution of scalar initial phases Φn,m is uniform within -π,π. Its purpose is to enable generation of different fading sequences on different uses of the model, but still maintaining the power angular distribution of the model. The scalar initial phases can be fixed (or removed) if completely deterministic process, i.e. exactly same fading sequences at each model use, is aimed at.

It follows from the above that we can expect the test to maintain preferred angular distribution.

Observation 4: SCM defined in TR 38.827 maintain a preferred angular distribution;

We also note that, to our understanding, this is not the case for the current TDL-based tests. Although they may imply a generalized "broadside" angular preference, as it is noted in other contributions, e.g. [3], their implementation in requirements testing prevents the UE from leveraging spatial processing. Consequently, this suggests that there should be no resultant benefits or drawbacks among various types of spatial processing algorithms implemented in the UE being tested.

Observation 5: TDL-based testing is not impacted by different implementations of UE algorithms for spatial processing;

So, we believe that RAN4 should investigate the potential interactions between this preferred angular distribution and UE-specific algorithm implementations, to make sure that one hand legal baseline implementations for UEs are not penalized and, on the other hand, that UEs do not exploit this knowledge to undermine the validity of the test.

Proposal 11: RAN4 to consider the impact on UE-specific implementation performances when testing requirements using an SCM with a preferred angular distribution;

- Conclusions

Proposal 1: For implementation alignment RAN4 should focus on discussing SCM details assuming FR1 conducted test first, according to the SI description;

Proposal 2: For FR1 Conducted Tests, RAN4 to consider CDL Channel Models parameters as defined in Section 7.2.1 of 38.827 as a starting point;

Proposal 3: RAN4 to discuss any proposed modifications to the Channel Models in 38.827 before moving to the simulation alignment step;

Proposal 4: Once agreements is reached on the SCM modeling, RAN4 should identify the benefits of the chosen approach (e.g. CDL) compared to existing models (ie. TDL);

Proposal 5: RAN4 to down-select a number of FR1 conducted UE-MIMO requirements in 38.101-4 as reference test cases for performance alignment in this SI.

Proposal 6: For the initial alignment RAN4 to focus on SU-MIMO requirements only, for both UE and BS Demodulation requirements. MU-MIMO test cases can be simulated after SU-MIMO results are reasonably aligned.

Observation 2: Channel Models defined in Section 7.2.1 of TR 38.827 do not include base station antenna filtering;

Proposal 7: RAN4 should discuss how to distinguish in the model, the application of the beamforming (intended as precoder selection at the gNB/TE TX antennas) and the application of the beamsteering (spatial beam direction towards the intended receiver);

Proposal 8:RAN4 should discuss the random components that influence the stochastic Spatial Channel Models under study and how to configure them during the simulation;

Proposal 9:RAN4 should consider the expected average and variance of the simulation results over multiple trials and simulation durations, and how this impacts the definition of the requirements;

Proposal 10: RAN4 to consider the impact on testing duration when using SCM within the framework of the current RAN5 procedure (using a single random seed), and evaluate whether to ask for RAN5’s input if necessary;

Observation 4: SCM defined in TR 38.827 maintain a preferred angular distribution;

Observation 5: TDL-based testing is not impacted by different implementations of UE algorithms for spatial processing;

Proposal 11: RAN4 to consider the impact on UE-specific implementation performances when testing requirements using an SCM with a preferred angular distribution;

- References

[1] RP-241610, “Study on spatial channel model for demodulation performance requirements”, 3GPP TSG RAN Meeting #104, Shanghai, China, 17th – 20th June, 2024;

[2] 3GPP TR 38.827 V16.8.0 (2022-09), “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR User Equipment (UE); (Release 16)”

[3] R4-2402277, “Discussion on 8Rx general demodulation aspects and spatial channel models”, 3GPP TSG-RAN WG4 Meeting #110, Athens, Greece Feb 26 – Mar 1, 2024