# 3GPP TSG-RAN WG4 Meeting # 112                                                         R4-2413061

# Maastricht, The Netherlands, 19 – 23  August 2024

# Title:	n28 PC2 NS_17 A-MPR

# Agenda Item:	7.5.3.1

# Source:	Skyworks Solutions, Inc.

# Document for:	Approval

# Introduction

At the last RAN plenary meeting, a new Work Item Description (WID) [1] was agreed upon to introduce Power Class 2 (PC2) operation for Band n28. One of the objectives of this WID is to consider no change to the existing PC3 A-MPR requirements up to Release 18.5.0 and with existing channel bandwidth (CBW) up to and including 30MHz. This document follows up the previous RAN4 meeting 110, 110-bis and meeting 111 agreements captured in the Way Forward (WF) [1,2,10]. In this document we present power amplifier (PA) back-off measurements to meet the NS_17 requirements.

# Discussion

As background information, for PC3 operation, RAN4 agreed that there is no need to specify A-MPR because RAN4 assumed a minimum of 12dB filter rejection in the NS_17 range. For the case of a UE implemented with a full Band n28 duplexer, no filter rejection can be assumed since the NS_17 upper range is located inside the passband bandwidth of the Band n28 uplink (UL) band.

On the other hand, it is commonly agreed that PC2 loses its benefits if the PC2 A-MPR is 3dB greater than the PC3 A-MPR. The RAN plenary agreement [1] to not revisit the PC3 NS_17 A-MPR agreement means that the PC2 A-MPR is bound to a maximum of 3dB for NS_17. The RAN plenary agreement also implies that the PC3 PA calibration deviates from the conventional RAN4 agreement to meet -30dBc ACLR at MPR1.

Observation: With no A-MPR agreed for NS_17 PC3 operation, the PC2 A-MPR cannot exceed 3dB.

## Power Amplifier Calibration

- The PA linearity is calibrated to meet -31dBc ACLR at MPR1 29dBm (4dB post PA insertion loss) output power using a 20 MHz, SCS=15 kHz, QPSK, DFT-S-OFDM, LCRB=100 (RBstart=0) waveform.

- Waveform impairments:

  - Local Oscillator (LO) leakage: -28 dBc;

  - IQ Image rejection: -28 dB;

  - C-IM3 -60dBc, C-IM5 -70dBc.

## PC2 Measurement Results

We present the measured PA output back-off (OBO) for only the worst-case waveform DFT-S-OFDM QPSK SCS=15kHz 10MHz CBW and Lcrb=50(RBstart=0) with UL carrier at Fc=723MHz and no Tx filter rejection. Our measurements indicate that a minimum of 2.3dB OBO is needed to meet the NS_17 requirements for 1Tx PC2 operation.

Proposal: For DFT-S-OFDM QPSK, consider adopting the following NS_17 PC2 A-MPR:

- 2.5dB A-MPR for UE supporting PC2 with single-Tx.

- 3.0dB A-MPR for UEs supporting PC2 with dual-Tx.

# Conclusion

This contribution presents PA back-off results to meet the NS_17 requirements assuming a full-band duplexer.

Observation: With no A-MPR agreed for NS_17 PC3 operation, the PC2 A-MPR cannot exceed 3dB.

Proposal: For DFT-S-OFDM QPSK, consider adopting the following NS_17 PC2 A-MPR:

- 2.5dB A-MPR for UE supporting PC2 with single-Tx.

- 3.0dB A-MPR for UEs supporting PC2 with dual-Tx.

# Reference

[1]	RP-241660 New WID: Introduction of Power Class 2 and UE 40MHz Channel Bandwidth in NR band n28, 3GPP TSG-RAN Meeting #104, Shanghai, CMCC, CBN.