# 3GPP TSG-RAN WG4 Meeting # 112                                                         R4-2413062

# Maastricht, The Netherlands, 19 – 23 August 2024

# Title:	n28 40MHz PC2 PC3 REFSENS

# Agenda Item:	7.20.3

# Source:	Skyworks Solutions, Inc.

# Document for:	Approval

# Introduction

This document presents our views on REFSENS requirements for the new request to support 40MHz channel bandwidth (CBW) in Band n28 for both Power Class 3 (PC3) and Power Class 2 (PC2) [1].

# Discussion

High self-desense “MSD”is expected for Band n28 40MHz CBW operation due to the overlap of the first adjacent channel of the uplink (UL) carrier with its own downlink (DL) carrier. We present power amplifier (PA) measurements to measure the level of interference that may affect the DL 40MHz Band n28 carrier for both PC3 and PC2.

## Power Amplifier Calibration

- The PA linearity is calibrated using a 20 MHz, SCS=15 kHz, QPSK, DFT-S-OFDM, LCRB=100 (RBstart=0) waveform to meet:

  - -30dBc ACLR at MPR1 26dBm (4dB post PA insertion loss) for PC2 operation;

  - -31dBc ACLR at MPR1 29dBm (4dB post PA insertion loss) for PC2 operation.

- Waveform impairments:

  - Local Oscillator (LO) leakage: -28 dBc;

  - IQ Image rejection: -28 dB;

  - C-IM3 -60dBc, C-IM5 -70dBc.

The measurements of the PA emissions are performed over the following range of UL RB allocations “LCRB” with UL RBs positioned closest to the DL band: 1,2,15,20,25,40,64,100,128,160,216.

## PC2 Measurement Results

Figure 1 below shows the measured noise in Rx band for PC3 (orange dots) vs. PC2 single Tx (red dots) vs. UL RB allocation LCRB. The Tx noise level is a minimum for close to 100 RBs for both power classes.

Figure 1: Measured PC3 and PC2 PA noise in band n28 Rx band for 40MHz CBW.

In Rel 18.6.0, the Band n28 REFSENS is verified using the UL RB configurations reproduced in Table 1 below. For 30MHz CBW LCRB = 25RB. For 40MHz CBW operation, we can consider adopting the same configuration as for 30MHz or a smaller RB allocation. The data collected in Figure 1 shows that Tx noise increases for LCRB < 25, we therefore propose to evaluate the MSD using LCRB = 25RB. For this UL configuration we measure the following Tx noise levels:

- PC3: -11.9dBm/MBW;

- PC2: -7.9 dBm/MBW.

Table 1 Existing UL configuration specifications for band n28 (Table 7.3.2-3) for CBW up to 30MHz.

| Operating band / SCS (kHz) / Channel bandwidth (MHz) / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 | Duplex Mode |
| n28 | 15 | 15 | 25 | 251 | 251 | 251 | 251 | 251 |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 101 | 101 | 101 | 101 | 101 |  |  |  |  |  |  |  |  |  |  |
| Note 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## PC3 REFSENS, PC2 RSD

For evaluation of Reference Sensitivity (REFSENS) and Reference Sensitivity Degradation (RSD), we assume:

- An RB scaled 40MHz REFSENS is -89.2dBm based on the agreed 3MHz CBW REFSENS level of -100.2dBm;

- 50dB and 45dB duplexer rejection from Tx to Rx and Tx to antenna respectively;

- 10dB antenna to antenna isolation; and

- 4dB front-end loss.

The PC3 40MHz REFSENS is estimated at -66.3dBm due to 22.9dB self-desense/MSD.

The PC2 single Tx 40MHz MSD is 26.9dB, corresponding to an RSD of 4dB.

The PC2 dual Tx 40MHz MSD is 31.4dB, corresponding to an RSD of 8.5dB

Proposal: For band n28 40MHz CBW operation, consider the following REFSENS requirement proposal for SCS 15kHz:

- PC3 REFSENS = -66.3dBm

- PC2 single Tx RSD = 4dB

- PC2 dual Tx RSD = 8.5dB

# Conclusion

Band n28 at 40MHz CBW may be subject to severe self-desense due to ACLR 1 overlap. In this paper we measure the noise emissions of a band n28 PA and propose the following REFSENS/RSD requirements.

Proposal: For Band n28 40MHz CBW operation, consider the following REFSENS requirement proposal for SCS 15kHz:

- PC3 REFSENS = -66.3dBm

- PC2 single Tx RSD = 4dB

- PC2 dual Tx RSD = 8.5dB

Proposal: For DC_n(8)AA BCS 0, consider the following proposal:

- Restrict operation of DC_(n)8AA to single-switched UL only.

- Evaluate MSD using the following test point.

| EN-DC configuration / channel allocations /MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC configuration | E-UTRA/NR band | FC (UL) (MHz) | Channel bandwidth (MHz) | UL allocation (LCRB) | FC (DL) (MHz) | MSD (dB) | Duplex mode |
| DC_(n)8AA | 8 | N/A | 10 | N/A | 935.5 | FFS | FDD |
|  | n8 | 905.4 | 20 | 20 (RBstart = 86) | 950.4 | FFS |  |
| NOTE 1: In accordance to BCS1, the NR uplink bandwidth is specified as in this table, but the corresponding NR downlink bandwidth is 5 MHz larger. NOTE 2: The transmitters powers shall be set to PUMAX, as defined in TS 38.101-1 [2], TS 38.101-2 [3], and TS 36.101 [4], with additional limits on configured maximum output power for the uplink according to clause 6.2B.4. NOTE 3: Applicable only to BCS 2. NOTE 4: Applicable only to BCS 1. NOTE 5: Applicable only to BCS 0. |  |  |  |  |  |  |  |

# Reference

[1]	RP-241660 New WID: Introduction of Power Class 2 and UE 40MHz Channel Bandwidth in NR band n28, 3GPP TSG-RAN Meeting #104, Shanghai, CMCC, CBN.