# 3GPP TSG-RAN WG4 Meeting # 112                                                         R4-2413063

# Maastricht, The Netherlands, 19th – 23rd August 2024

# Title:	UL harmonic MSD clean-up

# Agenda Item:	5.2

# Source:	Skyworks Solutions, Inc.

# Document for:	Approval

# Introduction

This document follows up the previous RAN4 meeting 110, 110-bis and meeting 111 agreements captured in the Way Forward (WF) [1,2,10]. A total of 306 uplink (UL) harmonic MSD test points are reviewed in three change requests: 114 PC3 NR-CA, 34 PC2 NR-CA, and 158 EN-DC MSD test points. In this paper we discuss only the key changes and key MSD test point corrections.

# Discussion

## Previous meeting agreements [1,2]

Agreement 1 [1]

- Near-miss MSD test points are only specified for UL2DL1

- Near-miss MSD test points are not needed when direct-hit collision occurs

Agreement 2 [1]

- The smallest supported UL CBW is specified for all MSD test points

Agreement 3 [2]

For UL Harmonic and Rx harmonic mixing MSD test points, RAN4 recommends that UL Lcrb are corrected as follows for SCS 15kHz / SCS 30kHz respectively:

- For UL2/DLx test point, Lcrb=12RB / 6RB

- For UL3/DLx test point, Lcrb=8RB / 4RB

- For UL4/DLx test point, Lcrb=6RB / 3RB

- For UL5/DLx test point, Lcrb=5RB / 2RB

Options to resolve issues with RBstart=0 [3]

- Option 1: RAN4 corrects all RBstart=0 values to ensure that the non-fully allocated RBs are centered in the middle of the UL channel. This means correcting several NR-CA, and EN-DC test points,

- Option 2: RAN4 removes RBstart=0 from all UL Harmonic and Rx Harmonic mixing MSD tables, and

Adds an equation in the core requirement text that guarantees the UL RBs are centered in the middle of the UL channel,

- Option 3: RAN4 removes RBstart=0 from all UL harmonic and Rx harmonic mixing MSD tables, and

Adds a sentence in the core requirement text that RAN4 MSD levels assume the UL RBs are centered in the middle of the UL channel. Informs RAN5 that RBstart are no longer specified and that RAN5 needs to specify RBstart in the test requirement technical specifications.

- Other options are not precluded.

Proposal 1: To resolve the issues with RBstart=0 [3], an alternative solution is proposed which removes RBstart=0 from all UL harmonic MSD tables (NR-CA and EN-DC). To ensure UL RBs are centered in the middle of the UL carrier, an equation that defines RBstart is added to the NOTE 1 of TS 38.101-1 Table 7.3A.4-1. It is also proposed to clarify that Note 1 is specific to the definition of direct-hit harmonic MSD test points. See Figure 1 – CR [3].

Figure 1: RBstart equation to ensure the UL RB allocation is centered in the UL channel [3] / direct-hit clarification.

## Impact of LCRB scaling on the Direct-hit 1st MSD test points.

Power class 3 (PC3) power amplifier (PA) measurements are presented to verify the impact of adopting the WF [2] Lcrb scaling rule onto the 1st test point MSD levels:

- PA is calibrated to meet ACLR 30dBc at 26dBm (PC3 MPR1) output power using DFT QPSK 20MHz Lcrb=100(RBstart=0) SCS 15kHz waveform.

- PA harmonic 2,3,4,5 (H2,H3,H4,H5) are measured using  5MHz UL CBW DFT QPSK SCS15kHz configuration and with centered UL RB allocations Lcrb=12,8,6,5 RBs [2].

PC3 MSD levels are re-evaluated assuming equal harmonic power affecting the primary and diversity antenna ports.

The NR-CA PC2 MSD levels are re-evaluated based on MSD simplification guidelines of [4].

### Impact of LCRB=12 on UL2/DL1 1st test points

Table 1 and Table 2 summarize respectively the measured H2 interference level change due to adopting LCRB=12 (SCS 15kHz) for each NR-CA PC3 band combination and each EN-DC combination. The CA_n2-n48, DC_2_n48, DC_48_n2 are special combinations which are treated in clause 2.4.1. The CA_n28-n50, CA_n28-n74, CA_n28-n75, CA_n28-n94 and DC_28_n50, DC_28_n51, DC_28_n75 are treated in separate clause 22.5.2.

Table 1 Impact of LCRB=12 on PC3 NR-CA UL2/DL1 direct-hit 1st MSD test points

| UL band | DL band | UL BW (MHz) | SCS of UL band (kHz) | Rel. 18.6.0 UL RB Allocation Lcrb | WF [2] UL RB Allocation Lcrb | DL BW (MHz) | Rel 18.6.0 MSD (dB) | Rel 18.6.0 H2 level (dBm) | Lcrb=12 H2 level (dBm) | H2 level change (dB) | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 | n77 | 5 | 15 | 25 (RBstart=0) | 12 | 10 | 23.9 | -13.9 | -13 | 0.9 | UL2/DL1 direct-hit |
| n2 | n48 | 5 | 15 | 25 (RBstart=0) | 12 | 5 | 27.1 | Removed: There is no direct hit – See clause 2.4.1. |  |  |  |
| n2 | n77 | 5 | 15 | 25 (RBstart=0) | 12 | 10 | 23.9 | -13.9 | -13 | 0.9 |  |
| n2 | n78 | 5 | 15 | 25 (RBstart=0) | 12 | 10 | 23.9 | -13.9 | -13 | 0.9 |  |
| n3 | n77 | 5 | 15 | 25 (RBstart=0) | 12 | 10 | 23.9 | -13.9 | -13 | 0.9 |  |
| n3 | n78 | 5 | 15 | 25 (RBstart=0) | 12 | 10 | 23.9 | -13.9 | -13 | 0.9 |  |
| n8 | n3 | N/A | N/A | N/A | 12 | N/A | N/A | N/A | N/A |  |  |
| n24 | n77 | 5 | 15 | 25 (RBstart=0) | 12 | 10 | N/A | N/A | N/A |  |  |
| n25 | n77 | 5 | 15 | 25 (RBstart=0) | 12 | 10 | 23.9 | -13.9 | -13 | 0.9 |  |
| n25 | n78 | 5 | 15 | 25 (RBstart=0) | 12 | 10 | 23.9 | -13.9 | -13 | 0.9 |  |
| n28 | n50 | 5 | 15 | 12 (RBstart=0) | 12 | 5 | 23 | Special combinations. See clause 22.5.2. |  |  |  |
| n28 | n74 | 5 | 15 | 12 (RBstart=0) | 12 | 5 | 23.1 |  |  |  |  |
| n28 | n75 | 5 | 15 | 12 (RBstart=0) | 12 | 5 | 28.1 |  |  |  |  |
| n28 | n94 | 5 | 15 | 12 (RBstart=0) | 12 | 5 | 28.1 |  |  |  |  |
| n66 | n48 | 5 | 15 | 12 (RBstart=0) | 12 | 5 | 27.1 | no Lcrb change |  |  |  |
| n66 | n77 | 5 | 15 | 25 (RBstart=0) | 12 | 10 | 23.9 | -13.9 | -13 | 0.9 |  |
| n66 | n78 | 5 | 15 | 25 (RBstart=0) | 12 | 10 | 23.9 | -13.9 | -13 | 0.9 |  |
| n70 | n77 | 5 | 15 | 25 (RBstart=0) | 12 | 10 | 23.9 | -13.9 | -13 | 0.9 |  |
| n70 | n78 | 5 | 15 | 25 (RBstart=0) | 12 | 10 | 23.9 | -13.9 | -13 | 0.9 |  |
| n78 | n104 | 10 | 15 | 50 (RBstart=0) | 12 | 20 | 37.5 | -14.4 | -12.9 | 1.5 |  |

Table 2 Impact of LCRB=12 on the EN-DC UL2/DL1 direct-hit 1st MSD test points

| UL band | DL band | UL BW (MHz) | SCS of UL band (kHz) | Rel. 18.6.0 UL RB Allocation Lcrb | WF [2] UL RB Allocation Lcrb | DL BW | Rel 18.6.0 MSD (dB) | Rel 18.6.0 direct-hit H2 level (dB) | Lcrb 12 direct hit H2 level (dB) | DH2_Lcrb (dB) | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n77 | 5 | 15 | 25 (RBstart=0) | 12 | 10 | 23.9 | -13.9 | -13 | 0.9 | UL2/DL1 direct-hit |
| 2 | n48 | 5 | 15 | 25 (RBstart=0) | 12 | 10 | 24.4 | Removed: There is no direct-hit. .See clause 2.4.1. |  |  |  |
| n2 | 48 | 5 | 15 | 12 (RBstart=0) | 12 | 5 | 27.1 |  |  |  |  |
| 2 | n77 | 5 | 15 | 25 (RBstart=0) | 12 | 10 | 23.9 | -13.9 | -13 | 0.9 |  |
| 2 | n78 | 5 | 15 | 25 (RBstart=0) | 12 | 10 | 23.9 | -13.9 | -13 | 0.9 |  |
| 3 | n77 | 5 | 15 | 25 (RBstart=0) | 12 | 10 | 23.9 | -13.9 | -13 | 0.9 |  |
| 3 | n78 | 5 | 15 | 25 (RBstart=0) | 12 | 10 | 23.9 | -13.9 | -13 | 0.9 |  |
| n3 | 42 | 5 | 15 | 12 (RBstart=0) | 12 | 5 | 27.3 | no Lcrb change |  |  |  |
| 4 | n78 | 5 | 15 | 25 (RBstart=0) | 12 | 10 | 23.9 | -13.9 | -13 | 0.9 |  |
| 8 | n3 | NA | NA | NA | 12 | NA | NA | N/A | N/A | N/A |  |
| n8 | 3 | NA | NA | NA | 12 | NA | NA | N/A | N/A | N/A |  |
| 25 | n77 | 5 | 15 | 25 (RBstart=0) | 12 | 10 | 23.9 | -13.9 | -13 | 0.9 |  |
| 25 | n78 | 5 | 15 | 25 (RBstart=0) | 12 | 10 | 23.9 | -13.9 | -13 | 0.9 |  |
| n25 | 48 | 5 | 15 | 12 (RBstart=0) | 12 | 5 | 27.1 | no Lcrb change |  |  |  |
| n28 | 11 | 5 | 15 | 12 (RBstart=0) | 12 | 5 | 24.8 | no Lcrb change |  |  |  |
| n28 | 21 | NA | NA | NA | 12 | NA | NA | N/A | N/A | N/A |  |
| n28 | 32 | 5 | 15 | 12 (RBstart=0) | 12 | 5 | 28.1 | no Lcrb change |  |  |  |
| 28 | n50 | 5 | 15 | 25 (RBstart=0) | 12 | 10 | 24.6 | Special combination treated in clause 22.5.2 |  |  |  |
| 28 | n51 | 5 | 15 | 12 (RBstart=0) | 12 | 5 | 27.8 |  |  |  |  |
| 28 | n75 | 5 | 15 | 12 (RBstart=0) | 12 | 5 | 28.1 |  |  |  |  |
| 66 | n48 | 5 | 15 | 12 (RBstart=0) | 12 | 10 | 24.4 | no Lcrb change |  |  |  |
| n66 | 48 | 5 | 15 | 12 (RBstart=0) | 12 | 5 | 27.1 | no Lcrb change |  |  |  |
| 66 | n77 | 5 | 15 | 25 (RBstart=0) | 12 | 10 | 23.9 | -13.9 | -13 | 0.9 |  |
| 66 | n78 | 5 | 15 | 25 (RBstart=0) | 12 | 10 | 23.9 | -13.9 | -13 | 0.9 |  |

An example of the difference in H2 power spectral density (PSD) decay is plotted in Figure 2 at 27dBm output power comparing LCRB =25 vs LCRB = 12 of a 5MHz CBW SCS15 kHz DFT-s-OFDM QPSK waveform.

Figure 2: Measured H2 in 10MHz BW at 27dBm for SCS15kHz, 5MHz UL CBW, LCRB=25 vs LCRB=12.

Observation 1: For the UL2/DL1 1st test points:

- For NR-CA (Table 1):

  - Adoption of LCRB = 12 increases the H2 level by 0.9dB for all band combinations except for CA_n78-n104 where H2 increases by 1.5dB.

  - Considering the MSD levels are already very high (23.9dB, 37dB etc…) we do not see the need to further increase the MSD to account for the slight H2 level increase (0.9dB or 1.5dB).

- For EN-DC (Table 2):

  - Same observation: Adoption of LCRB = 12 increases the H2 level by 0.9dB for band combinations affected by very high MSD levels (>=23.9dB). We believe there is no need to correct the MSD levels to reflect this increase of H2 level.

### Impact of LCRB=8 on UL3/DL1 1st test points

Table 3 and Table 4 summarize respectively the measured H2 interference level change due to adopting LCRB=8 (SCS 15kHz) for each NR-CA PC3 band combination and each EN-DC combination. The CA_n2-n71 and CA_n25-n71 band combinations which are treated in clause 2.5.1.

Table 3 Impact of LCRB=8 on PC3 NR-CA UL3/DL1 direct-hit 1st MSD test points.

| UL band | DL band | UL BW (MHz) | SCS of UL band (kHz) | Rel. 18.6.0 UL RB Allocation Lcrb | WF [2] UL RB Allocation Lcrb | DL BW (MHz) | Rel 18.6.0 MSD (dB) | Rel 18.6.0 H2 level (dBm) | Lcrb=8 H2 level (dBm) | H2 level change (dB) | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n5 | n41 | 5 | 15 | 16 (RBstart=0) | 8 | 10 | 10.3 | -45 | -45 | 0 | UL3/DL1 direct-hit |
| n8 | n7 | 5 | 15 | 8 (RBstart=0) | 8 | 5 | 10 | no Lcrb change |  |  |  |
| n8 | n41 | 5 | 15 | 16 (RBstart=0) | 8 | 10 | 13 | -45 | -45 | 0 |  |
| n12 | n66 | 5 | 15 | 8 (RBstart=0) | 8 | 5 | 10 | no Lcrb change |  |  |  |
| n28 | n1 | 5 | 15 | 8 (RBstart=0) | 8 | 5 | 10.2 | no Lcrb change |  |  |  |
| n71 | n2 | 5 | 15 | 8 (RBstart=0) | 8 | 5 | 10 | Special combinations. See clause 2.5.1 |  |  |  |
| n71 | n25 | 5 | 15 | 8 (RBstart=0) | 8 | 5 | 10 |  |  |  |  |
| n71 | n70 | 5 | 15 | 8 (RBstart=0) | 8 | 5 | 9.9 | no Lcrb change |  |  |  |
| n85 | n66 | 5 | 15 | 8 (RBstart=0) | 8 | 10 | 10 | no Lcrb change |  |  |  |
| n105 | n7 | 5 | 15 | 25 (RBstart=0) | 8 | 5 | 14.6 | -48.7 | -45.2 | 3.5 |  |

Table 4 Impact of LCRB=8 on EN-DC UL3/DL1 direct-hit 1st MSD test points.

| UL band | DL band | UL BW (MHz) | SCS of UL band (kHz) | Rel. 18.6.0 UL RB Allocation Lcrb | WF [2] UL RB Allocation Lcrb | DL BW (MHz) | Rel 18.6.0 MSD (dB) | Rel 18.6.0 H2 level (dBm) | Lcrb=8 H2 level (dBm) | H2 level change (dB) | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | n7 | 5 | 15 | 8 (RBstart=0) | 8 | 5 | 10 | no Lcrb change |  |  | UL3/DL1 direct-hit |
| 8 | n41 | 5 | 15 | 8 (RBstart=0) | 8 | 10 | 13 | no Lcrb change |  |  |  |
| n8 | 7 | 5 | 15 | 8 (RBstart=0) | 8 | 5 | 10 | no Lcrb change |  |  |  |
| 12 | n66 | 5 | 15 | 8 (RBstart=0) | 8 | 5 | 10 | no Lcrb change |  |  |  |
| n12 | 66 | 5 | 15 | 8 (RBstart=0) | 8 | 5 | 10 | no Lcrb change |  |  |  |
| 20 | n38 | 5 | 15 | 8 (RBstart=0) | 8 | 5 | 12.9 | no Lcrb change |  |  |  |
| 20 | n41 | 5 | 15 | 16 (RBstart=0) | 8 | 10 | 10.3 | -45 | -45 | 0 |  |
| 26 | n41 | 5 | 15 | 16 (RBstart=0) | 8 | 10 | 10.3 | -45 | -45 | 0 |  |
| 28 | n1 | 5 | 15 | 8 (RBstart=0) | 8 | 5 | 10.2 | no Lcrb change |  |  |  |
| n28 | 1 | 5 | 15 | 8 (RBstart=0) | 8 | 5 | 10.2 | no Lcrb change |  |  |  |
| n28 | 4 | 5 | 15 | 8 (RBstart=0) | 8 | 5 | 10.2 | no Lcrb change |  |  |  |
| n28 | 66 | 5 | 15 | 8 (RBstart=0) | 8 | 5 | 10.2 | no Lcrb change |  |  |  |
| 28 | n66 | 5 | 15 | 8 (RBstart=0) | 8 | 5 | 10.2 | no Lcrb change |  |  |  |
| 71 | n2 | 5 | 15 | 25 (RBstart=0) | 8 | 5 | 4.6 | Special combinations. See clause 2.5.1 |  |  |  |
| 71 | n2 | 5 | 15 | 25 (RBstart=0) | 8 | 20 | 0.6 |  |  |  |  |
| n71 | 2 | 5 | 15 | 25 (RBstart=0) | 8 | 5 | 4.6 |  |  |  |  |
| n71 | 2 | 5 | 15 | 25 (RBstart=0) | 8 | 20 | 0.6 |  |  |  |  |
| 71 | n25 | 5 | 15 | 25 (RBstart=0) | 8 | 5 | 4.6 |  |  |  |  |
| 71 | n25 | 5 | 15 | 25 (RBstart=0) | 8 | 20 | 0.6 |  |  |  |  |

Figure 3: Measured H2 in 5MHz BW at 27dBm for SCS15kHz, 5MHz UL CBW, LCRB=25 vs LCRB=8.

Observation 2: For the UL3/DL1 1st test points:

- For NR-CA (Table 3): With a 3.5dB increase of the H3 level, the adoption of LCRB=8 impacts the band n7 PC3 5MHz MSD for CA_n7-n105.

- For EN-DC (Table 4): The adoption of LCRB=8 does not impact any EN-DC combination.

The CA_n2-n71, CA_n25-n71, DC_71_n2, DC_2_n71, DC_71_n25 are special combinations which are treated in clause 2.5.1.

Proposal 2: Consider adopting the corrected the PC3 band n7 MSD for CA_n7-n105 of Figure 4.

Figure 4: CA_n7-n105 PC3 MSD correction due to adoption of LCRB=8.

### Impact of LCRB=6 on UL4/DL1 1st test points

Table 5 and Table 6 summarize respectively the measured H2 interference level change due to adopting LCRB=6 (SCS 15kHz) for each NR-CA PC3 band combination and each EN-DC combination.

Table 5 Impact of LCRB=6 on PC3 NR-CA UL4/DL1 direct-hit 1st MSD test points.

| UL band | DL band | UL BW (MHz) | SCS of UL band (kHz) | Rel. 18.6.0 UL RB Allocation Lcrb | WF [2] UL RB Allocation Lcrb | DL BW (MHz) | Rel 18.6.0 MSD (dB) | Rel 18.6.0 H2 level (dBm) | Lcrb=6 H2 level (dBm) | H2 level change (dB) | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n5 | n77 | 5 | 15 | 16 (RBstart=0) | 6 | 10 | 10.5 | -45.4 | -45.3 | 0.1 | UL4/DL1 direct-hit |
| n5 | n78 | 5 | 15 | 16 (RBstart=0) | 6 | 10 | 10.5 | -45.4 | -45.3 | 0.1 |  |
| n8 | n77 | 5 | 15 | 16 (RBstart=0) | 6 | 10 | 10.8 | -45.4 | -45.3 | 0.1 |  |
| n8 | n78 | 5 | 15 | 16 (RBstart=0) | 6 | 10 | 10.8 | -45.4 | -45.3 | 0.1 |  |
| n20 | n78 | 5 | 15 | 16 (RBstart=0) | 6 | 10 | 10.8 | -45.4 | -45.3 | 0.1 |  |
| n26 | n77 | 5 | 15 | 16 (RBstart=0) | 6 | 10 | N/A | -45.4 | -45.3 | 0.1 |  |
| n26 | n78 | 5 | 15 | 16 (RBstart=0) | 6 | 10 | 10.8 | -45.4 | -45.3 | 0.1 |  |
| n71 | n7 | 5 | 15 | 8 (RBstart=0) | 6 | 5 | 14.6 | -45.3 | -45.3 | 0 |  |
| n71 | n41 | 5 | 15 | 16 (RBstart=0) | 6 | 10 | 10.8 | -45.4 | -45.3 | 0.1 |  |
| n92 | n78 | 5 | 15 | 16 (RBstart=0) | 6 | 10 | 10.8 | -45.4 | -45.3 | 0.1 |  |
| n94 | n78 | 5 | 15 | 16 (RBstart=0) | 6 | 10 | 10.8 | -45.4 | -45.3 | 0.1 |  |

Table 6 Impact of LCRB=6 on EN-DC UL4/DL1 direct-hit 1st MSD test points.

| UL band | DL band | UL BW (MHz) | SCS of UL band (kHz) | Rel. 18.6.0 UL RB Allocation Lcrb | WF [2] UL RB Allocation Lcrb | DL BW (MHz) | Rel 18.6.0 MSD (dB) | Rel 18.6.0 H2 level (dBm) | Lcrb=6 H2 level (dBm) | H2 level change (dB) | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | n77 | 5 | 15 | 16 (RBstart=0) | 6 | 10 | 10.5 | -45.4 | -45.3 | 0.1 | UL4/DL1 direct-hit |
| 5 | n78 | 5 | 15 | 16 (RBstart=0) | 6 | 10 | 10.5 | -45.4 | -45.3 | 0.1 |  |
| 8 | n77 | 5 | 15 | 16 (RBstart=0) | 6 | 10 | 10.8 | -45.4 | -45.3 | 0.1 |  |
| 8 | n78 | 5 | 15 | 16 (RBstart=0) | 6 | 10 | 10.8 | -45.4 | -45.3 | 0.1 |  |
| 19 | n77 | 5 | 15 | 10 (RBstart=0) | 6 | 10 | 10.5 | -45.1 | -45.3 | -0.2 |  |
| 19 | n78 | 5 | 15 | 10 (RBstart=0) | 6 | 10 | 10.5 | -45.1 | -45.3 | -0.2 |  |
| 20 | n77 | 5 | 15 | 16 (RBstart=0) | 6 | 10 | 10.8 | -45.4 | -45.3 | 0.1 |  |
| 20 | n78 | 5 | 15 | 16 (RBstart=0) | 6 | 10 | 10.8 | -45.4 | -45.3 | 0.1 |  |
| 26 | n77 | 5 | 15 | 16 (RBstart=0) | 6 | 10 | 10.8 | -45.4 | -45.3 | 0.1 |  |
| 26 | n78 | 5 | 15 | 16 (RBstart=0) | 6 | 10 | 10.8 | -45.4 | -45.3 | 0.1 |  |
| 71 | n7 | 5 | 15 | 8 (RBstart=0) | 6 | 5 | 14.6 | -45.3 | -45.3 | 0 |  |
| n71 | 7 | 5 | 15 | 8 (RBstart=0) | 6 | 5 | 14.6 | -45.3 | -45.3 | 0 |  |
| 71 | n41 | 5 | 15 | 16 (RBstart=0) | 6 | 10 | 10.8 | -45.4 | -45.3 | 0.1 |  |
| n105 | 7 | 5 | 15 | 8 (RBstart=0) | 6 | 5 | 14.6 | -45.3 | -45.3 | 0 |  |

Figure 5 shows the measured H4 PSD of LCRB = 16 vs LCRB = 6 of a 5MHz CBW SCS15 kHz DFT-s-OFDM QPSK waveform.

Figure 5: Measured H2 in 10MHz BW at 27dBm for SCS15kHz, 5MHz UL CBW, LCRB=16 vs LCRB=6.

Observation 3: For the UL4/DL1 1st test points:

- For NR-CA (Table 5) and for EN-DC (Table 6), the adoption of LCRB=6 does not impact the 1st test point MSD of any band combination.

### Impact of LCRB=5 on UL5/DL1 1st test points

Table 7 Impact of LCRB=5 on PC3 NR-CA UL5/DL1 direct-hit 1st MSD test points.

| UL band | DL band | UL BW (MHz) | SCS of UL band (kHz) | Rel. 18.6.0 UL RB Allocation Lcrb | WF [2] UL RB Allocation Lcrb | DL BW (MHz) | Rel 18.6.0 MSD (dB) | Rel 18.6.0 H2 level (dBm) | Lcrb=5 H2 level (dBm) | H2 level change (dB) | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n5 | n77 | 5 | 15 | 16 (RBstart=0) | 5 | 10 | 10.4 | -64.4 | -64.1 | 0.3 | UL5/DL1 direct-hit |
| n8 | n79 | 5 | 15 | 16 (RBstart=0) | 5 | 10 | 12 | -64.4 | -64.1 | 0.3 |  |
| n12 | n48 | 5 | 15 | 10 (RBstart=0) | 5 | 10 | 10.4 | -64 | -64.1 | -0.1 |  |
| n12 | n77 | 5 | 15 | 10 (RBstart=0) | 5 | 10 | 10.4 | -64 | -64.1 | -0.1 |  |
| n12 | n78 | 5 | 15 | 10 (RBstart=0) | 5 | 10 | 10.4 | -64 | -64.1 | -0.1 |  |
| n13 | n77 | 5 | 15 | 10 (RBstart=0) | 5 | 10 | 10.4 | -64 | -64.1 | -0.1 |  |
| n14 | n77 | 5 | 15 | 10 (RBstart=0) | 5 | 10 | 10.4 | -64 | -64.1 | -0.1 |  |
| n18 | n77 | 5 | 15 | 16 (RBstart=0) | 5 | 10 | 10.4 | -64.4 | -64.1 | 0.3 |  |
| n28 | n77 | 5 | 15 | 10 (RBstart=0) | 5 | 10 | 10.4 | -64 | -64.1 | -0.1 |  |
| n28 | n78 | 5 | 15 | 10 (RBstart=0) | 5 | 10 | 10.4 | -64 | -64.1 | -0.1 |  |
| n71 | n77 | 5 | 15 | 10 (RBstart=0) | 5 | 10 | 10.4 | -64 | -64.1 | -0.1 |  |
| n71 | n78 | 5 | 15 | 10 (RBstart=0) | 5 | 10 | 10.4 | -64 | -64.1 | -0.1 |  |
| n85 | n77 | 5 | 15 | 10 (RBstart=0) | 5 | 10 | 10.4 | -64 | -64.1 | -0.1 |  |
| n105 | n78 | 5 | 15 | 25 (RBstart=0) | 5 | 10 | 10.4 | -65.6 | -64.1 | 1.5 |  |

Table 8 Impact of LCRB=5 on EN-DC UL5/DL1 direct-hit 1st MSD test points.

| UL band | DL band | UL BW (MHz) | SCS of UL band (kHz) | Rel. 18.6.0 UL RB Allocation Lcrb | WF [2] UL RB Allocation Lcrb | DL BW (MHz) | Rel 18.6.0 MSD (dB) | Rel 18.6.0 H2 level (dBm) | Lcrb=5 H2 level (dBm) | H2 level change (dB) | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | n77 | 5 | 15 | 10 (RBstart=0) | 5 | 10 | 10.4 | -64 | -64.1 | -0.1 | UL5/DL1 direct-hit |
| 8 | n79 | 5 | 15 | 10 (RBstart=0) | 5 | 10 | 12 | -64 | -64.1 | -0.1 |  |
| n12 | 48 | 5 | 15 | 5 (RBstart=0) | 5 | 5 | 13 |  |  |  |  |
| 12 | n77 | 5 | 15 | 10 (RBstart=0) | 5 | 10 | 10.4 | -64 | -64.1 | -0.1 |  |
| 12 | n78 | 5 | 15 | 10 (RBstart=0) | 5 | 10 | 10.4 | -64 | -64.1 | -0.1 |  |
| 13 | n77 | 5 | 15 | 10 (RBstart=0) | 5 | 10 | 10.4 | -64 | -64.1 | -0.1 |  |
| 14 | n77 | 5 | 15 | 10 (RBstart=0) | 5 | 10 | 10.4 | -64 | -64.1 | -0.1 |  |
| 18 | n77 | 5 | 15 | 10 (RBstart=0) | 5 | 10 | 10.4 | -64 | -64.1 | -0.1 |  |
| 18 | n78 | 5 | 15 | 10 (RBstart=0) | 5 | 10 | 10.4 | -64 | -64.1 | -0.1 |  |
| 19 | n77 | 5 | 15 | 10 (RBstart=0) | 5 | 10 | 10.4 | -64 | -64.1 | -0.1 |  |
| n28 | 42 | 5 | 15 | 10 (RBstart=0) | 5 | 5 | 14.1 | -65 | -64 | 1 |  |
| 28 | n77 | 5 | 15 | 10 (RBstart=0) | 5 | 10 | 10.4 | -64 | -64.1 | -0.1 |  |
| 28 | n78 | 5 | 15 | 10 (RBstart=0) | 5 | 10 | 10.4 | -64 | -64.1 | -0.1 |  |
| 71 | n77 | 5 | 15 | 10 (RBstart=0) | 5 | 10 | 10.4 | -64 | -64.1 | -0.1 |  |
| 71 | n78 | 5 | 15 | 10 (RBstart=0) | 5 | 10 | 10.4 | -64 | -64.1 | -0.1 |  |

Figure 6: Measured H5 in 10MHz BW at 27dBm for SCS15kHz, 5MHz UL CBW, LCRB=25 vs LCRB=5.

.

Observation 4: For the UL5/DL1 1st test points:

- For NR-CA (Table 7): With a 1.5dB increase of the H5 level, the adoption of LCRB=5 impacts the band n78 PC3 10MHz MSD for CA_n78-n105.

- For EN-DC (Table 8): With a 1dB increase of the H5 level, the adoption of LCRB=5 impacts the band 42 PC3 5MHz MSD for DC_42-n28.

Proposal 3: To account respectively for 1.5dB and 1dB increase of the H5 level, consider adopting the corrected band n78 10MHz PC3 MSD for CA_n78-n105 and band 42 5MHz MSD for DC_42_n28 of Figure 7.

Figure 7: MSD correction due to the adoption of LCRB=5 for CA_n78-n105 PC3 MSD and DC_42_n28.

## Correction to 2nd direct-hit MSD test points.

For direct-hit 2nd MSD test points, the CR [3] implements the following corrections:

- Based on agreement 2, the UL CBW are corrected to specify the MSD using the smallest UL CBW

- Whenever needed, the MSD level is corrected according to the principles of [5,6], i.e.:

  - Step 1: Calculate the harmonic interference that corresponds to the 1st MSD test point assuming MRC combining of equal interference affecting the primary/diversity antenna ports

  - Step 2: Calculate the 2nd test point MSD using the harmonic interference level found in step 1.

This approach is valid since the harmonic interference level is constant for all DL CBW thanks to the adoption on identical UL CBW and identical UL RB allocation for both the 1st and the 2nd MSD test point.

Examples of these MSD re-evaluations can be found in [5,6]. The corrected MSD levels can be found in [3].

## Impact of WF [2] LCRB scaling on near-miss MSD test points.

After implementation of agreement 1, the near-miss MSD test points become exception test cases as there remain only 3 test points for NR-CA (CA_n2-n48, CA_n25-n48 and CA_n7-n79) and 5 test points for EN-DC (DC_2_n48, DC_48_n2, DC_7_n79, DC_48_n25, DC_1_n105). PA harmonic measurements are used in this section to check whether the WF [2] LCRB scaling rule can be applied to near-miss MSD test points.

### CA_n2-n48 / CA_n25-n48 / DC_2_n48 / DC_48_n2.

One near-miss and two direct-hit UL2/DL1 MSD test points are currently specified for CA_n2-n48, DC_2_n48, DC_48_n2. For example, the agreed MSD test points for CA_n2-n48 are reproduced in Figure 8.

Figure 8: CA_n2-n48 agreed UL2/DL1 MSD test points (from Table 7.3A.4-1).

Issue #1 on direct-hit test points: It was shown in [5] that these band combinations do not mee the Note 1 direct-hit criteria of TS 38.101-1 Table 7.3A.4-1. These band combinations are affected only by near-miss harmonic MSD as shown in Figure 9 (reproduced from [5]).

Figure 9: The UL2/DL1 direct-hit criteria are not met for CA_n2-n48, DC_2_n48, DC_48_n2.

Proposal 4: Remove the direct-hit MSD test points from TS 38.101-1 for CA_n2-n48 and from TS 38.101-3 for DC_2_n48, DC_48_n2.

Issue #2 on the near-miss test point:  The MSD may need to be re-evaluated due to the following changes:

- The smallest supported of 5MHz for band n48 DL CBW should be specified instead of currently agreed test point at 10MHz CBW.

- According to the Note 6 of TS 38.101-1 Table 7.3A.4-1, the near-miss MSD level is specified assuming the affected DL band carrier frequency (high-band “HB”) is separated from the 2nd harmonic center frequency by a distance of 20MHz+BWChannelHB/2, i.e. by 22.5MHz for the case of band n28 5MHz CBW.

This raises two questions:

  - For band n109 which is also affected by UL2/DL1 direct-hit harmonic interference, it was agreed that the near-miss MSD can be neglected for - quoting NOTE 11 of TS 38.101-1 Table 7.3.2-1b -  “… DL channels for which channels edges are > 15 MHz away from 2xFUL at 15 kHz..”. For CA_n2-n48, the band n109 agreement means that the near-miss MSD of band n48 5MHz CBW can be neglected for carrier center frequency to harmonic center frequency separation distance of 15+5/2 = 17.5MHz.  This implies that Note 6 may need to be revisited.

  - According to WF [2], an UL RB allocation of LCRB=12 shall be specified for all UL2/DL1 MSD test points at SCS 15kHz. However, due to the sharp roll-off of the LCRB=12  2nd harmonic PSD, it is not clear at which frequency separation and which UL RB allocation can a near-miss MSD test point be specified.

Observation 5: For CA_n2-n48 and its EN-DC counterparts, the near-miss MSD needs to be re-evaluated to account for the lowest 5MHz CBW and to address the following questions:

- According to the band n109 agreements, the near-miss MSD can be neglected for the frequency separation distance currently specified in Note 6 of TS 38.101-1 Table 7.3A.4-1/ Note 6 of TS 38.101-3 Table 7.3B.2.3.1-1,

- According to WF [2], the near-miss UL2/DL1 should be specified with LCRB=12. However, due to the sharp roll-off of the 2nd harmonic PSD, it is not clear at which frequency separation and which UL RB allocation can a near-miss MSD test point be specified.

To answers these questions, we evaluate the H2 level in 5MHz CBW for 4 different frequency separation distances:

- 20MHz+BWChannelHB/2, i.e. at 22.5MHz offset (current Note 6),

- 15MHz+BWChannelHB/2, i.e. at 17.5MHz offset,

- 10MHz+BWChannelHB/2, i.e at 12.5MHz offset,

- at the minimum distance that separates the band n48 5MHz uppermost channel carrier frequency from the H2 center frequency, here 7.5MHz,

and for two 5MHz CBW UL waveforms: LCRB=12 (WF [2]) and LCRB=25 (current test point). Three of these test cases are illustrated with the sketches of Figure 10 for LCRB=12.

Figure 10: UL 5MHz LCRB=12 H2 measurements in 5MHz CBW vs different separation distances for CA_n2-n48.

We summarize in Table 9 our measurement results and corresponding band n48 uppermost 5MHz channel MSD. These measurements indicate that

- For LCRB=25RB

  - the MSD is close to 0dB for the current near-miss configuration of Note 6 20MHz+BWChannelHB/2. This confirms the band n109 agreements.

  - The MSD ranges from 1.3 to 18.2dB at minimum distance. At 12.5MHz frequency offset (separation distance of  10MHz+BWChannelHB/2 ), the MSD is 8.1dB,

- For LCRB=12RB, the rapid H2 PSD decay leads to ~0dB MSD for frequency offsets of 12.5, 17.5 and 22.5MHz. The MSD can only be verified for the minimum separation distance.

Table 9 H2 interference level for LCRB=12 and 25RB and corresponding n48 5MHz MSD vs frequency offsets

| LCRB = 12 (WF [2]) | Direct-hit | Near-miss H2 measurements |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 20MHz+BWChannelHB/2 | 15MHz+BWChannelHB/2 | 10MHz+BWChannelHB/2 | Minimum distance |
| Frequency offset (MHz) | 0 | -22.5 | -17.5 | -12.5 | -7.5 |
| H2 relative level to direct-hit for 5MHz CBW (dB) | 0 | -51.4 | -50.4 | -41.2 | -22.6 |
| n48 5MHz MSD (dB) | [27.1] | 0 | 0 | 0.1 | 5.8 |
| LCRB = 25 | Direct-hit | Near-miss H2 measurements |  |  |  |
|  |  | 20MHz+BWChannelHB/2 | 15MHz+BWChannelHB/2 | 10MHz+BWChannelHB/2 | Minimum distance |
| Frequency offset (MHz) | 0 | -22.5 | -17.5 | -12.5 | -7.5 |
| H2 relative level to direct-hit for 5MHz CBW (dB) | 0 | -38.4 | -31.4 | -19.7 | -8.9 |
| n48 5MHz MSD (dB) | [27.1] | 0.3 | 1.3 | 8.1 | 18.2 |

Based on these measurements, we make the following proposal to resolve the issues associated with the near-miss harmonic MSD test conditions.

Proposal 5: Adopt the following new test conditions for near-miss MSD test points.

- Near-miss MSD test points are specified only for UL2/DL1 harmonic interference and only when the band combination does not meet the direct-hit criteria.

- For near-miss MSD test points, the MSD is specified with;

  - UL CBW = 5MHz

  - UL LCRB = 25RB (fully allocated)

  - The Note 6 of Table 7.3A.4-1/ Note 6 of TS 38.101-3 Table 7.3B.2.3.1-1 is modified as:

NOTE 6: The near-miss requirements are only applicable when direct-hit requirements do not apply. These requirements should be verified for downlink channel bandwidths no larger than 10 MHz and with a carrier frequency at ±10+BWChannelHB/2 MHz offset from  in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively.

Proposal 6: To account for new near-miss test conditions (proposal 5), consider adopting the corrected band n48 5MHz PC3 near-miss MSD for CA_n2-n48 and for DC_2_n48, DC_48_n2 of Figure 11.

Figure 11: near-miss MSD correction for PC3 CA_n2-n48, DC_2-n48, DC_48_n2.

### CA_n7-n79, DC_7_n79

The 1.1dB near-miss UL2/DL1 MSD for band n79 10MHz CBW needs to be re-evaluated to account for new Note 6 / frequency separation distance requirements.

The band n79 MSD was copied from the DC_1_n77 MSD analysis [7]. Considering the uncertainty on the agreed 1.1dB near-miss MSD, we

- re-evaluate the band n79 MSD by calculating the H2 interference from the DC_1_n77 23.9dB direct-hit MSD

- evaluate from the measured LCRB=25 H2 PSD, the harmonic correction factor at 12.5MHz frequency offset for 10MHz CBW as captured in the Tables below.

| LCRB = 25 | Direct-hit | Near-miss |
| --- | --- | --- |
|  |  | 10MHz+BWChannelHB/2 |
| Frequency offset (MHz) | 0 | -12.5 |
| H2 relative level to direct-hit for 10MHz CBW (dB) | 0 | -15.1 |
| n79 10MHz MSD (dB) | [23.9] | 9.3 |

| Near-miss PC3 UL2/DL1 harmonic MSD for CA_n7_n79 | 1st MSD test point |  | near-miss test point |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Prim. | Div. | Prim. | Div. |  |
| Antenna referred in-channel harmonic interference levels: [primary/diversity] (dBm/MBW) | -70.9 | -70.9 | -84.2 | -84.2 |  |
| Ideal REFSENS: [n79 10MHz] (dBm/MBW) | -70.9 |  | -86.0 |  |
| Noise floor (dBm/MBW) | -91.8 | -91.8 | -91.8 | -91.8 |  |
| Total noise level at antenna connector (dBm/MBW) | -70.9 | -70.9 | -86.0 | -86.0 |  |
| Degradation: [n79 10MHz] (dB) | 21.0 | 21.0 | 6.8 | 6.8 |  |
| REFSENS MRC degradation: [n79 10MHz] (dB) | 23.9 |  | 9.3 |  |

Proposal 7: To account for new near-miss test conditions (proposal 5), consider adopting the corrected band n79 10MHz PC3 near-miss MSD for CA_n7-n79 of Figure 12. The same correction is proposed for DC_7_n79 in CR [3].

Figure 12: near-miss MSD correction for PC3 CA_n7-n79.

## PC3 Corrections for special band combinations

### CA_n2-n71 / CA_n25-n71 / DC_71_n2 / DC_2_n71 / DC_71_n25

Table 10 captures the agreed MSD test points for band 2/n2 25/n25 due UL3/DL1 interference from band 71/n71 where we have:

- 3 test points for PC3 NR-CA and associated footnotes 10, 11,

- 12 test points for EN-DC and associated footnotes 9, 10,

Table 10 PC3 UL3/DL1 MSD test points for CA_n2-n71 and CA_n25-n71

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n71 | n210 | 5 | 15 | 8 (RBstart=0) | 5 | 10 | NOTE 3 | UL3/DL1 direct-hit |
| n71 | n2510,11 | 5 | 15 | 8 (RBstart=0) | 5 | 10 | NOTE 3 | UL3/DL1 direct-hit |
| n71 | n2510,11 | 5 | 15 | 8 (RBstart=0) | 40 | 2.1 | NOTE 3 | UL3/DL1 direct-hit |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd / 3rd / 4th / 5th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 10: These requirements apply when the lower edge frequency of the 10 MHz, 15 MHz, or 20 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band n25 is located with its upper edge at 1995 MHz. NOTE 11: These requirements apply when the lower edge frequency of the uplink channel in Band n71 is located at or below 668 MHz and the downlink channel in Band n25 is located with its upper edge at 1990 MHz. |  |  |  |  |  |  |  |  |
| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| 71 | n29 | 5 | 15 | 25 (RBstart=0) | 5 | 4.6 | NOTE 3 | UL3/DL1 direct-hit |
| 71 | n210 | 5 | 15 | 8 (RBstart=0) | 5 | 1.7 | NOTE 3 | UL3/DL1 direct-hit |
| 71 | n29 | 5 | 15 | 25 (RBstart=0) | 20 | 0.6 | NOTE 3 | UL3/DL1 direct-hit |
| 71 | n210 | 5 | 15 | 8 (RBstart=0) | 20 | 0.6 | NOTE 3 | UL3/DL1 direct-hit |
| n71 | 29 | 5 | 15 | 25 (RBstart=0) | 5 | 4.6 | NOTE 3 | UL3/DL1 direct-hit |
| n71 | 210 | 5 | 15 | 8 (RBstart=0) | 5 | 1.7 | NOTE 3 | UL3/DL1 direct-hit |
| n71 | 29 | 5 | 15 | 25 (RBstart=0) | 20 | 0.6 | NOTE 3 | UL3/DL1 direct-hit |
| n71 | 210 | 5 | 15 | 8 (RBstart=0) | 20 | 0.6 | NOTE 3 | UL3/DL1 direct-hit |
| 71 | n259 | 5 | 15 | 25 (RBstart=0) | 5 | 4.6 | NOTE 3 | UL3/DL1 direct-hit |
| 71 | n2510 | 5 | 15 | 8 (RBstart=0) | 5 | 1.7 | NOTE 3 | UL3/DL1 direct-hit |
| 71 | n259 | 5 | 15 | 25 (RBstart=0) | 20 | 0.6 | NOTE 3 | UL3/DL1 direct-hit |
| 71 | n2510 | 5 | 15 | 8 (RBstart=0) | 20 | 0.6 | NOTE 3 | UL3/DL1 direct-hit |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd / 3rd / 4th / 5th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 9: These requirements apply when the lower edge frequency of the 5 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. NOTE 10: These requirements apply when the lower edge frequency of the 10 MHz, 15 MHz, or 20 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. |  |  |  |  |  |  |  |  |

As background information, the band 2 MSD due to band 71 H3 interference was first studied in detail for LTE CA_2-n71. For LTE, band 2 is affected by near-miss interference and the agreed 5MHz MSD is 4.6dB for a band 71 UL configuration of 5MHz CBW and Lcrb=25RB.

Observation 6: Several issues can be observed from Table 10 regarding the UL3/DL1 NR-CA/EN-DC MSD test points for aggregation of bands [2/n2, 25/n25] with bands [71/n71].

- Issue #1: For CA_n2-n71: the test point is specified as a direct-hit with Lcrb=8. Yet it is not clear if the UL/DL cell configurations fulfil the Note 1 “direct-hit” requirements,

- Issue #2: For CA_n2-n71, the band n2 MSD is specified as 10dB for 5MHz CBW. This is not aligned with the LTE agreement. Not that the B2 MSD for DC_2_n71 is aligned with the LTE agreement with 4.6dB for 5MHz CBW,

- Issue #3: For CA_n25-n71/ DC_25_n71, the band 25/n25 MSD is the same as that of band 2/n2. One would have expected that the band 25/n25 MSD would be greater than that of band 2/n2 since the band 25/n25 is affected by a greater H3 overlap than band 2/n2

- Issue #4: There are 4 test point per EN-DC band combination, and different Lcrb configurations: 25 or 8RBs. While direct-hit may occur for Lcrb=25, it is not clear if direct-hit conditions are met with Lcrb=8. It also seems that some test points could be removed.

- Issue #5: Associated footnotes 10/11 for NR-CA and 9/10 for EN-DC are confusing. Corrections are needed.

- Issue #6: Lcrb configurations are not consistent between NR-CA and their EN-DC counterparts.

We propose to resolve these issues one-by-one by:

- Step 1: Correct the UL RB allocation to LCRB=8 to align with WF[2] agreements, and analyse the H3/DL carrier Resource Element (RE) overlap to clarify if direct-hit or near-miss applies to each band combination,

- Step 2: Re-evaluate the MSD of the 1st test point based on legacy LTE agreements,

- Step 3: Align the MSD of the CA_n25-n71 2nd test point based on the interference level found at step 2,

- Step 4: Simplify the number of test points for EN-DC and cross-align with NR-CA

- Step 5: Clean-up associated footnotes.

Step 1:

- Figure 13-left shows the Band n2 and Band n25 uppermost 5MHz CBW DL carriers relative to the band n71 3rd harmonic of UL 5MHz CBW LCRB=8. It can be seen that:

  - Band n2 is not affected by direct-hit interference,

  - Band n25 is affected by direct-hit interference since its DL transmission bandwidth is partially overlapped by some RE of the band n71 H3 (dark blue boundaries – cf Note 1).

Figure 13: UL3/DL1 landscape for CA_n2-n71/CA_n25-n71 based on measured  band n71 H3 for 5MHz UL CBW and LCRB=8(RBstart=8) and LCRB=25(RBstart=0) .

- Figure 13-right shows the landscape according to corrected Note 6 conditions (proposal 5), i.e. with 12.5MHz frequency separation from DL center frequency to H3 center frequency of band n71 5MHz UL CBW LCRB=25.

Step 2:

Band n2 near-miss MSD re-evaluation

Figure 14 shows the Band 2 uppermost DL 5MHz channel is offset by 9MHz relative to the Band 71 H3 of UL 5MHz CBW LCRB=25 for the agreed B2 4.6dB MSD. To evaluate the B2 near-miss MSD we assume that for these conditions, the Band n2 MSD is also 4.6dB. Then:

- We calculate the H3 interference level that corresponds to 4.6dB n2 5MHz MSD by assuming equal H3 interference affecting both the primary and the diversity antenna ports. We find a total in channel level of -94.2dBm/MBW.

- We measure the H3 level in 4.515MHz measurement BW (UL LCRB=25) at -12.5 MHz offset (Figure 13-right) and at -9MHz offset (Figure 14) and calculate the H3 power ratio. Our measurements the H3 at -12.5MHz offset is 5.8dB lower than at -9MHz offset, i.e. the near-miss H3 level is ~ -100dBm.

- The band n2 near-miss MSD is estimated in Table 11 at 1.7dB.

Figure 14: UL3/DL1 landscape for the LTE CA_2-71 MSD test point based on measured band 71 for 5MHz UL CBW LCRB=25(RBstart=0).

Table 11 MSD re-evaluation for CA_n2-n71 near miss for corrected Note 6 (Proposal 5).

| Near-miss PC3 UL3/DL1 harmonic MSD for CA_n2_n71 | 1st test point |  | near-miss test point |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Prim. | Div. | Prim. | Div. |  |
| Antenna referred in-channel harmonic interference levels: [primary/diversity] (dBm/MBW) | -94.2 | -94.2 | -100.0 | -100.0 |  |
| Ideal REFSENS: [n2 5MHz] (dBm/MBW) | -98.0 |  | -98 |  |
| Noise floor (dBm/MBW) | -94.0 | -94.0 | -94.0 | -94.0 |  |
| Total noise level at antenna connector (dBm/MBW) | -94.2 | -94.2 | -100.0 | -100.0 |  |
| Degradation: [n2 5MHz] (dB) | 2.9 | 2.9 | 1.0 | 1.0 |  |
| REFSENS MRC degradation: [n2 5MHz] (dB) | 4.6 |  | 1.7 |  |

Band n2 ‘direct-hit’ MSD re-evaluation

Figure 13-left shows that even though band n25 fulfills the note 1 direct-hit conditions, its DL transmission BW does not entirely overlap the band n71 H3 of 5MHz UL CBW LCRB=8. To re-evaluate the band n25 MSD,

- We measure the H3 level in 4.515MHz measurement BW at -4 MHz offset (UL LCRB=8, Figure 13-left). The measurements indicate this H3 level is ~ 4.7dB higher than the level measured at -9MHz offset (Figure 14), i.e. the H3 level affecting the band n25 is estimated at -89.5dBm.

- The band n25 direct-hit MSD is estimated in Table 12 at 6.9dB.

Table 12 MSD re-evaluation for CA_n25-n71 direct-hit 1st test point.

| Near-miss PC3 UL3/DL1 harmonic MSD for CA_n25_n71 | direct-miss test point |  |  |
| --- | --- | --- | --- |
|  | Prim. | Div. |  |
| Antenna referred in-channel harmonic interference levels: [primary/diversity] (dBm/MBW) | -89.5 | -89.5 |  |
| Ideal REFSENS: [n25 5MHz] (dBm/MBW) | -96.5 |  |
| Noise floor (dBm/MBW) | -92.5 | -92.5 |  |
| Total noise level at antenna connector (dBm/MBW) | -89.5 | -89.5 |  |
| Degradation: [n25 5MHz] (dB) | 4.7 | 4.7 |  |
| REFSENS MRC degradation: [n25 5MHz] (dB) | 6.9 |  |

Step 3: 2nd MSD test point alignment.

Table 13 estimates the band n25 40MHz direct-hit MSD at ~0.3dB by assuming equal interference level affecting the primary and the diversity antenna ports. It is proposed to remove this test point.

Table 13 MSD re-evaluation for CA_n25-n71 direct-hit 2nd test point.

| Near-miss PC3 UL3/DL1 harmonic MSD for CA_n25_n71 | direct-miss test point |  |  |
| --- | --- | --- | --- |
|  | Prim. | Div. |  |
| Antenna referred in-channel harmonic interference levels: [primary/diversity] (dBm/MBW) | -89.5 | -89.5 |  |
| Ideal REFSENS: [n25 40MHz] (dBm/MBW) | -79.5 |  |
| Noise floor (dBm/MBW) | -75.5 | -75.5 |  |
| Total noise level at antenna connector (dBm/MBW) | -89.5 | -89.5 |  |
| Degradation: [n25 40MHz] (dB) | 0.2 | 0.2 |  |
| REFSENS MRC degradation: [n25 40MHz] (dB) | 0.3 |  |

Step 4: 2nd MSD test point simplification for EN-DC combinations.

Based on the MSD analyzes for NR-CA, we propose the simplifications of Table 14 corrections. For the MSD test point of DC_71_n25, the MSD at 20MHz CBW is estimated at 2.9dB based on the H3 interference levels of Table 13.

Table 14 MSD test point simplification proposal for EN-DC combinations.

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| 71 | n29 | 5 | 15 | 25 (RBstart=0) | 5 | 4.6 | NOTE 3 | UL3/DL1 direct-hit |
| 71 | n210 | 5 | 15 | 25 (RBstart=0) | 5 | 1.7 | NOTE 6 | UL3/DL1 near-miss |
| 71 | n29 | 5 | 15 | 25 (RBstart=0) | 20 | 0.6 | NOTE 3 | UL3/DL1 direct-hit |
| 71 | n210 | 5 | 15 | 8 (RBstart=0) | 20 | 0.6 | NOTE 3 | UL3/DL1 direct-hit |
| n71 | 29 | 5 | 15 | 25 (RBstart=0) | 5 | 4.6 | NOTE 3 | UL3/DL1 direct-hit |
| n71 | 210 | 5 | 15 | 8 (RBstart=0) | 5 | 1.7 | NOTE 6 | UL3/DL1 near-miss |
| n71 | 29 | 5 | 15 | 25 (RBstart=0) | 20 | 0.6 | NOTE 3 | UL3/DL1 direct-hit |
| n71 | 210 | 5 | 15 | 8 (RBstart=0) | 20 | 0.6 | NOTE 3 | UL3/DL1 direct-hit |
| 71 | n259 | 5 | 15 | 25 (RBstart=0) | 5 | 4.6 | NOTE 3 | UL3/DL1 direct-hit |
| 71 | n2510 | 5 | 15 | 8 (RBstart=0) | 5 | 6.9 | NOTE 3 | UL3/DL1 direct-hit |
| 71 | n259 | 5 | 15 | 25 (RBstart=0) | 20 | 0.6 | NOTE 3 | UL3/DL1 direct-hit |
| 71 | n2510 | 5 | 15 | 8 (RBstart=0) | 20 | 2.9 | NOTE 3 | UL3/DL1 direct-hit |

Step 5: Footnote clean-up, cross-alignment and text proposal for CR[3,12].

Proposal 8: Consider adopting the following MSD test point corrections for PC3 CA_n2_n71 and CA_n25-n71 and their EN-DC counter parts.

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n71 | n210 | 5 | 15 | 8 (RBstart=0) | 5 | 1.7 | NOTE 6 | UL3/DL1 near-miss |
| n71 | n2510,11 | 5 | 15 | 8 (RBstart=0) | 5 | 6.9 | NOTE 3 | UL3/DL1 direct-hit |
| n71 | n2510,11 | 5 | 15 | 8 (RBstart=0) | 40 | 2.1 | NOTE 3 | UL3/DL1 direct-hit |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd / 3rd / 4th / 5th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 10: These requirements apply when the upper edge frequency of the 5 MHz uplink channel in Band n71 is located at or below 668 MHz and the downlink channel in Band n2 is located with its upper edge at 1990 MHz. NOTE 11: These requirements apply when the upper edge frequency of the 5 MHz uplink channel in Band n71 is located at or below 668 MHz and the downlink channel in Band n25 is located with its upper edge at 1995 MHz. |  |  |  |  |  |  |  |  |
| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| 71 | n29 | 5 | 15 | 8 25 (RBstart=0) | 5 | 4.6 | NOTE 3 | UL3/DL1 direct-hit |
| 71 | n29 | 5 | 15 | 25 8 (RBstart=0) | 5 | 1.7 | NOTE 6 | UL3/DL1 near-miss |
| 71 | n29 | 5 | 15 | 8 25 (RBstart=0) | 20 | 0.6 | NOTE 3 | UL3/DL1 direct-hit |
| 71 | n210 | 5 | 15 | 8 (RBstart=0) | 20 | 0.6 | NOTE 3 | UL3/DL1 direct-hit |
| n71 | 29 | 5 | 15 | 8 25 (RBstart=0) | 5 | 4.6 | NOTE 3 | UL3/DL1 direct-hit |
| n71 | 29 | 5 | 15 | 25 8 (RBstart=0) | 5 | 1.7 | NOTE 6 | UL3/DL1 near-miss |
| n71 | 29 | 5 | 15 | 8 25 (RBstart=0) | 20 | 0.6 | NOTE 3 | UL3/DL1 direct-hit |
| n71 | 210 | 5 | 15 | 8 (RBstart=0) | 20 | 0.6 | NOTE 3 | UL3/DL1 direct-hit |
| 71 | n259 | 5 | 15 | 8 25 (RBstart=0) | 5 | 4.6 | NOTE 3 | UL3/DL1 direct-hit |
| 71 | n2510 | 5 | 15 | 8 (RBstart=0) | 5 | 6.9 | NOTE 3 | UL3/DL1 direct-hit |
| 71 | n259 | 5 | 15 | 8 25 (RBstart=0) | 20 | 0.6 | NOTE 3 | UL3/DL1 direct-hit |
| 71 | n2510 | 5 | 15 | 8 (RBstart=0) | 20 | 2.9 | NOTE 3 | UL3/DL1 direct-hit |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd / 3rd / 4th / 5th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 9: These requirements apply when the upper edge frequency of the 5 MHz uplink channel in Band 71/n71 is located at or below 668 MHz and the downlink channel in Band 2/n2 is located with its upper edge at 1990 MHz. NOTE 10: These requirements apply when the upper edge frequency of the 5 MHz uplink channel in Band 71 is located at 668 MHz and the downlink channel in Band n25 is located with its upper edge at 1995 MHz. |  |  |  |  |  |  |  |  |

### CA_n28-n50 / CA_n28-n74 / CA_n28-n75 / CA_n28-n94 / DC_28_n50 / DC_28_n51 / DC_28_n75

This discussion focuses on the NR-CA combinations only. We do not discuss the changes related to their EN-DC counterparts since the conclusions & proposals brought forward in this clause are equally applicable.

Depending on the DL affected bands, there are four different MSD levels resulting from the 2nd harmonic interference of the band n28 transmissions:

- 23 dB and 23.1 dB for band n50 and band n74 5MHz MSD in CA_n28-n50, CA_n28-n74

- 28.1 dB for band n75 and band n94 5MHz MSD in CA_n28-n75/DC_28_n75 and CA_n28-n94

- 27.8 dB for band n51 5MHz MSD in DC_28_n51

- 24.6 dB for band n50 10MHz MSD in DC_28_n50.

It should be also noted that for band n109 which may be regarded with similar assumptions than CA_n28-n75, the direct-hit MSD (REFSENS exception) for 5MHz SCS15kHz is also 28.1dB.

By researching the text proposals of each band combinations, each MSD difference is justified by use of different harmonic interference levels, different PCB isolation assumptions, and different diplexer rejections. Two MSD analyses are reproduced from [8,9] in the Annex from the original CA_n28-n75 text proposal (TP) that led to 28.1dB MSD and TP for CA_n28-n74 that justified the 23.1dB 5MHz equivalent MSD .

We believe that with the new rules on specifying a common UL configuration for these band combinations, the level of band 28/n28 H2 interference affecting any of these 1500MHz DL bands is the same and their 5MHz REFSENS levels are also nearly identical: either -100dBm for SDL bands and band n94, or -99.5dBm for band n74. It is therefore expected that the same MSD level should apply to the 1st test point of these DL bands for 5MHz CBW.

We consider that the direct hit 28.1dB MSD agreements for band n109 5MHz CBW is the most up to date RAN4 agreement. This MSD should be used as the common MSD level for bands n50, n51, n74, n75, n94 1st direct-hit test point and make the MSD correction of Table 15 which are based on the principles than those presented in [5,6].

Proposal 9: Consider adopting the PC3 UL2/DL1 MSD of Table 15 for CA_n28-n50, CA_n28-n74, CA_n28-n75, CA_n28-n94. Similar changes are ported for their EN-DC counterparts.

Table 15 PC3 MSD corrections for CA_n28-n50, CA_n28-n74, CA_n28-n75, CA_n28-n94.

| UL band | DL band | Test point # | UL BW (MHz) | SCS of UL band (kHz) | Rel. 18.6.0 UL RB Allocation Lcrb | DL BW (MHz) | Rel 18.6.0 MSD (dB) | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n28 | n50 | 1 | 5 | 15 | 12 (RBstart=0) | 5 | 28.1 23 | UL2/DL1 direct-hit |
| n28 | n50 | 2 | 5 | 15 | 12 (RBstart=0) | 80 | 15.8 10.8 |  |
| n28 | n74 | 1 | 5 | 15 | 12 (RBstart=0) | 5 | 28.1 23.1 |  |
| n28 | n74 | 2 | 5 | 15 | 12 (RBstart=0) | 20 | 18.0 16.8 |  |
| n28 | n75 | 1 | 5 | 15 | 12 (RBstart=0) | 5 | 28.1 |  |
| n28 | n75 | 2 | 5 | 15 | 12 (RBstart=0) | 50 | 17.9 18.7 |  |
| n28 | n94 | 1 | 5 | 15 | 12 (RBstart=0) | 5 | 28.1 |  |
| n28 | n94 | 2 | 5 | 15 | 12 (RBstart=0) | 50 | 17.9 18.7 |  |

## NR-CA PC2 Corrections

Table merge: In Rel 18.6.0, two separate tables are captured, one table capturing the 1Tx MSD test points, the other capturing the 2Tx MSD test points. At this meeting, we propose to merge the tables into a single table according to the WF [10]

PC2 MSD level alignment: We propose to align the PC2 1Tx and PC2 2Tx using:

- the corrected PC3 MSD levels presented in this document

- the PC2 1Tx and PC2 Tx simplification look-up table of [4]. For 2Tx, there was consensus at RAN4 # 111 to adopt the 9dB imbalance look-up table from [4]. Corrections in this document are based on this informal decision. MSD corrections less 0.3dB are not implemented.

Special cases:

- For CA_n25-n71, the 2nd MSD test point is deleted from the PC3 table due to MSD < 0.5dB. Based on [4] we propose a

  - 1Tx PC2 MSD = 0.7dB for band n25 40MHz CBW.

  - 2Tx PC2 MSD = PC3 MSD +6 = 12.9dB for band n25 5MHz CBW [4 – 9dB imbalance]

  - 2Tx PC2 MSD = 1.1dB for band n25 40MHz CBW.

- CA_n8-n79:

  - 1Tx PC2 MSD = PC3 MSD + 2 = 6.5dB for n79 100MHz CBW

  - Change from 16.7dB to 2Tx PC2 MSD = PC3 MSD + 7 = 19dB for n79 10MHz CBW.

  - No change for 2Tx PC2 MSD for n79 10MHz CBW

- CA_n8-n41:

  - 2Tx PC2 MSD change from 19.3dB to 20dB for 10MHz CBW

  - 2Tx PC2 MSD change from 8.3dB to 9.5dB for 100MHz CBW

- CA_n41-n71:

  - Change from 3.4dB to 2Tx PC2 MSD = PC3 MSD +3 = 6.1dB for band n41 100MHz CBW.

- CA_n3-n78:

  - the 2Tx PC2 MSD test point for n78 100MHz has gone missing in the implementation of Rel 18.6.0. We propose 2Tx PC2 MSD = PC3 MSD + 7dB = 20.8dB [4].

Table footnote alignment and clean-up:

Table 7.3A.4-2a is reproduced in from Rel 18.6.0. In the previous Release this table contained only Note 1 and Note 2. But in Rel 18.6.0, footnotes 3,4,5,6,7,8,9 are introduced, and all are voided. Some of these Notes should have been used to capture the UL3/DL1, UL4/DL1 etc… requirements.  In addition, and as highlighted in yellow:

- footnotes “Note 3” “Note 4” are captured twice in this table,

- the contents of “Note 1” are missing. This is particularly important now that the PC3 Note 1 defines the correct RBstart requirements.

Table 16 PC2 UL Harmonic MSD test points for 1Tx (Table 7.3A.4-2a).

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n3 | n78 | 5 | 15 | 25 (RBstart=0) | 10 | 27.1 | NOTE 2 | UL2/DL1 direct-hit |
| n3 | n78 | 10 | 15 | 50 (RBstart=0) | 100 | 16.6 | NOTE 2 | UL2/DL1 direct-hit |
| n25 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 26.9 | NOTE 2 | UL2/DL1 direct-hit |
| n25 | n77 | 10 | 15 | 50 (RBstart=0) | 100 | 16.8 | NOTE 2 | UL2/DL1 direct-hit |
| n25 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 2.0 | NOTE 6 | UL2/DL1 near-miss |
| n71 | n2510,11 | 5 | 15 | 8 (RBstart=0) | 5 | 12.8 | NOTE 3 | UL3/DL1 direct-hit |
| n71 | n2510,11 | 5 | 15 | 8 (RBstart=0) | 40 | 3.5 | NOTE 3 | UL3/DL1 direct-hit |
| n71 | n41 | 5 | 15 | 16 (RBstart=0) | 10 | 13.6 | NOTE 4 | UL4/DL1 direct-hit |
| n71 | n41 | 5 | 15 | 25 (RBstart=0) | 100 | 2.5 | NOTE 4 | UL4/DL1 direct-hit |
| n66 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 26.9 | NOTE 2 | UL2/DL1 direct-hit |
| n66 | n77 | 20 | 15 | 100 (RBstart=0) | 100 | 16.7 | NOTE 2 | UL2/DL1 direct-hit |
| n66 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 2.0 | NOTE 6 | UL2/DL1 near-miss |
| n71 | n77 | 5 | 15 | 10 (RBstart=0) | 10 | 13.2 | NOTE 5 | UL5/DL1 direct-hit |
| n8 | n79 | 5 | 15 | 16 (RBstart=0) | 10 | 14.9 | NOTE 2 | UL5/DL1 direct-hit |
| n8 | n79 | 5 | 15 | 25 (RBstart=0) | 100 | 6.2 | NOTE 2 | UL5/DL1 direct-hit |
| n8 | n41 | 5 | 15 | 16 (RBstart=0) | 10 | 15.9 | NOTE 2 | UL3/DL1 direct-hit |
| n8 | n41 | 5 | 15 | 25 (RBstart=0) | 100 | 6.2 | NOTE 2 | UL3/DL1 direct-hit |
| NOTE 1: Void NOTE 2: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 3: Void NOTE 4: Void NOTE 5: Void NOTE 6: Void NOTE 7: Void NOTE 8: Void NOTE 9: Void NOTE 10 NOTE 3: Void NOTE 4: Void: These requirements apply when the lower edge frequency of the 10 MHz, 15 MHz, or 20 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band n25 is located with its upper edge at 1995 MHz. NOTE 11: These requirements apply when the lower edge frequency of the uplink channel in Band n71 is located at or below 668 MHz and the downlink channel in Band n25 is located with its upper edge at 1990 MHz. |  |  |  |  |  |  |  |  |

We propose to:

- If allowed by RAN4, restore the contents of Note 1,3,4,5,6 from the PC3 table Table 7.3A.4-1. If not allowed, additional footnotes will have to be added to restore the contents of these footnotes.

- Assign only one footnote to CA_n25-n71 and align the wording with the Proposal 8 note 11.

- Assign “Applicable to UE supporting PC2 with single Tx” to Note 8 – see CR [11]

- Assign “Applicable to UE supporting PC2 with dual Tx” to Note 9 – see CR [11]

- Assign Note 10 to CA_n25-n71.

- Remove near-miss MSD test points when direct-hit interference occurs.

Proposal 10: Consider adopting the PC2 MSD requirements corrections captured in Table 17 . If replacing “Void” can not be agreed, we may consider revising this CR during the meeting to insert additional footnotes.

Table 17 Corrections for NR-CA PC2 MSD requirements and merge of Table 7.3A.4-2a and Table 7.3A.4-2b.

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD7 | MSD8 | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) | (dB) |  |  |
| n3 | n78 | 5 | 15 | 12 (RBstart=0) | 10 | 27.1 | 32.3 | NOTE 2 | UL2/DL1 direct-hit |
| n3 | n78 | 5 | 15 | 12 (RBstart=0) | 100 | 16.6 | 20.8 | NOTE 2 | UL2/DL1 direct-hit |
| n25 | n77 | 5 | 15 | 12 (RBstart=0) | 10 | 26.9 | 31.9 | NOTE 2 | UL2/DL1 direct-hit |
| n25 | n77 | 10 | 15 | 12 (RBstart=0) | 100 | 16.8 | 20.8 | NOTE 2 | UL2/DL1 direct-hit |
| n25 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 2.0 |  | NOTE 6 | UL2/DL1 near-miss |
| n71 | n259 | 5 | 15 | 8 (RBstart=0) | 5 | 9.4 | 12.9 | NOTE 3 | UL3/DL1 direct-hit |
| n71 | n259 | 5 | 15 | 8 (RBstart=0) | 40 | 0.7 | 1.1 | NOTE 3 | UL3/DL1 direct-hit |
| n71 | n41 | 5 | 15 | 6 (RBstart=0) | 10 | 13.6 | 17.8 | NOTE 4 | UL4/DL1 direct-hit |
| n71 | n41 | 5 | 15 | 6 (RBstart=0) | 100 | 5.1 | 6.1 | NOTE 4 | UL4/DL1 direct-hit |
| n66 | n77 | 5 | 15 | 12 (RBstart=0) | 10 | 26.9 | 31.9 | NOTE 2 | UL2/DL1 direct-hit |
| n66 | n77 | 20 | 15 | 12 (RBstart=0) | 100 | 16.7 | 20.8 | NOTE 2 | UL2/DL1 direct-hit |
| n66 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 2.0 |  | NOTE 6 | UL2/DL1 near-miss |
| n71 | n77 | 5 | 15 | 5 (RBstart=0) | 10 | 13.2 | 17.4 | NOTE 5 | UL5/DL1 direct-hit |
| n8 | n79 | 5 | 15 | 5 (RBstart=0) | 10 | 14.9 | 19 | NOTE 2 | UL5/DL1 direct-hit |
| n8 | n79 | 5 | 15 | 5 (RBstart=0) | 100 | 6.5 | 8.4 | NOTE 2 | UL5/DL1 direct-hit |
| n8 | n41 | 5 | 15 | 8 (RBstart=0) | 10 | 15.9 | 20 | NOTE 2 | UL3/DL1 direct-hit |
| n8 | n41 | 5 | 15 | 8 (RBstart=0) | 100 | 6.5 | 9.5 | NOTE 2 | UL3/DL1 direct-hit |
| NOTE 1: The direct-hit requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd / 3rd / 4th / 5th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. These requirements shall be verified with UL RB Allocation RBstart = floor((NRB-LCRB)/2), where floor(x) is the greatest integer less than or equal to x, and NRB is the maximum number of RBs for a given channel bandwidth and sub-carrier spacing defined in Table 5.3.2-1. NOTE 2: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 3: The requirements should be verified for UL NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with the carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the low band. NOTE 4: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 5: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 6: The near-miss requirements are only applicable when direct-hit requirements do not apply. These requirements should be verified for downlink channel bandwidths no larger than 10 MHz and with a carrier frequency at ±10+BWChannelHB/2 MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 7: Applicable to UEs supporting PC2 with single Tx. NOTE 8: Applicable to UEs supporting PC2 with dual Tx. NOTE 9: These requirements apply when the upper edge frequency of the 5 MHz uplink channel in Band n71 is located at or below 668 MHz and the downlink channel in Band n25 is located with its upper edge at 1995 MHz. NOTE 10 NOTE 3: Void NOTE 4: Void: These requirements apply when the lower edge frequency of the 10 MHz, 15 MHz, or 20 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band n25 is located with its upper edge at 1995 MHz. NOTE 11: These requirements apply when the lower edge frequency of the uplink channel in Band n71 is located at or below 668 MHz and the downlink channel in Band n25 is located with its upper edge at 1990 MHz. |  |  |  |  |  |  |  |  |  |

The PC2 1Tx MSD corrections are summarized in Figure 15 vs the proposed MSD simplifications of [4] (blue line). As a reminder, parameter PC2_1Tx is defined in [4] as  PC2_1Tx = PC2_1Tx MSD – PC3 MSD, i.e. the increase of the PC2 1Tx MSD from the agreed PC3 level.

Further alignment may be achieved if MSD corrections with 0.1dB granularity had been implemented.

Figure 15: Proposed PC2 1Tx corrections (dots) [11] vs MSD simplification (blue line) [4]

# Conclusion

In this contribution, measurements are used to complete the uplink harmonic MSD table clean-up according to the latest RAN4 agreements of WF [1,2]. Only key changes are discussed in this paper, the full set of corrections can be found in CR [3] for PC3 NR-CA band combinations and in two additional companion CRs to correct the PC2 NR-CA test points and the EN-DC test points. We make the following 9 proposals.

Proposal 1: To resolve the issues with RBstart=0 [3], an alternative solution is proposed which removes RBstart=0 from all UL harmonic MSD tables (NR-CA and EN-DC). To ensure UL RBs are centered in the middle of the UL carrier, an equation that defines RBstart is added to the NOTE 1 of TS 38.101-1 Table 7.3A.4-1. It is also proposed to clarify that Note 1 is specific to the definition of direct-hit harmonic MSD test points. See Figure 1 – CR [3].

Figure 1: RBstart equation to ensure the UL RB allocation is centered in the UL channel [3] / direct-hit clarification.

Proposal 2: Consider adopting the corrected the PC3 band n7 MSD for CA_n7-n105 of Figure 4.

Figure 4: CA_n7-n105 PC3 MSD correction due to adoption of LCRB=8.

Proposal 3: To account respectively for 1.5dB and 1dB increase of the H5 level, consider adopting the corrected band n78 10MHz PC3 MSD for CA_n78-n105 and band 42 5MHz MSD for DC_42_n28 of Figure 7.

Figure 7: MSD correction due to the adoption of LCRB=5 for CA_n78-n105 PC3 MSD and DC_42_n28.

Proposal 4: Remove the direct-hit MSD test points from TS 38.101-1 for CA_n2-n48 and from TS 38.101-3 for DC_2_n48, DC_48_n2.

Proposal 5: Adopt the following new test conditions for near-miss MSD test points.

- Near-miss MSD test points are specified only for UL2/DL1 harmonic interference and only when the band combination does not meet the direct-hit criteria.

- For near-miss MSD test points, the MSD is specified with;

  - UL CBW = 5MHz

  - UL LCRB = 25RB (fully allocated)

  - The Note 6 of Table 7.3A.4-1/ Note 6 of TS 38.101-3 Table 7.3B.2.3.1-1 is modified as:

NOTE 6: The near-miss requirements are only applicable when direct-hit requirements do not apply. These requirements should be verified for downlink channel bandwidths no larger than 10 MHz and with a carrier frequency at ±10+BWChannelHB/2 MHz offset from  in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively.

Proposal 6: To account for new near-miss test conditions (proposal 5), consider adopting the corrected band n48 5MHz PC3 near-miss MSD for CA_n2-n48 and for DC_2_n48, DC_48_n2 of Figure 11.

Figure 11: near-miss MSD correction for PC3 CA_n2-n48, DC_2-n48, DC_48_n2.

Proposal 7: To account for new near-miss test conditions (proposal 5), consider adopting the corrected band n79 10MHz PC3 near-miss MSD for CA_n7-n79 of Figure 12. The same correction is proposed for DC_7_n79 in CR [3].

Figure 12: near-miss MSD correction for PC3 CA_n7-n79.

Proposal 8: Consider adopting the following MSD test point corrections for PC3 CA_n2_n71 and CA_n25-n71 and their EN-DC counter parts.

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n71 | n210 | 5 | 15 | 8 (RBstart=0) | 5 | 1.7 | NOTE 6 | UL3/DL1 near-miss |
| n71 | n2510,11 | 5 | 15 | 8 (RBstart=0) | 5 | 6.9 | NOTE 3 | UL3/DL1 direct-hit |
| n71 | n2510,11 | 5 | 15 | 8 (RBstart=0) | 40 | 2.1 | NOTE 3 | UL3/DL1 direct-hit |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd / 3rd / 4th / 5th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 10: These requirements apply when the upper edge frequency of the 5 MHz uplink channel in Band n71 is located at or below 668 MHz and the downlink channel in Band n2 is located with its upper edge at 1990 MHz. NOTE 11: These requirements apply when the upper edge frequency of the 5 MHz uplink channel in Band n71 is located at or below 668 MHz and the downlink channel in Band n25 is located with its upper edge at 1995 MHz. |  |  |  |  |  |  |  |  |
| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| 71 | n29 | 5 | 15 | 8 25 (RBstart=0) | 5 | 4.6 | NOTE 3 | UL3/DL1 direct-hit |
| 71 | n29 | 5 | 15 | 8 (RBstart=0) | 5 | 1.7 | NOTE 6 | UL3/DL1 near-miss |
| 71 | n29 | 5 | 15 | 8 25 (RBstart=0) | 20 | 0.6 | NOTE 3 | UL3/DL1 direct-hit |
| 71 | n210 | 5 | 15 | 8 (RBstart=0) | 20 | 0.6 | NOTE 3 | UL3/DL1 direct-hit |
| n71 | 29 | 5 | 15 | 8 25 (RBstart=0) | 5 | 4.6 | NOTE 3 | UL3/DL1 direct-hit |
| n71 | 29 | 5 | 15 | 8 (RBstart=0) | 5 | 1.7 | NOTE 6 | UL3/DL1 near-miss |
| n71 | 29 | 5 | 15 | 8 25 (RBstart=0) | 20 | 0.6 | NOTE 3 | UL3/DL1 direct-hit |
| n71 | 210 | 5 | 15 | 8 (RBstart=0) | 20 | 0.6 | NOTE 3 | UL3/DL1 direct-hit |
| 71 | n259 | 5 | 15 | 8 25 (RBstart=0) | 5 | 4.6 | NOTE 3 | UL3/DL1 direct-hit |
| 71 | n2510 | 5 | 15 | 8 (RBstart=0) | 5 | 6.9 | NOTE 3 | UL3/DL1 direct-hit |
| 71 | n259 | 5 | 15 | 8 25 (RBstart=0) | 20 | 0.6 | NOTE 3 | UL3/DL1 direct-hit |
| 71 | n2510 | 5 | 15 | 8 (RBstart=0) | 20 | 2.9 | NOTE 3 | UL3/DL1 direct-hit |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd / 3rd / 4th / 5th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 9: These requirements apply when the upper edge frequency of the 5 MHz uplink channel in Band 71/n71 is located at or below 668 MHz and the downlink channel in Band 2/n2 is located with its upper edge at 1990 MHz. NOTE 10: These requirements apply when the upper edge frequency of the 5 MHz uplink channel in Band 71 is located at 668 MHz and the downlink channel in Band n25 is located with its upper edge at 1995 MHz. |  |  |  |  |  |  |  |  |

Proposal 9: Consider adopting the PC3 UL2/DL1 MSD of Table 15 for CA_n28-n50, CA_n28-n74, CA_n28-n75, CA_n28-n94. Similar changes are ported for their EN-DC counterparts.

Table 15 PC3 MSD corrections for CA_n28-n50, CA_n28-n74, CA_n28-n75, CA_n28-n94.

| UL band | DL band | Test point # | UL BW (MHz) | SCS of UL band (kHz) | Rel. 18.6.0 UL RB Allocation Lcrb | DL BW (MHz) | Rel 18.6.0 MSD (dB) | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n28 | n50 | 1 | 5 | 15 | 12 (RBstart=0) | 5 | 28.1 23 | UL2/DL1 direct-hit |
| n28 | n50 | 2 | 5 | 15 | 12 (RBstart=0) | 80 | 15.8 10.8 |  |
| n28 | n74 | 1 | 5 | 15 | 12 (RBstart=0) | 5 | 28.1 23.1 |  |
| n28 | n74 | 2 | 5 | 15 | 12 (RBstart=0) | 20 | 18.0 16.8 |  |
| n28 | n75 | 1 | 5 | 15 | 12 (RBstart=0) | 5 | 28.1 |  |
| n28 | n75 | 2 | 5 | 15 | 12 (RBstart=0) | 50 | 17.9 18.7 |  |
| n28 | n94 | 1 | 5 | 15 | 12 (RBstart=0) | 5 | 28.1 |  |
| n28 | n94 | 2 | 5 | 15 | 12 (RBstart=0) | 50 | 17.9 18.7 |  |

Proposal 10: Consider adopting the PC2 MSD requirements corrections captured in Table 17 . If replacing “Void” can not be agreed, we may consider revising this CR during the meeting to insert additional footnotes.

Table 17 Corrections for NR-CA PC2 MSD requirements and merge of Table 7.3A.4-2a and Table 7.3A.4-2b.

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD7 | MSD8 | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) | (dB) |  |  |
| n3 | n78 | 5 | 15 | 12 (RBstart=0) | 10 | 27.1 | 32.3 | NOTE 2 | UL2/DL1 direct-hit |
| n3 | n78 | 5 | 15 | 12 (RBstart=0) | 100 | 16.6 | 20.8 | NOTE 2 | UL2/DL1 direct-hit |
| n25 | n77 | 5 | 15 | 12 (RBstart=0) | 10 | 26.9 | 31.9 | NOTE 2 | UL2/DL1 direct-hit |
| n25 | n77 | 10 | 15 | 12 (RBstart=0) | 100 | 16.8 | 20.8 | NOTE 2 | UL2/DL1 direct-hit |
| n25 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 2.0 |  | NOTE 6 | UL2/DL1 near-miss |
| n71 | n259 | 5 | 15 | 8 (RBstart=0) | 5 | 9.4 | 12.9 | NOTE 3 | UL3/DL1 direct-hit |
| n71 | n259 | 5 | 15 | 8 (RBstart=0) | 40 | 0.7 | 1.1 | NOTE 3 | UL3/DL1 direct-hit |
| n71 | n41 | 5 | 15 | 6 (RBstart=0) | 10 | 13.6 | 17.8 | NOTE 4 | UL4/DL1 direct-hit |
| n71 | n41 | 5 | 15 | 6 (RBstart=0) | 100 | 5.1 | 6.1 | NOTE 4 | UL4/DL1 direct-hit |
| n66 | n77 | 5 | 15 | 12 (RBstart=0) | 10 | 26.9 | 31.9 | NOTE 2 | UL2/DL1 direct-hit |
| n66 | n77 | 20 | 15 | 12 (RBstart=0) | 100 | 16.7 | 20.8 | NOTE 2 | UL2/DL1 direct-hit |
| n66 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 2.0 |  | NOTE 6 | UL2/DL1 near-miss |
| n71 | n77 | 5 | 15 | 5 (RBstart=0) | 10 | 13.2 | 17.4 | NOTE 5 | UL5/DL1 direct-hit |
| n8 | n79 | 5 | 15 | 5 (RBstart=0) | 10 | 14.9 | 19 | NOTE 2 | UL5/DL1 direct-hit |
| n8 | n79 | 5 | 15 | 5 (RBstart=0) | 100 | 6.5 | 8.4 | NOTE 2 | UL5/DL1 direct-hit |
| n8 | n41 | 5 | 15 | 8 (RBstart=0) | 10 | 15.9 | 20 | NOTE 2 | UL3/DL1 direct-hit |
| n8 | n41 | 5 | 15 | 8 (RBstart=0) | 100 | 6.5 | 9.5 | NOTE 2 | UL3/DL1 direct-hit |
| NOTE 1: The direct-hit requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd / 3rd / 4th / 5th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. These requirements shall be verified with UL RB Allocation RBstart = floor((NRB-LCRB)/2), where floor(x) is the greatest integer less than or equal to x, and NRB is the maximum number of RBs for a given channel bandwidth and sub-carrier spacing defined in Table 5.3.2-1. NOTE 2: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 3: The requirements should be verified for UL NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with the carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the low band. NOTE 4: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 5: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 6: The near-miss requirements are only applicable when direct-hit requirements do not apply. These requirements should be verified for downlink channel bandwidths no larger than 10 MHz and with a carrier frequency at ±10+BWChannelHB/2 MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 7: Applicable to UEs supporting PC2 with single Tx. NOTE 8: Applicable to UEs supporting PC2 with dual Tx. NOTE 9: These requirements apply when the upper edge frequency of the 5 MHz uplink channel in Band n71 is located at or below 668 MHz and the downlink channel in Band n25 is located with its upper edge at 1995 MHz. NOTE 10 NOTE 3: Void NOTE 4: Void: These requirements apply when the lower edge frequency of the 10 MHz, 15 MHz, or 20 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band n25 is located with its upper edge at 1995 MHz. NOTE 11: These requirements apply when the lower edge frequency of the uplink channel in Band n71 is located at or below 668 MHz and the downlink channel in Band n25 is located with its upper edge at 1990 MHz. |  |  |  |  |  |  |  |  |  |

# Reference

[1]	R4-2406701 WF on Corrections to UL harmonic MSD, 3GPP TSG-RAN WG4 Meeting #110-bis, Changsha, Skyworks Solutions, Inc.

[2]	R4-2410651 WF on Harmonic MSD clean-up, 3GPP TSG-RAN WG4 Meeting #111, Fukuoka, Skyworks Solutions, Inc., AT&T, Qualcomm.

[3]	R4-241xxxx CR to TS 38.101-1 Rel-18 NR CA Uplink Harmonic clean-up PC3, 3GPP TSG-RAN WG4 Meeting #112, Maastricht, Skyworks Solutions Inc.

[4]	R4-2407156 Guidelines for FDD PC2 Harmonic MSD Analysis, 3GPP TSG-RAN WG4 Meeting #111, Fukuoka, Skyworks Solutions, Inc.

[5] 	R4-2405961 Corrections to UL harmonic MSD, 3GPP TSG-RAN WG4 Meeting #110-bis, Changsha, Skyworks Solutions, Inc.

[6] 	R4-2405453 Harmonic Mixing clean-up, 3GPP TSG-RAN WG4 Meeting #110-bis, Changsha, Qualcomm France.

[7]	R4-2119823 TP to TR 37.717-11-11: DC_7A_n79A and DC_7A_n79C, 3GPP TSG-RAN WG4 Meeting # 101-e, Electronic Meeting, Huawei, HiSilicon.

[8]	R4-1808057 TP on self-desense analysis for DC_20A_n28A-n75A, 3GPP TSG RAN WG4 #87, LG Electronics, Vodafone.

[9]	R4-1904562 MSD analysis for CA_n28_n50, 3GPP TSG-RAN WG4 Meeting #90bis, Huawei, HiSilicon.

[10]	R4-2403628 WF on PC2 HPUE MSD, 3GPP TSG-RAN WG4 Meeting #110, Skyworks Solutions Inc., Huawei, Qualcomm Inc.

[11]	R4-241xxxx CR to TS 38.101-1 Rel-18 NR CA Uplink Harmonic clean-up PC2, 3GPP TSG-RAN WG4 Meeting #112, Maastricht, Skyworks Solutions Inc.

[12]	R4-241xxxx CR to TS 38.101-3 Rel-18 NR CA Uplink Harmonic clean-up, 3GPP TSG-RAN WG4 Meeting #112, Maastricht, Skyworks Solutions Inc.

# Annex: UL2/DL1 MSD analysis for CA_n28-n75 and CA_n28-n74 [8,9]