# 3GPP TSG-RAN WG4 Meeting # 112                                                         R4-2413066

# Maastricht, The Netherlands, 19th – 23rd August 2024

# Title:	Further MSD simplifications

# Agenda Item:	10.1.1.1

# Source:	Skyworks Solutions, Inc.

# Document for:	Approval

# Introduction

This document presents our views to follow up the options captured at the last meeting in the agreed Way Forward (WF) [1].

# Discussion

The following three options were captured as the baseline for further discussions on simplifying HPUE MSD requirements:

- Option 1: RAN 4 defines only the default power class inter-band CA and intra-band CA MSD requirements, and the new HPUE MSD requirements are no longer specified in the TS. The legacy agreed HPUE MSD test points are not impacted and are kept in the TS.

- Option 2: RAN 4 defines the default power class inter-band CA and intra-band CA MSD requirements, and RAN4 specifies equations or simplified guidelines (like in [3,4]) or lookup tables “LUT” (like in [5]) that define the relationship between the default power class MSD and the HPUE MSD requirements:

  - Option 2a: Equations/LUT are captured in the RAN4 TS;

  - Option 2b: Equations/LUT are not captured in the RAN4 TS and are provided to RAN5.

- Option 3:  Keep the existing way of working, i.e., RAN4 analyses case-by-case all HPUE MSD requirements and experts flag TP for TRs for which MSD needs corrections/re-evaluations.

- Other options are not precluded.

Option 1 raise a lot of enthusiasm from many interested companies including ours. After review, it seems it is a bit too early to entirely support this option until:

- The impact of Option 1 on the entire ecosystem is fully understood, starting with the impact on conformance testing. The impact on the customer to system solution / components / chipset provider also needs to be carefully considered as in many instances the 3GPP requirements form the basis of discussions for commercial product development.

- The wide range of possible solutions to Option 2 are explored and discussed amongst companies. We provided an initial set of lookup tables to reduce the RAN4 workload on estimating the HPUE FDD band MSD requirements due to harmonic interference [2] at the last meeting. These guidelines have been used to introduce several band combinations in a short lapse of time. But further work is needed to refine and extend such type of approach to the remaining types of inter-band CA MSDs for HPUE. We bring updated harmonic MSD simplification guidelines at this meeting [3] and initial similar such input for MSD due to cross-band isolation. However, deriving a clear set of rules on how to reduce the workload and specification complexity for MSD test points due to dual UL IMD interference does not appear trivial. We therefore believe more studies are needed to draw a complete picture of pro & cons on Option 1 vs. Option 2.

Option 3 is not the desired way of working as the workload is becoming nearly impossible to handle.

Observation: With regards to WF [1]:

- Option 1: It is premature at this stage to support fully this option until the impact on the ecosystem is fully understood.

- Option 2: Only a few contributions have been presented so far at RAN4 towards reducing our workload  and the TS complexity. More studies are needed to converge towards a consensus. Some MSD are more difficult to simplify than others, e.g. MSD due to dual-UL IMD.

- We believe Option 3 should be ruled out.

Regarding the recently introduced MSD requirements for PC2 operation in FDD bands:

- In Rel 18, RAN4 has agreed to define two sets of inter-band CA MSD requirements for every band combination. Despite some effort to merge and minimize table size/complexity [5], the amount of work is still twice what was agreed upon for PC2 operation with TDD bands.

- There is no reason to keep on introducing two sets of MSD test points depending on if the PC2 power class in FDD bands is achieved via single Tx or dual Tx. Only one set of requirements should be captured for FDD band HPUE MSD requirements: the worst MSD requirements out of single Tx and dual TX.

Proposal: As an initial proposal towards Option 2, we propose that Rel-19 HPUE MSD requirements for inter-band CA with an FDD HPUE band are reduced to a single set of requirements; and, this single set captures the worst-case MSD requirements. This means that RAN4 no longer has to study two sets of MSD: one set of UE supporting single Tx and another set for UEs supporting dual Tx.

# Conclusion

This paper captures our views on simplifying MSD requirements for HPUE.

Observation: With regards to WF [1]:

- Option 1: It is premature at this stage to support fully this option until the impact on the ecosystem is fully understood.

- Option 2: Only a few contributions have been presented so far at RAN4 towards reducing our workload  and the TS complexity. More studies are needed to converge towards a consensus. Some MSD are more difficult to simplify than others, e.g. MSD due to dual-UL IMD.

- We believe Option 3 should be ruled out.

Proposal: As an initial proposal towards Option 2, we propose that Rel-19 HPUE MSD requirements for inter-band CA with an FDD HPUE band are reduced to a single set of requirements; and, this single set captures the worst-case MSD requirements. This means that RAN4 no longer has to study two sets of MSD: one set of UE supporting single Tx and another set for UEs supporting dual Tx.

# Reference

[1]	R4-2410605 WF on HPUE MSD Simplification, 3GPP TSG-RAN WG4 Meeting # 111, Skyworks Solutions, Inc., Murata, Qualcomm, Samsung, Nokia.

[2]    R4-2407156 Guidelines for FDD PC2 Harmonic MSD Analysis, 3GPP TSG-RAN WG4 Meeting # 111, Skyworks Solutions, Inc.

[3] 	 R4-2413068 Update on Harmonic MSD simplification, 3GPP TSG-RAN WG4 Meeting # 112, Skyworks Solutions, Inc.

[4] 	R4-2413065 Cross-band isolation MSD simplification, 3GPP TSG-RAN WG4 Meeting # 112, Skyworks Solutions, Inc.

[5]	 R4-2413022 CR to TS 38.101-1 Rel-18 NR CA Uplink Harmonic clean-up PC2, 3GPP TSG-RAN WG4 Meeting # 112, Skyworks Solutions, Inc.