3GPP TSG-RAN4 Meeting #112	R4-2413067

# Maastricht, Netherlands, 19th – 23rd August, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.852 | CR |  | rev |  | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TR 38.852: Clarification on PC1 Rx requirements for FRMCS operation in band n101 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Union Inter. Chemins de Fer, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RAIL_EU_1900MHz_TDD LTE_NR_HPUE_FWVM_R18-Core |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Current version of the TR does not address the issue of the missing Rx characteristics for PC1 cab-radio. In this CR we provide updates to clarify handling of Rx characteristics for PC1 cab-radio, as per ECC Decision (20)02. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 4.3.2: correction to address devices other thancab-radios.. New clause 4.3.3 and 7.2.3: cab-radio specific description added. Cross references corrections (drafting rules) and other editorial corrections |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TR would not capture decision on Rx characteristics for PC1 cab-radio. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.2, new 4.3.3, 7.2.1, new 7.2.3, 10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# <Start of Change>

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	ECC Decision (20)02: "Harmonised use of the paired frequency bands 874.4-880.0 MHz and 919.4-925.0 MHz and of the unpaired frequency band 1900-1910 MHz for Railway Mobile Radio (RMR) ".

[2]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[3]	RP-211496	Introduction of 1900MHz NR band for Europe for Rail Mobile Radio (RMR), WID

[4]	3GPP TS 38.104: " NR; Base Station (BS) radio transmission and reception".

[5]	European Commission implementing Decision (EU) 2021/1730 on the harmonised use of the paired frequency bands 874,4-880,0 MHz and 919,4-925,0 MHz and of the unpaired frequency band 1 900-1 910 MHz for Railway Mobile Radio, 28 September 2021.

[6]	ECC Report 318, Compatibility between RMR and MFCN in the 900 MHz range, the 1900-1920 MHz band and the 2290-2300 MHz band.

[7]	3GPP TR 37.829: " High-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands and NR bands for Rel-18".

[8]		R4-2400334	LS to 3GPP RAN4 on in-block output power requirements for bands n100 and n101 and on additional unwanted emission limits for band n100, LSin, CEPT ECC WG FM

[9]	ETSI TS 103 793: " Rail Telecommunications (RT); Future Rail Mobile Communication System (FRMCS); FRMCS Radio Characteristics"

# <End of Change>

# <Start of Change>

### 4.3.2	RMR terminals other than cab-radios using wideband technologies

The following technical conditions apply for RMR terminals other than cab-radios using wideband technologies:

Table 4.3.2-1: RMR terminals other than cab-radio requirements using wideband technologies

| Maximum output power | 23 dBm |
| --- | --- |
| ACLR | 30 dB minimum |
| Other | Uplink power control is mandatory and shall be activated. |

### 4.3.3	RMR cab-radios using wideband technologies

Cab-radios are high-power user equipments (i.e. HPUE) installed on trains with external antenna on top of the train roof at approximately 4m over groound.

ECC Decision (20)02 [1] has also captured transmit and receive requirements for cab-radios, which were not addressed in the Rel-17 WI. Those transmit requirements were captured in TR 37.829 [].

Table 4.3.3-1: RMR cab-radio requirements using wideband technologies

| Maximum output power | 31 dBm |
| --- | --- |
| ACLR | 37 dB minimum |
| Unwanted output power in 1920-1980 MHz |  -25 dBm/MHz maximum in 1920-1925 MHz;  -30 dBm/MHz maximum in 1925-1980 MHz; |
| Other | Uplink power control is mandatory and shall be activated. |

# <End of Change>

# <Start of Change>

### 7.2.1	Transmitter characteristics

Rel-17 UE requirements for RMR 1900 are summarized in the upcoming tables applicable for 3GPP TS 38.101-1 [2].

Table 7.2.1-1: UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n101 | 31 | +2/-3 |  |  |  |  | 23 | ±2 |

NOTE:	PC1 for cab-radio was introduced in Rel-18 inTR 37.829 [7].

# <End of Change>

# <Start of Change>

### 7.2.3	Additional requirements for RMR cab-radio receivers using wideband technologies

In ECC Decision (20)02 [1] Annex 3, additional requirements have been introduced for 1900 MHz RMR cab-radio receivers that reflect the specific train deployment case with UE antenna placed on top of the trains at approximately 4m over ground and having an effective antenna gain of 2dBi.

Table 7.2.3-1: RMR cab-radio receivers using wideband technologies [1]

| Level of the wanted signal | RefSens + 3 dB |
| --- | --- |
| Maximum 5 MHz LTE interfering signal in 1805-1880 MHz | -13 dBm |
| Maximum 5 MHz LTE interfering signal in 1920-1980 MHz | -39 dBm |
| The antenna connector of the radio module is the reference point. The reference sensitivity (RefSens) is the minimum mean power received at the antenna connector at which a specified minimum performance shall be met. These requirements cover both blocking and third-order intermodulation. |  |

The requirements in Table 7.2.3-1 are covered in the ETSI TS 103 793 [9].

# <End of Change>

# <Start of Change>

# 10	Conclusion

The study dealt with how the 1900-1910 MHz spectrum block (RMR1900) can be used for rail communication using 5G NR applicable in CEPT countries. The subject-areas 5G NR system, BS RF and UE RF were examined in detail.

General:	The focus was on the use of RMR1900 under uncoordinated conditions, assuming that RMR BS operates independently of other operators e.g. MFCN, however based on CEPT ECC WG FM clarification received in R4-2400334 [8], also coordinated deployments have to be considered. Co-location with other operators and resulting coordination is subject to national regulation and will be not further detailed by 3GPP RAN4.

System Parameter:	The RMR1900 spectrum block can be used by either a 10 MHz CBW or a 5 MHz CBW. The corresponding system parameters are to be transferred to 3GPP TS 38.101-1 [2] and 3GPP TS 38.104 [4].

BS RF:	The ECC Decision (20)02 [1], i.e. EIRP limits were analysed accordingly and the necessary conclusions for an uncoordinated operational approach were derived. However, based on CEPT ECC WG FM clarification received in R4-2400334 [8], also coordinated deployments have to be considered. These include the BS rated output power using corresponding assumptions, the resulting emission values of the transmitter and the specifications of the receiver. The agreed parameters are to be transferred to 3GPP TS 38.104 [4]. Furthermore, the coexistence with other 3GPP radio technologies has been included. These are to be captured in the corresponding normative 3GPP technical specifications.

UE RF:	Based on ECC Decision (20)02 [1], the EIRP limits were analysed accordingly, and the necessary conclusions were derived for the use of Power Class 3 (23dBm).

PC1 (31dBm) power class was introduced for the cab-radio after completion of Rel-17, with the derivation of Tx requirements captured in in TR 37.829 [7]. For related PC1 cab radio Rx characteristics for band n101 refer to ETSI TS 103 793 [9].

# <End of Change>