# 3GPP TSG-RAN WG4 Meeting # 112                                                         R4-2413068

# Maastricht, The Netherlands, 19 – 23 August 2024

# Title:	Update on Harmonic MSD simplification

# Agenda Item:	10.1.1.1

# Source:	Skyworks Solutions, Inc.

# Document for:	Approval

# Introduction

This document is a simplified version of our previous contribution based on additional harmonic measurements [1].

# Discussion

In our previous contribution, we provided several sets of lookup tables which help to quickly derive the PC2 FDD MSD level based on the band combination agreed upon for the PC3 level. These tables allow deriving both the PC2 single Tx and PC2 dual Tx MSD requirements. For dual Tx, 3 subsets were provided depending on which interference power balance is assumed between the primary antenna port and the diversity antenna port. In this contribution, we reduce the options to only 1 set of tables to derive the PC2 dual Tx MSD requirements.

We also bring more power amplifier harmonic measurement data since the derivation of our initial set of lookup tables relied on the assumption that the PC2 harmonic interference level is 3dB higher than the PC3 level. This assumption was based on the measurement of only 1 waveform at only 1 harmonic order. In this contribution we bring measurements across multiple waveforms at all harmonic orders ranging from harmonic 2 (H2) to harmonic 5 (H5).

## Power Amplifier Calibration & Results

- The PA linearity is calibrated using a 20 MHz, SCS=15 kHz, QPSK, DFT-S-OFDM, LCRB=100 (RBstart=0) waveform to meet:

  - -30dBc ACLR at MPR1 26dBm (4dB post PA insertion loss) for PC2 operation;

  - -31dBc ACLR at MPR1 29dBm (4dB post PA insertion loss) for PC2 operation.

- Waveform impairments:

  - Local Oscillator (LO) leakage: -28 dBc;

  - IQ Image rejection: -28 dB;

  - C-IM3 -60dBc, C-IM5 -70dBc.

The measurements of the PA harmonic emissions are performed from H2 to H5 using the following DFT-s-OFDM QPSK SCS=15kHz waveforms at PC3 MPR 0 and at PC2 MPR0:

- CBW=5MHz, LCRB=10(RBstart=0)

- CBW=5MHz, LCRB=12(RBstart=6)

- CBW=5MHz, LCRB=25(RBstart=0)

- CBW=20MHz, LCRB=50(RBstart=0)

Figure 1 plots the power difference between the power measured at MPR0 for PC3 and MPR 0 for PC2. The measurements indicate that the 3dB increase of harmonic interference power due to PC2 operation is valid up to harmonic 5 across a small set of waveforms.

Figure 1: Measured fundamental (H1), H2, H3, H4, H5 harmonic interference power difference between MPR0 PC3 and MPR0 PC2.

Observation 1: Under RAN4 PA linearity calibration test conditions, we may assume that the harmonic interference level for PC2 operation is approximately 3dB higher than for PC3 operation.

Based on Observation 1, the following proposals are reproduced and simplified from [1].

## PC2 single Tx lookup table proposal

Proposal 1: Consider adopting the PC2 singleTx MSD look-up of  Table 1 where the PC2 MSD is expressed as

PC2 single Tx MSD = PC3 MSD + PC2_1Tx.

Table 1: PC2 single-Tx simplified MSD using PC3 lookup tables

| Agreed PC3 MSD | Proposed PC2_1Tx |
| --- | --- |
| PC3 MSD ≥ 10dB | 3.0dB |
| 5.0dB ≤ PC3 MSD < 10dB | 2.5dB |
| 3.0dB ≤ PC3 MSD < 5.0dB | 2.0dB |
| 1.8dB ≤ PC3 MSD < 3.0dB | 1.5dB |
| 1.0dB ≤ PC3 MSD < 1.8dB | 1.0dB |
| PC3 MSD < 1.0dB or no PC3 MSD | Analysis case by case |
| NOTE 1: This set of PC2 single-Tx MSD class assumes an equal level of interference between the primary antenna and the diversity antenna. |  |

## PC2 dual-Tx lookup table proposal

Proposal 2: Consider adopting the PC2 dualTx MSD look-up of  Table 2 where the PC2 MSD is expressed as

PC2 dual Tx MSD = PC3 MSD + PC2_2Tx.

Table 2: PC2 dual-Tx simplified MSD using PC3 lookup tables

| Agreed PC3 MSD | PC2 2Tx MSD3 |
| --- | --- |
| [dB] |  |
| PC3 MSD ≥ 15dB | 8.0 |
| 8.5dB ≤ PC3 MSD < 15dB | 7.0 |
| 6.5dB ≤ PC3 MSD < 8.5dB | 6.0 |
| 4.5dB ≤ PC3 MSD < 6.5dB | 5.0 |
| 3.5dB ≤ PC3 MSD < 4.5dB | 4.0 |
| 2.5dB ≤ PC3 MSD < 3.5dB | 3.0 |
| 1.0dB ≤ PC3 MSD < 3.5dB | 2.0 |
| PC3 MSD < 1.0dB or no PC3 MSD | [FFS] |
| NOTE 3: This set of PC2 dual-Tx MSD assumes an 9dB lower interference on diversity than on primary antenna port. |  |

# Conclusion

In this contribution, we bring additional harmonic measurement data that confirms that the PC2 harmonic interference may be assumed 3dB greater than the PC3 level and we provided updated and simplified PC2 MSD lookup tables for single Tx and dual TX MSD requirements.

Observation 1: Under RAN4 PA linearity calibration test conditions, we may assume that the harmonic interference level for PC2 operation is approximately 3dB higher than for PC3 operation.

Proposal 1: Consider adopting the PC2 single Tx MSD look-up of  Table 1 where the PC2 MSD is expressed as

PC2 single Tx MSD = PC3 MSD + PC2_1Tx.

Table 1: PC2 single-Tx simplified MSD using PC3 lookup tables

| Agreed PC3 MSD | Proposed PC2_1Tx |
| --- | --- |
| PC3 MSD ≥ 10dB | 3.0dB |
| 5.0dB ≤ PC3 MSD < 10dB | 2.5dB |
| 3.0dB ≤ PC3 MSD < 5.0dB | 2.0dB |
| 1.8dB ≤ PC3 MSD < 3.0dB | 1.5dB |
| 1.0dB ≤ PC3 MSD < 1.8dB | 1.0dB |
| PC3 MSD < 1.0dB or no PC3 MSD | Analysis case by case |
| NOTE 1: This set of PC2 single-Tx MSD class assumes an equal level of interference between the primary antenna and the diversity antenna. |  |

Proposal 2: Consider adopting the PC2 dualTx MSD look-up of  Table 2 where the PC2 MSD is expressed as

PC2 dual Tx MSD = PC3 MSD + PC2_2Tx.

Table 2: PC2 dual-Tx simplified MSD using PC3 lookup tables

| Agreed PC3 MSD | PC2 2Tx MSD3 |
| --- | --- |
| [dB] |  |
| PC3 MSD ≥ 15dB | 8.0 |
| 8.5dB ≤ PC3 MSD < 15dB | 7.0 |
| 6.5dB ≤ PC3 MSD < 8.5dB | 6.0 |
| 4.5dB ≤ PC3 MSD < 6.5dB | 5.0 |
| 3.5dB ≤ PC3 MSD < 4.5dB | 4.0 |
| 2.5dB ≤ PC3 MSD < 3.5dB | 3.0 |
| 1.0dB ≤ PC3 MSD < 3.5dB | 2.0 |
| PC3 MSD < 1.0dB or no PC3 MSD | [FFS] |
| NOTE 3: This set of PC2 dual-Tx MSD assumes an 9dB lower interference on diversity than on primary antenna port. |  |

# Reference

[1]	R4-2407156 Guidelines for FDD PC2 Harmonic MSD Analysis, 3GPP TSG-RAN WG4 Meeting #111, Skyworks Solutions, Inc.

# Annex: