# 3GPP TSG-RAN WG4 Meeting # 112                                                         R4-2413069

# Maastricht, The Netherlands, 19 – 23rd August 2024

# Title:	CA_n5B BCS4/5 REFSENS

# Agenda Item:	7.3.2

# Source:	Skyworks Solutions, Inc.

# Document for:	Approval

# Introduction

This document presents our views on REFSENS requirements for newly requested CA_n5B BCS4/5 [1].

# Discussion

As background information, RAN4 recently introduced the REFSENS requirements for CA_n5B BCS1 with NOTE 5 as reproduced in Table 1 below.

Table 1 CA_n5B MSD test points for BCS0 and BCS1 (Table 7.3A.2.1)

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | UL PCC allocation (LCRB) | UL SCC allocation (LCRB) | PCC ΔRIBC (dB) | SCC ΔRIBC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n5B | 15/15 | 10MHz + 10MHz | 10 (RBstart = 0) | 10 (RBstart = 42) | 30.8 | 26.1 | FDD |
| CA_n5B5 | 15/15 | 5MHz + 20MHz | 4 (RBSTART = 0) | 16 (RBSTART = 90) | 44.6 | 23.0 | FDD |
| NOTE 1: All combinations of channel bandwidths defined in Table 5.5A.1-1. NOTE 2: The carrier centre frequency of SCC in the UL operating band is configured closer to the DL operating band. NOTE 3: The transmitted power over both PCC and SCC shall be set to PUMAX as defined in subclause 6.2A.4. NOTE 4: The PCC allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.3.2-1. NOTE 5: Applicable only to BCS 1. |  |  |  |  |  |  |  |

CA_n5B BCS1 introduces the support for 20MHz uplink (UL) channel bandwidth (CBW) are shown in Table 2.

Table 2 Rel 18.6.0 CA_n5B supported bandwidth combination sets.

| NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations or single uplink carrier5 | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
| CA_n5B | CA_n5B | 5, 10, 15 | 5, 10, 15 |  |  |  | 20 | 0 |
|  |  | 5, 10, 15, 20 | 5, 10, 15, 20 |  |  |  | 25 | 1 |

Considering Band n5 supports a maximum of 20MHz UL CBW, it can be concluded that the request for BCS4/5 does not introduce a new higher or a new lower CBW compared to BCS1 or even BCS0. Therefore the BCS1 REFSENS requirements are sufficient to cover the BCS4/5 request.

Observation: The request to introduce BCS4/5 for CA_n5B does not introduce a new lower or a new higher CBW. The CA_n5B BCS1 REFSENS requirements are applicable to BCS4/5.

Proposal: For CA_n5B BCS4/5, consider adopting the changes highlighted in yellow to the footnote 5 of Table 7.3A.2.1.

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | UL PCC allocation (LCRB) | UL SCC allocation (LCRB) | PCC ΔRIBC (dB) | SCC ΔRIBC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n5B | 15/15 | 10MHz + 10MHz | 10 (RBstart = 0) | 10 (RBstart = 42) | 30.8 | 26.1 | FDD |
| CA_n5B5 | 15/15 | 5MHz + 20MHz | 4 (RBSTART = 0) | 16 (RBSTART = 90) | 44.6 | 23.0 | FDD |
| NOTE 1: All combinations of channel bandwidths defined in Table 5.5A.1-1. NOTE 2: The carrier centre frequency of SCC in the UL operating band is configured closer to the DL operating band. NOTE 3: The transmitted power over both PCC and SCC shall be set to PUMAX as defined in subclause 6.2A.4. NOTE 4: The PCC allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.3.2-1. NOTE 5: Applicable only to BCS 1 and to BCS4/5. |  |  |  |  |  |  |  |

# Conclusion

The request to support BCS4/5 for CA_n5B does not introduce a new lower or a new lower CBW. Therefore the existing BCS1 REFSENS requirements are applicable to BCS4/5.

Proposal: For CA_n5B BCS4/5, consider adopting the changes highlighted in yellow to the footnote 5 of Table 7.3A.2.1.

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | UL PCC allocation (LCRB) | UL SCC allocation (LCRB) | PCC ΔRIBC (dB) | SCC ΔRIBC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n5B | 15/15 | 10MHz + 10MHz | 10 (RBstart = 0) | 10 (RBstart = 42) | 30.8 | 26.1 | FDD |
| CA_n5B5 | 15/15 | 5MHz + 20MHz | 4 (RBSTART = 0) | 16 (RBSTART = 90) | 44.6 | 23.0 | FDD |
| NOTE 1: All combinations of channel bandwidths defined in Table 5.5A.1-1. NOTE 2: The carrier centre frequency of SCC in the UL operating band is configured closer to the DL operating band. NOTE 3: The transmitted power over both PCC and SCC shall be set to PUMAX as defined in subclause 6.2A.4. NOTE 4: The PCC allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.3.2-1. NOTE 5: Applicable only to BCS 1 and to BCS4/5. |  |  |  |  |  |  |  |

# Reference

[1]	RP-241674 New WID: Rel-19 NR Carrier Aggregation (CA)/Dual Connectivity (DC) for x bands DL with y bands UL (x<7, y<3) and Supplementary Uplink (SUL) band combinations/CA band combinations with a single SUL or two SUL cells, 3GPP TSG-RAN Meeting #104, Shanghai, RAN4 Chair, Huawei.