# 3GPP TSG-RAN WG4 Meeting # 112                                                         R4-2413070

# Maastricht, The Netherlands, 19 – 23 August 2024

# Title:	DC_(n)8AA MSD

# Agenda Item:	7.2.2

# Source:	Skyworks Solutions, Inc.

# Document for:	Approval

# Introduction

This document presents our views on REFSENS requirements for the newly requested intra-band (IB) contiguous EN-DC combination in Band 8/n8 “DC_(n)8AA” [1].

# Discussion

As background information, we reproduce in Table 1 the existing IB EN-DC band combinations in Rel 18.6.0. At the exception of DC_(n)71AA, the operation of all other FDD band IB EN-DC combinations is restricted to single-switched UL operation due to very high self-desense caused by dual-UL intermodulation distortion. The agreed MSD for the exception case of DC_(n)71AA serves as a good reminder of the severity of self-desense for dual UL operation:

For DC_(n)71AA:

- LTE B71 10MHz MSD = 17.2dB;

- NR n71 10MHz MSD = 29.4dB.

The benefits of such deploying such a configuration in a commercial network are questionable. Band 8/n8 having similar duplex distance than Band 71/n71, it is expected that the conclusions reached for DC_(n)71AA would be equally applicable to DC_(n)8AA.

Table 1 Rel 18.6.0 intra-band contiguous EN-DC band combinations as of Rel 18.6.0 (Table 5.5B.2-1).

Observation: At the exception of DC_(n)71AA, the operation of all other intra-band contiguous FDD combinations is restricted to single switched uplink. This is mostly due to the very high dual UL self-desense. For example, DC_(n)71AA is affected by 17dB and 29dB MSD for LTE/NR cells. Similar issues are expected for DC_(n)8AA due to near identical Tx/Rx duplex distance.

## DC_(n)8AA IMD landscape

The requested bandwidth combination set (BCS) for DC_(n)8AA is reproduced in Table 2 below.

Table 2 Request bandwidth combination set for DC_(n)8AA [1]

| ENDC configuration | Uplink ENDC configurations | Component carriers in order of increasing carrier frequency |  |  | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Channel bandwidths for E-UTRA carrier (MHz) | Channel bandwidths for NR carrier (MHz) | Channel bandwidths for E-UTRA carrier (MHz) |  |  |
| DC_(n)8AA | DC_(n)8AA | 5, 10 | 5, 10, 15, 20 |  | 30 | 0 |
|  |  |  | 5, 10, 15, 20 | 5, 10 |  |  |

Dual-UL IMD landscape:

Figure 1 below shows that for dual-UL transmissions with configuration LTE 5MHz / NR 20MHz CBW, the downlink (DL) LTE cell may be affected by dual UL IMD3 and the NR cell by dual UL IMD5 products for LTE UL RB allocation LCRB=4(RBstart=0) and NR LCRB=16(RBstart=0) . The MSD that may result from such interference is expected to be even higher than that specified for DC_(n)71AA. Benefits of such deployment are questionable and only single switched uplink operation is recommended.

Figure 1: DC_(n)8AA dual-UL IMD sketch for LTE 5MHz / NR 20MHz CBW,

Single-UL IMD landscape:

Figure 2 below shows that for single-UL transmissions with configuration LTE 10MHz / NR 20MHz CBW, the downlink (DL) LTE cell may be affected by one UL IMD3 product and the NR cell by one UL IMD5 product. The MSD is expected moderately high but much lower than for the case of dual UL operation. Selecting LCRB=5 ensures the entire power spectral density (PSD) of the UL LCRB=5 IMD 3/5 product are entirely overlapped by the LTE/NR DL carriers respectively. Selecting the REFSENS UL configuration of LCRB=20 spreads the IMD product PSD slightly beyond the edges of the LTE/NR DL carrier, yet most of the IMD energy is expected to significantly affect each DL cell.

Figure 2: DC_(n)8AA single-UL IMD sketch for LTE 10MHz / NR 20MHz CBW: LCRB=5(RBstart=101) vs LCRB=20(RBstart=86)

Proposal: For DC_n(8)AA BCS 0, consider the following proposal:

- Restrict operation of DC_(n)8AA to single-switched UL only.

- Evaluate MSD using the following test point.

| EN-DC configuration / channel allocations /MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC configuration | E-UTRA/NR band | FC (UL) (MHz) | Channel bandwidth (MHz) | UL allocation (LCRB) | FC (DL) (MHz) | MSD (dB) | Duplex mode |
| DC_(n)8AA | 8 | N/A | 10 | N/A | 935.5 | FFS | FDD |
|  | n8 | 905.4 | 20 | 20 (RBstart = 86) | 950.4 | FFS |  |
| NOTE 1: In accordance to BCS1, the NR uplink bandwidth is specified as in this table, but the corresponding NR downlink bandwidth is 5 MHz larger. NOTE 2: The transmitters powers shall be set to PUMAX, as defined in TS 38.101-1 [2], TS 38.101-2 [3], and TS 36.101 [4], with additional limits on configured maximum output power for the uplink according to clause 6.2B.4. NOTE 3: Applicable only to BCS 2. NOTE 4: Applicable only to BCS 1. NOTE 5: Applicable only to BCS 0. |  |  |  |  |  |  |  |

# Conclusion

DC_(n)8AA may be subject to severe self-desense for dual UL operation. MSD greater than or equal to DC_(n)71AA 17dB/29.7dB is expected for DC_(n)8AA. We therefore make the following proposal.

Proposal: For DC_n(8)AA BCS 0, consider the following proposal:

- Restrict operation of DC_(n)8AA to single-switched UL only.

- Evaluate MSD using the following test point.

| EN-DC configuration / channel allocations /MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC configuration | E-UTRA/NR band | FC (UL) (MHz) | Channel bandwidth (MHz) | UL allocation (LCRB) | FC (DL) (MHz) | MSD (dB) | Duplex mode |
| DC_(n)8AA | 8 | N/A | 10 | N/A | 935.5 | FFS | FDD |
|  | n8 | 905.4 | 20 | 20 (RBstart = 86) | 950.4 | FFS |  |
| NOTE 1: In accordance to BCS1, the NR uplink bandwidth is specified as in this table, but the corresponding NR downlink bandwidth is 5 MHz larger. NOTE 2: The transmitters powers shall be set to PUMAX, as defined in TS 38.101-1 [2], TS 38.101-2 [3], and TS 36.101 [4], with additional limits on configured maximum output power for the uplink according to clause 6.2B.4. NOTE 3: Applicable only to BCS 2. NOTE 4: Applicable only to BCS 1. NOTE 5: Applicable only to BCS 0. |  |  |  |  |  |  |  |

# Reference

[1]	RP-241673 New WID: Rel-19 Dual connectivity (DC) of x LTE band(s), y NR band(s) (x<=6) and single or two NR Supplementary Uplink (SUL) bands, 3GPP TSG-RAN Meeting #104, Shanghai, RAN4 Chair, Huawei.