3GPP TSG-RAN WG4 Meeting #112	R4-2413071

# Maastricht, Netherlands, 19th – 23rd August, 2024

Source:	ZTE Corporation, Sanechips

Title:	Discussion on core the maintenance for Case 2

Agenda Item:	5.15.1

Document for:	Approval

- Introduction

- Around Case 1 and 2 in the WI, some progress were achieved in last meeting and captured in [1] as below.

Sub-topic 2-1: Collision handling for dynamic collisions - Agreement: - The above issues 2-1-1, 2-1-2, 2-1-3, and 2-1-4 are closed. The maintenance for core part to be discussed in CRs. Issue 2-1-5: [Case 1] - [New issue - dropping rule optimization] Whether to optimise the concurrent measurement gaps are collided when collided? - Agreement: - RAN4 not to consider optimizations of the collision handling for concurrent gaps in Rel-18. - This issue is closed. Issue 2-1-6: [Case 1] - [New issue - spec cleaning] This issue related to further cleaning in current spec writing [multiple options can be selected based on discussion]: - Agreement: Maintenance to be discussed in a CR directly. Issue 2-1-7: [Case 1] - [New issue – Dynamic collision] What is the UE behaviour when the UE doesn’t support dynamic collision FG? Agreement from online session: For UE not supporting dynamic collision, the MG will be drop if overlapped with Pre-MG, regardless whether Pre-MG (with higher priority) is activated or deactivated, including the case when the MG overlaps with the Pre-MG activation/deactivation procedure. Issue 3-2-1: [Case 2] When the UE is configured with Concurrent gaps with NCSG, what is the potential changes to UE behaviour for NCSG upon SCell activation (in Rel-18) Agreement from online session: - For UE configured with one NCSG and one Type 1/2 MG: All deactivated SCells are measured within NCSG, regardless of the reported UE capabilities [and gap association]. - Further details on the processing delay between NCSG and Type 1/2 MG can be further discussed. - For UE configured with 2 NCSG, deactivated SCells are measured with NCSG - If the association is provided, deactivated SCells are measured with NCSG according to gap association. - If the association is not provided, UE is not expected to cause interruption outside the VIL due to measurement on any of the deactivated SCells, and the existing measurement delay requirement does not apply to this case.

Besides these, still only one remaining issue need to be decided. In this paper, we provide our views on this remaining issue.

- Discussion

UE behavior for deactivated SCell measurements with NCSG

This issue was discussed in last meeting and concluded with the following progress:

Agreement from online session: - For UE configured with one NCSG and one Type 1/2 MG: All deactivated SCells are measured within NCSG, regardless of the reported UE capabilities [and gap association]. - Further details on the processing delay between NCSG and Type 1/2 MG can be further discussed. - For UE configured with 2 NCSG, deactivated SCells are measured with NCSG - If the association is provided, deactivated SCells are measured with NCSG according to gap association. - If the association is not provided, UE is not expected to cause interruption outside the VIL due to measurement on any of the deactivated SCells, and the existing measurement delay requirement does not apply to this case.

We highlighted the still open part. Regarding the part in square brackets, we believe the square brackets can be removed. For the case of one NCSG and one Type 1/2 MG configured, no matter the gap association configured, based on existing spec, the deactivated SCell can always be measured within NCSG given that all or part of SMTC occasions are overlapped with NCSG. As a result, even one activated SCell MO is associated with the Type 1/2 MG at first, after a while, the activated SCell is switched to deactivated status, performing this deactivated SCell measurement with in the NCSG, which could avoid additional interruption, it is beneficial to the system performance. Furthermore, which is allowed by existing spec. Therefore, we agree to ignore the association, the deactivated SCell can always be measured within NCSG.

Proposal 1: For UE configured with one NCSG and one Type 1/2 MG: All deactivated SCells are measured within NCSG, regardless of the reported UE capabilities [and gap association].

Regarding the processing delay between NCSG and Type 1/2 MG, For Case 2, no matter the case of NCSG + NCSG or the case of NCSG + Type 1/2 MG, not any additional processing delay was introduced compared with legacy. So we believe just following the existing processing delay is fine, not additional processing delay needed.  For an activated SCell, if it is configured to be associated with the Type 1/2 MG when it configured. But after the deactivation procedure, the status of this SCell is updated to deactivated, then this deactivated SCell would be measured within the NCSG instead of the Type 1/2 MG any more, such update of applicable gap for this SCell can be performed during the deactivated procedure or during the VIL of NCSG, Therefore, no additional processing delay is needed.

Observation 1: For an activated SCell, if it is configured to be associated with the Type 1/2 MG but after the deactivation procedure, the status of this SCell is updated to deactivated, then this deactivated SCell would be measured within the NCSG instead of the Type 1/2 MG any more. Such update of applicable gap for this SCell can be performed during the deactivated procedure or during the VIL of NCSG.

Proposal 2: For UE configured with one NCSG and one Type 1/2 MG: All deactivated SCells are measured within NCSG, regardless of the reported UE capabilities [and gap association]. No the processing delay between NCSG and Type 1/2 MG is needed.

- Conclusion

In this contribution, we have the following proposals for joint consideration around Case 2:

Proposal 1: For UE configured with one NCSG and one Type 1/2 MG: All deactivated SCells are measured within NCSG, regardless of the reported UE capabilities [and gap association].

Observation 1: For an activated SCell, if it is configured to be associated with the Type 1/2 MG but after the deactivation procedure, the status of this SCell is updated to deactivated, then this deactivated SCell would be measured within the NCSG instead of the Type 1/2 MG any more. Such update of applicable gap for this SCell can be performed during the deactivated procedure or during the VIL of NCSG.

Proposal 2: For UE configured with one NCSG and one Type 1/2 MG: All deactivated SCells are measured within NCSG, regardless of the reported UE capabilities [and gap association]. No the processing delay between NCSG and Type 1/2 MG is needed.

- Reference

- R4-2410376 “WF on NR_MG_enh2_part1”, MediaTek Inc