3GPP TSG-RAN WG4 Meeting #112	R4-2413073

# Maastricht, Netherlands, 19th – 23rd August, 2024

Source:	ZTE Corporation, Sanechips

Title:	Discussion on measurement without gaps for UEs reporting NeedForGapsInfoNR

Agenda Item:	5.15.1

Document for:	Discussion

- Introduction

In last RAN4 meeting, around the measurement without gap, the following progress were achieved in [1] as below. Besides these progress, some agreements around the UE feature were also approved.

Issue 1-1-3: Aligned DRX-on duration and SMTC for NFG measurements - Agreement - Interruption ratio requirement not based on DRX-on duration - Not define the interruption location Issue 1-3-3: Cross feature support - Agreement - Rel-18 requirements for UE supporting NFG and inter-RAT measurements without gap do not apply for FR2-2.

In this paper, we provide our views on that remaining issues.

- Discussion

- 2.1 DRX

Misalignment between DRX-on duration and SMTC for NFG measurements

DRX is an important technology in NR to achieve power saving, thus DRX based interruption requirement is worthy to be discussed. In last meeting, the following options were proposed and warmly discussed, but unfortunately no consensus achieved.

- Option 1: - Option 1a: Interruptions are always allowed outside DRX ON duration and it is according to Tcycle,i. - Option 1b: Interruptions are not allowed during DRX ON duration. - Option 1c: Interruptions are not allowed when DRX cycle is larger than 320ms. - Option 1d: Interruptions are not allowed in the DRX ON duration, excluding the time extended due to drx-inactivityTimer. - Option 2: Interruptions are allowed under the conditions of issues 1-1-2, 1-1-3, 1-1-4.

To our understand, If DRX cycle is larger than 320ms, a DRX cycle would have multiple SMTC occasions. The UE can select one SMTC occasion within DRX OFF duration to perform measurement on, which can reduce interruption ratio with the cost of power consumption.

If DRX cycle is smaller than 320ms, since SMTC is cell specific and DRX is UE specific, it is possible that the configured SMTC occasions are completely misaligned with DRX ON duration. In this case, the UE only can perform measurement on the DRX OFF duration, which can not cause any interruption. If the SMTC occasion is partially overlapping with DRX ON duration, the UE performs measurement during DRX OFF duration as much as possible, similar to legacy DRX based measurement requirement.

Based on such analysis, we prefer Option 1c and Option 1d. Option 1c aims to face the case of large DRX cycle, and Option 1d aims to face the case of small DRX cycle.

Proposal 1: When Misalignment between DRX-on duration and SMTC, for the case of DRX cycle larger than 320ms, interruptions are not allowed. For the case of DRX cycle not larger than 320ms, interruptions are not allowed in the DRX ON duration, excluding the time extended due to drx-inactivityTimer.

Aligned DRX-on duration and SMTC for NFG measurements

Regarding the aligned case, in last meeting, two basic principles were achieved:

- Agreement - Interruption ratio requirement not based on DRX-on duration - Not define the interruption location

Besides these principles, how to define the exact interruption ratio, which is still open. Considering the fact that the interruption location is not specified and depend on UE vendor implementation, only one thing can be sure that the interruption would be outside the SMTC. Here the DRX-on duration is aligned with SMTC, if they are equal in length, then no interruption. If the DRX-on duration is larger than SMTC, then it is possible there is interruption or not. So the situations are complicated. To move forward, it is better to split the discussion into two cases: the large DRX-on duration and the small DRX-on duration. But which is a bit conflict with the approved 1st principle.

From our view, we believe the interruption is harm to system performance, we prefer the interruption is not allowed at least for the small DRX-on duration. For the large DRX-on duration, we can agree that interruption is allowed but except for the last DL slot containing PDCCH in the ON duration.

Proposal 2: The interruption is not allowed at least for the small DRX-on duration. For the large DRX-on duration, we can agree that interruption is allowed but except for the last DL slot containing PDCCH in the ON duration.

- 2.2 Others

- Whether allow to enable R17 and R18 UE capabilities

Regarding this issue, some discussion were held in previous meeting and the following progress achieved in [1]:

Issue 1-3-2: 1-to-1 mapping between NeedForGaps and NCSG capabilities - Agreements - No need to establish the mapping between UE’s indication for NeedForGaps and NCSG.

During online discussion in last meeting,the issue of whether allow to enable both R17 NCSG and R18 NeedForGaps capabilities was also touched. Some company believe only enable one of the two UE capabilities is enough, we have different view.

Firstly from the signalling perspective, R17 NCSG capability includes semi-static UE capability nr-NeedForGapNCSG-Reporting-r17 and dynamic reporting NeedForGapNCSG-InfoNR-r17, whether UE  dynamically reports NeedForGapNCSG-InfoNR-r17, depends on NW request NeedForGapNCSG-ConfigNR. We listed all these R17 NCSG signallings defined in [2] as below:

UE capability(semi-staitic): nr-NeedForGapNCSG-Reporting-r17 nr-NeedForGapNCSG-Reporting-r17 ENUMERATED {supported} OPTIONAL,

UE reporting(dynamic): NeedForGapNCSG-InfoNR-r17 NeedForGapNCSG-InfoNR-r17 ::= SEQUENCE { intraFreq-needForNCSG-r17 NeedForNCSG-IntraFreqList-r17, interFreq-needForNCSG-r17 NeedForNCSG-BandListNR-r17 } NeedForNCSG-IntraFreqList-r17 ::= SEQUENCE (SIZE (1.. maxNrofServingCells)) OF NeedForNCSG-IntraFreq-r17 NeedForNCSG-BandListNR-r17 ::= SEQUENCE (SIZE (1..maxBands)) OF NeedForNCSG-NR-r17 NeedForNCSG-IntraFreq-r17 ::= SEQUENCE { servCellId-r17 ServCellIndex, gapIndicationIntra-r17 ENUMERATED {gap, ncsg, nogap-noncsg} } NeedForNCSG-NR-r17 ::= SEQUENCE { bandNR-r17 FreqBandIndicatorNR, gapIndication-r17 ENUMERATED {gap, ncsg, nogap-noncsg} }

NW request: needForGapNCSG-ConfigNR-r17 RRCReconfiguration-v1700-IEs ::= SEQUENCE { otherConfig-v1700 OtherConfig-v1700 OPTIONAL, -- Need M sl-L2RelayUE-Config-r17 SetupRelease { SL-L2RelayUE-Config-r17 } OPTIONAL, -- Need M sl-L2RemoteUE-Config-r17 SetupRelease { SL-L2RemoteUE-Config-r17 } OPTIONAL, -- Need M dedicatedPagingDelivery-r17 OCTET STRING (CONTAINING Paging) OPTIONAL, -- Cond PagingRelay needForGapNCSG-ConfigNR-r17 SetupRelease {NeedForGapNCSG-ConfigNR-r17} OPTIONAL, -- Need M needForGapNCSG-ConfigEUTRA-r17 SetupRelease {NeedForGapNCSG-ConfigEUTRA-r17} OPTIONAL, -- Need M musim-GapConfig-r17 SetupRelease {MUSIM-GapConfig-r17} OPTIONAL, -- Need M ul-GapFR2-Config-r17 SetupRelease { UL-GapFR2-Config-r17 } OPTIONAL, -- Need M scg-State-r17 ENUMERATED { deactivated } OPTIONAL, -- Need N appLayerMeasConfig-r17 AppLayerMeasConfig-r17 OPTIONAL, -- Need M ue-TxTEG-RequestUL-TDOA-Config-r17 SetupRelease {UE-TxTEG-RequestUL-TDOA-Config-r17} OPTIONAL, -- Need M nonCriticalExtension RRCReconfiguration-v1800-IEs OPTIONAL }

The dynamic UE reporting NeedForGapNCSG-InfoNR-r17 can be per band per BC level. So it is possible that UE report different status for different bands.

Regarding the R18 signalling, it is on top of R16 NFG, an additional interruption indication can be dynamic reported for the R16 and R18 capable UE. Both the NeedForGapsInfoNR-r16 and NeedForInterruptionInfoNR-r18 are dynamically reported by the UE if NW requests. The two reporting are both per band per BC level. We listed all the R16 and R18 signallings as below.

R16 NFG:

nr-NeedForGap-Reporting-r16 Indicates whether the UE supports reporting the measurement gap requirement information for NR target in the UE response to a network configuration RRC message. - nr-NeedForGap-Reporting-r16 ENUMERATED {supported} OPTIONAL

needForGapsConfigNR - Configuration for the UE to report measurement gap requirement information of NR target bands in the RRCReconfigurationComplete and RRCResumeComplete message. RRCReconfiguration-v1610-IEs ::= SEQUENCE { otherConfig-v1610 OtherConfig-v1610 OPTIONAL, -- Need M bap-Config-r16 SetupRelease { BAP-Config-r16 } OPTIONAL, -- Need M iab-IP-AddressConfigurationList-r16 IAB-IP-AddressConfigurationList-r16 OPTIONAL, -- Need M conditionalReconfiguration-r16 ConditionalReconfiguration-r16 OPTIONAL, -- Need M daps-SourceRelease-r16 ENUMERATED{true} OPTIONAL, -- Need N t316-r16 SetupRelease {T316-r16} OPTIONAL, -- Need M needForGapsConfigNR-r16 SetupRelease {NeedForGapsConfigNR-r16} OPTIONAL, -- Need M onDemandSIB-Request-r16 SetupRelease { OnDemandSIB-Request-r16 } OPTIONAL, -- Need M dedicatedPosSysInfoDelivery-r16 OCTET STRING (CONTAINING PosSystemInformation-r16-IEs) OPTIONAL, -- Need N sl-ConfigDedicatedNR-r16 SetupRelease {SL-ConfigDedicatedNR-r16} OPTIONAL, -- Need M sl-ConfigDedicatedEUTRA-Info-r16 SetupRelease {SL-ConfigDedicatedEUTRA-Info-r16} OPTIONAL, -- Need M targetCellSMTC-SCG-r16 SSB-MTC OPTIONAL, -- Need S nonCriticalExtension RRCReconfiguration-v1700-IEs OPTIONAL }

UE dynamic reporting: NeedForGapsInfoNR-r16 ::= SEQUENCE { intraFreq-needForGap-r16 NeedForGapsIntraFreqList-r16, interFreq-needForGap-r16 NeedForGapsBandListNR-r16 } NeedForGapsIntraFreqList-r16 ::= SEQUENCE (SIZE (1.. maxNrofServingCells)) OF NeedForGapsIntraFreq-r16 NeedForGapsBandListNR-r16 ::= SEQUENCE (SIZE (1..maxBands)) OF NeedForGapsNR-r16 NeedForGapsIntraFreq-r16 ::= SEQUENCE { servCellId-r16 ServCellIndex, gapIndicationIntra-r16 ENUMERATED {gap, no-gap} } NeedForGapsNR-r16 ::= SEQUENCE { bandNR-r16 FreqBandIndicatorNR, gapIndication-r16 ENUMERATED {gap, no-gap} }

R18 NFInerruption:

nr-NeedForInterruptionReport-r18 ENUMERATED {supported} OPTIONAL,

needForInterruptionConfigNR Indicates whether the UE shall report interruption requirement information of NR target bands in the RRCReconfigurationComplete and RRCResumeComplete message. The network sets this field to enabled only if the needForGapsConfigNR is configured. The network sets this field to disabled if the needForGapsConfigNR is released. RRCReconfiguration-v1800-IEs ::= SEQUENCE { needForInterruptionConfigNR-r18 ENUMERATED { enabled, disabled } OPTIONAL, -- Need M uav-Config-r18 SetupRelease { UAV-Config-r18 } OPTIONAL, -- Need M sl-IndirectPathAddChange-r18 SetupRelease { SL-IndirectPathAddChange-r18 } OPTIONAL, -- Need M n3c-IndirectPathAddChange-r18 SetupRelease { N3C-IndirectPathAddChange-r18 } OPTIONAL, -- Need M n3c-IndirectPathConfigRelay-r18 SetupRelease { N3C-IndirectPathConfigRelay-r18 } OPTIONAL, -- Need M otherConfig-v1800 OtherConfig-v1800 OPTIONAL, -- Need M srs-PosResourceSetLinkedForAggBWList-r18 SetupRelease { SRS-PosResourceSetLinkedForAggBWList-r18 } OPTIONAL,-- Need M ltm-Config-r18 SetupRelease {LTM-Config-r18} OPTIONAL, -- Need M nonCriticalExtension SEQUENCE {} OPTIONAL }

UE dynamic reporting: NeedForInterruptionInfoNR-r18 ::= SEQUENCE { intraFreq-needForInterruption-r18 NeedForInterruptionIntraFreqList-r18, interFreq-needForInterruption-r18 NeedForInterruptionBandListNR-r18 } NeedForInterruptionIntraFreqList-r18 ::= SEQUENCE (SIZE (1.. maxNrofServingCells)) OF NeedForInterruptionNR-r18 NeedForInterruptionBandListNR-r18 ::= SEQUENCE (SIZE (1..maxBands)) OF NeedForInterruptionNR-r18 NeedForInterruptionNR-r18 ::= SEQUENCE { interruptionIndication-r18 ENUMERATED {no-gap-with-interruption, no-gap-no-interruption} OPTIONAL }

If UE only support one of the alternatives between R17 capability and R16+R18 capability, then certainly NW can only request the supported reporting. While if UE supports both of R17 and R16+R18, from NW perspective, since not sure what status the UE would report for a certain band in the R17 and R16+R18 reporting, so the most reliable solution is to request both R17 and R16+R18 reporting.

Furthermore, the NW request is semi-statically configured, while the R17 and R16+R18 reporting are dynamic reported, so NW can not adaptively change the request depending on what status the UE reports. E.g. at first NW only request R17 reporting, and the UE reports ‘ncsg’ for a band. After a while, the UE reports ‘gap’ for the same band, then the NW does not know why the UE can not support ‘ncsg’ for the same band, and the NW wants to know whether the UE can support ‘no gap with interruption’, but unfortunately, the NW can not reconfigure the request dynamically. So as to avoid such case, it is better to allow the NW to request both R17 and R16+R18 reporting.

Another case which would be beneficial by requesting both R17 and R16+R18 is the handover case. Since it is possible that the source BS supports R16+R18 while the target BS only supports R17, if the source BS only requests R16+R18 instead of requests both R17 and R16+R18, after the UE handovers to the target BS, the UE still only report R16+R18 reporting to the target BS, but since not support R18 signalling, the target BS can not understand the actual status reported by UE.

Consequently, it is beneficial and flexible to allow the NW to request both R17 and R16+R18 reporting for the capable UE.

It has been apporved that not and mapping between the R17 and R16+R18 reporting would be defined, so the NW could decide whether to configure gap or NCSG or neither of them based on the received reporting.

Proposal 3: Allow to enable both R17 and R18 reporting.

- 2.3 UE capability

Relations between nr-NeedForGap-Reporting-r16 and nr-NeedForInterruptionReport-r18 and UE behaviours

This issue was warmly discussed during last meeting and the following options were captured in [1] as below:

- Option 1: A Rel-18 UE indicating support of nr-NeedForGap-Reporting-r16 shall also indicate support of nr-NeedForInterruptionReport-r18. - Option 2: When a Rel-18 UE only supports Rel-16 NFG capability but not supports Rel-18 NFI capability, the UE’s behaviour is the same as Rel-16 UE. - Option 2a: When a Rel-18 UE supports both Rel-16 NFG and Rel-18 NFI capabilities, but NW doesn’t configure Rel-18 needForInterruptionConfigNR, the UE’s behaviour is the same as Rel-16 UE. - Option 3: A Rel-18 UE indicating support of interRAT-NeedForGapsNR-r16 shall also indicate if interruptions are needed. - No new RRM test cases are defined for a Rel-18 UE supporting signalling above.

We can understand the intention of the proponent of Option 1, such binding of R18 NFI and R16 NFG tries to make the interruption allowed/no allowed more clear. We appreciate such intention, however we still believe the existing product should also be considered. As we said online in last meeting, we concern such binding would miss the performance gain caused by the R16 NFG if the UE vendor can not support R18 NFI.

Consequently, we still prefer Option 2 and 2a. Both the nr-NeedForGap-Reporting-r16 and nr-NeedForInterruptionReport-r18 are optional UE features. From UE perspective, R18 UE may support neither of them, only support R16 NFG, or support both R16 NFG and R18 NFI. From NW perspective, the R18 NW can request only R16 NFG reporting or request both R16 NFG and R18 NFI reporting for the capable UE.

Proposal 4: When a Rel-18 UE only supports Rel-16 NFG capability but not supports Rel-18 NFI capability, the UE’s behaviour is the same as Rel-16 UE. When a Rel-18 UE supports both Rel-16 NFG and Rel-18 NFI capabilities, but NW doesn’t configure Rel-18 needForInterruptionConfigNR, the UE’s behaviour is the same as Rel-16 UE.

- Conclusion

In this contribution, we have the following proposals:

Proposal 1: When Misalignment between DRX-on duration and SMTC, for the case of DRX cycle larger than 320ms, interruptions are not allowed. For the case of DRX cycle not larger than 320ms, interruptions are not allowed in the DRX ON duration, excluding the time extended due to drx-inactivityTimer.

Proposal 2: The interruption is not allowed at least for the small DRX-on duration. For the large DRX-on duration, we can agree that interruption is allowed but except for the last DL slot containing PDCCH in the ON duration.

Proposal 3: Allow to enable both R17 and R18 reporting.

Proposal 4: When a Rel-18 UE only supports Rel-16 NFG capability but not supports Rel-18 NFI capability, the UE’s behaviour is the same as Rel-16 UE. When a Rel-18 UE supports both Rel-16 NFG and Rel-18 NFI capabilities, but NW doesn’t configure Rel-18 needForInterruptionConfigNR, the UE’s behaviour is the same as Rel-16 UE.

- Reference

- R4-2410406 “WF on NR_MG_enh2_part2”, Intel Corporation

- TS 38.331-i00