3GPP TSG-RAN WG4 Meeting #112	R4-2413076

# Maastricht, Netherlands, 19th – 23rd August, 2024

Source:	ZTE Corporation, Sanechips

Title:	Discussion on CSSF optimization of R19 RRM enhancements

Agenda Item:	8.15.2.2

Document for:	Discussion

- Introduction

In recent RAN plenary 103 meeting, a new R19 WID of RRM enhancements has been approved in [1]. We summarize the objectives of the WID as below:

Objective of Core part WI - FR2-1 SSB based L3 measurement delay reduction for connected mode - For UE supporting multiple-Rx simultaneous reception on single carrier: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - Rx beam sweeping factor - For UE not in multiple-Rx simultaneous reception mode: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2 - Fast SCell activation for UE supporting Rel-18 EMR - Study and, if feasible, to reduce the SCell activation delay with valid EMR reporting - Apply fast scell activation in FR1 and FR2-1 - Note: RAN4 to start this work from Q3’2024 and aim for completion in Dec’2024. Workplan for this bullet can be discussed in May’2024 Objective of Performance part WI NOTE: Leave empty if the WI proposal does not contain a RAN performance part. Specify RRM performance requirements and test cases for the Core part enhancements.

It is obviously two sub topics are involved in this new WID. The 1st topic refers to the enhancements on L3 measurements, through multi-Rx and CSSF reduction to reduce the L3 measurement delay. And the 2nd topic aims to further accelerate the SCell activation procedure by applying the EMR reporting. Both the two sub topics are beneficial to reduce the latency and relieve the corresponding interruption impact on the system performance. After discussion in last meeting, the work plan was approved in [2], the 2nd topic would start from 112 meeting.

In this paper, we provide our views on the 2nd adaptation of the 1st topic.

- Discussion

Regarding this sub topic, we share our views from the following aspects:

- Applicability scenario

- Applicability condition

- Exact solution

- Applicability scenario

For the CSSF outside gap enhancement in CA/DC, for the UE supporting multiple carriers in CA/DC, restricted to the assumption of 2 searchers, one searcher is applied to PCC specially, and the other searcher is shared by all SCCs and neighbour cells, so CSSF is introduced in legacy L3 measurement in CA/DC. With the increase of carrier number, the CSSF may be large, Accordingly, the measurement/identification period would be pretty large. If the CSSF can be reduced with some solution, the L3 measurement period would be reduced.

In legacy, for the case of DC, EN-DC, NE-DC and NR-DC are all considered in the CSSF outside gap identification. Similarly, all these DC scenarios are included in this R19 scope for the CSSF enhancement.

Proposal 1: For the CSSF enhancement, CA, EN-DC, NE-DC and NR-DC are applicable scenarios.

- Applicability condition

- This issue was warmly discussed during 110bis meeting around the following option:

- Option 1 (ZTE): It should be clarified that “For UE not in multiple-Rx simultaneous reception mode” includes two cases: 1) The UE is not capable of multi-Rx; 2) The UE is capable of multi-Rx but work in single-Rx currently.

- During the discussion, companies reach consensus on 1) but still diverse on 2). So finally the following agreements achieved in [3]:

Agreement: - Rel-19 discussion on CSSF optimization starts for the case UE is not capable of Rel-18 multi-Rx simultaeous reception, further discuss whether/how it can be applied to the case UE is capable of Rel-18 multi-Rx simultaeous reception but work in single-Rx currently.

We believe the intention of this R19 WID aim to reach two optimization: 1) For the multi-Rx enabling case, optimizing the system performance by reducing the L3 measurement latency, since which is not covered by R18 multi-Rx. 2) For non multi-Rx case, i.e. normal case, optimizing the system performance by controlling the CSSF to accommodate more carriers configured. So actually the 2nd intention is irrelevant to whether multi-Rx enabling. However, so as to simplify the scenario and distinguish the two intentions, the 2nd intention is depicted as “For UE not in multiple-Rx simultaneous reception mode”.

So the above Option 1 proposed by us in 110bis meeting starts from the above intention of the two optimization, and try to make the wording of “For UE not in multiple-Rx simultaneous reception mode” more clear. If company has concern on how to understand “work in single-Rx”, we believe the wording can be refined to 2) The UE is capable of R18 multi-Rx but work in normal mode.

Proposal 2: It should be clarified that“For UE not in multiple-Rx simultaneous reception mode” includes two cases: 1) The UE is not capable of R18 multi-Rx; 2) The UE is capable of R18 multi-Rx but work in normal mode currently.

- Exact solution

- During previous meetings, multiple solutions proposed by companies. We classify them into multiple types:

- Type 1: Only measure one serving carrier per band

- Type 2: Dynamic searcher sharing between PCC and SCC

- Type 3: Adaptation on 2*NSCC_CSIRS

- Type 4: Adaptation on the assumption of searcher number

- From our point of view, we are open to all types of optimization. It seems that Type 1 and Type 2 can be deemed as the starting point.

- Observation 1: Multiple solutions proposed by companies. We classify them into multiple types:

- Type 1: Only measure one serving carrier per band

- Type 2: Dynamic searcher sharing between PCC and SCC

- Type 3: Adaptation on 2*NSCC_CSIRS

- Type 4: Adaptation on the assumption of searcher number

- Proposal 3: Type 1 and Type 2 can be deemed as the starting point.

- Conclusion

In this contribution, we have the following proposals on the RRM enhancements:

Proposal 1: For the CSSF enhancement, CA, EN-DC, NE-DC and NR-DC are applicable scenarios.

Proposal 2: It should be clarified that“For UE not in multiple-Rx simultaneous reception mode” includes two cases: 1) The UE is not capable of R18 multi-Rx; 2) The UE is capable of R18 multi-Rx but work in normal mode currently.(highlight the refining part)

- Observation 1: Multiple solutions proposed by companies. We classify them into multiple types:

- Type 1: Only measure one serving carrier per band

- Type 2: Dynamic searcher sharing between PCC and SCC

- Type 3: Adaptation on 2*NSCC_CSIRS

- Type 4: Adaptation on the assumption of searcher number

- Proposal 3: Type 1 and Type 2 can be deemed as the starting point.

- Reference

- RP-240830 “New WID: WI resulting from discussion on RRM enhancements”, vivo

- R4-2404368 “Work plan for R19 RRM Phase 5 WI”, Apple, CATT

- R4-2406392 “WF for [110bis][230] NR_RRM_Ph5”, Apple