3GPP TSG-RAN WG4 Meeting #112	R4-2413081

# Maastricht, Netherlands, 19th – 23rd August, 2024

Source:	ZTE Corporation, Sanechips

Title:	Discussion on RRM aspects of R19 SBFD

Agenda Item:	8.19.3

Document for:	Discussion

- Introduction

In RAN plenary 102 meeting, a R19 WI on Sub-band full duplex (SBFD) was approved in [1]. We summarize the objectives of the WI as below:

The objectives are as follows: - For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier: - Specify semi-static indication of time location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2] - Indication of time location of SBFD subbands in SIB is not precluded - Specify semi-static indication of frequency domain location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2] - Indication of frequency domain location of SBFD subbands in SIB is not precluded - Specify SBFD operation to support random access in SBFD symbols by UEs in RRC CONNECTED mode [RAN1, RAN2] - Study and specify, if justified, SBFD operation to UE in RRC_IDLE/INACTIVE mode for random access [RAN1, RAN2] - RAN#104 to check whether to proceed normative work - Specify UE transmission, reception and measurement behavior and procedures in SBFD symbols and/or non-SBFD symbols for SBFD aware UE [RAN1, RAN2] - Transmission and reception behaviours on SBFD subbands configured in DL and/or flexible symbol indicated by TDD-UL-DL-ConfigCommon - UL transmissions within UL subband only - DL receptions within DL subband(s) only, except for CLI measurement by the UE outside of the DL subbands Note: When flexible symbols are used, it is not expected that any legacy Uplink symbol is converted to Downlink/SBFD symbols - Enhancement on resource allocation in frequency domain in SBFD symbols, including - resource allocation in frequency domain for PDSCH/CSI-RS across two DL subbands in SBFD symbols - handling of unaligned boundaries between SBFD subband(s) and RBG, CSI reporting subband, CSI-RS resource, PRG - Enhancements on physical channels/signals and procedure across SBFD symbols and non-SBFD symbols in different slots, where each transmission/reception within a slot has either all SBFD or all non-SBFD symbols, including - resource allocation in frequency domain for transmission or reception in SBFD symbols and non-SBFD symbols with different available frequency resource in different slots - CSI report of which associated CSI-RS instances occur in both SBFD symbols and non-SBFD symbols in different slots - Configurations for SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols, e.g., resources, frequency hopping parameters, UL power control parameters and/or beam/spatial relation - Collision handling between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol - Followings are assumed based on TR 38.858 - SBFD at the gNB side - Half duplex operation at the UE side - FR1 and FR2-1 - SBFD operation Option 4, i.e., both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs - Coexistence between non-SBFD aware UEs (including legacy UEs) and SBFD aware UEs in the cell operating SBFD at gNB side - SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies - One UL subband for SBFD operation in an SBFD symbol (excluding legacy UL symbol/slot) within a TDD carrier - Mechanisms for SBFD operation shall also consider the adjacent channel coexistence between two operators - Specify enhancements for CLI handling [RAN1, RAN2, RAN3]: - Support gNB-to-gNB co-channel CLI handling scheme(s) (the detailed schemes are to be down-selected from those in TR38.858 by RAN1#117) - Support UE-to-UE co-channel CLI handling scheme(s) (the detailed schemes are to be down-selected from those in TR38.858 by RAN1#117) - Note: Without dedicated optimization for dynamic/flexible TDD. - Specify BS RF requirements for SBFD operation at gNB [RAN4] - Specify applicable RRM core requirements for co-channel CLI handling mechanisms [RAN4] - Specify other RRM core requirements for SBFD operation, if identified [RAN4]

We highlight the objectives relevant to RAN4 RRM session. For the 1st RAN4 RRM aspects, it clearly shows that RAN4 should define the requirements for CLI measurement. Based on the investigation during SI phase and the RAN1 progress until RAN1#117 meeting in R19 phase, the CLI measurement contains UE-to-UE CLI and gNB-to-gNB CLI. The 2nd RAN4 RRM task is general, all RRM aspects regarding the SBFD operation should be identified and concluded.

In this paper, we provide some of our initial views on the SBFD operation. Since comprehensive discussion has held in SI phase, so the progress of SI and RAN1 progress in R19 would be the basis of RRM discussion. We develop our discussion from the following aspects:

- Scenario discussion

- UE-to-UE CLI

- gNB-to-gNB CLI

- Discussion

- 2.1 Scenario discussion

Based on the scope of R19 WI and the extensive investigation in SI[2], we can summarize some high level points regarding the applicable scenario of SBFD.

- The frequency and time resource of SBFD subband is semi-statically indicated to SBFD aware UE in CONNECTED mode, i.e. dynamic SBFD/TDD operation is not considered

- SBFD at gNB side, and half duplex operation at UE side

- Consider FR1 and FR2-1

- Coexistence between non-SBFD aware UEs and SBFD aware UEs in a cell at gNB side

- SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies

- The SBFD subband applicable to UL operation only locates in D or F symbol

- Includes both single-operator and two-operator applications

- Both macro and micro are considered, i.e. the homogeneous and heterogeneous network are considered

Observation 1: Some high level key points of the applicable scenario of R19 SBFD are listed as below:

- The frequency and time resource of SBFD subband is semi-statically indicated to SBFD aware UE in CONNECTED mode, i.e. dynamic SBFD/TDD operation is not considered

- SBFD at gNB side, and half duplex operation at UE side

- Consider FR1 and FR2-1

- Coexistence between non-SBFD aware UEs and SBFD aware UEs in a cell at gNB side

- SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies

- The SBFD subband applicable to UL operation only locates in D or F symbol

- Includes both single-operator and two-operator applications

- Both macro and micro are considered, i.e. the homogeneous and heterogeneous network are considered

- 2.2 UE-to-UE CLI

Background of R16 CLI

Before we diving into the discussion of R19 CLI, we can review the legacy R16 CLI measurement first. In legacy R16 CLI, only UE-to-UE CLI measurement was introduced to support dynamic TDD. Through performing the CLI-RSSI and/or SRS-RSRP measurement on the configured resource within active DL BWP, the potential victim UE can detect the interference to its DL Rx cased by the UL Tx of the aggressor UE. Both the SRS-RSRP and CLI-RSSI measurements are L3 measurement, L3 filtering allowed to the measurement results. RAN4 decided the measurement requirements and accuracy requirements for the two types of CLI resource.

When identifying the accuracy of SRS-RSRP measurement, the side condition of timing offset between UE’s DL reference timing in the serving cell and SRS arrival time was decided as Terror_SRS_RSRP, defined in clause 10.1.22.1 in [6].

Since no QCL-type D info was configured for the CLI resource in FR2, so the beam assumption of CLI resource is defined in [6] as the UE can assume the configured CLI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET.

In R19 SBFD, both UE-to-UE CLI and gNB-to-gNB CLI are allowed, as shows in Figure.1.

- Figure 1. Both gNB-to-gNB CLI and UE-to-UE CLI

In this clause, we focus on the UE-to-UE CLI.

Based on RAN1 progress until RAN1#117 meeting[3][4][5], the framework of UE-to-UE CLI is depicted from the following aspects.

- Measurement type

- The UE-to-UE CLI is L1 measurement, no longer L3 measurement, which is distinguished from legacy R16 CLI;

- Measurement resource

- The UE-to-UE CLI also contains SRS-RSRP measurement and CLI-RSSI measurement. The measurement resource could be periodic, semi-persistent or aperiodic resource (set). The SRS-RSRP measurement resource is based on the legacy RS pattern, no new pattern introduced.

- How to determine the QCL typeD assumption of the CLI resource, still FFS.

- Measurement reporting

- At least aperiodic reporting supported.

- The report quantities includes: L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices.

Observation 2: The R19 UE-to-UE CLI measurement is determined as L1 measurement, both SRS-RSRP measurement and CLI-RSSI measurement are allowed, the resource can be configured as periodic, semi-persistent, or aperiodic resource. At least aperiodic reporting supported.

Observation 3: How to determine the QCL typeD assumption of the CLI resource, still FFS.

- From RAN4 perspective, we analysis the UE-to-UE CLI from the following the following aspects:

- Measurement period requirements

- Accuracy requirements

- Side condition

- Measurement period requirements

- For UE-to-UE CLI measurement, currently there are 4 methods are on the table:

Agreement For SBFD aware UEs, CLI measurements is performed within the active DL BWP and the following can be considered - Method#1: UE measures RSSI within DL subband - Method#2: UE measures RSRP of aggressor UE within UL subband - Method#3: UE measures RSSI within UL subband - Method#4: UE measures RSSI within guard band, if guard band exists Note: If DL subband, UL subband or guard band is outside the active DL BWP, the above methods does not apply. Note: Method#4 does not imply that guard band is explicitly configured.

- Within the 4 methods, only Method#1 is direct measurement, by which the victim UE can directly detect the RSSI in the DL subband, the interference to the UE Rx in DL subband can be inferred. The other three are all indirect measurement, victim UE senses the RSRP/RSSI of the UL subband/guard band so as to estimate the interference leakage to the Rx operation on the DL subband. We believe the measurement period requirements under different methods should be discussed one by one.

Observation 4: Four types of UE-to-UE CLI measurements are on the table, including both direct measurement on DL subband and indirect measurement on UL subband/guard band.

Proposal 1: The measurement period requirements for the four candidate UE-to-UE CLI measurements should be discussed one by one.

- For Method#1: UE measures RSSI within DL subband

- Configure RSSI resource in DL subband.

- The measurement period requirements can reuse legacy CLI-RSSI, i.e. one-shot measurement.

- For Method#2: UE measures RSRP of aggressor UE within UL subband

- Configure SRS-RSRP resource in UL subband.

- L1 measurement, so 1 or 3 samples, need to evaluate the accuracy performance

- For Method#3: UE measures RSSI within UL subband

- Configure RSSI resource in UL subband.

- The measurement period requirements can reuse legacy CLI-RSSI, i.e. one-shot measurement.

- UE needs post-operation based on this measurement result, to estimate the interference leakage at the DL subband. Whether to specify this post-operation at UE side?

- For Method#4: UE measures RSSI within guard band, if guard band exists

- Configure RSSI resource in guard band.

- Similar as Method#3.

- Accuracy requirements

- After the requirements of measurement period identified, than RAN4 can determine the accuracy requirements. Since the R19 UE-to-UE CLI is no longer L3 measurement, so the legacy UE-to-UE CLI accuracy requirements can not be reused directly. Some adaptation to facilitate L1 measurement should be considered.

Proposal 2: The R19 UE-to-UE CLI is L1 measurement, instead of L3 measurement, so the legacy UE-to-UE CLI accuracy requirements can not be reused directly. Some adaptation to facilitate L1 measurement should be considered.

- Side condition

- For Method#2, we believe the following side conditions are need to considered:

- Side condition of time offset between UE’s DL reference timing in the serving cell and SRS arrival time. We can not see the obvious difference between this case and the legacy R16 SRS-RSRP CLI measurement, so it is preferred to reuse the legacy time offset condition.

- Side condition of SINR. Since this is not direct interference measurement, when victim gNB obtain such interference in UL subband, the victim gNB still needs to estimate the leakage in DL subband, so how to define the SINR side condition should be determined. Technically the side condition can be relaxed compared with legacy, but how to relax, more discussion and study is needed.

- Side condition of maximum/minimum RSRP, to our understanding, which is similar as the SINR.

Proposal 3: The side condition for Method#2 should be considered from the following aspects:

- Side condition of time offset between UE’s DL reference timing in the serving cell and SRS arrival time

- Side condition of SINR

- Side condition of maximum/minimum RSRP

- 2.3 gNB-to-gNB CLI

The gNB-to-gNB CLI was studied during SI phase with the following general summary in [2]:

Measurement Resource, Performance Matric and Relevant information exchange In the study for gNB-to-gNB co-channel CLI measurement, it is considered as baseline to reuse existing DL channel(s)/signal(s)/measurement resource(s), for example, SSB, NZP/ZP-CSI-RS, DMRS for PDCCH/PDSCH, CSI-IM, RSSI measurement resource, etc. In the study, beam level (i.e., based on measurement result per SSB resource and/or per CSI-RS resource) CLI measurement can be considered. - In the study, RAN1 assumed that exchange of configuration for NZP CSI-RS /SSB can be an enabler for gNB-to-gNB CLI measurement and/or channel measurement.

The original motivation of gNB-to-gNB CLI is to estimate the interference leakage to the UL subband of victim gNB when it performs Rx due to the aggressor gNB Tx at the adjacent DL subband. While during the RAN1 discussion in R19, some spatial domain enhancements and power control scheme were touched. Until RAN1#117 meeting, only the scheme of gNB Tx power control was excluded, other schemes are still on the table, down-selection may be needed.

- gNB-to-gNB co-channel CLI and/or channel measurements

- Spatial domain based schemes

- Beam nulling

- Beam pairing

- Coordinated scheduling in time and/or frequency

- Power control based schemes

- UE Tx power control

To our understanding, the basic scheme of gNB-to-gNB CLI is similar as gNB-to-gNB CLI, aim to estimate the impact of interference leakage to the victim gNB UL subband, the aggressor gNB sends CSI-RS or SSB at the DL subband. Then victim gNB measures such gNB-to-gNB CLI resource at DL subband to further derive the interference leakage to the adjacent UL subband. During the whole procedure, RAN4 RRM could focus on the following aspects:

- The measurement purpose and measurement metric

- To derive the interference leakage to the adjacent UL subband, the victim gNB measures the CSI-RS or SSB resource to obtain the RSRP or RSSI.

- Besides, so as to perform some spatial domain scheme such as beam nulling, the victim gNB still need to estimate the channel info between the aggressor gNB to victim gNB, and report such channel info to aggressor gNB so that the aggressor gNB could determine the precoding with the consideration of zero impact to the victim gNB.

- Another spatial domain scheme is beam pairing, which is a bit different from beam nulling. The purpose of beam pairing is to guarantee the the precoding of two links as much as orthogonal, the 1st link is from aggressor gNB to UE1, the 2nd link is from UE2 to victim gNB. Such orthogonality could guarantee almost zero interference from aggressor gNB Tx to victim gNB Rx.

Observation 5: One main motivation of gNB-to-gNB CLI measurement is to help the victim gNB to derive the interference leakage to the UL subband since the aggressor gNB Tx at the DL subband. Besides, some spatial domain scheme could reduce or eliminate the interference.

- Based on the above motivation and schemes for gNB-to-gNB CLI, we can classify the gNB-to-gNB CLI measurement into the following types:

- Type 1：Victim gNB measures the CLI resource to derive the channel info from aggressor gNB to victim gNB

- Type 2：Aggressor gNB measures the CLI resource and based on the reciprocity to derive the channel info from aggressor gNB to victim gNB

- Type 3：Victim gNB measures the CLI resource to obtain the RSRP/RSSI/SINR of the link from aggressor gNB to victim gNB

- Type 4：Victim gNB measures the CLI resource to obtain the strongest/poorest beam of the link from aggressor gNB to victim gNB

- Regarding Type 1, 2, the candidate measurement resource includes SSB, CSI-RS and DMRS. The measurement metrics is the channel matrix H.

- For Type 3, the candidate measurement resource includes SSB or CSI-RS. Instead to estimate the channel matrix itself, the candidate measurement metrics includes RSRP, RSSI and SINR. Such type of measurement is similar as UE-to-UE CLI measurement.

- For Type 4, similar as legacy BM measurement, so the candidate measurement resource includes SSB or CSI-RS, and the measurement metrics could be RSRP. The report quantity could be RSRP and the relevant RS index. The victim gNB could report the strongest/poorest beam info to the aggressor gNB, when the aggressor gNB schedules its serving UEs, some interference avoidance strategy can be considered.

Observation 6: Based on all candidate schemes of gNB-to-gNB CLI, four types of measurements could be considered with different measurement purpose:

- Type 1：Victim gNB measures the CLI resource to derive the channel info from aggressor gNB to victim gNB

- Type 2：Aggressor gNB measures the CLI resource and based on the reciprocity to derive the channel info from aggressor gNB to victim gNB

- Type 3：Victim gNB measures the CLI resource to obtain the RSRP/RSSI/SINR of the link from aggressor gNB to victim gNB

- Type 4：Victim gNB measures the CLI resource to obtain the strongest/poorest beam of the link from aggressor gNB to victim gNB

- L3 measurement or L1 measurement

- It has been determined that the UE-to-UE CLI measurement in R19 is L1 measurement. For gNB-to-gNB CLI measurement, we do not have strong view to maintain L3 measurement or degrade to L1 measurement. Within all listed measurement type above, Type 1 and 2 are special, the channel matrix H is measured in the two types, it is hard to define as L3 or L1 measurement. While for the Type 3 and 4, both L3 or L1 are fine. From the perspective of NW, it would always transmit signal even though to different UE, so the interference would be stable relatively compared with the UE-to-UE interference. So L1 measurement is preferred. Furthermore, L1 based gNB-to-gNB CLI measurement is aligned with the already decided L1 based UE-to-UE CLI measurement.

Proposal 4: L1 based gNB-to-gNB CLI measurement is preferred due to the relatively stable interference circumstance. If channel matrix measurement allowed, need to discuss how to define the measurement type.

- How to ensure the measurement resource are non-overlapping between multiple aggressors

- For Type 1, 2, 4, it is better to use non-overlapping resource between multiple aggressor gNBs. The victim gNB can be configured with multiple SSB/CSI-RS resources, but who undertakes the configuration task? It seems some interaction between gNBs is needed. The multiple SSB/CSI-RS resources should be non-overlapping, so it is better to use NCD-SSB since the CD-SSB from multiple NBs may overlapping.

- For Type 3, if the gNB-to-gNB CLI measurement is only used for the victim gNB scheduling decision internally, no interaction with the aggressor gNBs, then overlapping CLI resource is preferred since the victim gNB needs to figure out the whole interference situation.

Observation 7: One victim gNB may interfered by multiple aggressors. So who undertakes the gNB-to-gNB CLI resource configuration task should be considered. Whether allow the interaction between gNBs should be determined, so wait for RAN3 conclusions.

Observation 8: Within all four types of measurement for gNB-to-gNB CLI, the Type 3 is not sensitive to single aggressor or multiple aggressors. While Type 1, 2, 4 are sensitive to single aggressor or multiple aggressors due to non-overlapping CLI resource are needed for multiple aggressors case.

- Side condition

- Similar as our analysis for UE-to-UE CLI, the side conditions including time offset, SINR/RSRP should be considered.

For Type 1, 3, the timing between receiving CLI SSB/CSI-RS and receiving normal UL signal may not aligned at the victim gNB, so how to track the two timing should be studied. The victim NB would receive UL transmission from its serving UE and CLI RS from aggressor gNB. So the time offset between the victim gNB receiving UL transmission and receiving aggressor gNB CLI resource is Tprop+NTA,offset*Tc.

If victim gNB and aggressor gNB share different NTA,offset, here the value of NTA,offset should follow the value of aggressor gNB. The component of Tprop is the propagation delay from aggressor NB to victim NB.  it depends on the ISD. For 500m ISD(UMA), if the two NBs’ separation is 2*ISD, propagation delay is about 3.33us.

- Conclusion

In this contribution, we have the following proposals on the RRM enhancements:

Observation 1: Some high level key points of the applicable scenario of R19 SBFD are listed as below:

- The frequency and time resource of SBFD subband is semi-statically indicated to SBFD aware UE in CONNECTED mode, i.e. dynamic SBFD/TDD operation is not considered

- SBFD at gNB side, and half duplex operation at UE side

- Consider FR1 and FR2-1

- Coexistence between non-SBFD aware UEs and SBFD aware UEs in a cell at gNB side

- SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies

- The SBFD subband applicable to UL operation only locates in D or F symbol

- Includes both single-operator and two-operator applications

- Both macro and micro are considered, i.e. the homogeneous and heterogeneous network are considered

Observation 2: The R19 UE-to-UE CLI measurement is determined as L1 measurement, both SRS-RSRP measurement and CLI-RSSI measurement are allowed, the resource can be configured as periodic, semi-persistent, or aperiodic resource. At least aperiodic reporting supported.

Observation 3: How to determine the QCL typeD assumption of the CLI resource, still FFS.

Observation 4: Four types of UE-to-UE CLI measurements are on the table, including both direct measurement on DL subband and indirect measurement on UL subband/guard band.

Proposal 1: The measurement period requirements for the four candidate UE-to-UE CLI measurements should be discussed one by one.

Proposal 2: The R19 UE-to-UE CLI is L1 measurement, instead of L3 measurement, so the legacy UE-to-UE CLI accuracy requirements can not be reused directly. Some adaptation to facilitate L1 measurement should be considered.

Proposal 3: The side condition for Method#2 should be considered from the following aspects:

- Side condition of time offset between UE’s DL reference timing in the serving cell and SRS arrival time

- Side condition of SINR

- Side condition of maximum/minimum RSRP

Observation 5: One main motivation of gNB-to-gNB CLI measurement is to help the victim gNB to derive the interference leakage to the UL subband since the aggressor gNB Tx at the DL subband. Besides, some spatial domain scheme could reduce or eliminate the interference.

Observation 6: Based on all candidate schemes of gNB-to-gNB CLI, four types of measurements could be considered with different measurement purpose:

- Type 1：Victim gNB measures the CLI resource to derive the channel info from aggressor gNB to victim gNB

- Type 2：Aggressor gNB measures the CLI resource and based on the reciprocity to derive the channel info from aggressor gNB to victim gNB

- Type 3：Victim gNB measures the CLI resource to obtain the RSRP/RSSI/SINR of the link from aggressor gNB to victim gNB

- Type 4：Victim gNB measures the CLI resource to obtain the strongest/poorest beam of the link from aggressor gNB to victim gNB

Proposal 4: L1 based gNB-to-gNB CLI measurement is preferred due to the relatively stable interference circumstance. If channel matrix measurement allowed, need to discuss how to define the measurement type.

Observation 7: One victim gNB may interfered by multiple aggressors. So who undertakes the gNB-to-gNB CLI resource configuration task should be considered. Whether allow the interaction between gNBs should be determined, so wait for RAN3 conclusions.

Observation 8: Within all four types of measurement for gNB-to-gNB CLI, the Type 3 is not sensitive to single aggressor or multiple aggressors. While Type 1, 2, 4 are sensitive to single aggressor or multiple aggressors due to non-overlapping CLI resource are needed for multiple aggressors case.

- Reference

- RP-234035, “New WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, 3GPP RAN#102, 2023.

- 3GPP TR 38.858_i10, Study on Evolution of NR Duplex Operation, (Release 18).

- Final_Minutes_report_RAN1#116_v100

- Final_Minutes_report_RAN1#116b_v100

- Final_Minutes_report_RAN1#117_v020

- TS 38.133_i50.