3GPP TSG-RAN WG4 Meeting #112	R4-2413124

Maastricht, Netherlands, Aug 19 – Aug 23, 2024

Source:	Nokia, Nokia Shanghai Bell

Title:	Considerations about neighbor cell satellite measurements

Agenda item:	5.30.2

Document for:	Discussion

Introduction

Enhanced RRM aspects for NB-IoT/eMTC operation were introduced in the IoT_NTN_enh WI [1], which included mobility enhancements and other aspects related to the GNSS operation. The work related to the core part of RRM requirements was finalized in RAN4 #109 [2], and most of the issues were addressed in previous discussions or in maintenance discussions of the same WI.

In a previous meeting, companies have discussed some aspects that were left for further discussion. In this document we keep the discussion of these maintenance aspects that are relevant for UE smooth operation in the network.

Our contribution focuses on:

- UE measurement behavior after very long GNSS-MGs

- Neighbor cell measurements right before t-serviceStart.

Discussion

UE measurement behavior after very long GNSS-MGs

RAN2 has recently decided to fasten the procedures for the UE to land on a different cell, when t-service expires and the current serving cell is not available anymore:

Agreements on RAN2 #122 2. If the serving cell t-service expires, stop T310 (if running) and start T311 (i.e. perform cell search and re-establishment without attempting to recover on the current cell for the duration of T310). FFS on discontinuous coverage

The idea from RAN2 is that the UE can speed up the process by knowing that the current serving cell is not available after t-service.

Regardless of whether the conditions of the serving cell right before t-service are good or bad, the UE knows (by the configuration of t-service) that the serving cell will disappear at t-service. Based on the average scenario it is even more likely that the conditions on the serving cell are good before the cell is discontinued.  Any last effort to collect serving cell measurements are not unnecessary, but it might hurt the timely cell reselection. If that’s the case, the UE will not perform cell reselection upon t-service, as the UE will rank the serving cell as very high. Only Tevaluate seconds after the last sample before t-service the UE will be required to perform the cell reselection.

Even more problematic: if the UE measures other cells (neighbor cells) associated to the same serving satellite, but also configured to disappear at t-service (for example, inter-frequency cells), the UE might decide to reselect to one of those cells right before t-service, which is inefficient as a new reselection procedure will take place when t-service is reached.

Cell reselection for those cells associated to the same serving satellite should not be allowed when t-service is approaching, as it can cause the problem described above.

When a UE starts inter-frequency neighbor cell measurements based on time-based measurement initiation (e.g. Ttrigger before t-service), and the inter-frequency neighbor cells associated to the same satellite are also configured with t-Service, then the UE is not required to measure these cells.

When t-service is reached, and the UE has initiated measurements based on time-based measurement initiation, the UE shall immediately perform cell reselection ignoring any previous measurement on the serving cell.

Conclusion

The paper has discussed the work item on enhancements of IoT/eMTC NTN deployments in special in what regards mobility aspects. The following observations and proposals are presented as an outcome of the discussions above:

Proposal 1: When a UE starts inter-frequency neighbor cell measurements based on time-based measurement initiation (e.g. Ttrigger before t-service), and the inter-frequency neighbor cells associated to the same satellite are also configured with t-Service, then the UE is not required to measure these cells.

Proposal 2: When t-service is reached, and the UE has initiated measurements based on time-based measurement initiation, the UE shall immediately perform cell reselection ignoring any previous measurement on the serving cell.

- References

- RP-220979– Revised WID on IoT NTN enhancements, Mediatek, RAN 95, Electronic Meeting, March 17 - 23, 2022

- R4-2321577, WF on R18 IoT NTN RRM requirements, Mediatek, RAN WG4 #109, Chicago, US, November 13 – 17, 2023

- R4-2405754, Maintenace on mobility requirements for IoT NTN enhancements, Nokia, RAN4 #110bis, Changsha, China, April 15 – April 19, 2024