3GPP TSG-WG4 Meeting #112	R4-2413127

# Maastricht, Netherland, Aug 19th – 23th, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR | 1297 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clean up of power class indication for DC configurations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-Core |  |  |  |  |  | Date: |  |  | 2024-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Note 23 indicated for some EN-DC configurations means that the minimum requirements for PC2 are applicable for this uplink EN-DC configuration, however, the superscipts also marked for the first column. No clear rule for power class relevant superscript for the 1st column, which is not supposed to have such superscript. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove superscripts of note 23 in first colum in the table for some band combination configurations, as the superscripts are power class relevant. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguity exists in the specificication. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5B.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-3 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change>

5.5B.4.1	Inter-band EN-DC configurations within FR1 (two bands)

Table 5.5B.4.1-1: Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed | DL interruption allowed (Note 14) |
| --- | --- | --- | --- |
| DC_1A_n3A DC_1C_n3A | DC_1A_n3A DC_1C_n3A | DC_1_n3 |  |
| DC_1A_n5A | DC_1A_n5A | No |  |
| DC_1A_n7A DC_1A_n7B | DC_1A_n7A DC_1A_n7B | No |  |
| DC_1A-1A_n7A DC_1A-1A_n7B | DC_1A_n7A | No |  |
| DC_1A_n8A | DC_1A_n8A | No |  |
| DC_1A_n20A | DC_1A_n20A | No |  |
| DC_1A_n28A | DC_1A_n28A | No |  |
| DC_1A_n26A | DC_1A_n26A | No |  |
| DC_1A-1A_n28A | DC_1A_n28A | No |  |
| DC_1A_n38A DC_1C_n38A | DC_1A_n38A | No |  |
| DC_1A_n40A DC_1A_n40B | DC_1A_n40A | No |  |
| DC_1A_n41A7 | DC_1A_n41A | No |  |
| DC_1A_n50A | DC_1A_n50A | No |  |
| DC_1A_n51A | DC_1A_n51A | No |  |
| DC_1A_n71A DC_1A_n71B | DC_1A_n71A | No |  |
| DC_1A_n77A7 DC_1A_n77C7 | DC_1A_n77A | DC_1_n77 | No |
| DC_1A_n77(2A)7,21 DC_1A_n77(3A)7 | DC_1A_n77A21 | DC_1_n77 | No |
| DC_1A_n78A7 DC_1A_n78C7, 21 | DC_1A_n78A21 | No | No |
| DC_1A_n78(2A)7,21 DC_1A_n78(A-C)7 | DC_1A_n78A21 | No | No |
| DC_1A-1A_n78A | DC_1A_n78A | No | No |
| DC_1A_n79A7 DC_1A_n79C7 | DC_1A_n79A | No | No |
| DC_1A_n105A | DC_1A_n105A | No |  |
| DC_2A_n5A | DC_2A_n5A | No |  |
| DC_2A-2A_n5A | DC_2A_n5A | No |  |
| DC_2A_n7A DC_2C_n7A | DC_2A_n7A | No |  |
| DC_2A_n7(2A) | DC_2A_n7A | No |  |
| DC_2A-2A_n7A | DC_2A_n7A | No |  |
| DC_2A_n12A | DC_2A_n12A | No |  |
| DC_2A-2A_n12A | DC_2A_n12A | No |  |
| DC_2A_n25A11, 13, 20 | N/A | N/A |  |
| DC_2A_n28A DC_2C_n28A | DC_2A_n28A | No |  |
| DC_2A_n30A | DC_2A_n30A | No |  |
| DC_2A-2A_n30A | DC_2A_n30A | No |  |
| DC_2A_n38A | DC_2A_n38A | No |  |
| DC_2A-2A_n38A | DC_2A_n38A | No |  |
| DC_2A_n41A DC_2A_n41C DC_2C_n41A | DC_2A_n41A DC_2C_n41A | No |  |
| DC_2A_n41(2A) | DC_2A_n41A | No |  |
| DC_2A-2A_n41A | DC_2A_n41A | No |  |
| DC_2A_n46A | DC_2A_n46A | No |  |
| DC_2A_n48A DC_2A_n48B | DC_2A_n48A | No |  |
| DC_2A_n66A | DC_2A_n66A | DC_2_n66 |  |
| DC_2A_n66(2A) | DC_2A_n66A | DC_2_n66 |  |
| DC_2A-2A_n66A | DC_2A_n66A | DC_2_n66 |  |
| DC_2A_n71A DC_2A_n71B DC_2C_n71A | DC_2A_n71A | No |  |
| DC_2A-2A_n71A | DC_2A_n71A | No |  |
| DC_2A_n77A DC_2A_n77C21 | DC_2A_n77A21 | DC_2_n77 |  |
| DC_2A_n77(2A)21 | DC_2A_n77A21 | DC_2_n77 |  |
| DC_2A-2A_n77A21 DC_2A-2A_n77C21 | DC_2A_n77A21 | DC_2_n77 |  |
| DC_2A-2A_n77(2A)21 | DC_2A_n77A21 | DC_2_n77 |  |
| DC_2A_n78A | DC_2A_n78A | DC_2_n78 |  |
| DC_2A-2A_n78(2A) | DC_2A_n78A | DC_2_n78 |  |
| DC_2A_n78(2A)21 | DC_2A_n78A21 | DC_2_n78 |  |
| DC_2A-2A_n78A | DC_2A_n78A | DC_2_n78 |  |
| DC_3A_n1A DC_3C_n1A | DC_3A_n1A DC_3C_n1A | DC_3_n1 |  |
| DC_3A-3A_n1A | DC_3A_n1A | DC_3_n1 |  |
| DC_3A_n5A DC_3C_n5A | DC_3A_n5A | DC_3_n5 |  |
| DC_3A_n7A DC_3A_n7B DC_3C_n7A DC_3C_n7B | DC_3A_n7A DC_3A_n7B DC_3C_n7A | No |  |
| DC_3A-3A_n7A DC_3A-3A_n7B | DC_3A_n7A | No |  |
| DC_3A_n8A | DC_3A_n8A | No |  |
| DC_3A-3A_n8A | DC_3A_n8A | No |  |
| DC_3A_n20A DC_3C_n20A | DC_3A_n20A | No |  |
| DC_3A_n26A DC_3C_n26A | DC_3A_n26A DC_3C_n26A | Yes |  |
| DC_3A_n28A DC_3C_n28A | DC_3A_n28A DC_3C_n28A | No |  |
| DC_3A_n34A | DC_3A_n34A | No |  |
| DC_3A_n38A DC_3C_n38A | DC_3A_n38A | No |  |
| DC_3A_n40A DC_3A_n40B DC_3C_n40A | DC_3A_n40A | No |  |
| DC_3A_n41A7 DC_3A_n41C DC_3C_n41A7 | DC_3A_n41A DC_3C_n41A | DC_3_n41 | No |
| DC_3A_n50A | DC_3A_n50A | No |  |
| DC_3A_n51A | DC_3A_n51A | No |  |
| DC_3A_n71A DC_3A_n71B | DC_3A_n71A | No |  |
| DC_3A_n77A7 DC_3A_n77C7 DC_3C_n77A7,21 | DC_3A_n77A21 DC_3C_n77A | DC_3_n77 | No |
| DC_3A_n77(2A)7,21 DC_3A_n77(3A)7 DC_3C_n77(2A)7,21 | DC_3A_n77A,21 DC_3C_n77A | DC_3_n77 | No |
| DC_3A-3A_n77A7 | DC_3A_n77A | DC_3_n77 | No |
| DC_3A_n78A7 DC_3A_n78C7 DC_3C_n78A7,21 | DC_3A_n78A,21,23 DC_3C_n78A | DC_3_n78 | No |
| DC_3A_n78(2A)7,21 DC_3A_n78(A-C)7 DC_3C_n78(2A)7,21 | DC_3A_n78A,21 DC_3C_n78A | DC_3_n78 | No |
| DC_3A-3A_n78A7, 21 | DC_3A_n78A21 | DC_3_n78 | No |
| DC_3A_n79A7 DC_3A_n79C7 DC_3C_n79A7 | DC_3A_n79A DC_3C_n79A | No | No |
| DC_3A-3A_n79A7 | DC_3A_n79A | No |  |
| DC_3A_n105A | DC_3A_n105A | No |  |
| DC_4A_n2A | DC_4A_n2A | No |  |
| DC_4A_n5A | DC_4A_n5A | DC_4_n5 |  |
| DC_4A_n7A | DC_4A_n7A | No |  |
| DC_4A_n28A | DC_4A_n28A | No |  |
| DC_4A_n38A | DC_4A_n38A | No |  |
| DC_4A_n41A | DC_4A_n41A | No |  |
| DC_4A_n78A | DC_4A_n78A | No |  |
| DC_4A_n78(2A) | DC_4A_n78A | No |  |
| DC_5A_n1A | DC_5A_n1A | No |  |
| DC_5A_n2A DC_5B_n2A | DC_5A_n2A | No |  |
| DC_5A_n2(2A) | DC_5A_n2A | No |  |
| DC_5A-5A_n2A | DC_5A_n2A | No |  |
| DC_5A_n3A | DC_5A_n3A | DC_5_n3 |  |
| DC_5A_n7A | DC_5A_n7A | DC_5_n7 |  |
| DC_5A_n7(2A) | DC_5A_n7A | DC_5_n7 |  |
| DC_5A_n12A | DC_5A_n12A | No |  |
| DC_5A_n25A | DC_5A_n25A | No |  |
| DC_5A_n28A | DC_5A_n28A | No |  |
| DC_5A_n30A | DC_5A_n30A | No |  |
| DC_5A_n38A | DC_5A_n38A | DC_5_n38 |  |
| DC_5A_n40A | DC_5A_n40A | No |  |
| DC_5A_n41A | DC_5A_n41A | No |  |
| DC_5A_n48A DC_5A_n48B | DC_5A_n48A | No |  |
| DC_5A_n66A DC_5B_n66A | DC_5A_n66A | DC_5_n66 |  |
| DC_5A-5A_n66A | DC_5A_n66A | DC_5_n66 |  |
| DC_5A_n77A DC_5A_n77C21 | DC_5A_n77A21 | No |  |
| DC_5A_n77(2A)21 DC_5A_n77(3A) | DC_5A_n77A21 | No |  |
| DC_5A_n71A | DC_5A_n71A | No |  |
| DC_5A_n78A7 DC_5A_n78C7 | DC_5A_n78A | No | No |
| DC_5A_n78(2A)7,21 DC_5A_n78(A-C)7 | DC_5A_n78A21 | No | No |
| DC_5A_n79A | DC_5A_n79A | No | No |
| DC_7A_n1A DC_7C_n1A | DC_7A_n1A DC_7C_n1A | No |  |
| DC_7A-7A_n1A | DC_7A_n1A | No |  |
| DC_7A_n2A DC_7C_n2A | DC_7A_n2A | No |  |
| DC_7A_n2(2A) | DC_7A_n2A | No |  |
| DC_7A_n3A DC_7C_n3A | DC_7A_n3A DC_7C_n3A | No |  |
| DC_7A_n5A DC_7C_n5A | DC_7A_n5A DC_7C_n5A | DC_7_n5 |  |
| DC_7A-7A_n5A | DC_7A_n5A | DC_7_n5 |  |
| DC_7A_n8A | DC_7A_n8A | No |  |
| DC_7A-7A_n8A | DC_7A_n8A | No |  |
| DC_7A_n12A | DC_7A_n12A | No |  |
| DC_7A-7A_n78(2A)7,21 | DC_7A_n78A21 | No |  |
| DC_7A_n20A | DC_7A_n20A | No |  |
| DC_7A_n25A DC_7C_n25A | DC_7A_n25A | No |  |
| DC_7A_n26A DC_7C_n26A | DC_7A_n26A DC_7C_n26A | Yes |  |
| DC_7A-7A_n25A | DC_7A_n25A | No |  |
| DC_7A_n28A DC_7C_n28A | DC_7A_n28A DC_7C_n28A | No |  |
| DC_7A_n40A | DC_7A_n40A | Yes |  |
| DC_7A-7A_n40A | DC_7A_n40A | Yes |  |
| DC_7A-7A_n28A | DC_7A_n28A | No |  |
| DC_7A_n51A | DC_7A_n51A | No |  |
| DC_7A_n66A DC_7C_n66A | DC_7A_n66A | No |  |
| DC_7A-7A_n66A | DC_7A_n66A | No |  |
| DC_7A_n71A | DC_7A_n71A | No |  |
| DC_7A_n77A7 DC_7C_n77A | DC_7A_n77A | No |  |
| DC_7A_n77(2A) DC_7A_n77(3A) DC_7C_n77(2A) | DC_7A_n77A | No |  |
| DC_7A-7A_n77A7 | DC_7A_n77A | No |  |
| DC_7A-7A_n77(2A) DC_7A-7A_n77(3A) | DC_7A_n77A | No |  |
| DC_7A_n78A7 DC_7C_n78A7,21 DC_7A_n78C7 | DC_7A_n78A21,23 DC_7C_n78A | No |  |
| DC_7A_n78(2A)7,21 DC_7A_n78(A-C)7 DC_7C_n78(2A)7, 21 | DC_7A_n78A21 DC_7C_n78A | No |  |
| DC_7A-7A_n78A7, 21 DC_7A-7A_n78C7 | DC_7A_n78A21 | No |  |
| DC_7A-7A_n78(A-C)7 | DC_7A_n78A | No |  |
| DC_7A_n79A DC_7A_n79C | DC_7A_n79A | No |  |
| DC_7A-7A_n79A | DC_7A_n79A | No |  |
| DC_7A_n105A | DC_7A_n105A | No |  |
| DC_8A_n1A DC_8B_n1A | DC_8A_n1A DC_8B_n1A | No |  |
| DC_8A_n2A | DC_8A_n2A | DC_8_n2 |  |
| DC_8A_n3A DC_8B_n3A | DC_8A_n3A | No |  |
| DC_8A_n7A | DC_8A_n7A | No |  |
| DC_8A_n20A | DC_8A_n20A | Yes |  |
| DC_8A_n28A | DC_8A_n28A | No |  |
| DC_8A_n34A | DC_8A_n34A | No |  |
| DC_8A_n38A | DC_8A_n38A | No |  |
| DC_8A_n39A | DC_8A_n39A | No |  |
| DC_8A_n40A7 | DC_8A_n40A | No |  |
| DC_8A_n41A7 DC_8A_n41C | DC_8A_n41A | No | No |
| DC_8A_n41(2A) | DC_8A_n41A | No | No |
| DC_8A_n77A7 DC_8B_n77A7 | DC_8A_n77A | No | No |
| DC_8A_n77(2A)7,21 DC_8B_n77(2A)7 DC_8A_n77(3A)7 | DC_8A_n77A21 | No | No |
| DC_8A_n78A7 DC_8B_n78A7, 21 | DC_8A_n78A21,23 DC_8B_n78A | No | No |
| DC_8A_n78(2A)7, 21 | DC_8A_n78A21 | No | No |
| DC_8A_n79A7 DC_8A_n79C | DC_8A_n79A DC_8A_n79C | No | No |
| DC_8A_n93A | DC_8A_n93A_ULSUP-TDM | N/A |  |
| DC_8A_n94A | DC_8A_n94A_ULSUP-TDM | N/A |  |
| DC_11A_n1A | DC_11A_n1A | No |  |
| DC_11A_n3A | DC_11A_n3A | No |  |
| DC_11A_n28A | DC_11A_n28A | No |  |
| DC_11A_n41A7 | DC_11A_n41A | No |  |
| DC_11A_n77A7 | DC_11A_n77A | No | No |
| DC_11A_n77(2A)7 DC_11A_n77(3A)7 | DC_11A_n77A | No | No |
| DC_11A_n78A7 | DC_11A_n78A | No | No |
| DC_11A_n78(2A) | DC_11A_n78A | No | No |
| DC_11A_n79A7 | DC_11A_n79A | No |  |
| DC_12A_n2A | DC_12A_n2A | No |  |
| DC_12A_n2(2A) | DC_12A_n2A | No |  |
| DC_12A_n5A | DC_12A_n5A | No |  |
| DC_12A_n7A | DC_12A_n7A | No |  |
| DC_12A_n7(2A) | DC_12A_n7A | No |  |
| DC_12A_n25A | DC_12A_n25A | No |  |
| DC_12A_n30A | DC_12A_n30A | No |  |
| DC_12A_n38A | DC_12A_n38A | No |  |
| DC_12A_n41A | DC_12A_n41A | No |  |
| DC_12A_n66A | DC_12A_n66A | No |  |
| DC_12A_n66(2A) | DC_12A_n66A | No |  |
| DC_12A_n71A | DC_12A_n71A18,19 | DC_12_n71 |  |
| DC_12A_n77A | DC_12A_n77A | DC_12_n77 |  |
| DC_12A_n77(2A)21 | DC_12A_n77A21 | DC_12_n77 |  |
| DC_12A_n78A | DC_12A_n78A | DC_12_n78 |  |
| DC_12A_n78(2A) | DC_12A_n78A | DC_12_n78 |  |
| DC_13A_n2A | DC_13A_n2A | No |  |
| DC_13A_n5A | DC_13A_n5A | DC_13_n5 |  |
| DC_13A_n7A | DC_13A_n7A | No |  |
| DC_13A_n7(2A) | DC_13A_n7A | No |  |
| DC_13A_n25A | DC_13A_n25A | No |  |
| DC_13A_n48A DC_13A_n48B | DC_13A_n48A | No |  |
| DC_13A_n66A | DC_13A_n66A | No |  |
| DC_13A_n71A | DC_13A_n71A | No |  |
| DC_13A_n77A DC_13A_n77C21 | DC_13A_n77A21 | No |  |
| DC_13A_n78A | DC_13A_n78A | No |  |
| DC_13A_n78(2A)21 | DC_13A_n78A21 | No |  |
| DC_14A_n2A | DC_14A_n2A | No |  |
| DC_14A_n5A | DC_14A_n5A | DC_14_n5 |  |
| DC_14A_n30A | DC_14A_n30A | No |  |
| DC_14A_n41A | DC_14A_n41A | No |  |
| DC_14A_n66A | DC_14A_n66A | No |  |
| DC_14A_n77A | DC_14A_n77A | No |  |
| DC_14A_n77(2A)21 | DC_14A_n77A21 | No |  |
| DC_18A_n3A | DC_18A_n3A | No |  |
| DC_18A_n28A | DC_18A_n28A | No |  |
| DC_18A_n41A16 | DC_18A_n41A | No |  |
| DC_18A_n77A7 DC_18A_n77(2A)7 | DC_18A_n77A | No | No |
| DC_18A_n78A7 | DC_18A_n78A | No | No |
| DC_18A_n78(2A)7 | DC_18A_n78A | No | No |
| DC_20A_n91A | DC_20A_n91A_ULSUP-TDM | N/A |  |
| DC_20A_n92A | DC_20A_n92A_ULSUP-TDM | N/A |  |
| DC_18A_n79A7 | DC_18A_n79A | No |  |
| DC_19A_n1A | DC_19A_n1A | No |  |
| DC_19A_n77A7 DC_19A_n77C7 | DC_19A_n77A | No |  |
| DC_19A_n77(2A)7, 21 | DC_19A_n77A21 | No |  |
| DC_19A_n78A7 DC_19A_n78C7 | DC_19A_n78A | No | No |
| DC_19A_n78(2A)7, 21 | DC_19A_n78A21 | No | No |
| DC_19A_n79A7 DC_19A_n79C7 | DC_19A_n79A | No | No |
| DC_20A_n1A | DC_20A_n1A | No |  |
| DC_20A_n3A | DC_20A_n3A | No |  |
| DC_20A_n7A | DC_20A_n7A | DC_20_n7 |  |
| DC_20A_n8A | DC_20A_n8A | DC_20_n8 |  |
| DC_20A_n28A8,11,13 | DC_20A_n28A | No |  |
| DC_20A_n38A | DC_20A_n38A | No |  |
| DC_20A_n40A | DC_20A_n40A | No |  |
| DC_20A_n41A | DC_20A_n41A | DC_20_n41 |  |
| DC_20A_n50A | DC_20A_n50A | No |  |
| DC_20A_n51A | DC_20A_n51A | No |  |
| DC_20A_n77A7 | DC_20A_n77A | No |  |
| DC_20A_n78A7 DC_20A_n78C7 | DC_20A_n78A23 | No |  |
| DC_20A_n78(2A)7 | DC_20A_n78A | No |  |
| DC_21A_n1A | DC_21A_n1A | No |  |
| DC_21A_n28A17 | DC_21A_n28A | DC_21_n28 |  |
| DC_21A_n77A7 DC_21A_n77C7 | DC_21A_n77A | No |  |
| DC_21A_n77(2A)7,21 | DC_21A_n77A21 | No |  |
| DC_21A_n78A7 DC_21A_n78C7 | DC_21A_n78A | No | No |
| DC_21A_n78(2A)7,21 | DC_21A_n78A21 | No | No |
| DC_21A_n79A7 DC_21A_n79C7 | DC_21A_n79A | No | No |
| DC_25A_n41A | DC_25A_n41A | No |  |
| DC_25A-25A_n41A | DC_25A_n41A | No |  |
| DC_25A_n77A | DC_25A_n77A | DC_25_n77 |  |
| DC_25A-25A_n77A | DC_25A_n77A | DC_25_n77 |  |
| DC_25A_n78A | DC_25A_n78A | DC_25_n78 |  |
| DC_25A-25A_n78A | DC_25A_n78A | DC_25_n78 |  |
| DC_26A_n25A | DC_26A_n25A | No |  |
| DC_26A_n41A | DC_26A_n41A | No |  |
| DC_26A_n77A7 | DC_26A_n77A | No |  |
| DC_26A_n78A7 | DC_26A_n78A | No |  |
| DC_26A_n78(2A) | DC_26A_n78A | No |  |
| DC_26A_n79A7 | DC_26A_n79A | No |  |
| DC_28A_n1A | DC_28A_n1A | No |  |
| DC_28A_n2A | DC_28A_n2A | No |  |
| DC_28A_n3A | DC_28A_n3A | No |  |
| DC_28A_n5A | DC_28A_n5A | No |  |
| DC_28A_n7A DC_28A_n7B | DC_28A_n7A DC_28A_n7B | No |  |
| DC_28A_n51A | DC_28A_n51A | No |  |
| DC_28A_n8A | DC_28A_n8A | No |  |
| DC_28A_n20A8,11,13 | DC_28A_n20A | No |  |
| DC_28A_n38A | DC_28A_n38A | No |  |
| DC_28A_n40A DC_28C_n40A | DC_28A_n40A | No |  |
| DC_28A_n41A7 | DC_28A_n41A | No |  |
| DC_28A_n50A | DC_28A_n50A | No |  |
| DC_28A_n66A | DC_28A_n66A | No |  |
| DC_28A_n77A7 DC_28A_n77C7 | DC_28A_n77A | No | No |
| DC_28A_n77(2A)7 | DC_28A_n77A | No | No |
| DC_28A_n78A7 DC_28A_n78C7 | DC_28A_n78A23 | No | No |
| DC_28A_n78(2A)7 | DC_28A_n78A | No | No |
| DC_28A_n79A7 DC_28A_n79C7 | DC_28A_n79A | No |  |
| DC_28A_n105A | DC_28A_n105A18 | DC_28_n105 |  |
| DC_30A_n2A | DC_30A_n2A | No |  |
| DC_30A_n5A | DC_30A_n5A | No |  |
| DC_30A_n66A | DC_30A_n66A | No |  |
| DC_30A_n77A | DC_30A_n77A | No |  |
| DC_30A_n77(2A)21 | DC_30A_n77A21 | No |  |
| DC_38A_n1A | DC_38A_n1A | No |  |
| DC_38A_n3A | DC_38A_n3A | No |  |
| DC_38A_n8A | DC_38A_n8A | No |  |
| DC_38A_n28A | DC_38A_n28A | No |  |
| DC_38A_n78A7 | DC_38A_n78A | No |  |
| DC_38A_n79A DC_38A_n79C | DC_38A_n79A | No |  |
| DC_39A_n40A3 | DC_39A_n40A | No |  |
| DC_39A_n41A DC_39C_n41A DC_39A_n41C | DC_39A_n41A DC_39C_n41A | No | No |
| DC_39A_n78A5,7 | DC_39A_n78A | No |  |
| DC_39A_n79A7 DC_39A_n79C7 | DC_39A_n79A | No | No |
| DC_40A_n1A DC_40C_n1A | DC_40A_n1A | No |  |
| DC_40A_n3A | DC_40A_n3A | No |  |
| DC_40A_n7A | DC_40A_n7A | No |  |
| DC_40A_n41A DC_40A_n41C DC_40C_n41A | DC_40A_n41A | No |  |
| DC_40A_n41(2A) | DC_40A_n41A | No |  |
| DC_40A_n77A DC_40A_n77C DC_40C_n77A21 DC_40C_n77C DC_40D_n77A21 | DC_40A_n77A21 | No |  |
| DC_40A_n78A DC_40A_n78C DC_40C_n78A21 DC_40C_n78C DC_40D_n78A21 | DC_40A_n78A21, 23 DC_40C_n78A | No |  |
| DC_40A_n78(2A) DC_40C_n78(2A) | DC_40A_n78A DC_40C_n78A | No |  |
| DC_40A_n79A7,12 DC_40A_n79C7,12 DC_40C_n79A7,12 | DC_40A_n79A | No | No |
| DC_41A_n1A DC_41C_n1A | DC_41A_n1A DC_41C_n1A | No | DC_41A_n1A DC_41C_n1A |
| DC_41A_n3A7 DC_41C_n3A7 | DC_41A_n3A DC_41C_n3A | No |  |
| DC_41A_n28A7 DC_41C_n28A7 | DC_41A_n28A DC_41C_n28A | No |  |
| DC_41A_n77A DC_41C_n77A21 | DC_41A_n77A21 DC_41C_n77A | No |  |
| DC_41A_n77(2A) DC_41C_n77(2A) | DC_41A_n77A DC_41C_n77A | No |  |
| DC_41A_n78A DC_41C_n78A DC_41D_n78A | DC_41A_n78A23 DC_41C_n78A | No |  |
| DC_41A_n78(2A) DC_41C_n78(2A) | DC_41A_n78A DC_41C_n78A | No |  |
| DC_41A_n79A6,7 DC_41A_n79C6,7 DC_41C_n79A6,7 | DC_41A_n79A DC_41A_n79C DC_41C_n79A | No | No |
| DC_42A_n1A7 DC_42C_n1A7 | DC_42A_n1A DC_42C_n1A | No |  |
| DC_42A_n3A7 DC_42C_n3A7 | DC_42A_n3A DC_42C_n3A | DC_42_n3 |  |
| DC_42A_n28A7 DC_42C_n28A7 | DC_42A_n28A DC_42C_n28A | No |  |
| DC_42A_n51A | DC_42A_n51A | No |  |
| DC_42A_n77A3,4,9,11,13 DC_42A_n77C3,4,9 DC_42C_n77A3,4,9,11 DC_42C_n77C3,4,9 DC_42D_n77A3,4,9,11 DC_42D_n77C DC_42E_n77A3,4,9,11 DC_42E_n77C | N/A | N/A |  |
| DC_42A_n77(2A)3,4,9 DC_42C_n77(2A)3,4,9 | N/A | N/A |  |
| DC_42A_n78A3,4,9,11,13 DC_42A_n78C3,4,9 DC_42C_n78A3,4,9,11 DC_42C_n78C3,4,9 DC_42D_n78A3,4,9,11 DC_42D_n78C3,4,9 DC_42E_n78A3,4,9,11 DC_42E_n78C3,4,9 | N/A | N/A |  |
| DC_42A_n79A9,15 DC_42A_n79C9,15 DC_42C_n79A9,15 DC_42C_n79C9,15 DC_42D_n79A9,15 DC_42D_n79C9,15 DC_42E_n79A9,15 DC_42E_n79C9,15 | N/A | N/A |  |
| DC_46A_n77A2 | N/A | N/A |  |
| DC_46A_n78A2 DC_46C_n78A2 DC_46D_n78A2 DC_46E_n78A2 | N/A | N/A |  |
| DC_48A_n2A DC_48C_n2A DC_48D_n2A DC_48E_n2A | DC_48A_n2A | No |  |
| DC_48A_n5A DC_48C_n5A DC_48D_n5A DC_48E_n5A | DC_48A_n5A | No |  |
| DC_48A_n12A | DC_48A_n12A | No |  |
| DC_48A_n25A DC_48C_n25A DC_48D_n25A | DC_48A_n25A | No |  |
| DC_48A_n46A DC_48B_n46A DC_48C_n46A DC_48D_n46A DC_48E_n46A DC_48A_n46B DC_48B_n46B DC_48C_n46B DC_48D_n46B DC_48E_n46B DC_48A_n46C DC_48B_n46C DC_48C_n46C DC_48D_n46C DC_48E_n46C DC_48A_n46D DC_48B_n46D DC_48C_n46D DC_48D_n46D DC_48E_n46D | DC_48A_n46A DC_48B_n46A | No |  |
| DC_48A_n66A DC_48C_n66A DC_48D_n66A DC_48E_n66A | DC_48A_n66A | No |  |
| DC_48A_n71A DC_48B_n71A DC_48C_n71A DC_48D_n71A | DC_48A_n71A | No |  |
| DC_48A-48A_n71A DC_48A-48A-48A_n71A | DC_48A_n71A | No |  |
| DC_48A_n77A3. 4. 9, 11 DC_48C_n77A3. 4. 9, 11 DC_48A_n77C3. 4. 9, 11 DC_48C_n77C3. 4. 9, 11 DC_48D_n77A3. 4. 9, 11 DC_48D_n77C3. 4. 9, 11 DC_48E_n77A3. 4. 9, 11 | N/A | N/A |  |
| DC_48A-48A_n77A | N/A | N/A |  |
| DC_48A-48A-48A_n77A | N/A | N/A |  |
| DC_66A_n2A DC_66B_n2A DC_66C_n2A | DC_66A_n2A | DC_66_n2 |  |
| DC_66A_n2(2A) | DC_66A_n2A | DC_66_n2 |  |
| DC_66A-66A_n2A | DC_66A_n2A | DC_66_n2 |  |
| DC_66A-66A-66A_n2A | DC_66A_n2A | DC_66_n2 |  |
| DC_66A_n5A DC_66B_n5A DC_66C_n5A | DC_66A_n5A | DC_66_n5 |  |
| DC_66A-66A_n5A | DC_66A_n5A | DC_66_n5 |  |
| DC_66A-66A-66A_n5A | DC_66A_n5A | DC_66_n5 |  |
| DC_66A_n7A | DC_66A_n7A | No |  |
| DC_66A_n7(2A) | DC_66A_n7A | No |  |
| DC_66A-66A_n7A | DC_66A_n7A | No |  |
| DC_66A-66A_n7(2A) | DC_66A_n7A | No |  |
| DC_66A_n12A | DC_66A_n12A | No |  |
| DC_66A_n25A | DC_66A_n25A | DC_66_n25 |  |
| DC_66A_n28A | DC_66A_n28A | No |  |
| DC_66A_n30A | DC_66A_n30A | No |  |
| DC_66A-66A_n30A | DC_66A_n30A | No |  |
| DC_66A_n38A | DC_66A_n38A | No |  |
| DC_66A-66A_n38A | DC_66A_n38A | No |  |
| DC_66A_n41A DC_66A_n41C | DC_66A_n41A | No |  |
| DC_66A_n41(2A) | DC_66A_n41A | No |  |
| DC_66A_n46A | DC_66A_n46A | No |  |
| DC_66A_n48A DC_66A_n48B | DC_66A_n48A | No |  |
| DC_66A-66A_n48A DC_66A-66A_n48B | DC_66A_n48A | No |  |
| DC_66A_n71A DC_66C_n71A DC_66A_n71B | DC_66A_n71A | No |  |
| DC_66A-66A_n71A | DC_66A_n71A | No |  |
| DC_66A_n77A DC_66A_n77C21 | DC_66A_n77A21 | DC_66_n77 |  |
| DC_66A_n77(2A)21 | DC_66A_n77A21 | DC_66_n77 |  |
| DC_66A-66A_n77A21 DC_66A-66A_n77C21 | DC_66A_n77A21 | DC_66_n77 |  |
| DC_66A-66A_n77(2A)21 | DC_66A_n77A21 | DC_66_n77 |  |
| DC_66A-66A-66A_n77A21 DC_66A-66A-66A_n77C21 | DC_66A_n77A21 | DC_66_n77 |  |
| DC_66A-66A-66A_n77(2A)21 | DC_66A_n77A21 | DC_66_n77 |  |
| DC_66A_n78A | DC_66A_n78A | No |  |
| DC_66A_n78(2A) 21 | DC_66A_n78A21 | No |  |
| DC_66A-66A_n78A21 | DC_66A_n78A21 | No |  |
| DC_66A-66A_n78(2A)21 | DC_66A_n78A21 | No |  |
| DC_71A_n2A | DC_71A_n2A | No |  |
| DC_71A_n2(2A) | DC_71A_n2A | No |  |
| DC_71A_n5A | DC_71A_n5A | No |  |
| DC_71A_n12A | DC_71A_n12A18,19 | Yes |  |
| DC_71A_n38A | DC_71A_n38A | No |  |
| DC_71A_n7A | DC_71A_n7A | No |  |
| DC_71A_n25A | DC_71A_n7A | No |  |
| DC_71A_n41A | DC_71A_n41A | No |  |
| DC_71A_n48A | DC_71A_n48A | No |  |
| DC_71A_n66A | DC_71A_n66A | No |  |
| DC_71A_n77A DC_71A_n77C | DC_71A_n77A | No |  |
| DC_71A_n77(2A) | DC_71A_n77A | No |  |
| DC_71A_n78A | DC_71A_n78A | No |  |
| DC_71A_n78(2A)21 | DC_71A_n78A21 | No |  |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Restricted to E-UTRA operation when inter-band carrier aggregation is configured. The downlink operating band for Band 46 is paired with the uplink operating band (external E-UTRA band) of the carrier aggregation configuration that is supporting the configured Pcell. NOTE 3: The minimum requirements apply only when there is non-simultaneous Tx/Rx operation between E-UTRA and NR carriers. This restriction applies also for these carriers when applicable EN-DC configuration is part of a higher order EN-DC configuration. NOTE 4: For a UE not capable of interBandMRDC-WithOverlapDL-Bands-r16 or a UE capable of interBandMRDC-WithOverlapDL-Bands-r16 and requirementTypeIndication-r18 and is provided with nonCollocatedTypeMRDC-r18, the minimum requirements for intra-band non-contiguous EN-DC apply for the Band 42/48 and Band n77/n78 combination. For a UE not capable of interBandMRDC-WithOverlapDL-Bands-r16 or a UE capable of interBandMRDC-WithOverlapDL-Bands-r16 and requirementTypeIndication-r18 and is provided with nonCollocatedTypeMRDC-r18, when UE capability interBandContiguousMRDC is indicated, the minimum requirements for intra-band contiguous EN-DC also should be met in addtion to intra-band non-contiguous EN-DC. The intra-band requirements also apply for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. NOTE 5: The frequency range above 3600 MHz for Band n78 is not used in this combination. NOTE 6: The frequency range below 2506 MHz for Band 41 is not used in this combination. NOTE 7: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability. NOTE 8: The frequency range in band n28 /28 is restricted for this band combination to 703 - 733 MHz for the UL and 758-788 MHz for the DL. This restriction also applies for any band combinations when DC_20_n28/DC_28_n20/CA_20-28/CA_n20-n28 is a subset of a higher order band combination. NOTE 9: The combination is not used alone as fall-back mode of other band combinations in which UL in Band 42 or Band 48 is not used. NOTE 10: Void. NOTE 11: For a UE not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements apply when the maximum power spectral density imbalance between downlink carriers is within 6 dB. For a UE capable ofinterBandMRDC-WithOverlapDL-Bands-r16 and not capable of requirementTypeIndication-r18 or a UE capable of interBandMRDC-WithOverlapDL-Bands-r16 and requirementTypeIndication-r18 but is not provided with nonCollocatedTypeMRDC-r18 and is configured with maxMIMO-Layers with value less than or equal to 2, the power imbalance requirement defined in clause 7.10B.3 apply. For a UE capable of both interBandMRDC-WithOverlapDL-Bands-r16 and requirementTypeIndication-r18 and is provided with nonCollocatedTypeMRDC-r18, the minimum requirements apply when the maximum power spectral density imbalance between downlink carriers is within 6 dB. For these UEs, the power spectral density imbalance condition also applies for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. NOTE 12: Applicable for frequency range above 4800 MHz for Band n79 in this combination. NOTE 13: For a UE not capable of interBandMRDC-WithOverlapDL-Bands-r16 or a UE capable of interBandMRDC-WithOverlapDL-Bands-r16 and requirementTypeIndication-r18 and is provided with nonCollocatedTypeMRDC-r18, the minimum requirements apply for synchronized DL carriers with a maximum receive time difference ≤ 3 usec. The requirements also apply for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. NOTE 14: Applicable when dynamic switching between two uplink carriers is conducted. The DL interruption requirements for NR DL carrier(s) and E-UTRA DL carrier(s) are specified in clause 8.2.1.2.14 of 38.133 [15] and clause 7.32.2.12 of 36.133 [16] respectively. NOTE 15: Simultaneous Rx/Tx capability does not apply for UEs supporting band 42 with a n77 implementation only. Same restrictions are applied to related higher order configurations. NOTE 16: The frequency range in band n41 is restricted for this band combination to 2595 – 2645 MHz. NOTE 17: The frequency range in band n28 is restricted for this band combination to 728 - 738 MHz for the UL and 783 - 793 MHz for the DL. This restriction applies also for these band combinations when applicable EN-DC configuration is part of a higher order EN-DC configuration. NOTE 18: Only single switched UL is supported. NOTE 19: The implementation with 4 antennas is targeted for FWA form factor for this band combination. NOTE 20: The combination is not used alone as fallback mode of other band combinations in which UL in Band 2 is not used. NOTE 21: Minimum requirements for PC2 are applicable for this uplink EN-DC configuration in this downlink/uplink EN-DC configuration with 1Tx antenna connector in each band. NOTE 22: The PC2 Uplink EN-DC configuration supported in Table 6.2B.1.3-1 is applicable to the same EN-DC configuration without additional indication of NOTE 21. NOTE 23: Minimum requirements for Power Class 2 are applicable for this uplink EN-DC configuration with 1Tx antenna connector in one band and 2Tx antenna connectors in the other band. |  |  |  |

<End of Change>